3GPP TSG-RAN WG2 Meeting #131	                                          R2-2505152

Bengaluru, India, 25 – 29, Aug, 2025

Source: 	Xiaomi

Title:  	Discussion on functionality management for event prediction model

Agenda Item:	8.3.2.2

Document for:	Discussion and Decision

Introduction

In this discussion, we discuss inference solution and functionality management for event prediction.

Discussion

Inference solution for event prediction

During the study phase, three options were studied regarding to how to decide the time point to transmit handover command. All solutions require UE to report the predicted measurement event when it’s predicted to be fulfilled. Then it can be up to NW to decide which option is used to decide the handover timing. Upon receiving the predicted measurement event, source cell may prepare the handover in advance. The handover preparation procedure between source and target cell can be reused as baseline.

Proposal 1: UE reports the predicted measurement event when it’s predicted to be fulfilled. It’s up to NW implementation when to transmit the handover command.

Event prediction can be achieved in either indirect or direct way. In indirect way, UE first predicts future L3 cell level measurement results, then evaluates the measurement report event. UE can obtain the timing of the predicted measurement event according to the time instance of predicted future L3 measurements. In direct way, UE directly predicts the measurement event occurrence. Based on this, UE can obtain the timing (window) of the predicted measurement and probability.

In legacy measurement report, UE would report the measurement results, MeasId and triggered cell ID. We believe the legacy measurement framework shall be reused in AI event prediction. If the event is predicted to be fulfilled, UE shall also report the MeasId and triggered cell ID of the predicted event for both indirect and direct prediction. Furthermore, UE can report the predicted measurement results if indirect prediction is used and the probability if direct prediction is used.

Proposal 2: UE reports the timing (window), MeasId and triggered cell ID of predicted measurement event in both indirect and direct event prediction.

Proposal 3: UE can also report the predicted measurement results if indirect prediction is used.

Proposal 4: UE can also report the probability if direct prediction is used.

During the study phase, the evaluation is mainly about the entering condition fulfilled, which can be used to determine the handover command transmission. Furthermore, we believe the leaving condition fulfilment prediction may also be useful. NW may not immediately send handover command to UE, due to handover preparation or load control. If the leaving condition of the triggered event would be fulfilled in future, the triggered cell would not be good anymore. NW can choose another cell as target cell and avoid handover UE to the non-optimum cell.

Proposal 5: RAN2 to support predict and report triggered event leaving condition fulfilled.

Unified functionality management solution should be used for both RRM and event prediction, unless specific requirement is identified for event prediction.

In AI handover option 1/2, NW may not immediately send handover command after UE reporting the inference result. UE can still perform measurement and inference after reporting. Since more measurement results are obtained at UE, UE may find the earlier reported predicted event would not occur. In such case, UE should be allowed to indicate NW to correct/cancel the predicted event. NW can cancel the handover preparation. Otherwise, HOF may occur due to incorrect prediction result.

Proposal 6: Allow UE to indicate NW to cancel the earlier predicted event according to latest prediction.

Inference control

It’s FFS when the UE can perform inference in AI mobility. Unlike BM, for case A/event prediction, the inference is expected to enhance handover, which occurs at cell edge. The inference result may be useless in cell centre and consumes UE power. However, NW may provide the inference configuration before UE moves to cell edge. Therefore, UE shall not be required to perform inference immediately after receiving the inference configuration. Alternatively, the UE can perform inference only when it’s about to perform handover at cell edge. UE can determine whether it’s at cell edge based on event (A2, A3, A5 alike event), which can be configured by NW.

Alternatively, the inference control can be based on the remaining TTT if PW is smaller than TTT. As depicted in the following fig, UE would not predict event occurrence before timing point A. Because the remaining TTT is larger than PW length. UE can calculate the remaining TTT based on the TTT running status. No additional effort is required, since UE would anyway maintain the TTT.

Proposal 7: RAN2 to discuss event inference control solution, including radio measurement result based and remaining TTT based solution.

Performance monitoring

For UE side mode performance monitoring, it’s agreed UE can calculate and report the performance metric to NW. In RRM, the metric may be average RSRP difference. During the study the performance of event prediction is evaluated by F1 score. Sophisticated F1 score would require large amount of event data. However, it may take long time for a single UE to collect enough event data and calculate the sophisticated F1 score. Also, NW may not collect the data from multiple UEs, since UEs may deploy different models. Alternatively, the performance monitoring can be done by NW. NW can configure UE to report the predicted event and still wait for real measurement report to check the accuracy. The performance monitoring can be pure NW implementation without spec impact.

Proposal 8: Rely on NW implementation to monitor the performance of event prediction.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Inference solution

Proposal 1: UE reports the predicted measurement event when it’s predicted to be fulfilled. It’s up to NW implementation when to transmit the handover command.

Proposal 2: UE reports the timing (window), MeasId and triggered cell ID of predicted measurement event in both indirect and direct event prediction.

Proposal 3: UE can also report the predicted measurement results if indirect prediction is used.

Proposal 4: UE can also report the probability if direct prediction is used.

Proposal 5: RAN2 to support predict and report triggered event leaving condition fulfilled.

Proposal 6: Allow UE to indicate NW to cancel the earlier predicted event according to latest prediction.

Inference Control

Proposal 7: RAN2 to discuss event inference control solution, including radio measurement result based and remaining TTT based solution.

Performance monitoring

Proposal 8: Rely on NW implementation to monitor the performance of event prediction.

Reference

[1] TR 38.843

[2] RP-242393 SID of AI/ML mobility