3GPP TSG-RAN WG2 Meeting #131	R2-2505177

Bangalore, India  Aug 25th – 29th , 2025

Agenda Item	: 8.1.4

Source	: LG Electronics Inc.

Title	: Discussion on UE-side data collection

Document for	: Discussion and Decision

Introduction

During the post e-mail discussion after RAN2#130 meeting [1], companies had the discussion about the container of candidate configuration and the content for candidate configuration.

Furthermore, at the RAN#106 meeting, the new SID for AI/ML air Phase 2(RP-243245) [2] was agreed and the further discussion for UE-side data collection was approved in RP-243307[3] with the following guidance for RAN2 during Rel-19:

- Study RAN aspects related to data transfer over UP - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection. - Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion). - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain. - Discuss on the scalability aspects of CP - RAN2 can Liaise with other WGs if/when needed

In this contribution, we will show our views regarding the topics of post e-mail discussion [1] and continue discuss the topics assigned to RAN2 and left FFS in previous RAN2#130 meeting [4].

Discussion

Post e-mail discussion: UE candidate data collection

According to the RAN2#129bis agreement below, it is possible for the UE to support multiple configurations for UE-side data collection, and it was agreed that the network can provide a list of (i.e., multiple) candidate configurations.

- RAN2#129bis meeting

- Agreements on data collection configuration

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

As discussed in the running CR of the Annex below, it would be natural for the UE to transmit multiple preferred configurations via UAI. Additionally, the UE's preferred configuration transmitted via UAI will include an indication of the start/stop of data collection and configuration IDs, based on the RAN2#129bis agreement mentioned above.

From the email discussion [1], it was noted that some companies believe additional information beyond the start/stop indication and configuration ID is necessary in the UAI. For example, one company argued about the need for additional information such as the priority of configurations. In our understanding, what this company is considering is that if there are multiple configurations preferred by the UE, and if the network can not configure all of the multiple configurations preferred by the UE when considering the current network resources, it would be necessary to ensure that the network configures the configuration by priorities among the UE's preferred configurations. However, such an operation is considered sufficiently feasible within the current operation without additional information like the priority of configurations. If the UE has transmitted multiple preferred configurations, the network will support only some of them based on the current network resource. In legacy operation, if the configuration with high priority from the UE's perspective has failed to be configured by the network, the UE can transmit the refined preferred configuration. This will allow the network to configure the configuration with the high priority from the UE’s perspective. Considering that UE-side data collection does not require urgency, such an action can be sufficiently implemented with the current settings. Also, any delays caused by the UE's configuration refinement will not be an issue in the UE-side data collection scenario. So, defining new additional information for an operation that can be sufficiently implemented with the current operation would be an overly optimized action as of now.

Therefore, when the UE transmits its preferred data collection configuration, no additional information beyond the start/stop indication and configuration ID is required. This would be sufficient for the network to determine a suitable configuration for UE-side data collection, considering its capabilities. Furthermore, excessive optimization for an operation that is already feasible with the current settings is unnecessary.

Proposal 1. [RRC3] No other information is needed for UE to report as part of preferred data collection configuration. (i.e., Start/stop indication and configuration ID are sufficient.)

During the post-email discussion, there was a discussion about the prohibit timer for UE indicating its preferred data collection configuration. We believe that the prohibit timer is not necessary, as the preferences for data collection configuration are unlikely to change frequently.

Proposal 2. [RRC3] Prohibit timer for UE indicating its preferred data collection configuration is not needed.

However, if RAN2 agrees to adopt the prohibit timer due to the concern from network vendors, clarification for the target of the prohibit timer will be required in advance. This is because the impact of the prohibit timer will vary depending on the target which the timer is applied to.

The subjects to which the prohibit timer applies can be broadly classified as:

- Method 1. Applying the prohibit timer for each candidate configuration

  - For example, if the UE prefers three candidate configurations (configuration #1, configuration #2, configuration #3) among five candidate configurations (configuration #1, configuration #2, configuration #3, configuration #4, configuration #5), a total of three prohibit timers will be applied for each preferred candidate configuration (configuration #1, configuration #2, configuration #3).

  - In this case, there will be signaling when preference changes for each candidate configuration. This will result in massive signaling overhead, and the losses will outweigh the gains.

- Method 2. Applying a single prohibit timer for all candidate configurations (i.e., a prohibit timer per UE)

  - For example, if the UE prefers three candidate configurations (configuration #1, configuration #2, configuration #3) among five candidate configurations (configuration #1, configuration #2, configuration #3, configuration #4, configuration #5), only one prohibit timer will be applied for multiple preferred candidate configuration (configuration #1, configuration #2, configuration #3).

  - In this case, if a preference change for one candidate configuration triggered a prohibit timer, UAI reporting for preference changes for other candidate configurations will not be triggered while the prohibit timer is running.

Therefore, if RAN2 agrees to introduce prohibit timer for indicating UE preferred data collection configuration to prevent repetitive preference reports to the network and minimize repetitive reconfiguration attempts by the network, it seems reasonable to apply a single prohibit timer for all candidate configurations.

Observation 1. [RRC3] For introducing prohibit timer to indicate UE preferred data collection configuration, the subjects to which the prohibit timer applies can be classified as:

- Method 1. Applying the prohibit timer for each candidate configuration

- Method 2. Applying a single prohibit timer for all candidate configurations (i.e., a prohibit timer per UE)

Proposal 3. [RRC3] If RAN2 agrees to introduce prohibit timer to indicate UE preferred data collection configuration, a single prohibit timer for all candidate configurations (i.e., a prohibit timer per UE) would be sufficient.

Three main solutions have been proposed regarding which RRC parameter should contain/signal the candidate UE data collection configuration:

Solution 1: OtherConfig contains a list of candidate configurations as a list with a new IE, where the new IE (i.e., each candidate configuration) contains at least the following set of parameters as agreed in RAN2#130, i.e., identifier of the candidate configuration, CSI-ResourceConfigId for Set A, CSI-ResourceConfigId for Set B, and related associated IDs [4]. An example of DataCollectionPreferenceConfig-r19 is shown as below:

DataCollectionPreferenceConfig-r19 :: = SEQUENCE { -- Solution 1 dataCollectionCandidateConfigList-r19 SEQUENCE (SIZE (1..maxCandidateConfig-r19)) OF DataCollectionCandidateConfig-r19 OPTIONAL, -- Need N }

Solution 2: OtherConfig contains a list of candidate configurations as a list of CSI-ReportConfigId, where the candidate configurations are provided by the gNB within csi-ReportConfigToAddModList under CSI-MeasConfig [5]. An example of DataCollectionPreferenceConfig-r19 is shown as below:

DataCollectionPreferenceConfig-r19 :: = SEQUENCE { -- Solution 2 dataCollectionCandidateConfigList-r19 SEQUENCE (SIZE (1.. maxCandidateConfig-r19)) OF CSI-ReportConfigId OPTIONAL, -- Need R }

Solution 3: OtherConfig contains a list of candidate configurations as a new list of CSI-ReportConfig, i.e., creating a list of CSI-ReportConfig in OtherConfig, which is independent from CSI-ReportConfig under CSI-MeasConfig. An example of DataCollectionPreferenceConfig-r19 is shown as below:

DataCollectionPreferenceConfig-r19 :: = SEQUENCE { -- Solution 3 dataCollectionCandidateConfigToAddModList SEQUENCE (SIZE (1..maxCandidateConfig -r19)) OF CSI-ReportConfig OPTIONAL, -- Need N }

For Solution 2, since the configuration is under CSI-MeasConfig (i.e., the configuration may impact the legacy CSI-ReportConfig, even if reportQuantity='none-BM-r19' is set), the RAN1 spec impact is unavoidable. Therefore, potential compatibility issues with legacy operations may exist due to the simultaneous configuration of legacy CSI-resourceConfig and CSI-ReportConfig without introducing new IEs or lists. Furthermore, since RAN1 concluded its release 19 discussions in August, it would be appropriate to avoid any RAN1 spec impact at this point.

On the other hand, for Solution 1 and Solution 3, the candidate configuration is independent of the legacy measurement configuration because the configuration is independent from CSI-MeasConfig. In other words, Solution 1 and Solution 3 will have no impact on RAN1.

For Solution 3, since OtherConfig contains a list of candidate configurations as a new list of CSI-ReportConfig, the gNB may need to reconfigure the CSI-ReportConfig to activate the resource for measurements. This means that configurations are duplicated with the legacy CSI-ReportConfig. This will result in a significant overhead compared to Solution 1.

Therefore, we believe that Solution 1 is the most suitable solution among the three solutions, as it has the lowest overhead and no impact on RAN1.

Proposal 4. [RRC3] RAN2 adopt Solution1 to include a list of candidate configurations in OtherConfig.

- Solution 1: OtherConfig contains a list of candidate configurations as a list of a new IE, where each candidate configuration contains at least an identifier of the candidate configuration, CSI-ResourceConfigId for Set A, CSI-ResourceConfigId for Set B, and related associated IDs, as agreed in RAN2#130.

According to the agreement of RAN2#130 meeting[4], for BM case, candidate UE data collection configuration must include at least CSI-ResourceConfigId of Set A&B, one/two associated IDs.

RAN2#130 meeting Agreements For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements

If RAN2 agrees to adopt Solution 1, besides the above parameters agreed in RAN2#130, we think at least the serving cell ID may be additionally needed. It is because the CSI-ReportConfig is configured per cell, while OtherConfig is configured per cell group. Therefore, the serving cell ID is absolutely necessary to specify to which cell the configuration of Solution1 should be applied.

Proposal 5. [RRC3] If Solution 1 is adopted, serving cell ID is additionally needed to be included in candidate UE data collection configuration.

Data transfer over UP

According to the RAN2#127 meeting [5], data transmission via the UP tunnel is considered for Option 1a and Option 1b, while it remains as FFS for Option 2 and Option 3. The table below illustrates a baseline example of the data transmission process through the UP tunnel for each option, depending upon the location of the server for UE data collection, whether it is located inside or outside the MNO. As shown in the table, when the server is located inside the MNO, the pathways of data transfer and the MNO's scope for Option 1b and Option 2 are identical. Likewise, when the server is located outside the MNO, the pathways of data transfer and the MNO's scope for Option 1a, Option 1b, and Option 2 are identical.

| Location of the server Option | Server inside the MNO | Server outside the MNO |
| --- | --- | --- |
| Option 1a | N/A |  |
| Option 1b |  |  |
| Option 2 |  |  |
| Option 3 |  |  |

From the RAN2 perspective, the UP tunnel facilitates data transmission from UE to the Core Network via the gNB and UPF. Consequently, the UP solutions for Option 1a, Option 1b, and Option 2 appear to be equivalent, irrespective of the location of the server. In contrast, Option 3 presents a different scenario, as the UP tunnel for this option routes data from the UE to the Core Network through the gNB and OAM, thereby distinguishing it from the other options.

As a result, the data transmission through the UP tunnel can be categorized into two distinct groups from the RAN2 perspective, based on the above analysis.

- data transfer through UP tunnel via UPF (i.e., Option 1a, Option 1b, Option 2)

- data transfer through UP tunnel via OAM (i.e., Option 3)

Proposal 6. From the RAN2 perspective, UP solution can be classified into two:

- Data transfer through UP tunnel via UPF (i.e., Option1a, Option1b, Option2)

- Data transfer through UP tunnel via OAM (i.e., Option3).

Discussion on the controllability aspects of CP&UP solutions

In terms of controllability, the MNO ensures full control of the CP solution, as it fully manages the transmission of SRBs. This extensive control empowers the MNO to efficiently handle communications, optimize resource allocation, and ensure high-quality service for users. By managing SRB transmission, the MNO can dynamically adjust parameters, handle signaling traffic, and respond in real-time to fluctuating network conditions, thereby significantly enhancing overall network performance and reliability.

Similarly, with respect to the UP solution, the NG-RAN also maintains full control, as the SMF within the core network takes charge of controlling the UPF and OAM. Whether the data collection server is located inside or outside the MNO, the MNO can achieve complete controllability by differentiating AIML data from non-AIML data, thus establishing separate PDU sessions with unique LCH, DRB, and QoS requirements.

Proposal 7. MNO can have full controllability over the data collection procedure through both CP solution and UP solution.

Discussion on the visibility aspects of CP & UP solutions

In addition to its controllability features, the NG-RAN also ensures full scalability for the transmission of standardized data through the CP solution, as the management of data transmission via SRBs falls under NG-RAN's purview. Similarly, the UP solution within the core network guarantees full scalability for standardized data transfer, given that each PDU session is under the control of the SMF. Moreover, full visibility for standardized data collection is already achievable in legacy operations, as outlined in the spec related to other relevant working groups. For instance, the TS 26.531 specifies that when the data collection server is located inside the MNO, full visibility for standardized data can be assured for UE-side model training. In this context, it is anticipated that full visibility for standardized AIML data will also be achievable.

Proposal 8. For both CP solution and UP solution, MNO can have full visibility over standardized data.

During the discussions at the RAN2#127 meeting regarding various data collection options for UE-side model training [7], it was noted that "RAN2 has not reached a conclusion on the necessity of partial and no visibility options.". To address these options effectively, it is essential to first define the content of partially standardized data and non-standardized data. Some companies advocate for the inclusion of partial or non-standardized data to protect proprietary information. However, the existing data transfer requirements, such as L1-RSRP values, are sufficiently fulfilled with standardized data. To advance discussions on partial and non-standardized data, it is essential to clarify which data types should be classified as partial or non-standardized and to specify the information that must be included for data transfer purposes. Should this clarification indicate a need for the introduction of partial or non-standardized data, further discussions can be scheduled at that time.

Proposal 9. For the discussion of transferring partial/non-standardized data of UE side data collection, RAN2 should first clarify which specific data needed as partial/non-standardized data.

According to the objective of the new SID (SP-250413) from SA plenary on March [6] and key issue #1 of TR23.700-04 [7], SA is only considering support for the standardized transfer of standardized data over the UP for UE data collection. Therefore, it would be prudent for RAN2 to also focus solely on standardized data during the Rel-19 discussion period, as SA is not considering partial or non-standardized data during this time.

Proposal 10. For Rel-19, RAN2 only focuses on standardized data, disregarding partial/non-standardized data.

Discussion on the scalability aspects of CP & UP solutions

At the RAN2#130 meeting [4], it was agreed that the CP solution's issues related to large amounts of data transmission would be captured in a TR, and the following TR contents were discussed through post-email discussion [8].

RAN2#130 meeting Agreements Capture in the TR issues with CP solution related to large amount of data (if there is a large amount of data) to be transferred for a UE, need for segmentation and it’s implications.

[POST130][033][AI PHY] UE Side data collection (Ericsson) - 7.2.1.3.2.1 Data collection for UE-side model training – CP solutions analysis Related to the solutions based on CP and captured in Section 7.2.1.3.2, the following challenges in the Table 7.2.1.3.2.1-1 have been identified, especially for the case of large amount of data to be transferred from a UE. Table 7.2.1.3.2.1-1. Data Collection CP solutions analysis Challenges; Description. UE memory allocation; The collected data may need to be stored in the access stratum (AS) buffer for the control plane-based data transfer via AS. Requirements on the UE memory were not discussed.. Segmentation for UE side data collection; A single RRC message can contain maximum about 9KB data. Thus RRC or higher layer segmentation is needed to transfer more data that can fit in a single RRC message The existing RRC message only supports up to 16 segments in UL, which amounts to a maximum of 144KB of collected data that can be transmitted. RAN2 has not analysed the time window during which this transmission would need to occur or if data could be partitioned and sent over a longer time window. Continuity of the collected data reporting; In case the collected data are not yet fully transferred before a handover, Xn / NG-AP signaling enhancements may be required for the continuity of the data reporting. Similarly, it could be further discussed the case of collected data not yet fully transmitted before a radio link failure, or before transitions to IDLE/INACTIVE mode. RAN2 has not assessed the impact on existing signalling, if any, or the extent of such impact..

The UP solution can effectively address these limitations by replacing the CP solution. This can be achieved by configuring legacy PDU sessions without requiring enhancements. The transfer of large volumes of AIML data can then be managed similarly to any other UP traffic. Based on the configuration of MNO, the network can establish a PDU session and manage this traffic with specific QoS flows and related QoS parameters. At this point, it is essential to ensure that the transmission of AIML data, which may have relaxed latency requirements, does not interfere with the transmission of other UP traffic. Therefore, considerations such as dedicated bearers may be explored, since they do not impose significant impacts on the specifications.

Proposal 11. The UP solution addresses the signaling overhead and scalability issues associated with the CP solution, allowing for the transmission of large volumes of AIML data using legacy methods without any spec impact.

Conclusion

Based on the above discussion, we made the following observations and proposals:

[About multiple preferred data collection configurations]

Proposal 1. No other information is needed for UE to report as part of preferred data collection configuration. (i.e., Start/stop indication and configuration ID are sufficient.)

Proposal 2. Prohibit timer for UE indicating its preferred data collection configuration is not needed.

Observation 1. For introducing prohibit timer to indicate UE preferred data collection configuration, the subjects to which the prohibit timer applies can be classified as:

- Method 1. Applying the prohibit timer for each candidate configuration

- Method 2. Applying a single prohibit timer for all candidate configurations (i.e., a prohibit timer per UE)

Proposal 3. If RAN2 agrees to introduce prohibit timer to indicate UE preferred data collection configuration, a single prohibit timer for all candidate configurations (i.e., a prohibit timer per UE) would be sufficient.

Proposal 4. RAN2 may adopt Solution1 to include a list of candidate configurations in OtherConfig.

- Solution 1: OtherConfig contains a list of candidate configurations as a list of a new IE, where each candidate configuration contains at least an identifier of the candidate configuration, CSI-ResourceConfigId for Set A, CSI-ResourceConfigId for Set B, and related associated IDs, as agreed in RAN2#130.

Proposal 5. If Solution 1 is adopted, serving cell ID is additionally needed to be included in candidate UE data collection configuration.

Data transfer over UP

Proposal 6. From the RAN2 perspective, UP solution can be classified into two:

- Data transfer through UP tunnel via UPF (i.e., Option1a, Option1b, Option2),

- Data transfer through UP tunnel via OAM (i.e., Option3).

Discussion on the controllability aspects of CP&UP solutions

Proposal 7. MNO can have full controllability over the data collection procedure through both CP solution and UP solution.

Discussion on the visibility aspects of CP&UP solutions

Proposal 8. For both CP solution and UP solution, MNO can have full visibility over standardized data.

Proposal 9. For the discussion of transferring partial/non-standardized data of UE side data collection, RAN2 should first clarify which specific data needed as partial/non-standardized data.

Proposal 10. For Rel-19, RAN2 only focuses on standardized data, disregarding partial/non-standardized data.

Discussion on the scalability aspects of CP solution

Proposal 11. The UP solution addresses the signaling overhead and scalability issues associated with the CP solution, allowing for the transmission of large volumes of AIML data using legacy methods without any spec impact.

References

- R2-250xxxx_[POST130][037][AI PHY] UE candidate data collection (XiaomiEricsson)_v16_Rapp.docx

- RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, Qualcomm.

- RP-243307, “Way forward on AIML UE sided data collection,” Dec 2024

- R2_130_Chair_minutes_2025-05-23_final

- R2_127_ChairNotes_24-08-23_17-00_eom.docx

- SP-250413, “New SID on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML) Phase 2”, Mar 2025

- TR23.700-04, “Study on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML); Phase2”

- [POST130][033][AI PHY] UE Side data collection (Ericsson)_v12_Rapp.docx

Annex: Part of ‘Draft TP for candidate UE data collection (Solution 1_2_3).docx’

- 6.2.2	Message definitions

<Text Omitted>

- –	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the network.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UEAssistanceInformation message

-- ASN1START

-- TAG-UEASSISTANCEINFORMATION-START

UEAssistanceInformation ::=         SEQUENCE {

criticalExtensions                  CHOICE {

ueAssistanceInformation             UEAssistanceInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UEAssistanceInformation-IEs ::=     SEQUENCE {

delayBudgetReport                   DelayBudgetReport                   OPTIONAL,

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1540-IEs   OPTIONAL

}

DelayBudgetReport::=                CHOICE {

type1                               ENUMERATED {

msMinus1280, msMinus640, msMinus320, msMinus160,msMinus80, msMinus60, msMinus40,

msMinus20, ms0, ms20,ms40, ms60, ms80, ms160, ms320, ms640, ms1280},

...

}

UEAssistanceInformation-v1540-IEs ::= SEQUENCE {

overheatingAssistance               OverheatingAssistance               OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1610-IEs   OPTIONAL

}

OverheatingAssistance ::=           SEQUENCE {

reducedMaxCCs                       ReducedMaxCCs-r16                   OPTIONAL,

reducedMaxBW-FR1                    ReducedMaxBW-FRx-r16                OPTIONAL,

reducedMaxBW-FR2                    ReducedMaxBW-FRx-r16                OPTIONAL,

reducedMaxMIMO-LayersFR1            SEQUENCE {

reducedMIMO-LayersFR1-DL            MIMO-LayersDL,

reducedMIMO-LayersFR1-UL            MIMO-LayersUL

} OPTIONAL,

reducedMaxMIMO-LayersFR2            SEQUENCE {

reducedMIMO-LayersFR2-DL            MIMO-LayersDL,

reducedMIMO-LayersFR2-UL            MIMO-LayersUL

} OPTIONAL

}

OverheatingAssistance-r17 ::=       SEQUENCE {

reducedMaxBW-FR2-2-r17              SEQUENCE {

reducedBW-FR2-2-DL-r17              ReducedAggregatedBandwidth-r17,

reducedBW-FR2-2-UL-r17              ReducedAggregatedBandwidth-r17

} OPTIONAL,

reducedMaxMIMO-LayersFR2-2          SEQUENCE {

reducedMIMO-LayersFR2-2-DL          MIMO-LayersDL,

reducedMIMO-LayersFR2-2-UL          MIMO-LayersUL

} OPTIONAL

}

ReducedAggregatedBandwidth ::= ENUMERATED {mhz0, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz200, mhz300, mhz400}

ReducedAggregatedBandwidth-r17 ::= ENUMERATED {mhz0, mhz100, mhz200, mhz400, mhz800, mhz1200, mhz1600, mhz2000}

UEAssistanceInformation-v1610-IEs ::= SEQUENCE {

idc-Assistance-r16                  IDC-Assistance-r16                  OPTIONAL,

drx-Preference-r16                  DRX-Preference-r16                  OPTIONAL,

maxBW-Preference-r16                MaxBW-Preference-r16                OPTIONAL,

maxCC-Preference-r16                MaxCC-Preference-r16                OPTIONAL,

maxMIMO-LayerPreference-r16         MaxMIMO-LayerPreference-r16         OPTIONAL,

minSchedulingOffsetPreference-r16   MinSchedulingOffsetPreference-r16   OPTIONAL,

releasePreference-r16               ReleasePreference-r16               OPTIONAL,

sl-UE-AssistanceInformationNR-r16   SL-UE-AssistanceInformationNR-r16   OPTIONAL,

referenceTimeInfoPreference-r16     BOOLEAN                             OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1700-IEs   OPTIONAL

}

UEAssistanceInformation-v1700-IEs ::= SEQUENCE {

ul-GapFR2-Preference-r17              UL-GapFR2-Preference-r17              OPTIONAL,

musim-Assistance-r17                  MUSIM-Assistance-r17                  OPTIONAL,

overheatingAssistance-r17             OverheatingAssistance-r17             OPTIONAL,

maxBW-PreferenceFR2-2-r17             MaxBW-PreferenceFR2-2-r17             OPTIONAL,

maxMIMO-LayerPreferenceFR2-2-r17      MaxMIMO-LayerPreferenceFR2-2-r17      OPTIONAL,

minSchedulingOffsetPreferenceExt-r17  MinSchedulingOffsetPreferenceExt-r17  OPTIONAL,

rlm-MeasRelaxationState-r17           BOOLEAN                               OPTIONAL,

bfd-MeasRelaxationState-r17           BIT STRING (SIZE (1..maxNrofServingCells)) OPTIONAL,

nonSDT-DataIndication-r17             SEQUENCE {

resumeCause-r17                       ResumeCause                       OPTIONAL

}                                                                           OPTIONAL,

scg-DeactivationPreference-r17        ENUMERATED { scg-DeactivationPreferred, noPreference }    OPTIONAL,

uplinkData-r17                        ENUMERATED { true }                   OPTIONAL,

rrm-MeasRelaxationFulfilment-r17      BOOLEAN                               OPTIONAL,

propagationDelayDifference-r17        PropagationDelayDifference-r17        OPTIONAL,

nonCriticalExtension                  UEAssistanceInformation-v1800-IEs     OPTIONAL

}

UEAssistanceInformation-v1800-IEs ::= SEQUENCE {

idc-FDM-Assistance-r18                IDC-FDM-Assistance-r18                          OPTIONAL,

idc-TDM-Assistance-r18                IDC-TDM-Assistance-r18                          OPTIONAL,

multiRx-PreferenceFR2-r18             ENUMERATED {single, multiple }                  OPTIONAL,

musim-Assistance-v1800                MUSIM-Assistance-v1800                          OPTIONAL,

flightPathInfoAvailable-r18           ENUMERATED {true}                               OPTIONAL,

ul-TrafficInfo-r18                    UL-TrafficInfo-r18                              OPTIONAL,

n3c-RelayUE-InfoList-r18              SEQUENCE (SIZE (0..8)) OF N3C-RelayUE-Info-r18  OPTIONAL,

sl-PRS-UE-AssistanceInformationNR-r18 SL-PRS-UE-AssistanceInformationNR-r18           OPTIONAL,

nonCriticalExtension                  UEAssistanceInformation-v19xy-IEs               OPTIONAL

}

UEAssistanceInformation-v19xy-IEs ::= SEQUENCE {

applicabilityReportList-r19           ApplicabilityReportList-r19                     OPTIONAL,

dataCollectionPreference-r19          DataCollectionPreference-r19                    OPTIONAL,

loggedDataCollectionAssistance-r19    LoggedDataCollectionAssistance-r19              OPTIONAL,

nonCriticalExtension                  SEQUENCE {}                                     OPTIONAL

}

IDC-Assistance-r16 ::=                  SEQUENCE {

affectedCarrierFreqList-r16             AffectedCarrierFreqList-r16               OPTIONAL,

affectedCarrierFreqCombList-r16         AffectedCarrierFreqCombList-r16           OPTIONAL,

...

}

AffectedCarrierFreqList-r16 ::= SEQUENCE (SIZE (1.. maxFreqIDC-r16)) OF AffectedCarrierFreq-r16

AffectedCarrierFreq-r16 ::=     SEQUENCE {

carrierFreq-r16                 ARFCN-ValueNR,

interferenceDirection-r16       ENUMERATED {nr, other, both, spare}

}

AffectedCarrierFreqCombList-r16 ::= SEQUENCE (SIZE (1..maxCombIDC-r16)) OF AffectedCarrierFreqComb-r16

AffectedCarrierFreqComb-r16 ::=     SEQUENCE {

affectedCarrierFreqComb-r16         SEQUENCE (SIZE (2..maxNrofServingCells)) OF  ARFCN-ValueNR    OPTIONAL,

victimSystemType-r16                VictimSystemType-r16

}

VictimSystemType-r16 ::=    SEQUENCE {

gps-r16                     ENUMERATED {true}        OPTIONAL,

glonass-r16                 ENUMERATED {true}        OPTIONAL,

bds-r16                     ENUMERATED {true}        OPTIONAL,

galileo-r16                 ENUMERATED {true}        OPTIONAL,

navIC-r16                   ENUMERATED {true}        OPTIONAL,

wlan-r16                    ENUMERATED {true}        OPTIONAL,

bluetooth-r16               ENUMERATED {true}        OPTIONAL,

...,

[[

uwb-r18                     ENUMERATED {true}        OPTIONAL

]]

}

DRX-Preference-r16 ::=              SEQUENCE {

preferredDRX-InactivityTimer-r16    ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1} OPTIONAL,

preferredDRX-LongCycle-r16          ENUMERATED {

ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512,

ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, spare12, spare11, spare10,

spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } OPTIONAL,

preferredDRX-ShortCycle-r16         ENUMERATED {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } OPTIONAL,

preferredDRX-ShortCycleTimer-r16    INTEGER (1..16)    OPTIONAL

}

MaxBW-Preference-r16 ::=            SEQUENCE {

reducedMaxBW-FR1-r16                ReducedMaxBW-FRx-r16                     OPTIONAL,

reducedMaxBW-FR2-r16                ReducedMaxBW-FRx-r16                     OPTIONAL

}

MaxBW-PreferenceFR2-2-r17 ::=       SEQUENCE {

reducedMaxBW-FR2-2-r17              SEQUENCE {

reducedBW-FR2-2-DL-r17              ReducedAggregatedBandwidth-r17       OPTIONAL,

reducedBW-FR2-2-UL-r17              ReducedAggregatedBandwidth-r17       OPTIONAL

} OPTIONAL

}

MaxCC-Preference-r16 ::=            SEQUENCE {

reducedMaxCCs-r16                   ReducedMaxCCs-r16                        OPTIONAL

}

MaxMIMO-LayerPreference-r16 ::=     SEQUENCE {

reducedMaxMIMO-LayersFR1-r16        SEQUENCE {

reducedMIMO-LayersFR1-DL-r16        INTEGER (1..8),

reducedMIMO-LayersFR1-UL-r16        INTEGER (1..4)

} OPTIONAL,

reducedMaxMIMO-LayersFR2-r16        SEQUENCE {

reducedMIMO-LayersFR2-DL-r16        INTEGER (1..8),

reducedMIMO-LayersFR2-UL-r16        INTEGER (1..4)

} OPTIONAL

}

MaxMIMO-LayerPreferenceFR2-2-r17 ::=    SEQUENCE {

reducedMaxMIMO-LayersFR2-2-r17          SEQUENCE {

reducedMIMO-LayersFR2-2-DL-r17          INTEGER (1..8),

reducedMIMO-LayersFR2-2-UL-r17          INTEGER (1..4)

} OPTIONAL

}

MinSchedulingOffsetPreference-r16 ::= SEQUENCE {

preferredK0-r16                       SEQUENCE {

preferredK0-SCS-15kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}              OPTIONAL,

preferredK0-SCS-30kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}              OPTIONAL,

preferredK0-SCS-60kHz-r16             ENUMERATED {sl2, sl4, sl8, sl12}             OPTIONAL,

preferredK0-SCS-120kHz-r16            ENUMERATED {sl2, sl4, sl8, sl12}             OPTIONAL

}                                                                                  OPTIONAL,

preferredK2-r16                       SEQUENCE {

preferredK2-SCS-15kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}             OPTIONAL,

preferredK2-SCS-30kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}             OPTIONAL,

preferredK2-SCS-60kHz-r16             ENUMERATED {sl2, sl4, sl8, sl12}            OPTIONAL,

preferredK2-SCS-120kHz-r16            ENUMERATED {sl2, sl4, sl8, sl12}            OPTIONAL

}                                                                                 OPTIONAL

}

MinSchedulingOffsetPreferenceExt-r17 ::=  SEQUENCE {

preferredK0-r17                           SEQUENCE {

preferredK0-SCS-480kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL,

preferredK0-SCS-960kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL

}                                                                                     OPTIONAL,

preferredK2-r17                           SEQUENCE {

preferredK2-SCS-480kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL,

preferredK2-SCS-960kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL

}                                                                                     OPTIONAL

}

MUSIM-Assistance-r17 ::=              SEQUENCE {

musim-PreferredRRC-State-r17          ENUMERATED {idle, inactive, outOfConnected}     OPTIONAL,

musim-GapPreferenceList-r17           MUSIM-GapPreferenceList-r17                     OPTIONAL

}

MUSIM-GapPreferenceList-r17 ::= SEQUENCE (SIZE (1..4)) OF MUSIM-GapInfo-r17

MUSIM-Assistance-v1800 ::=              SEQUENCE {

musim-GapPriorityPreferenceList-r18     MUSIM-GapPriorityPreferenceList-r18           OPTIONAL,

musim-GapKeepPreference-r18             ENUMERATED {true}                             OPTIONAL,

musim-CapRestriction-r18                MUSIM-CapRestriction-r18                      OPTIONAL,

musim-NeedForGapsInfoNR-r18             NeedForGapsInfoNR-r16                         OPTIONAL,

...

}

MUSIM-GapPriorityPreferenceList-r18 ::= SEQUENCE (SIZE (1..3)) OF GapPriority-r17

MUSIM-CapRestriction-r18 ::=            SEQUENCE {

musim-Cell-SCG-ToRelease-r18            MUSIM-Cell-SCG-ToRelease-r18                  OPTIONAL,

musim-CellToAffectList-r18              MUSIM-CellToAffectList-r18                    OPTIONAL,

musim-AffectedBandsList-r18             MUSIM-AffectedBandsList-r18                   OPTIONAL,

musim-AvoidedBandsList-r18              MUSIM-AvoidedBandsList-r18                    OPTIONAL,

musim-MaxCC-r18                         MUSIM-MaxCC-r18                               OPTIONAL

}

MUSIM-Cell-SCG-ToRelease-r18 ::=        SEQUENCE {

musim-CellToRelease-r18                 MUSIM-CellToRelease-r18                       OPTIONAL,

scg-ReleasePreference-r18               ENUMERATED {true}                             OPTIONAL

}

MUSIM-CellToRelease-r18 ::=             SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex

MUSIM-CellToAffectList-r18::=           SEQUENCE (SIZE (1..maxNrofServingCells)) OF MUSIM-CellToAffect-r18

MUSIM-CellToAffect-r18 ::=              SEQUENCE {

musim-ServCellIndex-r18                 ServCellIndex,

musim-MIMO-Layers-DL-r18                INTEGER (1..8)                                OPTIONAL,

musim-MIMO-Layers-UL-r18                INTEGER (1..4)                                OPTIONAL,

musim-SupportedBandwidth-DL-r18         SupportedBandwidth-v1700                      OPTIONAL,

musim-SupportedBandwidth-UL-r18         SupportedBandwidth-v1700                      OPTIONAL

}

MUSIM-AffectedBandsList-r18  ::=        SEQUENCE (SIZE (1..maxBandComb-MUSIM-r18)) OF MUSIM-AffectedBands-r18

MUSIM-AffectedBands-r18 ::=             SEQUENCE (SIZE (1..maxCandidateBandIndex-r18)) OF MUSIM-CapabilityRestrictedBandParameters-r18

MUSIM-CapabilityRestrictedBandParameters-r18 ::= SEQUENCE {

musim-bandEntryIndex-r18                MUSIM-BandEntryIndex-r18,

musim-CapabilityRestricted-r18          SEQUENCE {

musim-MIMO-Layers-DL-r18                INTEGER (1..8)                            OPTIONAL,

musim-MIMO-Layers-UL-r18                INTEGER (1..4)                            OPTIONAL,

musim-SupportedBandwidth-DL-r18         SupportedBandwidth-v1700                  OPTIONAL,

musim-SupportedBandwidth-UL-r18         SupportedBandwidth-v1700                  OPTIONAL

}

}

MUSIM-AvoidedBandsList-r18 ::=          SEQUENCE (SIZE (1..maxBandComb-MUSIM-r18)) OF MUSIM-AvoidedBands-r18

MUSIM-AvoidedBands-r18 ::=              SEQUENCE (SIZE (1..maxCandidateBandIndex-r18)) OF MUSIM-BandEntryIndex-r18

MUSIM-BandEntryIndex-r18 ::=            INTEGER(1.. maxCandidateBandIndex-r18)

MUSIM-MaxCC-r18 ::=                     SEQUENCE {

musim-MaxCC-TotalDL-r18                 INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-TotalUL-r18                 INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR1-DL-r18                  INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR1-UL-r18                  INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR2-1-DL-r18                INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR2-1-UL-r18                INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR2-2-DL-r18                INTEGER (1..32)                                OPTIONAL,

musim-MaxCC-FR2-2-UL-r18                INTEGER (1..32)                                OPTIONAL

}

ReleasePreference-r16 ::=           SEQUENCE {

preferredRRC-State-r16              ENUMERATED {idle, inactive, connected, outOfConnected}

}

ReducedMaxBW-FRx-r16 ::=            SEQUENCE {

reducedBW-DL-r16                    ReducedAggregatedBandwidth,

reducedBW-UL-r16                    ReducedAggregatedBandwidth

}

ReducedMaxCCs-r16 ::=               SEQUENCE {

reducedCCsDL-r16                    INTEGER (0..31),

reducedCCsUL-r16                    INTEGER (0..31)

}

SL-UE-AssistanceInformationNR-r16 ::= SEQUENCE (SIZE (1..maxNrofTrafficPattern-r16)) OF SL-TrafficPatternInfo-r16

SL-TrafficPatternInfo-r16::=          SEQUENCE {

trafficPeriodicity-r16                ENUMERATED {ms20, ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000},

timingOffset-r16                      INTEGER (0..10239),

messageSize-r16                       BIT STRING (SIZE (8)),

sl-QoS-FlowIdentity-r16               SL-QoS-FlowIdentity-r16

}

UL-GapFR2-Preference-r17::=           SEQUENCE {

ul-GapFR2-PatternPreference-r17       INTEGER (0..3)                     OPTIONAL

}

PropagationDelayDifference-r17 ::=  SEQUENCE (SIZE (1..4)) OF INTEGER (-270..270)

IDC-FDM-Assistance-r18 ::=            SEQUENCE {

affectedCarrierFreqRangeList-r18      AffectedCarrierFreqRangeList-r18               OPTIONAL,

affectedCarrierFreqRangeCombList-r18  AffectedCarrierFreqRangeCombList-r18           OPTIONAL,

...

}

IDC-TDM-Assistance-r18 ::=            SEQUENCE {

cycleLength-r18                       ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30,

ms32, ms35, ms40, ms60, ms64, ms70, ms80, ms96, ms100, ms128, ms160,

ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240},

startOffset-r18                       INTEGER (0..10239),

slotOffset-r18                        INTEGER (0..31),

activeDuration-r18                    CHOICE {

subMilliSeconds-r18 INTEGER (1..31),

milliSeconds-r18    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

...

}

AffectedCarrierFreqRangeList-r18 ::=  SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF AffectedCarrierFreqRange-r18

AffectedCarrierFreqRange-r18 ::=      SEQUENCE {

affectedFreqRange-r18                 AffectedFreqRange-r18,interferenceDirection-r18      ENUMERATED {nr, other, both, spare},

victimSystemType-r18                  VictimSystemType-r16                           OPTIONAL

}

AffectedCarrierFreqRangeCombList-r18 ::= SEQUENCE (SIZE (1..maxCombIDC-r16)) OF AffectedCarrierFreqRangeComb-r18

AffectedCarrierFreqRangeComb-r18 ::=  SEQUENCE {

affectedCarrierFreqRangeComb-r18      SEQUENCE (SIZE (2..maxNrofServingCells)) OF AffectedFreqRange-r18,

interferenceDirection-r18             ENUMERATED {nr, other, both, spare},

victimSystemType-r18                  VictimSystemType-r16                           OPTIONAL

}

AffectedFreqRange-r18 ::=             SEQUENCE {

centerFreq-r18                        ARFCN-ValueNR,

affectedBandwidth-r18                 ENUMERATED {khz200, khz400, khz600, khz800, mhz1, mhz2, mhz3, mhz4, mhz5, mhz6,

mhz8, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz200,

mhz300, mhz400, spare10, spare9, spare8, spare7, spare6, spare5, spare4,

spare3, spare2, spare1}

}

UL-TrafficInfo-r18 ::=                SEQUENCE (SIZE (1..maxNrofPDU-Sessions-r17)) OF PDU-SessionUL-TrafficInfo-r18

PDU-SessionUL-TrafficInfo-r18 ::=     SEQUENCE {

pdu-SessionID-r18                     PDU-SessionID,

qos-FlowUL-TrafficInfoList-r18        SEQUENCE (SIZE (1..maxNrofQFIs)) OF QOS-FlowUL-TrafficInfo-r18

}

QOS-FlowUL-TrafficInfo-r18 ::=        SEQUENCE {

qfi-r18                               QFI,

jitterRange-r18                       SEQUENCE {

lowerBound-r18                        JitterBound-r18,

upperBound-r18                        JitterBound-r18

}                                                                                    OPTIONAL,

burstArrivalTime-r18                  CHOICE {

referenceTime                         ReferenceTime-r16,

referenceSFN-AndSlot                  ReferenceSFN-AndSlot-r18

}                                                                                    OPTIONAL,

trafficPeriodicity-r18                INTEGER (1..640000)                            OPTIONAL,

pdu-SetIdentification-r18             BOOLEAN                                        OPTIONAL,

psi-Identification-r18                BOOLEAN                                        OPTIONAL,

...

}

ReferenceSFN-AndSlot-r18 ::= SEQUENCE {

referenceSFN-r18                 INTEGER (0..1023),

referenceSlot-r18                INTEGER (0..639)

}

JitterBound-r18 ::= ENUMERATED {ms0, ms0dot5, ms1, ms1dot5, ms2, ms2dot5, ms3, ms3dot5, ms4, ms4dot5, ms5, ms5dot5, ms6, ms6dot5, ms7, beyondMs7}

SL-PRS-UE-AssistanceInformationNR-r18 ::= SEQUENCE (SIZE (1..maxNrofSL-PRS-TxConfig-r18)) OF SL-PRS-TxInfo-r18

SL-PRS-TxInfo-r18 ::=                 SEQUENCE {

sl-PRS-Periodicity-r18                ENUMERATED {ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000, spare6,

spare5, spare4, spare3, spare2, spare1},

sl-PRS-Priority-r18                   INTEGER (1..8)                                                            OPTIONAL,

sl-PRS-DelayBudget-r18                INTEGER (0..1023)                                                         OPTIONAL,

sl-PRS-Bandwidth-r18                  ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40,

mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100, mhz200, mhz400,

spare15, spare14, spare13, spare12, spare11, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1}       OPTIONAL,

...

}

DataCollectionPreference-r19 ::= SEQUENCE {

dataCollectionStartStop-r19                      ENUMERATED {start, stop}                   OPTIONAL,

dataCollectionPreferredConfigurationList-r19         DataCollectionPreferredConfigurationList-r19             OPTIONAL,

...

}

LoggedDataCollectionAssistance-r19 ::=    SEQUENCE {

lowPowerState-r19                         ENUMERATED {true}                                                   OPTIONAL,

bufferStatus-r19                          ENUMERATED {full, aboveThreshold}                                   OPTIONAL,

...

}

-- Solution 1:

DataCollectionPreferredConfigurationList-r19 :: = SEQUENCE (SIZE (1..maxPreferredConfig-r19)) OF DataCollectionCandidateConfigId-r19

-- Solution 2/3:

DataCollectionPreferredConfigurationList-r19 :: = SEQUENCE (SIZE (1..maxPreferredConfig-r19)) OF CSI-ReportConfigId

-- TAG-UEASSISTANCEINFORMATION-STOP

-- ASN1STOP

UEAssistanceInformation field descriptions

activeDuration Indicates the UE's preferred active duration to resolve the IDC problem. Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on.

affectedBandwidth Indicates the bandwidth around the center frequency of the carrier frequency range which is affected by the IDC problem. Value mhz5 corresponds to 5 MHz, value mhz10 corresponds to 10 MHz and so on. If candidateBandwidth is not configured, the UE is allowed to report the frequency range for any bandwidth as indicated by affectedBandwidth, within the frequency band limitation as defined in TS 38.101-1 [15], TS 38.101-2 [39], TS 38.101-3 [34] and TS 38.101-5 [75].

affectedCarrierFreqList Indicates a list of NR carrier frequencies that are affected by IDC problem.

affectedCarrierFreqRangeList Indicates a list of NR carrier frequency ranges that are affected by IDC problem.

affectedCarrierFreqCombList Indicates a list of NR carrier frequency combinations that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from NR when configured with UL CA or NR-DC.

affectedCarrierFreqRangeCombList Indicates a list of NR carrier frequency range combinations that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from NR when configured with UL CA or NR-DC

bfd-MeasRelaxationState Indicates the relaxation state of BFD measurements. Each bit corresponds to a serving cell of the cell group. A serving cell is mapped to the (servCellIndex+1)-th bit, starting from MSB. A bit that is set to 1 indicates that the UE is performing BFD measurements relaxation on the serving cell mapped on the bit. A bit that is set to 0 indicates that the UE is not performing BFD measurements relaxation on the serving cell mapped on the bit. If a serving cell is not configured to the UE, the corresponding bit is set to 0.

centerFreq Indicates the center frequency of the carrier frequency range which is affected by the IDC problem.

cycleLength Indicates the UE's preferred cycle length to resolve the IDC problem. Value in ms. Value ms2 corresponds to 2 ms, value ms3 corresponds to 3 ms, and so on.

dataCollectionStartStop If it is set to 'start', it indicates the UE's preference to be configured with radio resources for UE data collection. If it is set to 'stop', it indicates the UE's preference to not be configured any longer with radio resources for UE data collection.

dataCollectionPreferredConfiguration Indicates the UE's preferred radio resource configuration(s) for UE data collection.

delayBudgetReport Indicates the UE-preferred adjustment to connected mode DRX.

interferenceDirection Indicates the direction of IDC interference. Value nr indicates that only NR is victim of IDC interference, value other indicates that only another radio is victim of IDC interference and value both indicates that both NR and another radio are victims of IDC interference. The other radio refers to either the ISM radio or GNSS (see TR 36.816 [44]).

loggedDataCollectionAssistance Indicates assistance information related to the logging of measurements for network data collection performed in accordance with CSI-LoggedMeasurementConfig.

lowPowerState It is set to 'true' if the UE determines to be in low power state. Editor's Note: FFS the need to clarify when/how this is signalled, e.g. when/how it is signalled that the power is not any longer low.

bufferStatus Indicates the status of the buffer reserved for the logging of L1 radio measurements. Editor's Note: FFS the encoding of the data availability indication and the cause value (full buffer, threshold).

<Text Omitted>