3GPP TSG-RAN WG2 Meeting #131                                                                    R2-2505219

Bengaluru, India, Aug 25th – 29th, 2025

Source:	CATT, Turkcell

Title:	Discussion on Network sided model

Agenda Item:	8.3.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we would like to discuss remaining issues on network sided model, including sub use cases to be supported, assistance information from the UE (e.g., measurements for inference or performance monitoring), and data collection.

- Discussion

  - Sub use cases

In the current running CR[1], the following 3 sub-use cases are considered for cell-level RRM measurement prediction:

-	Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated;

-	Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s);

-	Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

And the following 3 sub-use cases are considered for beam-level RRM measurement prediction:

-	Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated;

-	Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s);

-	Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

In the last RAN2#130 meeting, the issue of which sub-use cases could be supported was discussed, with the following agreement:

Agreements

- For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used.   For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero.   FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

Since we have already studied sub-use case 1 and sub-use case 3 and some companies have relevant simulations, sub-use case 1 and sub-use case 3 should be supported.

In the existing spec, the L1 beam-level measurement results are reported by UE to DU and the handover decision of LTM is executed at DU, while the RRM measurement results are reported by UE to CU and the handover decision is executed at CU. It is necessary to consider the cases of AI model at DU and CU separately for supporting sub-use case 1 and sub-use case 3. If AI model at DU, the specification impacts for sub-use case 1 is that L1 beam-level measurement results need to be reported to CU from DU, and the specification impacts for sub-use case 3 is that L3 cell-level measurement results need to be reported to CU from DU. If AI model at CU, the specification impacts for sub-use case 1 and sub-use case 3 is that L1 beam-level measurement result(s) need to be reported to CU from UE or DU. Based on the above analysis, if sub-use case 1 and sub-use case 3 are supported, the corresponding specification impact of the following two options should be considered:

- Option1: DU reports the measurement results to CU;

- Option2: UE reports L1 beam-level measurement results to CU.

Considering that option 2 is only used for the case of AI model at CU, we think that option 1 is more suitable.

Proposal 1: Sub-use case 1 and sub-use case 3 should be supported, and it’s sufficient that DU reports the available measurement results to CU.

  - Assistance information from the UE

During the last RAN2#130 meeting, we have the remained open issues as follows:

Agreements

3	FFS on UE awareness and preference for NW sided model

In the last meeting, some companies think that the UE should be aware that it is being asked to be provived measurements for performance monitoring, and some companies think that UE should report a preference to the NW on whether it would like to do performance monitoring. In our opinion, the NW side configures the measurement configuration according to its own needs, and the UE performs measurement and measurement reporting based on the measurement configuration of the NW. Therefore, there is no clear motivation for the UE to know that the measurement is for performance monitoring or inference, and NW does not need to know the preference of UE on whether it would like to do performance monitoring.

Proposal 2: UE performs measurement and measurement reporting based on the measurement configuration of the NW, UE awareness and preference for the NW side model are not supported.

  - Data collection

- Buffer

In the last RAN2 meeting, the following agreement was achieved.

Agreements - The availability indication can be triggered due to: - full buffer being reached (if configured), (FFS if buffer is per use case) - buffer threshold being reached (if configured), or (FFS if buffer is per use case) - low power (if configured)

Similar issues are also being discussed in AI/ML PHY, but there is no conclusion yet, so we can wait for the conclusion of RAN1.

###### Open issue RRC-34: Whether enhancements for NW-side data collection are per use case or common for all AI-related use cases Issue description: It can be discussed whether the enhancements for NW-side data collection are per use case or common for all AI-related use cases, e.g., AS layer memory, logged data availability, low-power state indication. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

Proposal 3: For the question of whether the buffer is per use case, wait for the conclusion of AI/ML PHY.

- CGI

In the last RAN2 #130 meeting, the following agreement was achieved and an FFS was remained.

Agreements - UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed)

In AI/ML PHY, similar issues were discussed and the following agreement were achieved:

Agreement

The UE should report the CGI of the serving cell whenever feasible. If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback. CGI：Cell Global Identifier

For AI mobility, CGI reporting for data logging should also be supported for RRM measurement prediction use case.

Proposal 4: UE can be configured to log and report the CGI of serving cell.  If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback.

- Handling of configuration/data during mobility/state transition

In AI/ML PHY, how to handle the configuration/data during mobility/state transition was discussed and the following agreements were achieved.

Agreements on configuration/data handling during mobility/state transition - UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE). - RRC-18) 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with synch is introduced. In case of HO, the source sends the 1-bit indication to target cell in HO preparation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data). We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it. Notify RAN3

An LS about logged Data Handling During Handover is sent from the AI/ML PHY to RAN3. We can wait for a response from RAN3. If RAN3 has no objections, the agreements above about configuration/data handling during mobility/state transition are also appropriate for NW-side data collection for RRM measurement prediction use case, thus, we propose,

Proposal 5: For handling of configuration/data during mobility/state transition, wait for the conclusion of AI/ML PHY.

- Starting/Stopping of logging

In AI/ML PHY, it was agreed that data collection is controlled by the network, the UE will not autonomously stop when low power state is detected. Also, when UE reaches its buffer limitation, the UE stops measurement for data collection purposes and logging.

Agreements on starting/stopping of logging - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

The agreements above about starting/stopping of logging are also appropriate for NW-side data collection for RRM measurement prediction use case, thus, we propose,

Proposal 6: The following agreements on starting/stopping of logging for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction:

- Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected;

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Sending of the collected data

In AI/ML PHY, it was agreed that on-demand reporting of the logged measurements will be specified, and the following agreements were achieved.

Agreements on sending of the collected data - As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT. - New SRB can be configured for NW-side data collection (with lower priority).

The agreements above about sending of the collected data are also appropriate for NW-side data collection for RRM measurement prediction use case, thus, we propose,

Proposal 7: The following agreements on sending of the collected data for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction:

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT;

- New SRB can be configured for NW-side data collection (with lower priority).

- Conclusion

According to the analysis in Section 2, our observations and proposals are summarized as follows:

Sub use cases

Proposal 1: Sub-use case 1 and sub-use case 3 should be supported, and it’s sufficient that DU reports the available measurement results to CU.

Assistance information from the UE

Proposal 2: UE performs measurement and measurement reporting based on the measurement configuration of the NW, UE awareness and preference for the NW side model are not supported.

Data collection

Proposal 3: For the question of whether the buffer is per use case, wait for the conclusion of AI/ML PHY.

Proposal 4: UE can be configured to log and report the CGI of serving cell.  If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback.

Proposal 5: For handling of configuration/data during mobility/state transition, wait for the conclusion of AI/ML PHY.

Proposal 6: The following agreements on starting/stopping of logging for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction:

- Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected;

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

Proposal 7: The following agreements on sending of the collected data for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction:

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT;

- New SRB can be configured for NW-side data collection (with lower priority).

- References

- TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR (Release 19), 3GPP