3GPP TSG-RAN WG2 Meeting #131	R2-2505232

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.17

Source: 	CATT

Title: 	Discussion on support of NB-IoT NTN TDD

Document for: 	Discussion and Decision

Introduction

In WID [1], RAN2 objectives are shown as follows:

This work item includes the following objectives: - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4]

This contribution analyzes potential RAN2 impacts to support NB-IoT TDD NTN on “other necessary impacts on higher layer” caused by the new frame structure of 90 ms period, based on the agreements of the RAN1 and RAN2 in the latest meeting.

Discussion

2.1 On PUR/UL SPS

This section tries to give some clarification on RAN2 agreements on PUR/UL SPS.

According to the current specification, if an uplink grant is indicated for a HARQ process for new transmission, the HARQ entity will obtain the MAC PDU to transmit from the "Multiplexing and assembly" entity, and then deliver the MAC PDU, the uplink grant and the HARQ information to the corresponding HARQ process for transmission.

The following agreements were achieved in RAN2#129bis for SPS and PUR:

4.	When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe

5.	When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

That is, the UE will postpone the PUR resource start subframe or the UL SPS resource to the next valid UL subframe of the new TDD structure.

Then what RAN2 needs to further clarify is how to understand the "postpone" in the agreements, when an PUR/SPS transmission occasion is overlapped with non-U NB-IoT subframes, i.e. whether the UE really obtains the MAC PDU for new transmit from the "Multiplexing and assembly" entity or not for this PUR/SPS transmission occasion.

There can be the following two understandings. If a PUR/UL SPS occasion determined based on current Spec is overlapped with non-U NB-IoT subframe:

- Understanding 1: The PUR/UL SPS occasion is not considered as a valid UL grant at all, and no MAC PDU for new transmission is obtained from the "Multiplexing and assembly" entity.

- Understanding 2: A MAC PDU for new transmission is still obtained and stored into the corresponding HARQ buffer for this PUR/UL SPS occasion, but the HARQ entity does not really perform the transmission of this MAC PDU on this specific occasion (so the transmission of the stored MAC PDU is delayed to later PUR/SPS UL grants with real UL NB-IoT subframe).

RAN2 may need to clarify which understanding is correct, w.r.t. the agreement of postponing PUR/UL SPS resources, because different understanding may result in different specification impacts. For example, for Understanding 1, the PUR/UL SPS occasion is not considered as a valid UL grant, so no UL grant should be delivered to HARQ entity; for Understanding 2, a UL grant is delivered to HARQ entity, but the HARQ entity should not try to perform actual transmission. That is, some statements corresponding to different understanding should be included in MAC specification.

Proposal 1: If a PUR/UL SPS occasion determined as per current Spec is overlapped with non-U subframe, RAN2 confirms which understanding is correct w.r.t. the agreement on postponing the PUR/SPS resources:

- Understanding 1: The PUR/UL SPS occasion is not considered as a valid UL grant, and no MAC PDU for new transmission is obtained from the "Multiplexing and assembly" entity for this PUR/UL SPS occasion.

- Understanding 2: A MAC PDU for new transmission is still obtained and stored in the corresponding HARQ buffer for this PUR/UL SPS occasion, but the HARQ entity does not really perform the transmission of this MAC PDU on this specific occasion (so the transmission of the stored MAC PDU is delayed to later PUR/SPS UL grants with real UL NB-IoT subframe).

2.2 On Open issue CAP-1

For the UE capability related with IoT NTN TDD mode, the following email discussion has been finished:

[Post130][314][IoT NTN TDD] capability CR (Samsung)

Scope: discuss the running capability CR

Deadline: long

And the following proposal was given in the summary:

Proposal 1: On the general IoT NTN TDD mode capability, RAN2 to decide on type of capability: - Conditionally mandatory (as in draft CR) - Optional capability without signalling

We think that UE not supporting IoT NTN TDD mode can identify a cell in IoT NTN TDD mode, and can avoid camping in that cell. So no explicit UE capability indication is needed to indicate the support of IoT NTN TDD mode, if the UE has try to access the network via IoT NTN TDD cell.

For the information from RAN1 that, the UE supporting band n249 must support this feature group, we think some statements can be added in the capability specification.

Proposal 2: (CAP-1) IoT NTN TDD mode capability can be optional capability without signaling, and a statement can be included in capability specification to declare that UE supporting band n249 must support IoT NTN TDD feature group.

Conclusion

In this contribution, the issue of RAN2 impact of TDD is discussed, with the following proposal about PUR/UL SPS:

Proposal 1: If a PUR/UL SPS occasion determined as per current Spec is overlapped with non-U subframe, RAN2 confirms which understanding is correct w.r.t. the agreement on postponing the PUR/SPS resources:

- Understanding 1: The PUR/UL SPS occasion is not considered as a valid UL grant, and no MAC PDU for new transmission is obtained from the "Multiplexing and assembly" entity for this PUR/UL SPS occasion.

- Understanding 2: A MAC PDU for new transmission is still obtained and stored in the corresponding HARQ buffer for this PUR/UL SPS occasion, but the HARQ entity does not really perform the transmission of this MAC PDU on this specific occasion (so the transmission of the stored MAC PDU is delayed to later PUR/SPS UL grants with real UL NB-IoT subframe).

Proposal 2: (CAP-1) IoT NTN TDD mode capability can be optional capability without signaling, and a statement can be included in capability specification to declare that UE supporting band n249 must support IoT NTN TDD feature group.

Reference

- RP-243264 Revised WID on introduction of IoT-NTN TDD_cl.

- Draft Report of 3GPP TSG RAN WG1 #121 v0.2.0.