3GPP TSG-RAN WG2 Meeting #131                                                                    R2-2505237

Bangalore, India, Aug 25th – 29th, 2025

Source:	CATT

Title:	Open issues on RRM Relaxation and Offloading in IDLE and INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our considerations on open issues of RRM relaxation and offloading in RRC_IDLE/INACTIVE.

Discussion

(38304-2) FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

R16 measurement relaxation criteria are used to evaluate for UE with low mobility and UE not at cell edge. When the R16 relaxation criteria are satisfied, the UE performs relaxed measurements on neighbor cells.

When the UE is performing serving cell measurement offloading, MR serving cell measurement results are not available. In this case both MR serving cell measurement and MR neighbor cell measurement are stopped.

Therefore, during the period of serving cell measurement offloading from MR to LR, it is meaningless to evaluate whether R16 criteria are met or not based on LR measurements, since the UE has already stopped the neighbor cell measurement.

Proposal 1: (38304-2) No need to consider enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when the UE is performing serving cell offloading from MR to LR.

According to existing agreements, both R19 relaxed measurement criterion and R16 relaxed measurement criterion are based on MR serving cell measurement results. When the UE exists serving cell offloading, the UE cannot obtain MR serving cell measurement results immediately. In this case, it is unclear whether/how the UE to determine the entry condition of R19 MR relaxed measurement or R16 relaxed measurement criterion is fulfilled. In our understanding, after the UE exits serving cell offloading, the UE would not stop measuring MR serving cell, instead it may perform relaxed MR serving cell measurement if the entry condition of R19 MR relaxed measurement is fulfilled. It is natural and simple that the UE starts to perform MR serving cell measurement when the UE exits serving cell offloading. Then the UE determines whether the entry condition of R19 MR relaxed measurement or R16 relaxed measurement criterion is fulfilled after the UE obtains MR serving cell measurement results. No need to enhance R16 relaxed measurement criteria or the entry condition of R19 MR relaxed measurement based on LR measurement results.

Proposal 2: (38304-2) No need to enhance R16 relaxed measurement criteria or the entry condition of R19 MR relaxed measurement based on LR measurement results when the UE exits serving cell offloading.

(38304-3/ RRC-7)The exit condition of MR RRM measurement relaxation

In RAN2#129 meeting, agreements on MR serving cell measurement relaxation included:

The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

In RAN2#129bis meeting, it was agreed to merge entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion). But it is still FFS on exit condition for MR RRM measurement relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition includes MR and/or LR-based measurements.

The possible options for exit condition of MR measurement relaxation include:

- Option 1: No separate exit condition.

- Option 2: Only based on LR serving cell quality.

- Option 3: Only based on MR serving cell quality.

During MR measurement relaxation period, MR serving cell quality is relaxed with longer measurement cycle. MR serving cell quality cannot be updated timely. Hence, it is not reasonable to consider MR serving cell quality only for exit condition of MR serving cell measurement relaxation. Option 3 is not preferred. Option 1 is similar as R16 RRM relaxation criteria. But different from R16 RRM relaxation for which only neighbor cell quality is relaxed, for LP-WUS MR serving cell quality is relaxed as well. It is questionable to reuse the principle of R16 RRM relaxation to guarantee the measurement accuracy. While, Option 2 is aligned with both LP-WUS monitoring and serving cell measurement offloading. It is preferred.

Proposal 3: (38304-3/RRC-7) The exit condition of MR measurement relaxation is only based on LR serving cell quality.

(38304-6/RRC-10) Whether UE low mobility criterion or stationary criterion should be considered for RRM relaxation/offloading.

According to current RAN4’s agreement, the scaling factor for relaxed serving cell and neighbouring cell measurements on MR with LP-WUS is 16, which is already large enough. There is no motivation to relax serving cell and neighbouring cell measurements on MR further. Hence, the motivation to introduce additional UE low mobility criterion or stationary criterion is not strong. In addition, R16 UE low mobility criterion is complexity. It is preferred to keep the solution as simple as possible.

Proposal 4: (38304-6/RRC-10) UE low mobility criterion or stationary criterion for RRM relaxation/offloading for LP-WUS is not supported.

(38304-12) FFS the impact on legacy low-mobility criteria for MR with LP-WUS

The legacy low-mobility criteria are captured in [1]:

5.2.4.9.1 Relaxed measurement criterion for UE with low mobility The relaxed measurement criterion for UE with low mobility is fulfilled when: - (SrxlevRef – Srxlev) < SSearchDeltaP, Where: - Srxlev = current Srxlev value of the serving cell (dB). - SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows: - After selecting or reselecting a new cell, or - If (Srxlev - SrxlevRef) > 0, or - If the relaxed measurement criterion has not been met for TSearchDeltaP: - The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

During the post email discussion of open issues on TS 38.304 [2], the impact on legacy low-mobility criteria with LP-WUS was raised. When the UE exits fully offloading case, the UE turns on the MR. The UE can start the re-evaluation for legacy low mobility criteria for legacy relaxed measurement if configured. For such case, it is unclear whether to clarify that SrxlevRef shall set the value of SrxlevRef to the current Srxlev value of the serving cell if the UE turns on the MR when UE exits using fully offloading case.

When R16 low mobility criterion is broadcast in system information, it is not required a UE to evaluate the criterion continuously. Then it is possible that a UE camped on a cell broadcast R16 low mobility criterion starts to evaluate the low mobility criterion at some time. In this case, the UE needs to set SrxlevRef to a value. In our understanding, in such case, the condition “if the relaxed measurement criterion has not been met for TSearchDeltaP” is fulfilled. Then the UE will set the value of SrxlevRef to the current Srxlev value of the serving cell. Or it is possible that a UE camped on a cell broadcast R16 low mobility criterion may stop the evaluation of the low mobility criterion at T1 and re-start the evaluation of R16 low mobility criterion at T2. In this case, the UE will update the value of SrxlevRef to the current Srxlev value of the serving cell when the condition “if the relaxed measurement criterion has not been met for TSearchDeltaP” is fulfilled.

Observation 1: It is possible that a UE (re)-starts the evaluation of R16 low mobility criterion after camping on a cell for a period.

After the UE exits fully offloading case, the UE may (re)-start the evaluation of R16 low mobility criterion. It is similar to the case that a UE (re)-starts the evaluation of R16 low mobility criterion after camping on a cell for a period. Then the UE will set the value of SrxlevRef to the current Srxlev value of the serving cell when the condition “if the relaxed measurement criterion has not been met for TSearchDeltaP” is fulfilled. There is no specification impact for R16 low mobility criterion when UE exits using fully offloading.

Proposal 5: RAN2 confirm the case that the UE (re)-starts the evaluation of R16 low mobility criterion after the UE exits fully offloading case is similar to the case that the UE (re)-starts the evaluation of R16 low mobility criterion after camping on a cell for a period. There is no specification impact for R16 low mobility criterion when UE exits using fully offloading.

Conclusion

According to the analysis in Section 2, we observe and propose:

Observation 1: It is possible that a UE (re)-starts the evaluation of R16 low mobility criterion after camping on a cell for a period.

Proposal 1: (38304-2) No need to consider enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when the UE is performing serving cell offloading from MR to LR.

Proposal 2: (38304-2) No need to enhance R16 relaxed measurement criteria or the entry condition of R19 MR relaxed measurement based on LR measurement results when the UE exits serving cell offloading.

Proposal 3: (38304-3/RRC-7) The exit condition of MR measurement relaxation is only based on LR serving cell quality.

Proposal 4: (38304-6/RRC-10) UE low mobility criterion or stationary criterion for RRM relaxation/offloading for LP-WUS is not supported.

Proposal 5: RAN2 confirm the case that the UE (re)-starts the evaluation of R16 low mobility criterion after the UE exits fully offloading case is similar to the case that the UE (re)-starts the evaluation of R16 low mobility criterion after camping on a cell for a period. There is no specification impact for R16 low mobility criterion when UE exits using fully offloading.

Reference

- TS 38.304

- R2-2505235, Report of [Post130][211][LPWUS] Running CR for 38.304 (CATT)