3GPP TSG RAN WG2 Meeting #131                                                            R2-2505261

# Bangalore, India Aug 25th – 29th, 2025

Agenda item:	8.7.5

Source:	Ofinno

Title:	RLC Enhancements for XR

Document for:	Discussion and decision

# Introduction

This contribution aims to discuss the following issues for XR-related RLC enhancements:

- Polling and STATUS Reporting for Remaining-time-based RLC Polling: current remaining-time-based RLC polling in Rel-19 is intended to accelerate feedback for delay-critical data, but STATUS reporting triggered by such polls is still delayed by legacy t-StatusProhibit restrictions, limiting the benefit.

- STATUS Reporting Triggered by t-RxDiscard:  When the Rx side discards obsolete PDUs based on the t-RxDiscard, STATUS reports are still subject to legacy t-StatusProhibit constraints, reducing the intended advantage of quickly informing the Tx side to stop retransmissions.

- Unnecessary Polling triggered by t-PollRetransmit:

  - [RLC-11] The existing RLC might trigger unnecessary polling (via the t-PollRetransmit) even when the Tx side already knows the status of all relevant RLC SDUs, potentially causing issues at the Rx.

- Relationships between Tx Stop-retransmission and Rx Discard detection:

  - (UE capability-04) issue about whether support for rxRLC-Discard should be a pre-requisite for txRLC-StopReTxDiscardedSDU.

  - [RLC-10] Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and t-RxDiscard

# Discussion

## Polling and STATUS Reporting for Remaining-time-based RLC Polling

TS 38.323 [1] 5.x Triggering of RLC functions For the purpose of remaining-time-based RLC polling, if the remaining time till discardTimer expiry becomes less than the remainingTimeBasedPollingThreshold for the PDCP SDU for which the corresponding PDCP Data PDU has already been submitted to lower layers but for which successful delivery has not been confirmed by lower layers, the transmitting PDCP entity shall: - indicate to lower layers that the condition for remaining-time-based RLC polling is met for the corresponding PDCP Data PDU. TS 38.322 [2] ### 5.3.3 Polling Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); or - if an indication is received from upper layer (e.g., PDCP) that the condition for remaining-time-based RLC polling has been met for an RLC SDU: - include a poll in the AMD PDU as described below. 5.3.4 Status reporting An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them). Triggers to initiate STATUS reporting include: - Polling from its peer AM RLC entity: - … When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall: - if t-StatusProhibit is not running: - at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer. - else: - at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

In Rel-19, XR introduced a remaining-time-based RLC polling mechanism aimed at ensuring timely retransmission of delay-critical data by accelerating the polling process. The underlying motivations for this enhancement include:

- (1) Expedited polling for delay-critical data: the Tx side facilitates a “fastened” polling approach for data with stringent latency requirements. Rather than relying on the accumulation of a certain number of PDUs or bytes exceeding configured thresholds (e.g., pollPDU, pollByte), polling is triggered proactively when the remaining time associated with an RLC SDU falls below a specific remaining time threshold configured in PDCP.

- (2) Prevention of data obsolescence: for the XR traffic, the integrity of a PDU set (comprising multiple PDUs) is crucial, and the entire set may be deemed obsolete if any constituent part/PDU fails to be retransmitted promptly. By enabling earlier polling, the Tx side can anticipate a quicker reception of the STATUS report from the Rx side, allowing the Tx side to ascertain which PDUs within the set have not been successfully received by the Rx side and to ensure the timely retransmission of the remaining portions/PDUs.

- (3) Reduction of delay gap: to minimize the temporal gap between the initial transmission of an RLC SDU and its subsequent retransmissions, it is necessary to rapidly execute both the RLC polling process at the Tx side and the corresponding STATUS report at the Rx side.

However, the current standard treats polls triggered by the remaining-time-based RLC polling the same as legacy polls. As a result, when the Rx side receives such a poll and initiating a STATUS report, the Rx side applies the same restrictions (e.g., the t-StatusProhibit) that govern the transmission of conventional STATUS reports. Specifically, the Rx side is prevented from transmitting the STATUS report while the t-StatusProhibit is running, thereby negating the intended advantage of faster polling for delay-critical data.

To fully realize the potential of remaining-time-based RLC polling to ensure timely RLC retransmissions, it is essential to expedite the associated STATUS reporting. Consequently, the following enhancements are proposed:

- Remaining-time-based RLC polling should be distinguished as a specific type of poll (e.g., a “delay-critical poll”) to reflect its association with delay-critical data.

- Upon receiving this specific poll (e.g., the “delay-critical poll”), the Rx side should trigger a corresponding specific STATUS reporting (e.g., a “delay-critical STATUS reporting”) that is controlled by a separate t-StatusProhibit with a shorter value.

This approach ensures that the transmission of the delay-critical STATUS report is not constrained by the legacy STATUS reporting triggered by conventional polling, and its transmission is solely controlled by a separate t-StatusProhibit with a shorter value than the legacy t-StatusProhibit, operating independently from the legacy STATUS reporting, thus enabling faster feedback for delay-critical data.

For remaining-time-based RLC polling, Tx side of RLC includes a delay-critical poll in the AMD PDU.

When Rx side of RLC receives a delay-critical poll in the AMD PDU, triggers a delay-critical STATUS reporting.

The delay-critical STATUS reporting is restricted by a separate t-StatusProhibit with a shorter value than the legacy t-StatusProhibit.

## STATUS Reporting Triggered by t-RxDiscard

TS 38.323 [1] 5.3.4 Status reporting An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them). Triggers to initiate STATUS reporting include: - … - Detection of discard of an AMD PDU: - The receiving side of an AM RLC entity shall trigger a STATUS report when t-RxDiscard expires.

In Rel-19, an approach to prevent unnecessary RLC retransmissions is to introduce a new RLC discard timer on Rx side, denoted as t-RxDiscard. On the Rx side, an AMD PDU deemed obsolete can be discarded based on the expiration of t-RxDiscard. Upon detecting the obsolete/discard of an AMD PDU due to the expiry of t-RxDiscard, the Rx side should trigger a STATUS report and positively acknowledge the discarded PDU within that STATUS report.

The rationale behind introducing t-RxDiscard extends beyond simply avoiding unnecessary RLC retransmissions; it also aims to prevent resource wastage. When t-RxDiscard expires, the Rx side concludes that certain SDUs are “obsolete” or “outdated,” implying they are missing and likely to be discarded by higher layers due to exceeding their permissible delay budget. To prevent the Tx side from wasting resources by retransmitting these SDUs that are already considered obsolete, the Rx side explicitly “acknowledges” them (e.g., referred to as a “fake ACK”) in the STATUS report. This “fake ACK” serves as a signal to the Tx side, indicating that it no longer needs to retransmit these specific SDUs. This proactive notification helps avert resource wastage that would otherwise occur if the Tx side continued to retransmit data that the Rx side would ultimately discard anyway.

However, in the current standard, the STATUS report triggered by t-RxDiscard is indistinguishable from the legacy STATUS report. Analogous to the previously discussed polling issue, since this STATUS report is viewed as a normal STATUS report, its transmission is subject to the same restrictions, such as t-StatusProhibit. This means that the Rx side is unable to transmit the STATUS report while t-StatusProhibit is running, thereby diminishing the intended benefit of triggering the STATUS report upon t-RxDiscard expiry.

To fully realize the potential of t-RxDiscard, it would be advantageous to expedite this type of STATUS reporting as well. For instance, the Rx side could trigger a specific STATUS reporting (e.g., a “delay-critical STATUS reporting”) when t-RxDiscard expires, with its transmission restricted by a separate t-StatusProhibit configured with a shorter value than the legacy t-RxDiscard.

When t-RxDiscard expires, Rx side of RLC triggers a delay-critical STATUS reporting.

## Unnecessary Polling Triggered by t-PollRetransmit:

TS 38.322 [2] 5.3.3.2 Transmission of a AMD PDU Upon submission of an AMD PDU including a poll to lower layer, the transmitting side of an AM RLC entity shall: - set POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer; - if t-PollRetransmit is not running: - start t-PollRetransmit. - else: - restart t-PollRetransmit. 5.3.3.3 Reception of a STATUS report Upon reception of a STATUS report from the receiving RLC AM entity the transmitting side of an AM RLC entity shall: - if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN: - if t-PollRetransmit is running: - stop and reset t-PollRetransmit. 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission (excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1); or - consider any RLC SDU which has not been positively acknowledged for retransmission (excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1). - include a poll in an AMD PDU, if any, as described in clause 5.3.3.2.

RAN2#130 Agreements Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

During the RAN2#130 meeting, the case where only SDUs whose (re-)transmission has been stopped due to discard indications from PDCP are buffered was discussed. The concern was that the absence of an SDU to include a poll for transmission could lead to window stalling. However, the conclusion reached was that window stalling would not occur due to the STATUS reporting triggered from the Rx side. Consequently, RAN2 made a working assumption that addressing window stalling issues for polling retransmission was unnecessary. Based on this, the RLC running CR added an “if any” condition for including a poll in an AMD PDU upon expiry of t-PollRetransmit, acknowledging the possibility that no AMD PDU is available to include the poll upon t-PollRetransmit expiry.

In the legacy RLC behavior, t-PollRetransmit is stopped and reset when the STATUS report ACKs or NACKs the RLC SDU with a SN equal to POLL_SN, indicating that the STATUS report has responded to the poll request for all SNs. Given the understanding that an AMD PDU may not be available to include a poll for transmission when all RLC SDUs have been stopped for (re-)transmission, it is worth to discuss whether the UE should stop t-PollRetransmit when the RLC entity already knows the status (ACKed, NACKed, or (re)-transmission is stopped) of all RLC SDUs up to POLL_SN.

In this case, we consider that RLC should stop and reset t-PollRetransmit. The primary purpose of t-PollRetransmit is to ensure that the receiving RLC entity receives the poll request from the Tx side, or the poll can be transmitted again. However, if the Tx side already possesses knowledge of the status of all RLC SDUs up to POLL_SN, there is no justification for retransmitting the poll. Such unnecessary polling could trigger t-StatusProhibit at the Rx side, potentially blocking the transmission of STATUS reporting triggered by t-RxDiscard (which is crucial for delay-critical data) during the t-StatusProhibit running period. Moreover, as previously mentioned, an AMD PDU may not be available to include the poll when all RLC SDUs are stopped for (re-)transmission, rendering the expiry of t-PollRetransmit ineffective as the poll cannot be transmitted regardless.

[RLC-11] RLC stops and resets t-PollRetransmit when all RLC SDUs with SNs, ranging from TX_Next_Ack to POLL_SN, either have been ACKed/NACKed via STATUS report or their (re-)transmission has been stopped (due to discard indications from PDCP).

## Relationships between Tx stop-retransmission and Rx Discard Detection

TS 38.306 [3] txRLC-StopReTxDiscardedSDU-r19 Indicates whether the UE supports stopping RLC transmission and retransmission of discarded SDUs at the transmitting side of an AM RLC entity, as specified in TS 38.322 [36]. rxRLC-Discard-r19 Indicates whether the UE supports detecting of discard of AMD PDU(s) based on timer t-RxDiscard at the receiving side of an AM RLC entity, as specified in TS 38.322 [36].

RAN2#130 meeting discussed whether a UE that supports stopping RLC (re-)transmissions of discarded SDUs at the Tx side should also necessarily indicate its support for detecting the discard of AMD PDU(s) based on the t-RxDiscard at the Rx side. Some companies raised concerns regarding potential window stalling issues if the Tx side of the UE supports stopping RLC (re-)transmissions, but the UE does not support discard detection on the Rx side. However, we consider this window stalling issue is not valid. First, it’s important to note that UE capabilities are indicated to reflect the UE’s own support. The window stalling issue could only arise in the following two scenarios:

- UL: UE supports txRLC-StopReTxDiscardedSDU, but NW does not support rxRLC-Discard.

- DL: NW supports txRLC-StopReTxDiscardedSDU, but UE does not support rxRLC-Discard.

In essence, the critical point is whether the network can correctly configure or activate both functionalities. For example:

- UL: If txRLC-StopReTxDiscardedSDU is configured for the UE, t-RxDiscard should also be activated on the network side.

- DL: t-RxDiscard should be configured for the UE if txRLC-StopReTxDiscardedSDU is activated on the network side.

Appropriate network implementation can ensure the alignment of configuration and activation for the limitations/associations between stopReTxDiscardedSDU and t-RxDiscard for both UL and DL.

(UE capability-04) UE capability of rxRLC-Discard does not need to be pre-requisite of txRLC-StopReTxDiscardedSDU.

[RLC-10] The associations between stopReTxDiscardedSDU and t-RxDiscard, considering both UL and DL, are ensured by NW implementation.

# Conclusion

Based on the analysis above, we have made the following proposals:

Proposal 1	For remaining-time-based RLC polling, Tx side of RLC includes a delay-critical poll in the AMD PDU.

Proposal 2	When Rx side of RLC receives a delay-critical poll in the AMD PDU, triggers a delay-critical STATUS reporting.

Proposal 3	The delay-critical STATUS reporting is restricted by a separate t-StatusProhibit with a shorter value than the legacy t-StatusProhibit.

Proposal 4	When t-RxDiscard expires, Rx side of RLC triggers a delay-critical STATUS reporting.

Proposal 5	[RLC-11] RLC stops and resets t-PollRetransmit when all RLC SDUs with SNs, ranging from TX_Next_Ack to POLL_SN, either have been ACKed/NACKed via STATUS report or their (re-)transmission has been stopped (due to discard indications from PDCP).

Proposal 6	(UE capability-04) UE capability of rxRLC-Discard does not need to be pre-requisite of txRLC-StopReTxDiscardedSDU.

Proposal 7	[RLC-10] The associations between stopReTxDiscardedSDU and t-RxDiscard, considering both UL and DL, are ensured by NW implementation.

# Reference

[POST130][507][XR] PDCP running CR and open issues (LGE)

[POST130][508][XR] RLC running CR and open issues (vivo)

[POST130][509][XR] UE capabilities CRs (Xiaomi)