3GPP TSG-RAN WG2 Meeting #131											R2-2505271

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.7.5

Source: 	Sharp

Title:  	Leftover issues on RLC enhancements

Document for: 	Discussion

# Introduction

In RAN2#130 meeting, RAN2 made the following agreements:

(RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this (RLC-9) No additional conditions are needed for the polling enhancement. RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT. (PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP SDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed. Autonomous retransmission and polling is triggered only based on discardTimer, i.e. not based on discardTimerForLowImprotance The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity. (RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission. Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report. Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

This document discusses leftover open issues on RLC enhancements for XR.

# Discussion

## (RLC-10) Configuration of stopRxTxObsoleteSDU in Uplink and t-RxDiscard in Downlink

Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard.

The RLC specification is written from the UE perspective. The specification does not mandate any network behaviour, so the network behaviour is left up to each network implementation. Rel-19 XR WI introduced unnecessary retransmission avoidance mechanisms with stopRxTxObsoleteSDU in uplink and t-RxDiscard in downlink. All those mechanisms are for the UE side. Thus, it is clear that for the same uplink data stream or downlink data stream, either TX mechanism or RX mechanism can be configured in each direction. Thus, there is no actual relationship from the radio bearer point of view. As RAN2 already agreed separate UE capabilities bits, i.e. txRLC-StopReTxDiscardedSDU and of rxRLC-Discard-r19, no configuration restriction is necessary.

(RLC-10) Configurations of stopReTxObsoleteSDU for UL and t-RxDiscard for DL are independent.

## (UE Capability-4) Capability Restriction of txRLC-StopReTxDiscardedSDU and of rxRLC-Discard-r19

Whether a UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19.

It may be true that supporting stopRxTxObsoleteSDU at the UE requires a t-RxDiscard-like mechanism at the gNB in uplink transmission. However, whether to implement a t-RxDiscard-like mechanism is purely up to the network implementation. t-RxDiscard is purely a downlink operation. There is no relationship with the uplink operation. Thus, we do not see any necessity of capability restriction of txRLC-StopReTxDiscardedSDU and of rxRLC-Discard-r19. Those can be independent with each other.

(UE Capability-4) Capabilities of txRLC-StopReTxDiscardedSDU and of rxRLC-Discard-r19 are independent.

## (RLC-11) Polling When There Are Only Stopped SDUs in the Buffer

FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”

The above open issue (RLC-11) was discussed in last meeting and following working assumption was made:

Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

The working assumption is based on the understanding that window stalling will not happen at expiry of t-PollRetransmit. But this is not the case. The situation is very similar to legacy. In legacy, window stalling may happen at expiry of t-PollRetransmit and a poll always will be transmitted even though t-Reassembly which used by Rx side to trigger a STATUS Report was defined. Receiving UE starts t-Reassembly only if a SN gap is detected. So there is a case that window stalling happens in the transmitting UE but no SN gap is detected by the receiver, i.e. gNB. For example, all the RLC PDUs with SN in the transmission window are received except the last one with the largest SN. In this case, no SN gap is detected in receiving UE and t-Reassembly does not start. that the consequence is that no STATUS Report can be sent by the receiving UE, so window stalling happens.

The difference between here and legacy is that another timer t-RxDiscard is defined which is also used by Rx side to trigger a STATUS Report. Actually, there is no big difference in triggering of STATUS Report between t-Reassembly and t-RxDiscard. Both of them are started based on detecting of a SN gap in the Rx side. If SN gap is not detected, UE starts neither of them. So, no STATUS Report can be sent by the receiving UE, window stalling happens as in legacy.

In addition, not sending a STATUS Report will delay the SN gap detection in the receiving UE. For example, after sending PDU with SN=0, 1, 2, 3, 4, 5, transmission buffer and retransmission buffer become empty, but only SN=0, 1, 2, 3 are received successfully by receiving UE. No STATUS Report can be sent for that no SN gap is detected by receiving UE. RLC SDU with SN=0, 1, 2, 3, 4, 5 are all indicated to be discard. After a long time, PDU with SN=6 is arrived and transmitted, SN gap is detected by receiving UE. Then receiving UE starts t-Reassembly and t-RxDiscard. A STATUS Report will not transmitted until the t-Reassembly expires. The Transmission window cannot be moved until the STATUS Report is received. The buffered RLC SDUs with SNs fall outside the transmission window cannot be transmitted.

(RLC-11) To avoid window stalling, a poll is always transmitted at the expiry of t-PollRetransmit for the following two cases:

- In both the transmission buffer and the retransmission buffer, the transmission and retransmission of all the RLC SDUs or RLC SDU segments are stopped.

- In the retransmission buffer, the transmission and retransmission of all the RLC SDUs or RLC SDU segments are stopped when no new RLC SDU or RLC SDU segment can be transmitted.

When there is no other RLC SDUs or RLC SDU segments whose transmission and retransmission are not stopped, we need to allow retransmission of a stopped SDU:

(RLC-11) When there is no other RLC SDUs or RLC SDU segments whose transmission and retransmission are not stopped, one of the following RLC SDUs are considered for retransmission:

- The RLC SDU which has been stopped for transmission or retransmission and with the highest SN among the RLC SDUs submitted to lower layer; or

- Any RLC SDU which has been stopped for transmission or retransmission and which has not been positively acknowledged.

## (RLC-12) PDCP SN Gap Reporting During Mobility

FFS how to handle PDCP SN gap report handling during UE mobility

This issue was triggered by a RAN3 LS [1] and discussed as a Rel-18 correction. The Rel-18 conclusion was that RAN2 did not see a big problem. Instead, another note was captured that Rel-19 XR WI can discuss this problem whether it is a critical problem on top of Rel-19 features. The relevant agreements in RAN2#129 are as follows:

From RAN2 point of view, for Rel-18 alternative 3 should not be supported. RAN2 doesn’t see a big problem to solve for Rel-18. RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements. If it is a problem further discussions are needed on the alternatives.

The problematic scenario is inter-gNB handover where the target gNB may not be aware of discarded packets reported to the source gNB before. The PDCP SN gap report is to let the PDCP receiving entity at the gNB moves the PDCP reception window. However, the PDCP reception window can move upon expiry of t-reordering timer even without PDCP SN gap report. That is why RAN2 agreed not to introduce anything in Rel-18.

The next question should be whether the problem becomes more critical in Rel-19. PDCP SN gap may be created when PSI-based discard is configured and the PDCP discard timer expires. Rel-19 XR WI does not have any mobility-related features. In short, the scenario is the same in Rel-19.

(RLC-12) No additional PDCP SN gap reporting during mobility is introduced.

## (PDCP-2) PDCP SN Gap Reporting for Segments

Whether the PDCP triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have been submitted by any RLC entity to lower layers.

According to their transmission status, the discarded PDCP SDUs can be divided into the following types:

- Type 1: PDCP SDUs that have not be submitted to RLC.

- Type 2: PDCP SDUs that have been submitted to RLC and have not been submitted by any RLC entity to lower layers.

- Type 3: PDCP SDUs that have been partially submitted by RLC to lower layers.

- Type 4: PDCP SDUs that have been submitted by RLC to lower layers and they or segments thereof are NACKed but retransmission has not been performed by RLC.

- Type 5: PDCP SDUs that have been submitted by RLC to lower layers but they or segments thereof have not been ACKed.

In Rel-18, a PDCP SN gap report is triggered only for the first two types of PDCP SDU (i.e., Type 1 and Type 2) that have not been submitted by any RLC entity to lower layers. This condition is related to RLC SN assignment as Rel-18 RLC AM always guarantees 100% successful delivery for RLC SDU that has been submitted to lower layer even if indicated to be discarded by PDCP entity. So, in Rel-18, a PDCP SN gap report is triggered for the discarded PDCP SDUs that the transmitting PDCP entity can confirm that they will not be successfully received by the receiver.

The Rel-19 WI feature should apply the same principle that a PDCP SN gap report is triggered for these PDCP SDUs that cannot be received successfully by the receiver. In Rel-19, with the configuration of stopRxTxObsoleteSDU, the last three types of PDCP SDUs will not be transmitted completely even if they or segments thereof have been submitted to MAC layer. For Type 3 and Type 4 PDCP SDUs, transmitting PDCP entity can confirm that they cannot be received successfully by the receiver. So, the PDCP SN gap report should also be triggered for Type 3 and Type 4 PDCP SDUs.

(PDCP-2) If stopRxTxObsoleteSDU is configured, PDCP SN gap report is triggered for the PDCP SDUs that cannot be received successfully:

- - PDCP SDUs that have been partially submitted by RLC to lower layers.

- - PDCP SDUs that have been submitted by RLC to lower layers and they or segments thereof are NACKed but retransmission has not been performed by RLC.

## (RLC-14) Submitted SDU Discard upon Discard Indication

If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU, whether the transmitting side of an AM RLC entity shall discard the corresponding RLC SDU or RLC SDU segment(s) if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers.

The current SDU procedure specifies that the discard indication triggers an SDU discard only if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The reason was that the discard indication does not allow the case that sequence number is used for multiple RLC SDUs.

In UE implementation, there are other cases that the transmitting RLC entity performs an SDU discard, e.g. memory overload or receiving positive acknowledgement. However, those cases are not specified in the RLC specification and whether or when to discard is left up to the UE implementation. If we keep the same principle, the UE implementation can decide the SDU discard regardless of stopReTxDiscardSDU.

(RLC-14) RLC SDU discard procedure is not impacted by stopReTxDiscardSDU.

## Discard Indication for Pending Considered Retransmission

During the RLC CR discussion, some companies raised the issue that a pending considered retransmission is performed regardless of the discard indication and the corresponding procedure captured in the CR [2]. Figure 1 describes the problematic scenario. The example assumes that the transmitter receives a negative ACK for SN=8, which triggers to consider it for retransmission as follows:

[Consideration of retransmission by a negative ACK] When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer; and - if stopReTxDiscardedSDU is configured and no discard indication for the RLC SDU has been received from upper layers, or if stopReTxDiscardedSDU is not configured: - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

The considered retransmission is pending. Later a discard indication of SN=8 is delivered from the PDCP entity. Then, the transmitter shall not consider SN=8 for retransmission as follows:

[No consideration of retransmission by a discard indication] If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment(s) for transmission or retransmission.

However, it is not clear whether already considered pending retransmission is also cancelled. The current CR may mislead that the pending retransmission cannot be cancelled by the discard indication as shown in Figure 1. However, this unnecessary retransmission should not be performed, and this is not a desirable UE behaviour. Thus, the CR needs a clarification.

The RLC CR clarifies that a discard indication shall cancel pending considerations for retransmission.

The corresponding TP can be as follows:

[No consideration of retransmission by a discard indication] If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment(s) which have been already considered or pending for transmission or retransmission.

Figure 1. Pending retransmission not affected by discard indication

# Conclusions

Based on the discussion given above, RAN2 is requested to agree the following proposals:

- (RLC-10) Configurations of stopReTxObsoleteSDU for UL and t-RxDiscard for DL are independent.

(UE Capability-4) Capabilities of txRLC-StopReTxDiscardedSDU and of rxRLC-Discard-r19 are independent.

(RLC-11) To avoid window stalling, a poll is always transmitted at the expiry of t-PollRetransmit for the following two cases:

- In both the transmission buffer and the retransmission buffer, the transmission and retransmission of all the RLC SDUs or RLC SDU segments are stopped.

- In the retransmission buffer, the transmission and retransmission of all the RLC SDUs or RLC SDU segments are stopped when no new RLC SDU or RLC SDU segment can be transmitted.

(RLC-11) When there is no other RLC SDUs or RLC SDU segments whose transmission and retransmission are not stopped, one of the following RLC SDUs are considered for retransmission:

- The RLC SDU which has been stopped for transmission or retransmission and with the highest SN among the RLC SDUs submitted to lower layer; or

- Any RLC SDU which has been stopped for transmission or retransmission and which has not been positively acknowledged.

(RLC-12) No additional PDCP SN gap reporting during mobility is introduced.

(PDCP-2) If stopRxTxObsoleteSDU is configured, PDCP SN gap report is triggered for the PDCP SDUs that cannot be received successfully:

- - PDCP SDUs that have been partially submitted by RLC to lower layers.

- - PDCP SDUs that have been submitted by RLC to lower layers and they or segments thereof are NACKed but retransmission has not been performed by RLC.

(RLC-14) RLC SDU discard procedure is not impacted by stopReTxDiscardSDU.

The RLC CR clarifies that a discard indication shall cancel pending considerations for retransmission.

# Reference

[1] R2-2500023	LS on PDCP SN gap report handling during UE mobility		RAN3

[2] R2-2505402	Introduction of R19 XR enhancements for RLC spec.	vivo