3GPP TSG-RAN2#131										                             R2-2505287

Bangalore, India  Aug 25th – 29th , 2025

Source: 			ZTE Corporation, Sanechips

Title: 	                  Consideration on IoT- NTN TDD mode

Agenda item:	      8.17

Document for:       Discussion and Decision

- Introduction

In RAN#106, the revised WID on introduction of IoT-NTN TDD mode[1] has been agreed to support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period[2]. Most  agreements have been made for supporting the IoT-NTN TDD mode.

In this contribution, we will further discuss the remaining issues for IoT-NTN TDD mode related RAN2 impacts and give our proposals.

- Discussion

# Issue 1: About the type of UE capability

In the TS 36.306 running CR[2], there is the following FFS to be determined:

Editors’s Note: FFS whether the capability is a Conditionally Mandatory feature under section 7 or an Optional feature without UE radio access capability under section 6.

And rapporteur made the following proposal:

Proposal 1: On the general IoT NTN TDD mode capability, RAN2 to decide on type of capability: - Conditionally mandatory (as in draft CR) - Optional capability without signalling

If the IoT NTN TDD UE only supports the IoT NTN TDD access technique, both option can work, and the effect is the same. But considering that in TS 36.102, band n249 shall only used for IoT NTN TDD, if the UE supports both IoT NTN TDD and other RAT(e.g. NB-IoT TN, NB-IoT NTN etc), the conditionally mandatory method (as in the TS36.306 running CR) is beneficial for network to know the UE’s IoT NTN TDD capability when it access the other RAT(e.g. NB-IoT TN, NB-IoT NTN etc), and the network can redirect the UE to IoT NTN TDD carrier, e.g. the load of the other RAT is high.

Observation 1: The conditionally mandatory method (as in the TS36.306 running CR) is more flexible and more future proof.

Proposal 1: Define the type of UE capability as conditionally mandatory method (i.e. as in the TS36.306 running CR).

Issue 2: About SC-PTM impact

In NB-IoT SC-PTM, the repetition numbers(e.g. npdcch-NumRepetitions-SC-MCCH, npdcch-NumRepetitions-SC-MTCH etc) are defined. Considering that SC-PTM are transmitted on common channel, e.g. SC-MCCH and SC-MTCH, similar as the SI message transmission, one or more repetitions of SC-PTM message transmission falling on the non-D subframes are postponed to the next valid D subframe within the transmission Period (e.g. defined by sc-mcch-RepetitionPeriod etc).

Proposal 2: The repetitions of SC-PTM message transmission falling on the non-D subframes are postponed to the next valid D subframe within the transmission Period.

- Issue 3: About IoT NTN TDD definition

In the IoT NTN TDD WID[2], it is described that”The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, ...”. In the current specification, lots of procedures and/or features are defined for NB-IoT, NB-IoT FDD and/or NB-IoT over NTN, which are captured in the specification by “For NB-IoT,...”, “For NB-IoT,..., for FDD...”, “For non-terrestrial networks...” and so on, these procedures and/or features also apply to IoT NTN TDD unless specifically stated otherwise.

Observation 2: In the current specification, lots of procedures and/or features defined for NB-IoT, NB-IoT FDD and/or NB-IoT in NTN can be applied to IoT NTN TDD unless specifically stated otherwise.

In TS 36.321 running CR[3], it is specified that “ IoT-NTN TDD mode applies to NB-IoT unless specified otherwise. IoT-NTN TDD mode does not apply to TDD or TDD mode unless specified otherwise. IoT-NTN TDD mode does not apply to TDD or TDD mode unless specified otherwise.”, we think that similar clarification is also necessary in TS 36.331 and 36.304. Since only FDD is supported in NB-IoT NTN, we can simply say that ”All the NB-IoT NTN features and/or procedures applies to IoT NTN TDD unless specified otherwise”. This can be captured in stage 2 specification or in each stage 3 specification.

Proposal 3: Captured in the stage 2 specification or in each state 3 specification that “All the features and/or procedures applicable to NB-IoT in NTN apply to IoT NTN TDD unless specified otherwise”.

Based on the proposal 2 and proposal 3, the related TPs for TS 38.331[4] and TS 38.300[5] are provided in Appendix for reference.

- Conclusion

The following observations and proposals are made:

Observation 1: The conditionally mandatory method (as in the TS36.306 running CR) is more flexible and more future proof.

Observation 2: In the current specification, lots of procedures and/or features defined for NB-IoT, NB-IoT FDD and/or NB-IoT in NTN can be applied to IoT NTN TDD unless specifically stated otherwise.

Proposal 1: Define the type of UE capability as conditionally mandatory method (i.e. as in the TS36.306 running CR).

Proposal 2: The repetitions of SC-PTM message transmission falling on the non-D subframes are postponed to the next valid D subframe within the transmission Period.

Proposal 3: Captured in the stage 2 specification or in each state 3 specification that “All the features and/or procedures applicable to NB-IoT NTN apply to IoT NTN TDD unless specified otherwise”.

Based on the proposal 2 and proposal 3, the related TPs for TS 38.331[4] and TS 38.300[5] are provided in Appendix for reference.

- References

- RP-243293 Revised WID on introduction of IoT-NTN TDD mode, RAN#106

- R2-250xxxx Introduction of capabilities for IoT NTN TDD_v05_rapp

- Draft CR IoT-NTN TDD mode 36321-i40_v04_Rapporteur

- TS 36.331, V18.6.0 (2025-06), Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification(Release 18)

- R2-25xxxxx CR 36300 IoT NTN TDD_v6_Rapp

- Appendix

Text Proposal for TS 38.331[4] based on Proposal 2

## 5.8a SC-PTM ### 5.8a.1 Introduction #### 5.8a.1.1 General SC-PTM control information is provided on a specific logical channel: the SC-MCCH. The SC-MCCH carries the SCPTMConfiguration message which indicates the MBMS sessions that are ongoing as well as the (corresponding) information on when each session may be scheduled, i.e. scheduling period, scheduling window and start offset. The SCPTMConfiguration message also provides information about the neighbour cells transmitting the MBMS sessions which are ongoing on the current cell. In this release of the specification, an SC-PTM capable UE is only required to support reception of a single MBMS service at a time, and reception of more than one MBMS service in parallel is left for UE implementation. A limited amount of SC-PTM control information is provided on the BCCH or BR-BCCH. This primarily concerns the information needed to acquire the SC-MCCH. NOTE: For BL UEs and UEs in CE, SC-MCCH transmission uses a 1.4 MHz channel bandwidth and a maximum TBS of 936 bits, see TS 36.213 [23]. For NB-IoT UEs, the maximum TBS for SC-MCCH transmission is 680 bits, see TS 36.213 [23]. #### 5.8a.1.2 SC-MCCH scheduling The SC-MCCH information (i.e. information transmitted in messages sent over SC-MCCH) is transmitted periodically, using a configurable repetition period. SC-MCCH transmissions (and the associated radio resources and MCS) are indicated on PDCCH. For IoT-NTN TDD mode, one or more repetitions of SC-MCCH transmissions falling on the non-D subframes are postponed to the next valid D subframe within the configurable repetition period. #### 5.8a.1.3 SC-MCCH information validity and notification of changes Change of SC-MCCH information only occurs at specific radio frames, i.e. the concept of a modification period is used. Within a modification period, the same SC-MCCH information may be transmitted a number of times, as defined by its scheduling (which is based on a repetition period). The modification period boundaries are defined by SFN values for which SFN mod m= 0, where m is the number of radio frames comprising the modification period. The modification period is configured by means of SystemInformationBlockType20 (SystemInformationBlockType20-NB in NB-IoT). If H-SFN is provided in SystemInformationBlockType1-BR, modification period boundaries for BL UEs or UEs in CE are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0. The modification period boundaries for NB-IoT UEs are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0. When the network changes (some of) the SC-MCCH information, it notifies the UEs, other than BL UEs, UEs in CE or NB-IoT UEs, about the change in the first subframe which can be used for SC-MCCH transmission in a repetition period. LSB bit in 8-bit bitmap when set to '1' indicates the change in SC-MCCH. Upon receiving a change notification, a UE interested to receive MBMS services transmitted using SC-PTM acquires the new SC-MCCH information starting from the same subframe. The UE applies the previously acquired SC-MCCH information until the UE acquires the new SC-MCCH information. When the network changes (some of) the SC-MCCH information for start of new MBMS service(s) transmitted using SC-PTM, it notifies BL UEs, UEs in CE or NB-IoT UEs about the change in every PDCCH which schedules the first SC-MCCH in a repetition period in the current modification period. The notification is transmitted with 1 bit. The bit, when set to '1', indicates the start of new MBMS service(s), see TS 36.212 [22], clauses 5.3.3.1.14 and 6.4.3.3. Upon receiving a change notification, a BL UE, UE in CE or NB-IoT UE interested to receive MBMS services transmitted using SC-PTM acquires the new SC-MCCH information scheduled by the PDCCH. The BL UE, UE in CE or NB-IoT UE applies the previously acquired SC-MCCH information until the BL UE, UE in CE or NB-IoT UE acquires the new SC-MCCH information. When the network changes SC-MTCH specific information e.g. start of new MBMS service(s) transmitted using SC-PTM or change of ongoing MBMS service(s) transmitted using SC-PTM, it notifies the BL UEs, UEs in CE or NB-IoT UEs in the PDCCH which schedules the SC-MTCH in the current modification period. The notification is transmitted with a 2 bit bitmap. The LSB in the 2-bit bitmap, when set to '1', indicates the change of the on-going MBMS service and the MSB in the 2-bit bitmap, when set to '1', indicates the start of new MBMS service(s), see TS 36.212 [22], clauses 5.3.3.1.12, 5.3.3.1.13 and 6.4.3.2. In the case the network changes an on-going SC-MTCH transmission in the next modification period, it notifies the BL UEs, UEs in CE or NB-IoT UEs in the PDCCH which schedules this SC-MTCH in the current modification period. In the case the network starts new MBMS service(s) transmitted using SC-PTM, the network notifies the UEs which have on-going SC-MTCH in the PDCCH scheduling each of the SC-MTCH. Upon receiving such notification, a BL UE, UE in CE or NB-IoT UE acquires the new SC-MCCH information at the start of the next modification period. The BL UE, UE in CE or NB-IoT UE applies the previously acquired SC-MCCH information until the BL UE, UE in CE or NB-IoT UE acquires the new SC-MCCH information. For IoT-NTN TDD mode, one or more repetitions of SC-MTCH transmissions falling on the non-D subframes are postponed to the next valid D subframe within the modification period. ...

Text Proposal for TS 38.300[5] based on Proposal 3

## 23.21 Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks ## 23.21.1 General Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks (see clause 4.12) is only applicable to E-UTRA connected to EPC. UEs not supporting NTN are barred from accessing an NTN cell. In NTN, only BL UEs, UEs in enhanced coverage and NB-IoT UEs with GNSS capability are supported in this release of the specification. To accommodate long propagation delays in NTN, increased timer values and window sizes, or delayed starting times are supported for the physical layer and for higher layers. UL segmented transmission is supported for UL transmission with repetitions. The UE shall apply UE pre-compensation per segment of UL transmission of PUSCH/PUCCH/PRACH for BL UEs or UEs in enhanced coverage and NPUSCH/NPRACH for NB-IoT UEs from one segment to the next segment. In this release of the specification, NTN system only supoorts FDD mode and IoT NTN TDD mode. In NTN, the distance refers to Euclidean distance. In this release of the specification, all the features and/or procedures applicable to NB-IoT in NTN apply to IoT NTN TDD unless specified otherwise.