3GPP TSG RAN WG2 #131	R2-2505302

Bengaluru, India, Aug 25th–29th, 2025

Agenda Item:	8.1.2.3

Source: 	Xiaomi

Title:  	Discussion on AI positioning case 1

Document for:	Discussion and Decision

# Introduction

After RAN2#130 meetings, following post email discussions were used to discuss the running CRs and open issues for AI/ML positioning:

[POST130][023][AI PHY] 38.305 CR (CATT)

[POST130][025][AI PHY] 37.355 CR (Qualcomm)

In this contribution, we mainly focus on how to solve the remaining open issues for AI/ML positioning case 1.

# Discussion

LPP-6a: Number of PRUs in NR-PRU-DL-Info

During RAN2 #130 meeting, NR-PRU-DL-Info is agreed to be applicable to NR AI/ML positioning use case, while whether more PRUs are needed is FFS:

[LPP-6] The IE NR-PRU-DL-Info is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3 can be removed.   FFS if more PRUs are needed to be included for training purposes

In our understanding, whether multiple PRUs (compared to single PRU) is needed depends on whether there’s additional performance gain when comparing to single PRU assistance data as training input. During Rel-18 and Rel-19, there’s no study or simulation from RAN1 to compare whether assistance information from multiple PRUs could be helpful or not.

Observation 1: (LPP-6a) There’s no study in RAN1 on whether NR-PRU-DL-Info from multiple PRUs can provide additional gain compared to NR-PRU-DL-Info from single PRU.

On the other hand, even multiple PRUs can be helpful, no matter LMF use single ProvideAssistanceData message to carry multiple PRU DL information, or use multiple ProvideAssistanceData messages to carry multiple PRUs’ DL information separately, UE needs to differentiate assistance data from different PRUs. This further needs LMF provides PRU IDs to the target UE. Even though PRU is a special type of UE with known location, same as normal UE, it is still not suitable to share PRU ID to other target UEs. Without knowing which PRU provides the associated measurements and information, it is difficult for UE to leverage such information, especially when PRU is moving.

Proposal 1: (LPP-6a) NR-PRU-DL-Info from multiple PRUs is not supported.

LPP-20: LPP impacts related to case 3a/3b

Following RAN1 agreements were reached for case 3a/3b data collection:

Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed

RAN1 further agreed that SRS measurements are used for Case 3a and Case 3b. Therefore, in our understanding, Part A measurement information can reuse the existing SRS configuration and measurement reporting, without further impact to UE.

Observation 2: (LPP-20) Part A information is generated by gNB/TRP based on existing SRS configuration and reporting.

Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications Agreement For AI/ML positioning Case 3b, for gNB channel measurements reported to LMF, regarding the power information (if included), - reuse the existing measurement report mapping table for SRS-RSRPP in 38.133.

As for Part B information, for Case 3a (gNB-based model) and Case 3b (LMF-based model), it is possible that Part B information can be obtained from either LMF or UE (only LMF for Case 3a). When Part B information needs to be transferred from UE to LMF, the existing ProvideLocationInformation procedure of other positioning methods can be reused.

Observation 3: (LPP-20) Part B information is either available at LMF or reported by UE to LMF via existing ProvideLocationInformation procedure.

Based on above two observations, there’s no additional LPP impacts related to Case 3a and Case 3b.

Proposal 2: (LPP-20) Case 3a and Case 3b can be supported without new impact to LPP.

LPP-21: "Associated ID" for TRP Location Coordinates (IE TRP-ImplicitLocationInfo-r19)

Following questions were summarized as part of issue LPP-21:

- Must the "Associated ID" for the TRP coordinates be associated with a "cell" (e.g., NCGI)? If so, how should PRS-only TPs (which are not associated with a cell) being handled? And how to implement RAN1 agreement that UE does not expect to receive different values of associated ID for TRPs belonging to the same cell? - Is the "Associated ID" only for the TRP location coordinates, or also for the associated DL-PRS Resource Set/DL-PRS Resource ARPs? - Is the value range of 0..255 for the "Associated ID" sufficient? In particular if the "Associated ID" should be "configured per cell"? - Are the explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) mutually exclusive, or can a deployment provide both? - If a UE supports explicit TRP location info (NR-TRP-LocationInfo-r16), must a UE support also implicit location info (nr-TRP-ImplicitLocationInfo-r19) (and vice versa)?

Following agreements on associated ID are reached during RAN1 #121 meeting:

Agreement For AI/ML based positioning Case 1, regarding Info #7 in the assistance information from legacy UE-based DL-TDOA, - If implicitly provided, the implicit indication of Info #7 is via associated ID. - For given TRP(s), same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. - The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). - an associated ID is configured per-cell (e.g., NCGI-r15) - UE does not expect to receive different values of associated ID for TRPs belonging to the same NCGI-r15 - Associated ID can be realized by an identifier of N bits (e.g., 8 bits)

In our understanding, during RAN1 discussion, PRS-only TP was not in the scope of evaluation and discussion. The associated ID is proposed to implicitly indicate geographical coordinates of the TRP, instead of NCGI. Additionally, it was agreed by RAN1 that the associated ID is configured per-cell. Considering RAN1 has concluded their WI, we suggest to consider PRS-only TP out of scope of this release.

Proposal 3: (LPP-21) RAN2 confirms associated ID is only associated to the geographical coordinates that can be considered consistent by the UE. PRS-only TP is not supported in Rel-19 AI/ML based positioning.

Regarding the explicit TRP location info and associated ID as implicit location information, following agreements were reached by RAN1:

Working Assumption For AI/ML based positioning Case 1, regarding info #7 in the assistance information from legacy UE-based DL-TDOA, it can be provided as in legacy UE-based DL-TDOA or implicitly. Agreement Above Working Assumption is confirmed.

In our understanding, to make sure the consistent between training and inference, either explicit TRP location info or associated ID should be indicated to the UE. Since RAN1 further agreed associated ID is cell-specific, it is up to network whether both assistance information or only signal one of them.

Proposal 4: (LPP-21) LMF needs to provide either explicit TRP location info or implicit associated ID for consistency between training and inference. It is up to network whether to provide both.

# Conclusions

In this contribution, we discussed LCM for AI positioning case 1 and provide the following observations and proposals:

Observation 1: (LPP-6a) There’s no study in RAN1 on whether NR-PRU-DL-Info from multiple PRUs can provide additional gain compared to NR-PRU-DL-Info from single PRU.

Proposal 1: (LPP-6a) NR-PRU-DL-Info from multiple PRUs is not supported.

Observation 2: (LPP-20) Part A information is generated by gNB/TRP based on existing SRS configuration and reporting.

Observation 3: (LPP-20) Part B information is either available at LMF or reported by UE to LMF via existing ProvideLocationInformation procedure.

Proposal 2: (LPP-20) Case 3a and Case 3b can be supported without new impact to LPP.

Proposal 3: (LPP-21) RAN2 confirms associated ID is only associated to the geographical coordinates that can be considered consistent by the UE. PRS-only TP is not supported in Rel-19 AI/ML based positioning.

Proposal 4: (LPP-21) LMF needs to provide either explicit TRP location info or implicit associated ID for consistency between training and inference. It is up to network whether to provide both.

# References

- TS 38.305,