3GPP TSG-RAN WG2 Meeting #131	R2-2505339

Bangalore, India  Aug 25th – 29th , 2025

Agenda Item:	8.5.3

Source:	vivo

Title:	Remaining issues of on-demand SIB1

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the remaining issues of on-demand SIB1.

Discussion

UE&NW behavior after reception of SI change notification

Background

According to the legacy UE behavior, if the UE receives SI change notification, the UE will receives SIB1 according to the Coreset#0 and Searchspace#0 parameters provided in MIB of the CD-SSB [1]. As for an OD-SIB1 cell, RAN2 has previously agreed that SIB1 update in OD-SIB1 cell also relies on SI change notification [2][3]:

RAN2#126 (202405) SIB1 acquisition upon SIB change notification in NES cell Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell. RAN2#127 (202408) OD-SIB1 Validity: 1.Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

However, according to the running CR to TS 38.331 [4], if the OD-SIB1 UE receives short message notifying SI change in the OD-SIB1 cell, the UE will be requesting OD-SIB1 because the kssb is indicating SIB1 is not scheduled in the cell, rather than monitoring SIB1 according to the OD-SIB1 request configuration, which is against with the previous RAN2 agreement:

5.2.2.2.2 SI change indication and PWS notification If the UE receives a Short Message, the UE shall: <omit irrelavant text> 1> if the UE does not operate an IDLE eDRX cycle longer than the modification period and the systemInfoModification bit of Short Message is set: 2> apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next modification period; 1> if the UE operates an IDLE eDRX cycle longer than the modification period and the systemInfoModification-eDRX bit of Short Message is set: 2> apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next eDRX acquisition period boundary. 5.2.2.3 Acquisition of System Information 5.2.2.3.1 Acquisition of MIB and SIB1 1> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 1> if the UE is in RRC_CONNECTED while T311 is running: (Note: legacy UE behavior when it receives SI change notification in a normal cell) 2> if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE: 3> acquire the SIB1, which is scheduled as specified in TS 38.213 [13]; 3> if the UE is unable to acquire the SIB1: 4> perform the actions as specified in clause 5.2.2.5; 3> else: 4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2. (Note: OD-SIB1 UE behavior when it receives SI change notification in the OD-SIB1 cell) 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> if the UE has a stored valid version of od-SIB1-Config for this cell: 4> perform the actions as specified in clause 5.2.2.3.3x; 3> else: 4> perform the actions as specified in clause 5.2.2.5. NOTE x: It is up to UE’s implementation to check if SIB1 is currently being broadcasted for that cell before triggering OD-SIB1 request procedure of that cell.

During last meeting, at offline session [AT130][104][NES] (Ericsson) [5],  P6 in R2-2504037 [6] about the UE behavior after receiving SI change notification was discussed. Rapporteur responded that a condition was added to address the issue at that time:

5> if SIB1 is not broadcasted 6> perform the actions as specified in clause 5.2.2.3.3x;

And therefore, at the Comeback session for R19 NES, it was first agreed that [7]:

Keep RRC CR as it is for the short message and UE behaviour.

Then, it is commented by Samsung [7] at the Comeback session that ‘5> if SIB1 is not broadcasted’ mandates the UEs to check whether SIB1 is broadcasting before requesting OD-SIB1 (not the case upon receiving SI change notification), which is against the RAN1 agreement that it is up to the UE’s implementation to check if SIB1 is broadcasting. Thus, ‘5> if SIB1 is not broadcasted’ was removed, and instead the NOTE x ‘It is up to UE’s implementation to check if SIB1 is currently being broadcasted for that cell before triggering OD-SIB1 request procedure of that cell.’ is added in the running CR.

As a result, the issue raised by P6 in R2-2504037 [6] about the UE behavior after receiving SI change notification becomes open again and is confirmed by the Rapporteur. Thus, we propose RAN2 to review the agreement ‘Keep RRC CR as it is for the short message and UE behaviour.’ at last meeting.

Observation 1: With the latest RRC running CR which removes the condition ‘5> if SIB1 is not broadcasted’ before performing OD-SIB1 request according to the RAN2 agreement at last meeting, the UE will perform OD-SIB1 request upon reception of SI change notification if the NES cell sends NCD-SSB, which is not aligned with the agreement ‘Keep RRC CR as it is for the short message and UE behaviour.’

UE&NW behavior

RAN2#126 [2] agreed that:

SIB1 acquisition upon SIB change notification in NES cell Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

However, there are several understandings of the UE&NW behavior among the companies so far during offline discussion regarding this agreement:

- Understanding 1: The UE assumes the NES cell will always transmits CD-SSB after sending SI change notification, the UE acquires SIB1 with the Coreset#0 and Searchspace#0 parameters in MIB as legacy;

- Understanding 2: It is up to the NES cell to transmits CD-SSB or NCD-SSB after sending SI change notification. If the NES cell transmits CD-SSB, the UE acquires SIB1 with the Coreset#0 and Searchspace#0 parameters in MIB as legacy; and if the NES cell transmits NCD-SSB,

- 2a: the UE assumes the Coreset#0 and Searchspace#0 parameters in the stored OD-SIB1 configuration are still valid, and acquires the updated SIB1 with the stored Coreset#0 and Searchspace#0 parameters from the start of the of the next modification period;

- 2b: the UE assumes the Coreset#0 and Searchspace#0 parameters in the stored OD-SIB1 configuration are no longer valid, and performs cell reselection to other cells;

- 2c: the UE assumes the stored OD-SIB1 request configuration is still valid, and performs OD-SIB1 request as the current procedure in the running CR;

Therefore, we propose RAN2 to discuss which understanding we should follow.

Proposal 1: RAN2 to discuss how to understand the previous agreement ‘Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’:

- Understanding 1: The UE assumes the NES cell will always transmits CD-SSB after sending SI change notification, the UE acquires SIB1 with the Coreset#0 and Searchspace#0 parameters in MIB as legacy;

- Understanding 2: It is up to the NES cell to transmits CD-SSB or NCD-SSB after sending SI change notification. If the NES cell transmits CD-SSB, the UE acquires SIB1 with the Coreset#0 and Searchspace#0 parameters in MIB as legacy; and if the NES cell transmits NCD-SSB,

- 2a: the UE assumes the Coreset#0 and Searchspace#0 parameters in the stored OD-SIB1 configuration are still valid, and acquires the updated SIB1 with the stored Coreset#0 and Searchspace#0 parameters from the start of the of the next modification period;

- 2b: the UE assumes the Coreset#0 and Searchspace#0 parameters in the stored OD-SIB1 configuration are no longer valid, and performs cell reselection to other cells;

- 2c: the UE assumes the stored OD-SIB1 request configuration is still valid, and performs OD-SIB1 request as the current procedure in the running CR;

OD-SIB1 request received by multiple NES cells sharing the same UL WUS configuration

During the RAN2#129bis, it was agreed that the UL WUS configuration can be common to multiple NES cells if their corresponding UL WUS configuration is the same [8]:

=> WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

When multiple NES cells enable OD-SIB1 operation, it is possible that OD-SIB1 request from a certain UE can be received by multiple adjacent NES cells. If all the adjacent NES cells respond to the same OD-SIB1 request, it will degrade the overall NES gain, as only one NES cell should be the target NES cell that satisfies the UE’s cell reselection criteria.

Observation 2: Multiple adjacent OD-SIB1 cells sharing the same UL WUS configuration may receive the same OD-SIB1 request and respond to it, causing unnecessary NES gain loss as only one of them is the target cell for the cell reselection procedure of a UE.

Figure 1. Multiple adjacent OD-SIB1 cells sharing same WUS configuration receive the same OD-SIB1 request

Therefore, we propose RAN2 to discuss how to address the issue. From our perspective, we can either rely on NW implementation to not configure the same UL WUS configuration for adjacent cells so that the issue can be avoided, or pursue further optimization like WUS information exchange among the NES cells.

Proposal 2: RAN2 to discuss how to address the issue that multiple adjacent OD-SIB1 cells sharing the same UL WUS configuration may receive the same OD-SIB1 request and respond to it:

- Alt.1: Leave it to NW implementation, e.g. adjacent OD-SIB1 cells are not expected to share the same OD-SIB1 request configuration;

- Alt.2: OD-SIB1 cells sharing the same OD-SIB1 request configuration can exchange information to avoid simultaneous response to the OD-SIB1 request. Send LS to RAN1/RAN3 to trigger the discussion for the exchanged information.

Conclusion

Based on the discussion, we have the following observation and proposals:

UE&NW behavior after reception of SI change notification

Observation 1: With the latest RRC running CR which removes the condition ‘5> if SIB1 is not broadcasted’ before performing OD-SIB1 request according to the RAN2 agreement at last meeting, the UE will perform OD-SIB1 request upon reception of SI change notification if the NES cell sends NCD-SSB, which is not aligned with the agreement ‘Keep RRC CR as it is for the short message and UE behaviour.’

Proposal 1: RAN2 to discuss how to understand the previous agreement ‘Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’:

- Understanding 1: The UE assumes the NES cell will always transmits CD-SSB after sending SI change notification, the UE acquires SIB1 with the Coreset#0 and Searchspace#0 parameters in MIB as legacy;

- Understanding 2: It is up to the NES cell to transmits CD-SSB or NCD-SSB after sending SI change notification. If the NES cell transmits CD-SSB, the UE acquires SIB1 with the Coreset#0 and Searchspace#0 parameters in MIB as legacy; and if the NES cell transmits NCD-SSB,

- 2a: the UE assumes the Coreset#0 and Searchspace#0 parameters in the stored OD-SIB1 configuration are still valid, and acquires the updated SIB1 with the stored Coreset#0 and Searchspace#0 parameters from the start of the of the next modification period;

- 2b: the UE assumes the Coreset#0 and Searchspace#0 parameters in the stored OD-SIB1 configuration are no longer valid, and performs cell reselection to other cells;

- 2c: the UE assumes the stored OD-SIB1 request configuration is still valid, and performs OD-SIB1 request as the current procedure in the running CR;

OD-SIB1 request received by multiple NES cells sharing the same UL WUS configuration

Observation 2: Multiple adjacent OD-SIB1 cells sharing the same UL WUS configuration may receive the same OD-SIB1 request and respond to it, causing unnecessary NES gain loss as only one of them is the target cell for the cell reselection procedure of a UE.

Proposal 2: RAN2 to discuss how to address the issue that multiple adjacent OD-SIB1 cells sharing the same UL WUS configuration may receive the same OD-SIB1 request and respond to it:

- Alt.1: Leave it to NW implementation, e.g. adjacent OD-SIB1 cells are not expected to share the same OD-SIB1 request configuration;

- Alt.2: OD-SIB1 cells sharing the same OD-SIB1 request configuration can exchange information to avoid simultaneous response to the OD-SIB1 request. Send LS to RAN1/RAN3 to trigger the discussion for the exchanged information.

References

- TS 38.331, V18.6.0

- R2-2405701	Report from session on V2X/SL, R19 NES and MOB	Vice Chairman (Samsung)

- R2-2407571	Report from session on V2X/SL, R19 NES and MOB	Vice Chairman (Samsung)

- [POST130][107][NES] Running RRC CR (Ericsson)

- [AT130][104][NES] (Ericsson)

- R2-2504037	Remaining issues on OD-SIB1		vivo

- R2-2504671	Report from session on R18 SL, R18/19 MOB, and R19 NES	Vice Chairman (Samsung)

- R2-2502981	Report from session on V2X/SL, R19 NES and MOB	Vice Chairman (Samsung)