3GPP TSG-RAN WG2 Meeting #131	R2-2505341

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.13.2

Source:	OPPO

Title:	Discovery and relay (re)selection for multi-hop U2N relay

Document for:	Discussion, Agreement

Introduction

The objective of multi-hop U2N relay is as follows, this paper will discuss the discovery and relay (re)selection for multi-hop U2N relay.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3]

Discussion

Open issue RRC-5/11/12: NotificationMessageSidelink message handling

For the notification message transmission at the intermediate relay UE, the following trigger condition has been agreed

- L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design): - cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network. - release of RRC connection between an intermediate relay UE and the network. - PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s). - FFS: link quality degradation of the upstream links resulting in a notification. - FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE). - FFS on the signalling contents of the PC5-RRC indication from the first relay UE. - FFS applicability to L3. - Upon intermediate relay UE performing relay (re)selection, it can notify its child node. If the cause of (re)selection already causes a notification, there is no double-trigger of the notification.

For PC5 RLF/release case, although it is new for U2N Relay, it is an existing scenario in U2U Relay with the cause value relayUE-PC5-RLF defined.

(RRC-5) Upon PC5 RLF/release happens at the PC5 link between the Intermediate relay UE and its parent UE, intermediate relay UE send Notification message to the child UE with existing cause value relayUE-PC5-RLF.

In the single hop U2N Relay, the following conditions are specified:

The Relay UE may initiate the procedure when one of the following conditions is met: 1> if the UE is acting as U2N Relay UE: 2> upon Uu RLF as specified in 5.3.10; 2> upon reception of an RRCReconfiguration including the reconfigurationWithSync; 2> upon cell reselection; 2> upon L2 U2N Relay UE's RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5;

As an intermediate relay, since it may also experience reconfigurationWithSync or RRC failure, both trigger conditions can be reused by the intermediate relay UE directly.

(RRC-12) As in Release 17 U2N Relay, the intermediate Relay UE in Release 19 upon reconfigurationWithSync or RRC failure, trigger a Notification message to the child UE using the legacy cause value.

In addition, an intermediate Relay UE may undergo cell selection during path switching from an indirect path to a direct path. Since both relay reselection (indirect-to-indirect path switching) and cell selection (indirect-to-direct path switching) are considered path switching procedures, a unified handling approach should be applied. Specifically, both scenarios trigger either a PC5 link release or the transmission of a notification message from the intermediate Relay UE to the child UE. The same cause value can be used to represent both cases.

When relay reselection or cell selection occurs at an IDLE or INACTIVE intermediate Relay UE, a unified handling approach should be applied: 1) both scenarios shall trigger either a PC5 link release or the transmission of a notification message from the intermediate Relay UE to the child UE and 2) a common cause value can be used to represent both cases.

For relay reselection or cell selection, on the cause value to be used, regardless of whether cell selection or relay reselection, and what triggers such scenario (e.g., RLF, mobility), the PC5 connection to the source Parent Relay UE always be released, so the simplest way would be to reuse the same cause value as PC5 connection release case (relayUE-PC5-RLF).

Since both the cell selection and relay reselection at the RRC IDLE/INACTIVE intermediate relay UE will result in the release of parent PC5 link, align with existing PC5 connection release, legacy relayUE-PC5-RLF cause value as defined in R18 can be reused.

(RRC-5, RRC-12) RAN2 to discuss when PC5 link with the parent UE is released due to cell selection or relay reselection at the intermediate relay UE, the intermediate relay UE sends Notification message to the child UE with existing cause value “relayUE-PC5-RLF” to maintain alignment with PC5 link release case.

For whether the notification message can be saved if the AS-visible parameters that could affect relay reselection are not changed. The following criteria are defined by AS and upper layer:

Table 1. Relay (re)selection criteria

| Criteria | Parameter | Whether visible to AS |
| --- | --- | --- |
| AS criteria | Cell ID/PLMN ID | Y |
|  | PC5 RSRP quality | Y |
| Upper layer criteria | Hop count | N |
|  | Accumulated QoS | N |
|  | Root relay info/path info, i.e., User Info ID | N |

As highlighted in the table above, AS layer has no visibility into the upper layer parameter accumulated QoS and root relay/path info. Consequently, it cannot assess whether a new link meets the Remote UE’s suitability requirements. Therefore, notifications about path changes must always be propagated to ensure the Remote UE can independently evaluate and act on updated network conditions.

For User Info, there is a discussion on the visibility to AS layer in R18 U2U Relay, and the understanding/conclusion from both RAN2 and SA2 are User Info is not visible to AS layer, and as replied in the SA2 LS (R2-2400072), there is a concern for providing User Info to AS layer[1].

SA2 also discussed the alternative option of RAN2 (i.e. Option 1 captured in R2-2311275 [1]): • According to SA2's understanding, this option can operate as below: Local ID and User Info ID (included in a container) of target Remote UE is provided by the UE-to-UE Relay to source Remote UE by using the RRCReconfigurationSidelink message. Then, the Local ID (included in a container) and User Info ID (included in a container) are provided from the AS layer to the ProSe layer of the source Remote UE. When sending e2e DCR message to the target Remote UE via the UE-to-UE Relay, the ProSe layer of the source Remote UE can pass the Local ID (included in a container) associated with the target Remote UE's User Info ID to the AS layer. • Therefore, this option does not require change on the existing SA2 defined procedures but requires some interaction between ProSe layer and AS layer which means SA2 specification change is required to support this option. However, some companies have concern about the inter layer interaction at UE-to-UE Relay and Remote UEs and providing User Info ID to the AS layer.

Hop number, accumulated QoS information, root relay info and path info are invisible to AS layer, but will affect relay reselection decision.

For the root relay info and path info which are indicated via User Info ID, SA2 has expressed the concern for providing such information to AS layer.

(RRC-11) RAN2 to confirm AS-layer notification message is always sent by an intermediate relay UE in RRC_IDLE/RRC_INACTIVE upon relay reselection.

Open issue RRC-6: Whether intermediate relay UE first access the network as the remote UE

For the PC5 threshold condition, RAN2 agree no PC5 threshold condition is needed for Model-A and Model-B when the PC5 link has been established. And for the case when the PC5 link between the intermediate relay and its parent is not established in Model-B, PC5 threshold condition is agreed, but FFS whether this case is a valid case

Agreements:

For discovery model B when there is no established PC5 link between the remote UE and the intermediate relay UE, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE.  FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first.

For discovery model B, the quality of the PC5 links is assumed already to have been checked when the solicitation messages were forwarded, and no AS criterion is needed for the intermediate Relay UE(s) to forward the response message towards the Remote UE.

For L2 Relay case, a UE with a SIM card always needs to find a cell for camping (i.e., paging/SIB reception). For a UE who can act as intermediate relay which is out of coverage, no matter it is acting as intermediate relay (with remote UE connected) or not yet, it needs to rely on a relay link for SI/Paging/data reception/transmission, which means it firstly acts as a remote UE.

The following procedures describe the reasonable UE (with SIM card) behavior which is captured in TS 38.304, the UE should always search for cell upon switching on.

When a UE is switched on, a public land mobile network (PLMN) or a SNPN is selected by NAS. For the selected PLMN/SNPN, associated RAT(s) may be set, as specified in TS 23.122 [9]. The NAS shall provide a list of equivalent PLMNs or a list of equivalent SNPNs, if available, that the AS shall use for cell selection and cell reselection. With cell selection, the UE searches for a suitable cell of the selected PLMN or selected SNPN, chooses that cell to provide available services, and monitors its control channel. This procedure is defined as "camping on the cell". <…> The purpose of camping on a cell in RRC_IDLE state and RRC_INACTIVE state is as follows: a) It enables the UE to receive system information from the PLMN or the SNPN. b) When registered and if the UE wishes to establish an RRC connection or resume a suspended RRC connection, it can do this by initially accessing the network on the control channel of the cell on which it is camped. c) If the network needs to send a message or deliver data to the registered UE, it knows (in most cases) the set of tracking areas (in RRC_IDLE state) or RNA (in RRC_INACTIVE state) in which the UE is camped. It can then send a "paging" message for the UE on the control channels of all the cells in the corresponding set of areas. The UE will then receive the paging message and can respond. d) It enables the UE to receive ETWS and CMAS notifications. e) It enables the UE to receive MBS broadcast services. f) It enables the UE to receive MBS multicast services in RRC_INACTIVE state.

Figure 1. Procedure for a switch-on UE

For L2 Multi-hop U2N Relay, the intermediate relay UE first acts as a remote UE for SI/Paging/data reception/transmission.

In other words, in L2 muti-hop U2N Relay, a UE should always have an available link towards the network before it serves the remote UE no matter for Model-B or Model-A. Which means the FFS case is not reasonable for L2 multi-hop U2N Relay.

In L2 multi-hop U2N Relay, a UE shall always have an available link towards the network before it serves a remote UE regardless of Model-B or Model-A. Which means the FFS case (PC5 link not established) is not a reasonable case for L2 multi-hop U2N Relay.

(RRC-6) For L2 U2N Relay, RAN2 consider the intermediate relay UE always establishes connection with a parent relay by acting as a remote UE first, so that the PC5 RSRP threshold at the last relay for Model-B discovery response message transmission is not needed.

Another issue is whether the intermediate Relay UE needs to check the Uu threshold. It is understood that the Uu threshold is intended to prevent cell-centre UEs from acting as intermediate Relay UEs, as this could lead to significant interference and inefficient resource utilization. This restriction should apply regardless of the discovery model used and irrespective of whether a PC5 connection to the parent UE is established. The same principle is already adopted in Release 17 for U2N Relay, where the U2N Relay UE checks the Uu threshold condition for each discovery transmission.

In R17 U2N Relay, the relay UE checks Uu threshold condition for each discovery transmission.

(RRC-6) RAN2 to confirm that Uu threshold condition checking is required for a UE acting as an intermediate Relay UE for each discovery transmission, irrespective of the discovery model and regardless of the presence of a PC5 connection with the parent UE.

Other issue: Avoiding multi-path topology

It was agreed that no cross-path/multipath topology is supported in L2 multi-hop U2N Relay as follows:

- Cases with two indirect paths to the gNB for the same remote UE are not supported. - If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

To avoid multi-path topology at the Remote UE and intermediate Relay UE, it needs to be excluded at the relay discovery stage, which is more of SA2 scope, so it can be up to SA2 to enforce, e.g.,

- For Model-A, the intermediate relay can only forward the announcement message from the connected parent relay;

- For Model-B,

  - for solicitation message forwarding, 1) if the intermediate relay already connects with a parent UE, the intermediate relay should only forward the solicitation message to the connected parent UE (i.e., unicast); 2) otherwise, it forward the solicitation message in broadcast manner.

  - for response message forwarding, 1) if the intermediate relay already connects with/selects a parent UE, the intermediate relay should only forward the response message from the connected parent UE; 2) otherwise, it forward the all the received response message.

The conclusion on excluding multi-path topology at the intermediate relay UE needs to be captured in discovery transmission mechanism, which falls into SA2 scope.

RAN2 to send LS to SA2 on the conclusion of excluding multi-path scenario and the possible impact to discovery transmission mechanism as in annex.

Conclusion

We have the following observations:

Observation 1	When relay reselection or cell selection occurs at an IDLE or INACTIVE intermediate Relay UE, a unified handling approach should be applied: 1) both scenarios shall trigger either a PC5 link release or the transmission of a notification message from the intermediate Relay UE to the child UE and 2) a common cause value can be used to represent both cases.

Observation 2	Since both the cell selection and relay reselection at the RRC IDLE/INACTIVE intermediate relay UE will result in the release of parent PC5 link, align with existing PC5 connection release, legacy relayUE-PC5-RLF cause value as defined in R18 can be reused.

Observation 3	Hop number, accumulated QoS information, root relay info and path info are invisible to AS layer, but will affect relay reselection decision.

Observation 4	For the root relay info and path info which are indicated via User Info ID, SA2 has expressed the concern for providing such information to AS layer.

Observation 5	For L2 Multi-hop U2N Relay, the intermediate relay UE first acts as a remote UE for SI/Paging/data reception/transmission.

Observation 6	In L2 multi-hop U2N Relay, a UE shall always have an available link towards the network before it serves a remote UE regardless of Model-B or Model-A. Which means the FFS case (PC5 link not established) is not a reasonable case for L2 multi-hop U2N Relay.

Observation 7	In R17 U2N Relay, the relay UE checks Uu threshold condition for each discovery transmission.

Observation 8	The conclusion on excluding multi-path topology at the intermediate relay UE needs to be captured in discovery transmission mechanism, which falls into SA2 scope.

We have the following proposals:

Proposal 1	(RRC-5) Upon PC5 RLF/release happens at the PC5 link between the Intermediate relay UE and its parent UE, intermediate relay UE send Notification message to the child UE with existing cause value relayUE-PC5-RLF.

Proposal 2	(RRC-12) As in Release 17 U2N Relay, the intermediate Relay UE in Release 19 upon reconfigurationWithSync or RRC failure, trigger a Notification message to the child UE using the legacy cause value.

Proposal 3	(RRC-5, RRC-12) RAN2 to discuss when PC5 link with the parent UE is released due to cell selection or relay reselection at the intermediate relay UE, the intermediate relay UE sends Notification message to the child UE with existing cause value “relayUE-PC5-RLF” to maintain alignment with PC5 link release case.

Proposal 4	(RRC-11) RAN2 to confirm AS-layer notification message is always sent by an intermediate relay UE in RRC_IDLE/RRC_INACTIVE upon relay reselection.

Proposal 5	(RRC-6) For L2 U2N Relay, RAN2 consider the intermediate relay UE always establishes connection with a parent relay by acting as a remote UE first, so that the PC5 RSRP threshold at the last relay for Model-B discovery response message transmission is not needed.

Proposal 6	(RRC-6) RAN2 to confirm that Uu threshold condition checking is required for a UE acting as an intermediate Relay UE for each discovery transmission, irrespective of the discovery model and regardless of the presence of a PC5 connection with the parent UE.

Proposal 7	RAN2 to send LS to SA2 on the conclusion of excluding multi-path scenario and the possible impact to discovery transmission mechanism as in annex.

Reference

[1] R2-2400072 Reply LS on L2ID and User Info for L2 based U2U

3GPP TSG-RAN WG2 Meeting #131	R2-250xxxx

Bangalore, India Aug 25th – 29th, 2025

Title:	LS on the discovery transmission for L2 multi-hop U2N Relay

Response to:	-

Release:	Release 19

Work Item:	NR_SL_relay_multihop-Core

Source:	OPPO [To be RAN2]

To:	SA2

Cc:	CT1

Contact Person:

Contact Person:

#### Name:	Bingxue Leng

E-mail Address:	<lengbingxue@oppo.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

It was agreed in RAN2 that each UE in L2 multi-hop U2N Relay can only have one path towards the network:

- Cases with two indirect paths to the gNB for the same remote UE are not supported. - If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

To prevent multi-path topology at the Remote UE and intermediate Relay UE, RAN2 understand that each Layer 2 intermediate Relay UE shall forward the discovery message (for both Model-A and Model-B) received from its single connected parent Relay UE, and transmit the discovery solicitation message exclusively to its single connected parent Relay UE. Based on this understanding, the following agreements were established:

Agreements to be added:

…

2. Actions:

To SA WG2

ACTION: 	RAN2 would like to ask SA2 to take the above agreements into account in their work.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#131 bis	from 2025-10-13 to 2025-10-17		                 Prague, CR

RAN2#132	from 2025-11-17 to 2025-11-21		                 Dallas, US