3GPP TSG-RAN WG2 Meeting #131	R2-2505346

Bangalore, India, Aug. 25th – 29th, 2025

Agenda item:	8.1.3

Source:	Samsung

Title:	Discussion on NW side data collection

Document for:	Discussion & Decision

Introduction

This document discusses on NW side data collection for Rel-19 AI/ML air interface WI.

Discussion

Memory capability

###### Open issue RRC-38: Minimum AS layer memory size supported by the UE Issue description: The following agreement was made in RAN2#127, to specify a minimum AS layer memory size across all use cases, similar to logged MDT. And the specific value of the memory size is FFS, but it has not been addressed yet. UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases. This issue can be part of the capability discussions. Proposed resolution: It is suggested to address this open issue in the capability discussions

In [POST130][038][AI PHY] UE capabilities (Xiaomi), RAN2 has discussed UE memory capability for NW-side data collection. Based on the companies input, rapporteur made two proposals:

Proposal 1: It is mandatory for UE to support the minimum AS layer memory size of 64kB for UE supporting AI/ML based beam management, which is shared across all use cases with NW-sided model. Proposal 2: FFS on whether UE can support other memory sizes and indicate to network via optional capability signaling.

In our understanding, 64KB may not be enough. Regarding beam management use-case, as RAN1 has indicated in LS (i.e., R1-2310681), a single data sample for L1-RSRP of 128 resources accounts for about 500 bits. However, L1-RSRP combinations for 128 resources a UE can observe could be so large due to multi-path fading and UE’s location/pose and surroundings/environment. So, we assume numerous data samples would be needed to train a practical AI/ML model showing a quality of prediction performance. Besides, more data generally is beneficial to model performance. Moreover, we assume memory sharing among use-cases (i.e., not only beam management but also CSI prediction) for efficient use of limited memory of UE. Considering the RAN1 LS indicated 1.5Mbits per data sample as an example for CSI prediction, 64KB is not sufficient at all.

Thus, while we support to adopt 64KB as the baseline mandatory capability, UE should also be able to indicate some larger sizes (e.g., hundreds of KB or tens of MB) if supported. Otherwise, NW cannot distinguish the UE with large memory. For example, even if NW needs large size of data (e.g., to train model for CSI prediction) for a UE, there is no way for NW to choose UEs with larger memory. The optional values could be 128KB, 256KB, 512KB, …, 16MB i.e., 3 bits are needed to represent these 8 values.

Proposal 1 (RRC-38). Adopt P1 in [POST130][038][AI PHY] UE capabilities (Xiaomi) i.e., It is mandatory for UE to support the minimum AS layer memory size of 64kB for UE supporting AI/ML based beam management, which is shared across all use cases with NW-sided model.

Proposal 2 (RRC-38). UE can support other memory sizes and indicate to network via optional capability signaling (i.e., One of value from 128KB, 256KB, … 16MB).

RRE and RRC_IDLE/RRC_INACTVE

###### Open issue RRC-28: Handling of configuration to report data availability and low power state during RRCReestablishment, transition to RRC_INACTIVE etc Issue description: Handling of configuration for UE assistance information, to report data availability and low power state (LoggedDataCollectionAssistanceConfig in RRC running CR) during RRCReestablishment and in transition to RRC_INACTIVE state needs to be specified. In the rapporteur’s view, handling the UAI configuration for reporting data availability and low power state should follow the way that the logging configurations are handled, which depends on the solution that RAN2 will adopt based on the email discussion [POST130][031][AI PHY] NW side data collection (Ericsson/ZTE). Proposed resolution: Adopt the proposal below. Summary: - 6/11 companies agree to the proposal from the rapporteur - 5/11 companies replied with comments - 3 companies suggest that the UE assistance information to report data availability and low power should follow the behaviour of other configurations in otherConfig - 1 company suggests that the UAI configuration and logging configuration should be released on the transition to IDLE/INACTIVE and RRE, since the logged data is discarded upon transition to IDLE/INACTIVE and RLF. Regarding Apple’s comment, the rapporteur’s intention was to say that for the logging configuration, RAN2 first needs to decide where to add this configuration (based on the email discussion), then when this configuration should be discarded, and then the UAI configuration for the availability indication should follow the same behaviour. The intention of the proposal was to avoid cases where the logging configuration is kept, but the UAI configuration for availability indication is discarded. This is because the UE wouldn’t have any means to indicate that it has logged data again, unless the NW sends again the UAI configuration. Thus, both the logging configuration and the UAI configuration should be discarded or kept in the same situations. Given the comments above, the rapporteur updated the proposals to: - (RRC-28) RAN2 to discuss whether logging configurations for NW-side data collection are discarded upon IDLE/INACTIVE/RLF. - (RRC-28) RAN2 to discuss whether handling the configuration for UE assistance information to report data availability and low power state upon IDLE/INACTIVE/RLF follows the UE behaviour for handling the logging configurations.

In the last meeting, RAN2 agreed to discard any logged data when going to RRC_IDLE and RRC_INACTIVE and in case on RLF.

<Agreement (RAN2#129bis)> - Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

We first would like to clarify there are various failures in NR other than RLF (e.g., re-configuration with sync failure, mobility from NR failure, RRC connection reconfiguration failure). Upon detecting all these failures, UE triggers RRC connection re-establishment procedure according to 5.3.7.2 in RRC. We understand UE should discard any logged data upon these failures, so we first propose to apply the agreement to generic RRE initiation.

Proposal 3 (RRC-28). Upon RRE initiation (including RLF detection), UE discards any logged data.

According to RRC spec, all of legacy UAI configurations (i.e., otherConfig) are released and legacy CSI measurement/reporting configuration included in spCellConfig is also released upon RRE initiation.

<TS 38.331> ### 5.3.7 RRC connection re-establishment <…> #### 5.3.7.2 Initiation <…> Upon initiation of the procedure, the UE shall: <…> 1> if UE is not configured with attemptCondReconfig; and 1> if UE is not configured with attemptLTM-Switch: 2> reset MAC; 2> release spCellConfig, if configured; <…> 2> release delayBudgetReportingConfig, if configured and stop timer T342, if running; 2> release overheatingAssistanceConfig, if configured and stop timer T345, if running; <…>

Considering it is the last meeting for Rel-19 functional freeze, we also would like to keep the same principle for configuration of NW-side data collection (i.e., NSDC) and UAI configuration for NSDC.

Proposal 4 (RRC-28). Upon RRE initiation, UE releases 1) the configuration of NW-side data collection and 2) UAI configuration for NW-side data collection.

When going to RRC_IDLE, we understand the configuration of NW-side data collection and UAI configuration for NW-side data collection are released according to the following legacy text.

<TS 38.331> ### 5.3.11 UE actions upon going to RRC_IDLE The UE shall: <…> 1> release all radio resources, including release of the RLC entity, the BAP entity, the MAC configuration and the associated PDCP entity and SDAP for all established RBs (except for broadcast MRBs), BH RLC channels, Uu Relay RLC channels, PC5 Relay RLC channels and SRAP entity;

Proposal 5 (RRC-28). When going to RRC_IDLE, UE releases 1) the configuration of NW-side data collection and 2) UAI configuration for NW-side data collection (i.e., No spec change is needed).

In the case of RRC_INACTIVE, we should first discuss whether to store in UE Inactive AS Context 1) the configuration of NW-side data collection and 2) UAI configuration for NW-side data collection. We already agreed to discard logged data, and so it is simple not to store the configuration of NW-side data collection in UE Inactive AS Context. Meanwhile, we prefer to follow the same principle with legacy UAI configuration i.e., UE first stores UAI configuration in UE Inactive AS Context upon reception of RRCRelease message, but it is released upon initiation of RRC connection resume.

Proposal 6 (RRC-28). When going to RRC_INACTIVE, UE does not store the configuration of NW-side data collection in UE Inactive AS Context.

Proposal 7 (RRC-28). When going to RRC_INACTIVE, UE first stores UAI configuration for NW-side data collection in UE Inactive AS Context and it is released upon initiation of RRC connection resume.

Timestamp

###### Open issue RRC-21: Time related content of logged data Issue description: It has not yet been clarified what information needs to be included with the logged data, to indicate a time gap between the logged data entries (i.e. a gap that is longer than the logging data periodicity). This issue refers to the RAN2#129bis agreement: - For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit RAN2#130 agreement: Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.. The issue is captured as an editor’s note in the running CR, in clause 5.7.10.3 and 6.2.2. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

In the last meetings, RAN2 discussed on whether timestamp for logged data is need or not, and agreed NW should be known whether there is temporal gap more than configured periodicity, and the gap should be indicated.

<Agreement (RAN2#129bis)> - For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit <Agreement (RAN2#130)> - Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.

There were a few of proposals from companies to introduce timestamp in reports to indicate when the data is measured/logged. It is obvious any kind time information is needed for NW to train model at least for temporal prediction. However, we do not think the explicit timestamp is needed. i.e., NW can understand the time information implicitly.

In case of periodic logging, for each CSI-RS or SSB resource, UE logs data (e.g., L1-RSRP) periodically. As NW configures the periodicity of the resource or logging periodicity, it is obviously known by NW so it is enough for UE to contain/report the data (e.g., L1-RSRP) in chronological order in a list. Then, NW can understand the differential time between two consecutive entries in the list is same as the configured periodicity.

On the other hand, in case of event-based logging, data logging may not be uniformly periodic, as there could be the period during which UE does not measure the configured resources. For example,

- 1) When the event is fulfilled, UE starts logging data periodically (i.e., 1st logging instance).

- 2) When the event is not fulfilled any longer, UE stops measurement/logging.

- 3) When the event is fulfilled again, UE resumes measurement and logs data periodically again (i.e., 2nd logging instance).

There is a similar issue when we consider full buffer case, even if periodic logging is configured. For example,

- 1) When UE is configured with data collection (e.g., periodic logging), UE logs data periodically (i.e., 1st logging instance).

- 2) When UE’s buffer becomes full, UE stops measurement/logging.

- 3) After some data reporting to NW, some buffer becomes available and UE resumes measurement and logs data periodically again (i.e., 2nd logging instance).

In the two aforementioned examples, if UE contains the logged data for a common list sequentially, the time difference between the last data for the 1st logging instance and the first data of the 2nd logging instance may not be same as the configured periodicity of resources. i.e., the “gap” occurs. To indicate this gap, one solution could be adding a timestamp to represent a gap between two consecutive logging instances (i.e., timestamp per gap). However, we do not think the length of gap is so important in model training. Given the length of gap could be large enough (e.g., tens of seconds), there is little correlation between two L1-RSRP values before and after the gap. Therefore, it is inefficient for model to try to train such a little correlation.

Hence, we propose the solution without explicit timestamp i.e., UE contains the data in a separate data list per each logging instance. For example,

- When UE performs measurement periodically (i.e., within a logging instance), UE contains the data sequentially in a common data list.

- Whenever UE suspends and resumes measurement (due to event or full buffer), UE starts logging the subsequent data using a separate data list

Figure 1. Illustration for Proposal 8

Proposal 8. (RRC-21) No explicit timestamp is needed for logged data to indicate time gap between two consecutive samples. Instead, separation of data list is used to indicate the gap.

- When UE performs measurement periodically, UE contains the data sequentially in a common data list.

- Whenever UE suspends and resumes measurement (depending on whether event is fulfilled and whether the buffer is full), UE starts logging the subsequent data using a separate data list.

Meanwhile, if NW wants to collect to sufficient labelled data for good quality of model training, the data collection could be configured to UEs long enough (e.g., tens of seconds). However, the interval for logging or measurement could be short (e.g., a tens of millisecond). As a result, numerous data could be collected by UE. In that sense, we would like to emphasize that signalling reduction is essential to minimize the size of report and collect more data with UE’s limited size of memory. So, we propose to include “absolute” L1-RSRP only in the 1st entry of the data list, but “differential” L1-RSRPs for the other entries. Furthermore, we propose the differential L1-RSRP is calculated compared to L1-RSRP of the previous entry, as the two consecutive samples are likely to have similar L1-RSRPs, which requires less bits to represent differential L1-RSRP in ASN.1.

Figure 2. Illustration for Proposal 9

Proposal 9. (RRC-21) UE includes L1-RSRPs sequentially in each data list for each beam, where the n-th entry of the data list is set as follows:

- If n=1, absolute L1-RSRP is set

- Otherwise (i.e., n>1), differential L1-RSRP is set compared to L1-RSRP of (n-1)th entry

Measurement resumption

RAN2 agreed UE autonomously stops measurement for NW-side data collection in case of full buffer and it is indicated to NW via UAI.

<Agreement (RAN2#127)> - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. <Agreement (RAN2#129bis)> - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication

RAN2 needs to discuss on when UE resumes suspended measurement due to full buffer. This is needed because if we are to specify UE stops measurement due to full buffer in the specification, when UE resumes measurement should be also specified. In our understanding, it is straightforward UE resumes after any memory becomes available (e.g., after any data report is sent to NW after full buffer). In the perspective of NW, it can also implicitly know UE’s resumption based on UE’s data reporting after the full buffer indication. Therefore, we propose UE autonomously resumes measurement after any memory becomes available (i.e., after any data report is sent to NW after full buffer). We categorize this proposal as RRC-24 based on RRC rapporteur’s guidance.

###### Open issue RRC-24: Where to include the logging configuration from NW to UE Issue description: It is FFS whether the logging configuration is included in the CSI framework (whether in CSI-ReportConfig or directly under CSI-MeasConfig) or at L3. This issue was discussed in RAN2#129bis and the following outcome was captured: Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.. The issue is captured as an editor’s note in the running CR, in clause 6.3.2. Configuration details of events for event-based logging configuration, e.g. whether we reuse configuration in reportConfig, can be included. Proposed resolution: This open issue is addressed in the email discussion [POST130][031][AI PHY] NW side data collection (Ericsson/ZTE). Companies can also discuss relevant aspects in their contributions

Proposal 10. (RRC-24) UE autonomously resumes measurement/logging paused due to full buffer, when any memory becomes available.

Mobility from NR

###### Open issue RRC-51: Discard logged data upon inter-RAT handover Issue description: RAN2 agreed that 1 bit indication (whether to retain logged data during handover) needs to be provided by the target cell at the time of handover based on indication from the source cell, the indication, if needed, has to be provided by E-UTRA RRCConnectionReconfiguration for Inter-RAT handover. Since 36.331 changes are not in the scope of WI, RAN2 may need to add some specification text for discarding in TS 38.331, e.g. as below: 5.4.3.4 Successful completion of the mobility from NR Upon successfully completing the handover, at the source side the UE shall: - discard the logged measurement entries included in VarCSI-LogMeasReport, if any;. Please note that without this change, we may also need more changes in 36.331 to discard VarCSI-LogMeasReport during E-UTRA RLF and E-UTRA state transition to RRC_IDLE. Proposed resolution: It is suggested that companies address this issue in their contributions.

RAN2 has agreed the 1 bit indication whether to retain or discard the logged data for intra-NR handover.

<Agreement (RAN2#130bis)> - 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with synch is introduced. In case of HO, the source sends the 1-bit indication to target cell in HO preparation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data). We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it. Notify RAN3

If 1-bit indication is absent in RRCReconfiguration message, UE discards logged data.

<RRC Running CR> #### 5.3.5.3 Reception of an RRCReconfiguration by the UE <…> 3> if the RRCReconfiguration includes retainLoggedMeasurements: 4> if the UE has logged measurement entries available in VarCSI-LogMeasReport: 5> include the csi-LogMeasAvailable in the RRCReconfigurationComplete message; 3> else: 4> discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

However, we should consider mobility from NR to other RAT (i.e., LTE). In our understanding, as the target LTE node does not support NW-side data collection, it cannot retrieve logged data. So, it is reasonable UE discards logged data. However, in case of mobility from NR to LTE, UE receives the LTE message (i.e., RRCConnectionReconfiguration) for handover (i.e., rather than NR RRCReconfiguration message). It means UE’s behavior upon reception of the RRCConnectionReconfiguration message is specified in LTE spec (i.e., 36.331). So, there are two options on the table to make UE discard the logged data.

- Option 1. Specify the UE behaviour in 36.331

- Option 2. Specify the UE behaviour in 38.331 (i.e., 5.4.3.5 Successful completion of the mobility from NR)

We assume the update of 36.331 is not scope of this WI, so we propose Option2.

Proposal 11 (RRC-51). Specify in 38.331 (i.e., 5.4.3.5 Successful completion of the mobility from NR), UE discards the logged data.

Conclusion

RAN2 is requested to discuss and agree the following proposals:

Proposal 1 (RRC-38). Adopt P1 in [POST130][038][AI PHY] UE capabilities (Xiaomi) i.e., It is mandatory for UE to support the minimum AS layer memory size of 64kB for UE supporting AI/ML based beam management, which is shared across all use cases with NW-sided model.

Proposal 2 (RRC-38). UE can support other memory sizes and indicate to network via optional capability signaling (i.e., One of value from 128KB, 256KB, … 16MB).

Proposal 3 (RRC-28). Upon RRE initiation (including RLF detection), UE discards any logged data.

Proposal 4 (RRC-28). Upon RRE initiation, UE releases 1) the configuration of NW-side data collection and 2) UAI configuration for NW-side data collection.

Proposal 5 (RRC-28). When going to RRC_IDLE, UE releases 1) the configuration of NW-side data collection and 2) UAI configuration for NW-side data collection (i.e., No spec change is needed).

Proposal 6 (RRC-28). When going to RRC_INACTIVE, UE does not store the configuration of NW-side data collection in UE Inactive AS Context.

Proposal 7 (RRC-28). When going to RRC_INACTIVE, UE first stores UAI configuration for NW-side data collection in UE Inactive AS Context and it is released upon initiation of RRC connection resume.

Proposal 8. (RRC-21) No explicit timestamp is needed for logged data to indicate time gap between two consecutive samples. Instead, separation of data list is used to indicate the gap.

- When UE performs measurement periodically, UE contains the data sequentially in a common data list.

- Whenever UE suspends and resumes measurement (depending on whether event is fulfilled and whether the buffer is full), UE starts logging the subsequent data using a separate data list.

Proposal 9. (RRC-21) UE includes L1-RSRPs sequentially in each data list for each beam, where the n-th entry of the data list is set as follows:

- If n=1, absolute L1-RSRP is set

- Otherwise (i.e., n>1), differential L1-RSRP is set compared to L1-RSRP of (n-1)th entry

Proposal 10. (RRC-24) UE autonomously resumes measurement/logging paused due to full buffer, when any memory becomes available.

Proposal 11 (RRC-51). Specify in 38.331 (i.e., 5.4.3.5 Successful completion of the mobility from NR), UE discards the logged data.

References