3GPP TSG-RAN WG2 Meeting #131	R2-2505379

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	8.4.2

Source:	Huawei, HiSilicon

Title:	Further discussion on LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

# Introduction

In this contribution, we further discuss the open issues related to LP-WUS in RRC_IDLE/INACTIVE mode.

# Discussion

## Threshold for Entry Condition for OFDM-only scenario

RAN1 agreed to support both OOK-based and OFDM-based receivers for LP-WUS monitoring. Since the OFDM-based LP-WUR has stronger capability for LP-WUS signal reception and processing, it is possible for NW to configure a shorter LP-WUS to wake up only OFDM-based receiver to reduce the resource overhead. In this case, the UE with OFDM-based receiver can benefit from the low-complexity operation on LP-WUS, e.g., to cancel any OOK sequence/pattern detection or implement termination of LP-WUS procedure by early detection. The UE with OOK-based receiver will skip the current LP-WUS. Thus, the NW should be able to support the scenario that only UEs with OFDM-based LP-WUR are paged, e.g., by sending a shorter LP-WUS that cannot be received by OOK-based receiver.

Observation 1: It is possible that only OFDM-based LP-WUR is supported in a cell based on network configuration.

Currently there is a consensus that the OFDM-based LP-WUR can also receive LP-SS for DL synchronization and measurement since RAN1 agreed that LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. Compared to SSB, LP-SS provides significant power-saving advantages due to its measurement efficiency. Since one LP-SS cycle contains larger number of occasions for LP-SS reception, a single LP-SS based measurement is sufficient to meet performance requirements.

The thresholds defined to evaluate entry condition [3] are given below:

entryEvaluationOnMR-ForLR-OnLPSS Indicates the threshold(s) for a UE supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS to determine whether the entry condition for using LP-WUS is fulfilled or not based on the serving cell measurement on MR (see TS 38.304 [20], clause xxxx).

entryEvaluationOnMR-ForLR-OnSSB Indicates the threshold(s) for a UE supporting OFDM based LP-WUR measuring on SSB to determine whether the entry condition for using LP-WUS is fulfilled or not based on the serving cell measurement on MR (see TS 38.304 [20], clause xxxx).

entryEvaluationOnLR-ForLR-OnLPSS Indicates the threshold(s) for a UE supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS to determine whether the entry condition for using LP-WUS is fulfilled or not based on the serving cell measurement on LP-WUR (see TS 38.304 [20], clause xxxx). This field is optional present for the cell supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS. It is absent otherwise.

entryEvaluationOnLR-ForLR-OnSSB Indicates the threshold(s) for a UE supporting OFDM based LP-WUR measuring on SSB to determine whether the entry condition for using LP-WUS is fulfilled or not based on the serving cell measurement on LP-WUR (see TS 38.304 [20], clause xxxx). This field is optional present for the cell supporting OFDM based LP-WUR measuring on SSB. It is absent otherwise.

There is a case where the network can use shorter LP-WUS to wake-up UEs with OFDM-based LR. In this case, if the network provides thresholds for entry condition based on LP-SS (i.e., entryEvaluationOnMR-ForLR-OnLPSS, entryEvaluationOnLR-ForLR-OnLPSS), UEs with OOK-based LR will enter LP-WUS monitoring if the entry conditions are met but cannot decode LP-WUS intended only for OFDM based LR and thus will remain in the LP-WUS monitoring state potentially missing paging.

Observation 2: When network provides thresholds for entry condition based on LP-SS, UEs with OOK-based LR may remain in LP-WUS monitoring state if network transmits shorter LP-WUS intended only for OFDM based LR.

To avoid this, a clarification is needed whether the thresholds based on LP-SS are applicable to UEs with OOK-based LR. Alternatively, network should not provide thresholds based on LP-SS if it supports waking up only UEs with OFDM-based LR.

Proposal 1: Clarify whether the thresholds based on LP-SS are applicable to UEs with OOK-based LR.

## LP-WUS entry/exit condition

RAN2 has discussed whether the entry condition can be also based on the measurement result of LP-WUR in RAN2#127 and RAN2#127bis, and made the following working assumption and agreements.

Working Assumption in RAN2#127 - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. Agreement in RAN2#127bis - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - The LPWUS monitoring exit condition does not include MR measurements.

If the NW configures the LP-WUS entry thresholds for both MR and LP-WUR measurements, it should be up to UE implementation to obtain the measurement results on LP-WUR, i.e., the metrics and requirement of LR measurement should wait for RAN4’s discussion and there is no need to define in RAN2 spec the specific UE behavior on LP-WUR measurement for LP-WUS entry condition.

Proposal 2: If the LP-WUS entry threshold based on LP-WUR is configured, it is up to UE implementation how to obtain the LP-WUR measurement results used for LP-WUS entry condition.

## Issue on prioritizing the frequencies supporting LP-WUS

Based on RAN Plenary discussion and way forward, RAN2 discussed the following solutions.

- Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band).

- Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria).

- Solution 3: MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation).

The conclusions in RAN2#129 meeting were:

RAN2 understands that UE can report which band(s) is supported by LR to NW. RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

If the solution suggested in [1] cannot be agreed, to let LP-WUS UE camp on the cell supporting LP-WUS, the gNB can only rely on RRC release message, e.g., to indicate dedicated cellReselectionPriorities with LP-WUS frequency(ies) set to higher priority. To implement it, the gNB has to know whether a UE supports IDLE/INACTIVE LP-WUS and which band(s) is/are supported by the UE for IDLE/INACTIVE LP-WUS. If the UE does not support LP-WUS, the gNB does not need to assign the dedicated cellReselectionPriorities for LP-WUS.

Observation 3: To assign proper dedicated cellReselectionPriorities for LP-WUS to the UE, the gNB needs to know whether the UE supports IDLE/INACTIVE LP-WUS and which band(s) is/are supported by the UE for IDLE/INACTIVE LP-WUS.

However, based on the agreement, the UE capability for IDLE/INACTIVE LP-WUS will be included in Rel-19 paging container using OCTET STRING.

Agreements

- Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced.   This is applicable only to new Rel-19 and future capabilities added to paging capabilities.  This will only work for Rel-19 gNB

According to the analysis in [2], “Since the “container” is defined as OCTET STRING, the gNB can receive the container from UE, and transparently forward it to CN (without decoding the content). We note that the “cost” of this mechanism is that the UE would need to signal a specific capability twice, as a “UE capability” and as a “UE Radio Paging Capability” (in the “container”)”. Since the gNB does not decode the content of Rel-19 paging container but needs to know the UE capability for IDLE/INACTIVE LP-WUS, the UE capability for IDLE/INACTIVE LP-WUS needs to be added outside the Rel-19 paging container as well. To reduce the signaling overhead, only the necessary UE capability for IDLE/INACTIVE LP-WUS needs to be included outside the Rel-19 paging container, e.g., the supported band(s) for LP-WUS operation in IDLE/INACTIVE.

Proposal 3: If RRC release message is used to prioritize LP-WUS frequency, the UE capability for LP-WUS operation in IDLE/INACTIVE should also be added outside the Rel-19 paging container in UE capability information message, e.g., the supported band(s) for LP-WUS.

# Conclusion

In this contribution, we discuss the open issues of LP-WUS for IDLE/INACTIVE modes, and have the following proposals:

Observation 1: It is possible that only OFDM-based LP-WUR is supported in a cell based on network configuration.

Observation 2: When network provides thresholds for entry condition based on LP-SS, UEs with OOK-based LR may remain in LP-WUS monitoring state if network transmits shorter LP-WUS intended only for OFDM based LR.

Proposal 1: Clarify whether the thresholds based on LP-SS are applicable to UEs with OOK-based LR.

Proposal 2: If the LP-WUS entry threshold based on LP-WUR is configured, it is up to UE implementation how to obtain the LP-WUR measurement results used for LP-WUS entry condition.

Observation 3: To assign proper dedicated cellReselectionPriorities for LP-WUS to the UE, the gNB needs to know whether the UE supports IDLE/INACTIVE LP-WUS and which band(s) is/are supported by the UE for IDLE/INACTIVE LP-WUS.

Proposal 3: If RRC release message is used to prioritize LP-WUS frequency, the UE capability for LP-WUS operation in IDLE/INACTIVE should also be added outside the Rel-19 paging container in UE capability information message, e.g., the supported band(s) for LP-WUS.

# References

R2-25056229, Discussion on prioritizing the frequencies supporting LP-WUS, Huawei, HiSilicon, vivo, CMCC, CATT, Nokia, Samsung, LG Electronics Inc., Apple, Ericsson, OPPO, Sharp, NEC.

R2-2502576, UE Radio Paging Capability, Ericsson.

R2-2505392 RRC Running CR for LP-WUS WUR