3GPP TSG-RAN WG2 Meeting #131		R2-2505405

Bengaluru, India, 25th – 29th Aug. 2025

Source:	vivo

Title:	Discussion on RLC enhancement for XR

Agenda Item:	8.7.5

Document for:	Discussion and Decision

- Introduction

In RAN2#130, the following agreements were made for RLC enhancement [1]:

- (RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this - (RLC-9) No additional conditions are needed for the polling enhancement. - RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT. - (PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP SDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed. - Autonomous retransmission and polling is triggered only based on discardTimer, i.e. not based on discardTimerForLowImprotance - The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity. - (RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission. - Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) - Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work - Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report. - Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

In this paper, we will discuss the remaining open issues for the RLC enhancement of XR summarized in [2].

- Discussion

  - Remaining issues for avoiding unnecessary retransmissions

    - RLC-10

The issue of RLC-10 is described below [2]:

| RLC-10 | Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard. - As raised by ZTE below | Issue Type: Not essential not important How to address it: based on companies’ contribution |
| --- | --- | --- |

In the RLC open issue discussion, ZTE has proposed that “if DL stop ReTx Obsolete SDU is supported, DL t-RxDiscard should be configured at the UE; and if UL stopReTxObsoleteSDU is configured for the UE, UL timer-based Rx discard should be activated at the NW”. This is technically right, but we think this could be ensured by the network implementation and no spec impact is needed.

Proposal 1: (RLC-10) RAN2 assumes network implementation should ensure without any specification impact:

- If DL stop retransmission obsolete SDU is supported, the DL t-RxDiscard should be configured at the UE;

- If UL stopReTxObsoleteSDU is configured, timer-based Rx discard should be activated in the network.

    - RLC-11

The issue of RLC-11 is described below [2]:

| RLC-11 | FFS how to handle the case “if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with” - As raised by Nokia and Apple below | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |

And in the last meeting, the following agreements were made regarding this issue:

Agreements for RLC-11

- Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting)

- Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

Agreements for RLC-11

- Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting)

- Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

There are two scenarios regarding this issue:

- Case 1: Only SDUs whose transmissions have been stopped are buffered, with no window stalling occurring;

- Case 2: Only SDUs whose transmissions have been stopped are buffered while window stalling occurs;

For case 1, since new SDUs from PDCP with SNs higher than those in the buffer can still be transmitted to the receiving side, the t-RxDiscard will be initiated upon detecting the SN gap. Once the t-RxDiscard expires, a STATUS report will be sent to the transmitter, resulting in the discarding of the SDUs whose transmissions have been stopped. Therefore, polling is not needed, and there is no issue in Case 1.

Observation 1: There is no issue regarding polling retransmission in the scenario where only SDUs with stopped transmissions are buffered, and no window stalling occurs.

For case 2, we consider this to be a corner case that does not require resolution. If we were to address this case, the SDUs whose transmissions have been stopped would need to be transmitted to resolve the window stalling, which is unexpected. Therefore, we propose to confirm the working assumption made in the last meeting.

Proposal 2: (RLC-11) RAN2 confirms that there is no need to address the window stalling issue with polling retransmission.

    - RLC-12

The issue of RLC-12 is described below [2]:

| RLC-12 | FFS how to handle PDCP SN gap report handling during UE mobility - As raised by Qualcomm below. The corresponding Rel-18 agreement is: - RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements. If it is a problem further discussions are needed on the alternatives. | Issue Type: Not essential not important? How to address it: based on companies’ contribution |
| --- | --- | --- |

This is a Rel-18 leftover issue about the PDCP SN gap report handling during UE mobility. We have agreed that this is not a big problem to solve in Rel-18, and RAN2 can discuss it in the context of Rel-19 RLC enhancements.

RAN2#129 Agreement

- From RAN2 point of view, for Rel-18 alternative 3 should not be supported.   RAN2

doesn’t see a big problem to solve for Rel-18.

- RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem

to solve due to RLC enhancements. If it is a problem, further discussions are needed on the alternatives.

RAN2#129 Agreement

- From RAN2 point of view, for Rel-18 alternative 3 should not be supported.   RAN2

doesn’t see a big problem to solve for Rel-18.

- RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem

to solve due to RLC enhancements. If it is a problem, further discussions are needed on the alternatives.

In the context of Rel-19, this issue remains unchanged from Rel-18, meaning that if it was not critical for Rel-18, it would also remain non-critical for Rel-19. Additionally, in Rel-19, the mechanism to avoid unnecessary retransmissions may also lead to missing PDCP SN gap reports (i.e., the control PDU for the PDCP SN gap report may be discarded when the t-RxDiscard timer expires). We have agreed that special handling to prevent the discard of PDCP control PDUs is unnecessary, same principle should be applied for the case here. Thus, we prefer to follow the previous agreement.

RAN2#128 Agreement

- Special handling to avoid PDCP control PDU discard is not needed.

RAN2#128 Agreement

- Special handling to avoid PDCP control PDU discard is not needed.

Proposal 3: (RLC-12) No special handling is required in Rel-19 for PDCP SN gap report during UE mobility.

    - RLC-14

In the RLC open issue discussion, NEC proposed that the transmitting side of RLC should discard the RLC SDU or segment(s) upon receiving a discard indication from PDCP, without waiting for the STATUS report from the Rx side.

| RLC-14 | If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU, whether the transmitting side of an AM RLC entity shall discard the corresponding RLC SDU or RLC SDU segment(s) if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers. As raised by NEC below | Issue Type: Not essential not important How to address it: based on companies’ contribution |
| --- | --- | --- |

In our view, this is beneficial at UE side because RLC SDUs indicated for discard by the upper layer are unnecessary and useless at the RLC layer. Therefore, there is no need for the transmitting side to buffer these RLC SDUs or segments until receiving a fake acknowledgment from the receiving side.

Proposal 4: (RLC-14) When indicated from upper layer to discard a particular RLC SDU, the transmitting side of RLC entity shall discard the corresponding RLC SDU or RLC SDU segment(s) if those have been submitted to lower layers.

    - UE capability-04

During the RAN2#130 meeting, it was discussed whether a UE that supports the Tx-side capability for avoiding unnecessary retransmissions should also support the Rx-side capability for the same purpose. An FFS was made:

- Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report. - Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

The Tx-side and Rx-side aspects are defined for different traffic directions (i.e., one for UL traffic and one for DL traffic), making them independent of each other. Therefore, we believe there should be no coupling between the Tx side and Rx side regarding the avoidance of unnecessary retransmissions.

Proposal 5: (UE capability-04) A UE that supports the feature of stopping Tx-side transmission of an outdated SDU is not required to support the feature of Rx-side abandonment of an outdated SDU based on a new RLC timer.

    - Impact on PDCP t-Reordering timer for avoiding unnecessary RLC retx

When t-RxDiscard expires, the RLC entity stops waiting to receive the obsolete RLC SDUs, and the corresponding RLC SDUs will be positively acknowledged in the STATUS report. However, due to the fake ACK, the transmitting PDCP entity will consider these RLC SDUs as being successfully received based on the STATUS report, while the receiving PDCP entity will still be waiting to receive the RLC SDUs from the RLC entity until the t-Reordering timer expires. In this case, HFN asynchronization between the transmitting PDCP entity and the receiving PDCP entity may occur. For example, the transmitting PDCP entity may consider the current HFN for a PDCP SDU to be X+1 based on the fake ACK from the STATUS report, while the receiving PDCP entity may consider it to be X.

Observation 2: HFN asynchronization between the transmitting PDCP entity and receiving PDCP entity may occur when the feature of avoiding unnecessary RLC retransmission is configured.

Furthermore, if t-RxDiscard expires while t-Reordering is still running and the PDCP entity is waiting for the outdated SDUs from the RLC layer, this can lead to unnecessary delays in packet delivery.

Observation 3: If the t-RxDiscard expires while t-Reordering is still running and receiving PDCP entity is waiting for the outdated SDUs from the RLC layer, it will cause unnecessary packet delivery delay.

To solve this problem, a straightforward solution is to make the receiving RLC entity indicate SNs of obsolete RLC SDUs when the t-RxDiscard expires, so that the receiving PDCP entity can update its state variable to maintain HFN synchronization with the transmitting PDCP entity and also deliver the buffered PDCP SDUs quickly to the upper layer.

Proposal 6: The receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard  expires for HFN synchronization and avoiding packet delivery delay.

  - Remaining issues for ensuring timely RLC retransmissions

    - Coexistence of remaining time based RLC retransmission and polling

When both remaining time based RLC polling and remaining time based RLC retransmission are configured for the same Tx RLC entity, under reasonable network configuration, the threshold for enabling remaining time based RLC retransmission is expected to be set lower than that for enabling remaining time based RLC polling. Otherwise, the configuration for enabling remaining time based RLC polling will become useless since remaining time based RLC retransmission would have already been triggered before the UE polls the Rx RLC entity to request the STATUS report.

Additionally, unnecessary retransmission can reduce overall system capacity. Therefore, remaining time based RLC retransmission should only be utilized when the RLC SDU(s) are particularly delay-critical. For RLC SDU(s) that are not delay-critical, polling enhancement should be preferred to avoid resource wastage caused by unnecessary retransmission.

Consequently, when both remaining time based RLC retransmission and remaining time based RLC polling are configured by the network, the threshold for enabling remaining time based RLC retransmission should be lower than that for enabling remaining time based RLC polling (refer to Figure 1 below).

Figure 1. Different thresholds for enabling polling enhancement and retransmission enhancement

Proposal 7: The threshold for enabling remaining time based RLC retransmission is expected to be lower than the threshold for enabling remaining time based RLC retransmission when both are configured to the same RLC entity.

    - Coexistence of remaining time based RLC retransmission and ARQ

During the post-meeting email discussion, Apple proposed a coexistence issue regarding remaining time based RLC retransmission and ARQ. Specifically, it was suggested that the pending remaining time based RLC retransmission should be canceled upon receiving a positive ack for the corresponding RLC SDU. This proposal is technically right, as RLC retransmission for the RLC SDU is unnecessary once a positive ACK is received.

If we assume that upon receiving a positively ACK for an RLC SDU, the corresponding RLC SDU in the transmission buffer and the retransmission buffer will be discarded, then there will be no opportunity to initiate a remaining time based RLC retransmission for the corresponding SDU. This implies that the pending remaining time based RLC retransmission for the SDU will also be canceled. However, whether specification impact is required may need to be discussed by RAN2.

Proposal 8: The pending remaining time based RLC retransmission should be cancelled when the RLC entity receives a positively ACK for the corresponding RLC SDU, RAN2 to discuss whether there are any specification impacts.

- Conclusion

In this contribution, we discuss the two directions of RLC AM enhancements, and we have the following observations and proposals:

Remaining issues for ensuring timely RLC retransmissions

Observation 1: There is no issue regarding polling retransmission in the scenario where only SDUs with stopped transmissions are buffered, and no window stalling occurs.

Observation 2: HFN asynchronization between the transmitting PDCP entity and receiving PDCP entity may occur when the feature of avoiding unnecessary RLC retransmission is configured.

Observation 3: If the t-RxDiscard expires while t-Reordering is still running and receiving PDCP entity is waiting for the outdated SDUs from the RLC layer, it will cause unnecessary packet delivery delay.

Proposal 1: (RLC-10) RAN2 assumes network implementation should ensure without any specification impact:

- If DL stop retransmission obsolete SDU is supported, the DL t-RxDiscard should be configured at the UE;

- If UL stopReTxObsoleteSDU is configured, timer-based Rx discard should be activated in the network.

Proposal 2: (RLC-11) RAN2 confirms that there is no need to address the window stalling issue with polling retransmission.

Proposal 3: (RLC-12) No special handling is required in Rel-19 for PDCP SN gap report during UE mobility.

Proposal 4: (RLC-14) When indicated from upper layer to discard a particular RLC SDU, the transmitting side of RLC entity shall discard the corresponding RLC SDU or RLC SDU segment(s) if those have been submitted to lower layers.

Proposal 5: (UE capability-04) A UE that supports the feature of stopping Tx-side transmission of an outdated SDU is not required to support the feature of Rx-side abandonment of an outdated SDU based on a new RLC timer.

Proposal 6: The receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard  expires for HFN synchronization and avoiding packet delivery delay.

Remaining issues for avoiding unnecessary retransmissions

Proposal 7: The threshold for enabling remaining time based RLC retransmission is expected to be lower than the threshold for enabling remaining time based RLC retransmission when both are configured to the same RLC entity.

Proposal 8: The pending remaining time based RLC retransmission should be cancelled when the RLC entity receives a positively ACK for the corresponding RLC SDU, RAN2 to discuss whether there are any specification impacts.

- References

- R2_130_ChairNotes.

- R2-2505403, Summary of RLC open issue list for R19 XR, vivo.