3GPP TSG-RAN WG2 Meeting #131	R2-2505433

Bengaluru, India, 25-29 August, 2025

Agenda Item:	 8.13.2

Source:	 Huawei, HiSilicon

Title:	 Relay discovery and (re)selection for multi-hop Relay

Document for:  Discussion and Decision

# 1	Introduction

In RAN2 #130 meeting, we had made some progress on the Relay discovery and (re)selection procedures for multi-hop relay, and reached the following agreements. The FFSs and open issues are highlighted in yellow [1].

RAN2#130 meeting agreements [1]:

Agreements on Relay discovery and (re)selection: - The notification message is used for the L2 child UE in case of upstream relay (re)selection triggered by AS layer failure cases (Uu/PC5 link failure/release, using legacy cause value) where the upper layer does not trigger link release towards the child. For reselection triggered by upper layers we rely on the upper-layer notification/release mechanism already defined. - Remote UE handling of the notification is up to upper layer implementation. - Further handling of any inter-layer issues on this topic can be done in maintenance phase. - LS to SA2 to notify them of these agreements. - Leaving remote UE handling of the notification to implementation applies to the upper-layer notification for remote UEs in RRC_IDLE/RRC_INACTIVE. UEs in RRC_CONNECTED follow the later agreement to trigger RRC reestablishment. No case is expected where a UE in RRC_CONNECTED receives the upper-layer notification. - Notification is always sent for AS failure cases at the intermediate relay UE (e.g., upstream RLF that prevents the intermediate relay from having a path to the network), using the legacy mechanism. FFS new cause values. - In principle, RAN2 consider that no AS-layer notification is sent by an intermediate relay UE in RRC_IDLE/RRC_INACTIVE when the AS-visible parameters that could affect relay reselection are not changed. FFS which parameters are related. This issue can be revisited if it is found that there are parameters not visible to the AS layer that would affect reselection and force the remote UE to be notified. FFS if notification behaviour can be optimized for intermediate relay UEs in RRC_CONNECTED. - Upon receiving the notification message or upon PC5 link release, remote UE in RRC_CONNECTED needs to initiate RRC Reestablishment as legacy. - When the intermediate Relay UE in RRC_CONNECTED receives the notification message from its parent intermediate Relay UE/last Relay UE or when the PC5 link is released, it initiates RRC Reestablishment.

In this paper, we discuss the FFSs and open issues that are listed in [2] for the Relay discovery and (re)selection procedures.

# 2	Discussion

## Open issues related to the Relay discovery and (re)selection

### Open issue RRC-6: Discovery Model B with no PC5 Link in multi hop scenario

In the previous meeting there were discussions on Discovery Model B, specifically for scenarios where no PC5 link is established between a parent Relay UE and the Remote UE. It was agreed that in Discovery Model B, when there is no established PC5 link between the Remote UE and the Intermediate Relay UE, the Intermediate Relay UE should forward the solicitation message only if the PC5 RSRP between the Remote UE (or First Relay UE) and the Intermediate Relay UE is above a defined threshold.

However, one open point that needs further discussion is regarding the scenario when  there is no established PC5 link between the last relay UE and the intermediate relay UE, whether the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE.

First, we need to determine whether this scenario is even possible, or whether the Intermediate Relay UE always first establishes itself as a Remote UE. One potential scenario is that the intermediate relay UE has just powered on and before establishing itself as a Remote UE, it receives a discovery solicitation message from a nearby Remote UE. If the SD-RSRP of the received discovery solicitation message is above a pre-configured threshold, the Intermediate Relay UE may. send/forward the solicitation message to nearby candidate last relay UEs.

In such case it seems reasonable that the candidate last relay UE should also check if the SD-RSRP of the received discovery solicitation message is above a pre-configured threshold. This ensures that the quality of resulting multi-hop path is  sufficient for  communication.

Another possible scenario involves a UE that is capable of functioning as an intermediate relay UE (i.e., it supports the Rel-19 multi-hop relay feature), but is not currently operating as a relay in a multi-hop configuration, as illustrated in Figure 1. In this figure, the UE is capable of acting as an intermediate relay UE but is currently connected to a Rel-17 single-hop relay. In such a scenario, if a multi-hop remote UE attempts to discover a valid multi-hop path, the intermediate relay UE can participate in Model B discovery by forwarding the discovery message. This enables the remote UE to identify potential relay paths.

Since the PC5 link between the intermediate relay UE and the last relay UE has not yet been established, the last relay UE will evaluate the AS conditions between itself and the intermediate relay UE before deciding whether to send a response message.

Figure 1. Intermediate Relay UE participating in Model B multi-hop relay discovery.

Observation 1: The scenario where there is no established PC5 link between the last relay UE and the intermediate relay UE, and the intermediate relay UE sends/forwards the discovery solicitation message received from a nearby Remote UE before establishing itself as a Remote UE could happen when

- The intermediate relay UE has just powered on.

- A UE that is capable of functioning as an intermediate relay UE, but not currently operating as a relay in a multi-hop configuration, participates in Model B discovery by forwarding the discovery Solicitation message.

Given that such scenario is possible, the next question is whether the same SD-RSRP threshold (in SL-RelayUE-ConfigMH-r19) — that is used by the Intermediate Relay UE to decide whether to forward the solicitation message—can also be applied by the Last Relay UE when determining whether to send a discovery response message to the Intermediate Relay UE. Based on our understanding, if support for this scenario is required, it would be straightforward for the Last Relay UE to apply the same threshold when deciding whether to send the discovery response message.

Proposal 1: If the Last Relay UE needs to determine whether to send a discovery response message to the Intermediate Relay UE, it is proposed that the same SD-RSRP threshold condition used by the Intermediate Relay UE to decide whether to forward the solicitation message be applied. This ensures consistency and simplifies the decision-making process for discovery response handling.

### Open Issue RRC-12: FFS Cause Value / Indication Type for Notifications from Intermediate Relay UE

When an intermediate relay UE receives a NotificationMessageSidelink message from the parent, it has two options

Option 1 –  Forward a copy of the original notification (with the same cause) to its child UEs.
Option 2 –  Generate a new notification message and send it to the child UEs, with an updated cause that reflects its own response, such as relay reselection or re-establishment.

Based on our understanding the issue with Option 1 is that it will lead to duplicate notifications being sent by the intermediate relay UE: first, a forwarded copy of the parent’s original notification, and then a second notification reflecting the intermediate relay’s own actions. This redundancy is clearly undesirable as it does not provide the child UE with any additional information.

To avoid this, we suggest adopting Option 2, where the intermediate relay UE generates a new notification message. This message would reflect the intermediate relay's own status or reaction (e.g., relay reselection) and is always sent, allowing the Remote UE to take appropriate action based on the most relevant and current context of its parent UE.

Based on our understanding this approach also aligns with the principles established in Rel-17 and helps minimize the impact on the specifications. Hence we propose

Proposal 2: Upon receiving a NotificationMessageSidelink from the Last Relay UE, the Intermediate Relay UE generates a new notification message reflecting its own status or reaction (e.g., relay reselection). This newly generated message is forwarded to the downstream child UE, enabling the directly or indirectly connected Remote UE to take appropriate action based on the most current and relevant context of its parent UE.

If we agree to go with option 2 as described above, where the intermediate relay UE generates a new notification message and this message would reflect the intermediate relay's own status or reaction (e.g., relay reselection) we can introduce a new indication Type set to e.g. relayUE-RelayReselection

Proposal 3: Upon receiving a NotificationMessageSidelink from the Last Relay UE, the Intermediate Relay UE generates a new notification message reflecting its own status or reaction (e.g., relay reselection) with the new indication Type set to e.g. relayUE-RelayReselection

The Child UE on receiving the message takes appropriate action and performs a relay reselection to re-establish the connectivity with the network. This does not preclude the Child UE to reselect the same parent Relay UE.

Proposal 4: Upon receiving a NotificationMessageSidelink from the parent Relay UE with the Indication Type set to e.g. relayUE-RelayReselection, the child UE performs a relay reselection to re-establish the connectivity with the network.

## Detailed content of the discovery message for relay selection and reselection

There may be several candidate U2N Relay UE(s) or candidate Intermediate Relay UE(s) near the U2N Remote UE during selection and reselection. According to the current spec, the only information available to the Remote UE regarding these candidate relay UE(s) is the PC5 link quality between the remote UE and candidate relay UE. For multi-hop relay, this information may not be sufficient to select a good relay UE.

Considering the case where the Remote UE discovers the following candidate relay UE(s).

- Relay UE 1: The SD-RSRP between the remote UE and the candidate relay UE is value 1, and the relay UE 1 is connected to the gNB through a U2N Relay UE. Value 1 satisfies the relay selection criterion, as it is above the configured threshold.

- Relay UE 2: The SD-RSRP between the remote UE and the candidate relay UE is value 2, and the relay UE 2 is connected to the gNB through an Intermediate Relay UE and then a U2N Relay UE. Value 2 satisfies the relay selection criterion, and is larger than value 1.

Based on the PC5 link quality, the Remote UE is likely to select the relay UE 2, as the PC5 link quality between the Remote UE and the relay UE2 is better. However, from the QoS perspective, the Relay UE 1 may be better since the hop count between the relay UE 1 and the network is smaller. Therefore, we propose that the hop number of each candidate Intermediate Relay UE be indicated to the U2N Remote UE during the discovery procedure. This additional information could help the Remote UE to select a better relay UE.

Proposal 5: RAN2 should discuss whether/how to support indicating the hop count between  the candidate Intermediate Relay UE and  the U2N Remote UE during the discovery procedure in AS layer.

Further, consider the following cases that remote UE discovers the candidate relay UE 3 and candidate relay UE 4:

- Relay UE 3: The SD-RSRP between the remote UE and the candidate relay UE 3 is value 1, and the relay UE 3 is connected to the gNB through an Intermediate Relay UE 5 and a last Relay UE Relay UE 6. Value 1 satisfies the relay selection criterion, as it is above the configured threshold.

- Relay UE 4: The SD-RSRP between the remote UE and the candidate relay UE 4 is value 1, and the relay UE 2 is connected to the gNB through an Intermediate Relay UE 7 and a last Relay UE 8. Value 2 satisfies the relay selection criterion, and is larger than value 1.

In above two cases, remote UE would consider relay UE 3 and relay UE 4 are both good, and select one of them based on the hop count information and the SD-RSRP between the remote UE and the candidate relay UE. However, the sidelink RSRP and Uu RSRP of Intermediate Relay UE 5 and last Relay UE Relay UE 6 may be better than the sidelink RSRP and Uu RSRP of Intermediate Relay UE 7 and last Relay UE Relay UE 8. Actually, it is better to select relay UE 3 since the link quality on each hop over the path of relay UE 3 is better than that of relay UE 4. Therefore, it is better that the remote UE can know the condition of each hop of the path related to a candidate relay UE.

Proposal 6. It is beneficial for the Remote UE to know the link condition (SD-RSRP or SL-RSRP) of each hop over the path to select a better candidate relay UE or path.

## 2.4 Support of multiple RSC (Relay Service Code)

SA2 has agreed that the policy of RSC (Relay Service Code) based on single-hop relay [3] can be reused in multi-hop relay [3].

A 5G ProSe Remote UE and a 5G ProSe UE-to-Network Relay shall set up a separate PC5 unicast links if an existing unicast link(s) was established with a different Relay Service Code or without a Relay Service Code.

NOTE 5:	A single PC5 unicast link is established between a 5G ProSe Layer-2 UE-to-Network Relay and a 5G ProSe Layer-2 Remote UE, as specified in TS 38.300 [12], for supporting PDU sessions of the 5G ProSe Layer-2 Remote UE.

Each PC5 unicast link for 5G ProSe UE-to-Network Relay is associated with a Unicast Link Profile, which includes:

-	User Info ID and Layer-2 ID of 5G ProSe Remote UE; and

-	User Info ID and Layer-2 ID of 5G ProSe UE-to-Network Relay; and

-	Relay Service Code; and

-	In the case of 5G ProSe Layer-3 UE-to-Network Relay, the network layer protocol and the information about PC5 QoS Flow(s).

Based on the above policy, we need to consider two cases for a multi-hop path:

- All remote UEs on this multi-hop relay path use the same RSC to establish a PC5 connection with the first relay UE on this path.

Case 1

- The remote UEs on this multi-hop relay path use different RSCs to establish their own PC5 connection with the first relay UE on this path.

Case 2

In our views, we need to discuss whether the SA2 policy for RSC has spec impacts on RAN2, especially the Case 2 where the multiple PC5 connections may be established with different RSCs between two relay UEs. Whether RAN2 needs to specify anything to support the Case 2.

Proposal 7: RAN 2 needs to discuss whether support for multiple PC5 connections, each associated with different RSCs, is required between two relay UEs.

# 3	Conclusion

Open issues items related to the Relay discovery and (re)selection.

Open issue RRC-6: Discovery Model B with no PC5 Link in multi hop scenario

Observation 1: The scenario where there is no established PC5 link between the last relay UE and the intermediate relay UE, and the intermediate relay UE sends/forwards the discovery solicitation message received from a nearby Remote UE before establishing itself as a Remote UE could happen when

- The intermediate relay UE has just powered on.

- A UE that is capable of functioning as an intermediate relay UE, but not currently operating as a relay in a multi-hop configuration, participates in Model B discovery by forwarding the discovery Solicitation message.

Proposal 1: If the Last Relay UE needs to determine whether to send a discovery response message to the Intermediate Relay UE, it is proposed that the same SD-RSRP threshold condition used by the Intermediate Relay UE to decide whether to forward the solicitation message be applied. This ensures consistency and simplifies the decision-making process for discovery response handling.

Open Issue RRC-12: FFS Cause Value / Indication Type for Notifications from Intermediate Relay UE

Proposal 2: Upon receiving a NotificationMessageSidelink from the Last Relay UE, the Intermediate Relay UE generates a new notification message reflecting its own status or reaction (e.g., relay reselection). This newly generated message is forwarded to the downstream child UE, enabling the directly or indirectly connected Remote UE to take appropriate action based on the most current and relevant context of its parent UE.

Proposal 3: Upon receiving a NotificationMessageSidelink from the Last Relay UE, the Intermediate Relay UE generates a new notification message reflecting its own status or reaction (e.g., relay reselection) with the new indication Type set to e.g. relayUE-RelayReselection

Proposal 4: Upon receiving a NotificationMessageSidelink from the parent Relay UE with the Indication Type set to e.g. relayUE-RelayReselection, the child UE performs a relay reselection to re-establish the connectivity with the network.

Detailed content of the discovery message for relay selection and reselection

Proposal 5: RAN2 should discuss whether/how to support indicating the hop number of the candidate Intermediate Relay UE to the U2N Remote UE during the discovery procedure in AS layer.

Proposal 6. It is beneficial for the Remote UE to know the link condition (SD-RSRP or SL-RSRP) of each hop over the path to select a better candidate relay UE or path.

Support of multiple RSC (Relay Service Code)

Proposal 7: RAN 2 needs to discuss whether support for multiple PC5 connections, each associated with different RSCs, is required between two relay UEs.

# 4	Reference

- Session Chair Notes on Relay and Positioning RAN2 #130;

- R2-2505432 Open issues for Multi hop Sidelink Relay in TS 38.331

- TS 23.304 V19.3.0, Proximity based Services (ProSe) in the 5G System (5GS).