3GPP TSG-RAN WG2 Meeting #131	R2-2505436

Bengaluru, India, 25-29 August, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on Uplink Capacity Enhancements

Agenda Item:	8.8.3

Document for:	Discussion and decision

# 1	Introduction

The NR NTN WID includes the uplink capacity enhancement as follows [1].

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

RAN1 discussed this objective for several meetings. In RAN1#119, RAN1 agreed to support inter-slot OCC for length 2 and length 4 [2].

Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact. - RAN1 assumes no specification impact for CFO grouping - RAN1 does not pursue closed-loop frequency adjustment commands. - RAN1 assumes that RAN4 does not define new UE requirements for CFO.

In RAN1#120bis [3], the following conclusions and agreements were made:

Agreement OCC length and OCC sequence for OCC with CG PUSCH Type 1 is configured by RRC higher-layers. - Up to RAN2 whether to signal this with one or two RRC parameters - Note: OCC lengths and sequences to be provided in the table of RRC parameters to be prepared by the WI rapporteur Agreement For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI - FFS: with a new field or reusing an existing field

RAN 2 has incorporated OCC related parameters from the R1 parameter list received in R1-2503243 in the RRC Running CR for NR NTN. There is one additional aspects which needs to be agreed and incorporated in the specs which is discussed in this paper.

# 2	Discussion

In the RRC Running CR the following OCC related parameters have been introduced

- occ-LengthAndSequenceIndex - indicates the inter-slot OCC length and sequence index for a Type 1 CG configuration included in ConfiguredGrantConfig

- occ-Length - indicates the inter-slot OCC length for a dynamic grant or a Type 2 CG configuration has been included in PUSCH-TimeDomainResourceAllocationList. For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI.

Additional aspects that needs to be incorporated in the TS38.306 and TS 38.331 running CR is to introduce UE capability signalling for OCC, in line with the RAN 1 agreements.

As noted in RAN1 agreement, separate UE capabilities are proposed to be introduced for OCC length 2 and OCC length 4, with UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Hence we propose

Proposal 1: RAN2 to introduce separate UE capabilities for OCC length 2 and OCC length 4, and UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

# 3	Conclusion

This contribution analyses the current status of OCC signalling, based on RAN1 agreements and the ongoing RRC and UE Capability running CR for the NTN, and proposes the following:

Proposal 1: RAN2 to introduce separate UE capabilities for OCC length 2 and OCC length 4, and UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

# 4	References

- RP-241789, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- Session notes for 9.11 Non-Terrestrial Networks (NTN) for NR Phase 3, Internet of Things (IoT) Phase 3, and IoT-NTN TDD mode, Ad-Hoc Chair (Huawei), RAN1#119

- Chair’s notes for RAN#1 120 bis, Wuhan, China, April 7th-11th, 2025