3GPP TSG-RAN WG2 Meeting #131	R2-2505449

Bengaluru, India, 25-29 August 2025

Agenda item:	8.2.4

Source:	Apple

Title:	Discussion on A-IoT MAC message formats and data transmission

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#130 meeting [1], the following agreements about Data transmission and MAC has been reached:

- Use as baseline the following message names, field names and definitions are to be used in A-IoT MAC: - Message name: A-IoT Paging message, Trigger message (CB exact word), Random ID message, Random ID Response message, R2D Upper Layer Data Transfer message, D2R Upper Layer Data Transfer message. - Field name: R2D Message Type, RA Type, Indication of Paging ID Presence, Length of Paging ID, Paging ID, Transaction ID, Number of Access Occasions, D2R Scheduling Info, Random ID, Echoed Random ID, AS ID, Assigned AS ID, More Data Indication, Length [CB after MAC PDU format discussion], MAC Padding, Received Data Size. - Definitions: - Access occasion: A time-frequency resource for device(s) to transmit Msg1 (i.e., the Random ID message) during a CBRA procedure. - AS ID: The AS layer identifier to address the specific device for R2D reception and D2R scheduling - One bit indication is needed for each echoed random ID in Msg2 to indicate whether AS ID is present (i.e., assigned by reader) for this random ID. - NACK feedback is defined as an explicit message (i.e. new message type). [CB]where AS ID(s) is/are included to indicate the failure for given device(s). - Assume two transport channels are introduced between A-IoT MAC and PHY. One is for R2D, and the other is for D2R. Neither logical channel concept nor SAP is defined for the interface between A-IoT MAC and upper layers. - R2D message scheduling non-first segment (re)transmission does not include upper layer command. - For the first segment and unsegmented packet (re)transmission, the “offset” indicator in R2D is not present. - This implies that the R2D message will either have command or offset (but not both). FFS whether we define two message types or one message type with optional fields. - The device is expected to send a MAC response to the reader in the D2R occasion. The MAC response contains the NAS message if available at the D2R occasion. If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed. - Send LS to CT1 to inform the agreement 1 to CT1 and explain that we have an issue with delayed NAS write success response. RAN2 would prefer that this is handled by CT1 (and give the example of sending NAS response upon successful reception of write command). Ask if this can be handled by CT1 - A mandatory length field directly indicates the length of D2R data MAC SDU to support varying lengths of D2R data. The size of length field is 7-bit in bytes. - The offset indication for transmission/retransmission of the segments after the first segment of a D2R message is 7-bit length in bytes. Segmented SDUs are also byte aligned. - FFS D2R message type. Current running CR will capture no message type, but we can revisit this next meeting and also consider if any other bits are needed for the MAC header - The length field inside MAC for SDU is not needed for R2D messages, assuming R2D MAC padding is not needed. FFS can come back if padding is needed depending on granularity of TBS (only if needed)The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

In this paper, we discuss some remaining issues for the normative work for Ambient IoT MAC layer design, especially on MAC message formats and Data transmission procedures.

# 2 Discussion

### 2.1 D2R Scheduling Information

In current MAC running CR, for the “Frequency Resource Indication” in D2R scheduling info field, it is proposed to have an 8-bit bitmap to represent the frequency domain configurations for D2R scheduling. Note that using a 8-bit bitmap will contain 1-8 possible value for access attempt in frequency domain. This will allow multiple D2R resource to be scheduled at the same time. However, it is also agreed in RAN1 that the actual set of R values are dependent on the configured “Bit duration”, as shown in one of the agreements made in the last RAN1 meeting [2].

For indication of frequency domain resources for Msg 1 transmissions when Y≥1, the reader indicates  a single bit duration Tb which is same for all frequency domain resources  a set of R values, where the possible R values correspond to the agreed table of values of Tb, Tchip, and R § note: the set of R values could be signalled using a bitmap The detailed signalling design is left to RAN2.

Since this single bit duration Tb is fixed for each time the “D2R Scheduling info” is provided, it is possible to let it to be jointly coded with the bitmap for “Frequency Resource Indication”, instead of being separately coded with 3-bit. For example, the following 9-bit coding scheme can work flawlessly, without using 3+8=11 bits “D2R scheduling Info”.

| Code point | Bit Duration | Frequency Resource Indication bitmap |
| --- | --- | --- |
| 9-bit begin MSB= “1” | 2τμs | 8-bit LSBs |
| 9-bit begin MSB= “01” | τμs | 7-bit LSBs |
| 9-bit begin MSB= “001” | τ/2μs | 6-bit LSBs |
| 9-bit begin MSB= “0001” | τ/4μs | 5-bit LSBs |
| 9-bit begin MSB= “00001” | τ/8μs | 4-bit LSBs |
| 9-bit begin MSB= “000001” | τ/16μs | 3-bit LSBs |
| 9-bit begin MSB= “0000001” | τ/32μs | 2-bit LSBs |
| 9-bit begin MSB= “00000001” | τ/96μs | N/A |

As shown above, the 9-bit structure above can represent all possible combinations of “Bit Duration” and “Frequence Resource Indication”.

Proposal 1 	Use a 9-bit field to jointly code “Bit Duration” and “ Frequence Resource Indication “ for D2R scheduling Information, if bitmap is used to indicate frequency resource.

Note that there is also CFRA paging scenario where D2R scheduling information aims to schedule a single resource for a single paged device. Similarly for the command procedure, an MAC layer message containing upper layer data is to be shared with a single device by the D2R transmission on a single D2R resource, which is to be scheduling by the R2D message triggering such a command response. In this case, there is no need to have a bitmap structure to represent the frequency domain resource(s) because only one single frequency shift is to be indicated. Thus, a 3-bit field is enough for those cases.

Proposal 2 	When a single D2R resource is signaled in R2D message, use 3-bit integer to represent “Frequence Resource Indication”.

### 2.2 D2R Message Type

It is assumed that D2R message type is not really needed in Rel-19 A-IoT design because for each “scheduled” D2R transmission resource, the reader is very clear which kind of D2R message is allowed and only one single type of D2R message is allowed in each scheduled resource. For example, any resource indicated in Paging and Access Triger will be only to be used for sending Msg1. From this perspective, it is true that the D2R message type can be omitted.

However, it is unclear whether this assumption is still true in Rel-20 A-IoT system. It is plausible that a Rel-20 D2R resource (dedicatedly scheduled or commonly configured) is allowed to be used by device to send more than one kind of D2R message. Therefore, to avoid a complete re-design of every D2R message format in Rel-20, we prefer to introduce the D2R message type in Rel-19.

Also, it is commonly assumed that a Rel-20 active A-IoT device could be backward-compatible, e.g.,  able to support Rel-19 A-IoT scenarios w/o having a stand-alone Rel-19 A-IoT implementation. In other words, it is highly questionable whether a Rel-20 A-IoT device will need to have two A-IoT radios to communicate with Rel-19 reader and Rel-20 reader in different PDRCH channels.

In general, unable to discern different signaling messages from signaling format design, (but relying on when and where the signaling message is transmitted)  is an exception and risky practice in L2/L3 protocol design for a communication system. Hence, we prefer to have a D2R message type in D2R messages.

Proposal 3 	Message Type is included in D2R message for Rel-19 A-IoT.

### 2.3 “R2D TBS” indication

RAN1 [2] has agreed the following agreement for the R2D message TBS size and requires upper layer signaling.

Agreement R2D postamble is specified with 4 ON chips corresponding to M value of the PRDCH - R2D postamble is added immediately after the PRDCH - R2D postamble has always 4 ON chips - Note: For M=24, 2 ON chips at the end of OFDM symbol for CP handling are in addition to R2D postamble, and are not part of the R2D postamble - R2D padding duration is determined after R2D postamble insertion TBS information for R2D is supported via higher layer R2D control signalling. - Send LS to RAN2 asking to include R2D TBS information (excluding CRC length) in higher layer signaling, at least for messages with variable size. Note: Exact method for determining the end of PRDCH at the device is not specified.

Note that it has been agreed to have a maximum 1000-bit size R2D message and the R2D message is to be byte aligned. So, it is reasonable to assume that a 7-bit “R2DTBS” field could be appended to each R2D message to indicate the size in bytes and satisfy the request of RAN1.  However, there is a discussion on whether we need to make an exception for “Access Trigger” message because this message has a fixed size. Theoretically, “Access Trigger” is a periodically broadcasting DL message which could be extended to contain a variety of information from the reader MAC layer. However, in current Rel-19 design, there is no other functions introduced for this message. So, RAN2 can either decide to make an exception for this message or not. Note that the drawback of making this exception is that the “Access Trigger” message must be first singled out by a device from a PRDCH transmission. From this perspective, it is more convenient for UE to first determine the exact message type of an incoming R2D message and then determine whether this field is absent or present.

Also, since this field is not always present, if not included in Access Trigger, then the meaning of this field is more like a L2 message size field, instead of a L1 TBS parameter. So, we suggest to change it formally as “R2D message size”.

Proposal 4 	If “R2D TBS” is not always included (i.e., only included for variable-size R2D message(s)), this field shall be placed after “R2D Message Type” and renamed as “R2D message size”.

### 2.4 “0 SDU” Message Format

In RAN2#130 meeting [1], RAN2 agreed that when he device is expected to send a MAC response to the reader in the D2R occasion but the NAS messages not available to transmit at the D2R occasion, the device will then send the response contains MAC with 0 SDU and padding as needed. However, there is a controversy issue about whether the “More Data Indication (MDI)” bit should be set or not.

First, it is MAC layer responsibility to set “MDI” bit, not NAS layer. Logically speaking, when “0 SDU” message is sent, the MAC layer is not aware of whether/when the upper layer command response data will be available. Thus, MAC layer should not falsely set this bit to 1 to indicate there is more “data” in MAC buffer waiting to be transmitted in PDRCH.

Second, the “More Data Indication“ is solely introduced for segmentation purpose. In this “0 SDU” message transmission scenario, there is no segmentation involved.

Finally, we think the reader’s MAC layer can understand the meaning of “0 SDU” when it expects to receive a longer message. So, the only possible valid interpretation of “0 SDU” reception is the device is not ready to send an upper layer response yet. There is no need to indicate this meaning by setting “MDI” bit to 1.

Based on the above reasons, we think the MDI bit is to be kept as “0” for “0 SDU” message.

Proposal 5 	More Data Indication should be set to “0” for “0 SDU” MAC message, if included.

Also, it is true that whether the “0 SDU” message is really needed depends on the reply LS from CT1. As RAN2 has already indicated that a NAS layer solution is preferred for this case, maybe there could be an upper-layer based solution w/o introducing signaling in A-IoT MAC layer.

Proposal 6 	Decide whether to include “0 SDU” message in Re-19 MAC layer based on CT1 reply LS.

# 3 Conclusion

In this contribution, we discuss the paging issue for Ambient IoT, and have the following proposals:

Proposal 1 	Use a 9-bit field to jointly code “Bit Duration” and “ Frequence Resource Indication “ for D2R scheduling Information, if bitmap is used to indicate frequency resource.

Proposal 2 	When a single D2R resource is signaled in R2D message, use 3-bit integer to represent “Frequence Resource Indication”.

Proposal 3 	Message Type is included in D2R message for Rel-19 A-IoT.

Proposal 4 	If “R2D TBS” is not always included (i.e., only included for variable-size R2D message(s)), this field shall be placed after “R2D Message Type” and renamed as “R2D message size”.

Proposal 5 	More Data Indication should be set to “0” for “0 SDU” MAC message, if included.

Proposal 6 	Decide whether to include “0 SDU” message in Re-19 MAC layer based on CT1 reply LS.

# 4 References

[1] Chairman’s note R2-130

[2] Chairman’s note R1-121