3GPP TSG-RAN2 Meeting #131	R2-2505459

# Bengalruru, India, Aug 25th – 29th, 2025

Agenda item:		8.11.2 (NR_duplex_evo)

Source:	LG Electronics Inc.

Title: 	Remaining issues on Random Access procedure for SBFD

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 SBFD [1], it is agreed to support the Random Access procedure using the SBFD symbols.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - … - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - …

In MAC running CR discussion [2-3], following open issues are remained:

[Issues for further discussion via contributions] [Open Issue MAC-2-1] Do companies think any additional MAC specification changes (compared to the latest version of the MAC running CR) are needed for capturing the condition of the agreement: SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA? [Open Issue MAC-2-2] If so, please clarify what the changes should be. [Open Issue MAC-3-1] Do companies think any additional specification changes (in MAC and/or RRC) are essential to address the issue on how to derive the SBFD RO locations for CFRA (ZTE’s concern)? Please provide your input with the rationale. [Open Issue MAC-3-2] If so, please provide your preferred solution with the rationale, including but not limited to: - Network can indicate RO type as SBFD RO for CFRA, only if an SBFD RACH configuration is configured for CBRA (proposed by LGE). [Open Issue MAC-4-1] Do companies think the parameter initialization is needed after RO type switching in the case of RACH configuration Option 2? Please provide your input with the rationale. [Open Issue MAC-4-2] If so, please further clarify whether all parameters in RACH-ConfigCommon should be considered for the initialization, or if certain parameters should be excluded from the initialization, along with the rationale. [Open Issue MAC-5] Do companies support introducing a power offset to compensate the power ramping difference between the two RO types (in SBFD RACH configuration Option 2) for RO type switching? Please provide your input with the rationale.

This document discusses highlighted open issues and further correction on RA procedure for SBFD.

2.	Discussion

[MAC-2/3] SBFD RO type indication for CFRA

In RAN2#130 meeting [4], it is agreed that the CBRA resource with same RO type is used for the fallback case from CFRA to CBRA (e.g., if the RSRP is less than the threshold for BFR and L3 HO case), if the network configures the CBRA resource for the same RO type.

SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

However, during the MAC running CR discussion, there may be different understandings on the yellow part, e.g.,

- Q1) whether it is possible that the network indicates one RO type (e.g., SBFD RO) for CFRA without providing CBRA resource for the same RO type (e.g., SBFD RO), and

- Q2) if Q1 is yes, the UE behavior to determine the Random Access resources when the RO type is indicated for CFRA but the UE fallbacks from CFRA to CBRA, e.g., whether the UE is allowed to use the other RO type in this case when the RACH resources for the same RO type is NOT provided for CBRA.

For Q1, in our understanding, it is not reasonable network operation to indicate a specific RO type for CFRA procedure without configuring CBRA resource for the same RO type. Further, if the UE is allowed to use the other RO type for CFRA to CBRA fallback case when the RACH resource for the same RO type is not provided for CBRA (as in Q2), the UE may use different RO type for each RA attempt. For example, if the network indicates the SBFD RO for BFR procedure without configuring CBRA resource with SBFD RO, when the UE initiates a Random Access procedure for BFR, the used RO type may be different for each RA attempt as follows:

- For RA attempt 1, the UE selects the SBFD RO based on the network indication. Then, based on at least one SSB with SS-RSRP above rsrp-ThresholdSSB, the UE performs the CFRA procedure using the SBFD RO. If the RA attempt 1 fails (e.g., due to RAR reception failure), the UE may re-attempt the RA procedure.

- For RA attempt 2, SS-RSRP of all SSB may become lower than rsrp-ThresholdSSB, the UE may perform CBRA procedure using the non-SBFD RO. Further, if the RA attempt 2 fails, the UE may re-attempt the RA procedure.

- For RA attempt 3, SS-RSRP for one SSB may become above rsrp-ThresholdSSB, the UE may perform the CFRA procedure using the SBFD RO again.

However, if UE switches between the SBFD RO and non-SBFD RO for each RA attempt, it may cause CLI impact due to continuously switching the RO type. In addition, it also causes additional UE complexity to determine the power-related parameter, which can be different for each RO type.

Meanwhile, in Rel-17 RACH partitioning, it was discussed which set of Random Access resources should be selected for RedCap UE, in order to determine which RA parameter configured in RACH-ConfigCommon should be used for the fallback case from CFRA to CBRA, considering that multiple RACH-ConfigCommon IEs are defined for feature/feature combinations in RACH partitioning framework [5]. Based on the discussion in RAN2#118 meeting[6], it was agreed to select the set of Random Access resource associated with RedCap indication for CFRA cases (if it is configured), in order to use the RA parameter in RACH-ConfigCommon IE associated with RedCap indication for the fallback case from CFRA to CBRA.

RAN2#118 meeting agreement For the fallback cases from CFRA to CBRA, RedCap UE should select the RedCap specific RACH resource, if it is configured (adopt the text similar to the one in R2-2205941)

Similar to this principle agreed for RedCap, the selection procedure of set of Random Access resource for other feature (e.g. eRedCap, Msg1 repetition) is defined as well based on feature applicability and resource configuration, in order to keep the same feature combination for the fallback case from CFRA to CBRA.

Similarly, for the fallback case from CFRA to CBRA, same RO type should be used in order to avoid additional complexity explained above. However, it is also not reasonable to modify RO type selection procedure for CFRA to consider other than network indication, e.g., further considering whether the same RO type is configured in CBRA resource or not. Therefore, in order to keep the same RO type for the fallback case from CFRA and CBRA without adding additional UE complexity, it would be simpler to indicate SBFD RO for CFRA case only if the SBFD RO is configured for CBRA as well.

Proposal 1. [MAC-3-2] Network can indicate RO type as SBFD RO for CFRA, only if an SBFD RACH configuration is configured for CBRA.

If proposal 1 is agreed, it is not needed to define additional UE behavior considering the case when the network indicates a RO type for CFRA procedure while the RACH resources for the same RO type is NOT provided for CBRA.

Proposal 2. [MAC-2] No need to define a UE behavior for the case that the network indicates a RO type for CFRA procedure without providing the CBRA RACH resources for the same RO type.

During the MAC open issue discussion [3], one question is raised on how determine SBFD RO for CFRA for BFR or L3 HO cases, considering different RO determination rules for each RACH configuration option (i.e., Option 1 or Option 2). However, it should be also noted that only one RACH configuration option is supported in the cell, as agreed in RAN2#128 [7].

Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

Therefore, even though RACH-ConfigGerneric IE is explicitly configured in CFRA configuration (e.g., in RACH-ConfigDedicated IE or in BeamFailureRecoveryConfig IE) it is natural to follow the cell specific configuration to determine SBFD RO, e.g., whether the sbfd-RACHSingleConfig or sbfd-RACHDualConfig is configured in the cell. Therefore, there is no need to further clarify on how to determine SBFD RO for CFRA cases.

Proposal 3. [MAC-3-1] No need to clarify on how to derive the SBFD RO locations for CFRA.

[MAC-4] Re-initialization of RA parameter after RO type switch

According to the current Random Access procedure, after selecting the set of Random Access resources, the UE initialize the RA parameter based on the values for the selected set of Random Access procedure configured by the RRC:

When a Random Access procedure is initiated, UE selects a set of Random Access resources as specified in clause 5.1.1b and initialises the following parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources: - prach-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble for Msg1. These are also applicable to the MSGA PRACH if the PRACH occasions are shared between 2-step and 4-step RA types; (…omitted)

In addition, for Msg1 repetition, if the Msg1 repetition number is increased during the Random Access procedure, the UE re-initialize the RA parameter corresponding to new Msg1 repetition number, in order to select the Random Access resources based on the new RA parameter. Note that only four parameters are re-initialized to increase Msg1 repetition number (i.e., startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup), since the Msg1 repetition number can be increased only in shared RO case and the UE will reuse the other RA parameters within same RACH-CongfigCommon IE.

2> if the Random Access procedure is not completed: 3> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure: 4> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 4> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure; 6> initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

On the other hand, for RACH configuration Option 2, different RACH-ConfigCommon IEs will be used for SBFD RO and non-SBFD RO. In detail, RA parameters for SBFD RO are configured by RACH-ConfigCommon included in SBFD-RACH-DualConfig IE, while the RA parameters for non-SBFD RO are configured by RACH-ConfigCommon IE included directly in BWP-UplinkCommon IE or in AdditionalRACH-Config IE, based on feature combination. In other words, different RA parameters should be applied when the UE selects different RO type between the SBFD RO and non-SBFD RO for RACH configuration Option 2.

Therefore, in order to apply the appropriate RA parameter after RO type is switched from SBFD RO to non-SBFD RO or from non-SBFD RO to SBFD RO, the re-initialization of Random Access parameters is essential at least for RACH configuration Option 2.

Proposal 4. [MAC 4-1] At least for RACH configuration Option 2, re-initialize RA parameter after the UE switches RO type.

For MAC open issue 4-2 (which parameter should be considered for initialization), it should be noted that all parameters can be separately configured except rsrp-ThresholdSSB-SUL (i.e., RSRP threshold for UL carrier selection), as agreed in RAN1#121 meeting [8].

Agreement For RACH configuration Option 2, all parameters in rach-ConfigCommon except for rsrp-ThresholdSSB-SUL can be included in the additional RACH configuration, i.e., sbfd-RACHDualConfig.

Therefore, in order to apply the appropriate RA parameters after RO type switch, all the parameters which can be separately configured for SBFD RO should be re-initialized, i.e., all parameters except rsrp-ThresholdSSB-SUL.

Proposal 5. [MAC 4-2] At least for RACH configuration Option 2, all parameters except for rsrp-ThresholdSSB-SUL should be re-initialized after the UE switches RO type.

[Others] applicability of Msg1 repetition number on SBFD RO

In RAN1#120 meeting [9], it was agreed that if the separated RSRP threshold to determine Msg1 repetition number (i.e., sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8) is not configured for SBFD RO, the RSRP threshold to determine Msg1 repetition number configured for non-SBFD RO (i.e., rsrp-ThresholdMsg1-RepetitionNum2/4/8) is reused to determine applicable Msg1 repetition number(s) for RACH configuration Option 1.

Agreement For RACH configuration Option 1, when separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs is not configured, the rsrp-ThresholdMsg1-RepetitionNum2/4/8 configured for PRACH transmission with preamble repetitions within legacy-ROs is reused for additional-ROs.

Note that reusing the rsrp-ThresholdMsg1-RepetitionNum2/4/8 defined for non-SBFD RO is only agreed for RACH configuration Option 1 based on discussion in [10], since the same RACH partitions (with Msg1 repetition numbers) would be configured in RACH Configuration Option 1, as both RO types share the same RACH-ConfigCommon IE.

(…unrelated part omitted) In last RAN1 meeting, we agreed the separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for RACH configuration Option 1/2. In this meeting, [NTT DOCOMO] propose the clarify the UE behavior when separate rsrp-ThresholdMsg1-RepetitionNum2/4/8 is not configured. According to TS 38.311, rsrp-ThresholdMsg1-RepetitionNum2/4/8 is mandatory if this repetition number is configured and RA resource without Msg1 repetition number or RA resource with lower Msg1 repetition number is also configured. Since same msg1-RepetitionNum is used for legacy-Ros and additional-Ros, it makes sense that re-use the rsrp-ThresholdMsg1-RepetitionNum2/4/8 of legacy-Ros to additional-Ros. Moderator suggests Initial proposal 2-2-1. (…unrelated part omitted) 1st Round Proposals (closed) #### Initial proposal 2-2-1: For RACH configuration Option 1, when separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs is not configured, the rsrp-ThresholdMsg1-RepetitionNum2/4/8 configured for PRACH transmission with preamble repetitions within legacy-ROs is reused for additional-ROs.

However, in the current MAC running CR [2], above agreement is not correctly implemented, since the rsrp-ThresholdMsg1-RepetitionNum2/4/8 defined for non-SBFD RO is applied when there is no separated RSRP threshold to determine Msg1 repetition number for SBFD RO, regardless of which RACH configuration Option is configured in the cell.

2> if the RO_TYPE is set to 2nd-RO: 3> if the BWP selected for the Random Access procedure is configured with set(s) of Random Access resources associated with Msg1 repetition number 8, and the RSRP of the downlink pathloss reference is less than sbfd-RSRP-ThresholdMsg1-RepetitionNum8 if configured, or less than rsrp-ThresholdMsg1-RepetitionNum8 otherwise: 4> assume Msg1 repetition is applicable and Msg1 repetition number applicable for the current Random Access procedure includes 8. 3> if the BWP selected for the Random Access procedure is configured with set(s) of Random Access resources associated with Msg1 repetition number 4, and the RSRP of the downlink pathloss reference is less than sbfd-RSRP-ThresholdMsg1-RepetitionNum4 if configured, or less than rsrp-ThresholdMsg1-RepetitionNum4 otherwise: 4> assume Msg1 repetition is applicable and Msg1 repetition number applicable for the current Random Access procedure includes 4. 3> if the BWP selected for the Random Access procedure is configured with set(s) of Random Access resources associated with Msg1 repetition number 2, and the RSRP of the downlink pathloss reference is less than sbfd-RSRP-ThresholdMsg1-RepetitionNum2 if configured, or less than rsrp-ThresholdMsg1-RepetitionNum2 otherwise: 4> assume Msg1 repetition is applicable and Msg1 repetition number applicable for the current Random Access procedure includes 2. 3> else if the RSRP of the downlink pathloss reference is not less than any configured sbfd-RSRP-ThresholdMsg1-RepetitionNumX, and not less than any configured rsrp-ThresholdMsg1-RepetitionNumX if the sbfd-RSRP-ThresholdMsg1-RepetitionNumX is not configured for the corresponding Msg1 repetition number: 4> assume Msg1 repetition is not applicable for the current Random Access procedure.

Reusing the rsrp-ThresholdMsg1-RepetitionNum2/4/8 for non-SBFD RO to determine Msg1 repetition number in RACH configuration Option 2 may cause an issue, since the different Msg1 repetition number(s) can be configured in SBFD RO and non-SBFD RO. For example, assume that the RACH partition(s) (i.e., set(s) of Random Access resources) are configured as follows for SBFD RO and non-SBFD RO:

- For SBFD RO,

  - RACH partition A1: Msg1 repetition only, RepNum = 4

- For non-SBFD RO

  - RACH partition B1: Msg1 repetition only, RepNum = 2

  - RACH partition B2: Msg1 repetition only, RepNum = 4

Then, for non-SBFD RO, rsrp-ThresholdMsg1-RepetitionNum4 is needed for non-SBFD RO in order to select between RACH partition B1 and RACH partition B2 based on the channel condition. On the other hand, for SBFD RO, sbfd-RSRP-ThresholdMsg1-RepetitionNum4 for SBFD RO is not needed since the UE will select RACH partition A1 regardless of channel condition, once the UE selects the SBFD RO.

However, if rsrp-ThresholdMsg1-RepetitionNum4 defined for non-SBFD RO is reused to determine Msg1 repetition number in this case, the SBFD-aware UE may not be able to select the Msg1 repetition number 4, even though only Msg1 repetition number 4 is configured in this BWP for SBFD RO. The correct behaviour for this case is to always determine that Msg1 repetition number 4 is applicable if the UE selects the SBFD RO (without apply any RSRP threshold for Msg1 repetition number), not to reuse the rsrp-ThresholdMsg1-RepetitionNum4 defined for non-SBFD RO.

Therefore, the MAC specification should be changed in order to reuse the rsrp-ThresholdMsg1-RepetitionNum2/4/8 defined for non-SBFD RO only in RACH configuration Option 1, which is exactly same as RAN1 agreement.

Proposal 6. Only for RACH configuration Option 1, reuse the rsrp-ThresholdMsg1-RepetitionNum2/4/8 to determine Msg1 repetition number in SBFD RO if sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 is not configured.

Then, it should be determined how to determine the Msg1 repetition number if sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 is not configured in RACH Configuration Option 2. In order to avoid any additional issue considering various cases for RACH partition configuration, it would be safer to determine Msg1 repetition number in SBFD RO only based on the separated threshold (i.e., sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8).

Proposal 7. For RACH configuration Option 2, determine Msg1 repetition number only by sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 (i.e., not reuse rsrp-ThresholdMsg1-RepetitionNum2/4/8).

3.	Conclusion

In this contribution, it is discussed about RAN2 aspects on Random Access operation for SBFD. This document includes following proposals.

Proposal 1. [MAC-3-2] Network can indicate RO type as SBFD RO for CFRA, only if an SBFD RACH configuration is configured for CBRA.

Proposal 2. [MAC-2] No need to define a UE behavior for the case that the network indicates a RO type for CFRA procedure without providing the CBRA RACH resources for the same RO type.

Proposal 3. [MAC-3-1] No need to clarify on how to derive the SBFD RO locations for CFRA.

Proposal 4. [MAC 4-1] At least for RACH configuration Option 2, re-initialize RA parameter after the UE switches RO type.

Proposal 5. [MAC 4-2] At least for RACH configuration Option 2, all parameters except for rsrp-ThresholdSSB-SUL should be re-initialized after the UE switches RO type.

Proposal 6. Only for RACH configuration Option 1, reuse the rsrp-ThresholdMsg1-RepetitionNum2/4/8 to determine Msg1 repetition number in SBFD RO if sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 is not configured.

Proposal 7. For RACH configuration Option 2, determine Msg1 repetition number only by sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 (i.e., not reuse rsrp-ThresholdMsg1-RepetitionNum2/4/8).

4.	Reference

[1] RP‑241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2] R2-2505575 Introduction of Rel-19 Evolution of NR duplex operation (SBFD) for MAC spec

[3] R2-2505560 Summary of the SBFD open issues in MAC

[4] Report of 3GPP TSG RAN WG2 meeting #130,St Julian's, Malta, 19 - 23 May, 2025

[5] R2-2206482 [AT118-e][508][RA Part] UP open issues and CR 38.321 (ZTE) - Report

[6] Report of 3GPP TSG RAN WG2 meeting #118-e, Online, 9 - 20 May, 2022

[7] Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA, 18 - 22 November, 2024

[8] Report of 3GPP TSG RAN WG1 #121

[9] Report of 3GPP TSG RAN WG1 #120

[10] R1-2501416 Summary#2 on SBFD random access operation