3GPP TSG-RAN2#131 	R2-2505463

Bengaluru, India, 25th Aug – 29th Aug, 2025

Agenda item:		8.4.4 (NR_LPWUS-Core)

Source:	LG Electronics Inc.

Title: 	Remaining issues on LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

1.	Introduction

In this contribution, we present our view on LP-WUS with dual DRX groups in RRC_CONNECTED and issue related to collision case.

2.	Discussion

In post e-mail discussion [1], RAN2 discussed open issues on LP-WUS, and one scope is LP-WUS with dual DRX group.

In the last meeting, RAN2 made WA to support LP-WUS with dual DRX group as follows.

Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime. When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

In addition, in RAN1#121, RAN1 agreed that the UE is not able to operate LR and MR simultaneously in Rel-19.

Agreement As the reply to RAN2 LS in R1-2503616, RAN1 assumes that UE is not able to operate LR and MR simultaneously in Rel-19. RAN1 understanding is that the terminology of LR and MR operations are for discussion purpose and will not be specified - LR operation is the UE operation for LP-WUS monitoring - MR operation is the UE operation for all other NR signals/channels transmissions/receptions in connected mode

During [1], RAN2 discussed whether to monitor LP-WUS on PCell if the secondary DRX group is in Active time. Based on the RAN1 agreement, RAN2 thought that if any of DRX groups is in Active time, the UE is using MR and cannot use LR, and, rapporteur suggested proposals as follows.

- Proposal 1: Confirm the following working assumption to support LP-WUS with dual DRX group

- Proposal 2: If secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time.

However, one concern was raised whether the restriction that the UE cannot operate MR and LR simultaneously is applied to DC or not. If applied, when the UE uses MR for one cell group, the UE cannot use LR for the other cell group even if the other cell group is not in Active time. i.e. MR state of one cell group impacts to LR state of the other cell group. Thus, in order to avoid LR and MR simultaneous operation, it is required that MN and SN coordinate scheduling.

In addition, it was also argued that the restriction cannot be applied to dual DRX group with FR1-FR2. This is because LR can be separately implemented for FR1-FR2, so it would be possible that MR and LR operate in FR1 and FR2 differently, e.g. MR and LR operate simultaneously in FR1 and FR2 respectively.

For further discussion, rapporteur captured a proposal as follows.

- Proposal 2a: If secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time. Regarding the RAN1 agreement on not supporting simultaneous LR and MR operation, further check with RAN1 whether the agreement is applicable to DC and CA, and whether it has any impact on per CG DRX operation.

First, currently, MR is separately implemented per cell group for DC case. Thus, the raised concern is happened only when one LR is shared by both MR. However, in our understanding, RAN1 assumes that LR is also implemented per cell group. Thus, MR state of one cell group does not impact to LR state of the other cell group.

Anyway, we think whether LR is separately implemented per cell group or not should be clarified, and RAN2 sends LS to ask this. If RAN1 confirm that LR is separately implemented per cell group in DC case, RAN2 applies the restriction only for the same cell group.

Proposal 1. RAN2 sends LS to RAN1 to ask whether LR is separately implemented per cell group in DC case.

Proposal 2. If RAN1 confirms that LR is separately implemented per cell group in DC case, the restriction that UE cannot operate MR and LR simultaneously is applied only for the same cell group.

Next, regarding separate LR for FR1 and FR2, this is RAN1 scope and is discussing in RAN1. In addition, even if the simultaneous operation is not allowed with the separate LR, we don't see any issue on Rel-19 LP-WUS operation.

Therefore, we propose not to consider the simultaneous operation of MR and LR for dual DRX group with FR1-FR2 until there is RAN1 input.

Proposal 3. RAN2 does not consider the simultaneous operation of MR and LR for dual DRX group with FR1-FR2 until there is RAN1 input.

In addition to the simultaneous operation, there is some remaining issues for CA case.

For Option 1-1 with single DRX group, we think that the UE behaviour is clearly defined, e.g. when the UE monitors LP-WUS and when the UE starts drx-onDurationTimer.

- If the UE can monitor LP-WUS (i.e. in non-Active time)

  - If LP-WUS is detected, the UE starts drx-onDurationTimer.

  - If LP-WUS is not detected, the UE does not start drx-onDurationTimer.

- If the UE cannot monitor LP-WUS (i.e. in Active time)

  - The UE starts drx-onDurationTimer.

In addition, with dual DRX group, the UE behavior is also clearly defined for the case where the UE can monitor LP-WUS. However, there is no clear agreement on how the UE handles drx-onDurationTimer when the UE is not able to monitor LP-WUS. As MR and LR cannot operate simultaneously, if at least one DRX group is in Active time, the UE cannot monitor LP-WUS. Thus, "the UE is not able to monitor LP-WUS" includes "both DRX groups are in Active time" or "only one DRX group is in Active time".

- If the UE can monitor LP-WUS (i.e. both DRX group is in non-Active time)

  - If LP-WUS is detected, the UE starts drx-onDurationTimer in both DRX group.

  - If LP-WUS is not detected, the UE does not start drx-onDurationTimer in both DRX group.

- If the UE cannot monitor LP-WUS (i.e. at least one DRX group is in Active time)

  - ???

For dual DRX groups, we think that the same principle as single DRX group can be applied easily, i.e., when the UE is not able to monitor LP-WUS, the UE starts the next drx-onDurationTimer in both DRX groups.

Proposal 4. In Option 1-1, when at least one DRX group is in Active time and LP-WUS occasion is overlapped with the Active time, the UE starts the next drx-onDurationTimer in both DRX groups.

Note that the discussion on the UE behaviour in Option 1-2 is related to potential collision case.

The other scope in [1] is the UE operation for potential collision case, and there is some agreement in RAN2#129bis and RAN1#121.

[RAN2#129bis] Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

[RAN1#121] Agreement As the initial reply to RAN2 LS in R1-2503616, RAN1 confirms that at least the collision with Active Time, measurement gap, and RAR window monitoring for BFR can be considered for the cases/scenarios on when the UE is not able to monitor LP-WUS.

In [1], for Option 1-1, all companies agree to confirm above WA and we think that there is no issue on Option 1-1.

However, for Option 1-2, companies’ views were diverse on whether to start lpwus-PDCCHMonitoringTimer. Some companies thought that lpwus-PDCCHMonitoringTimer should be started to avoid scheduling delay as in Option 1-1 because lpwus-PDCCHMonitoringTimer would be shorter than the periodicity of LP-WUS occasion. The other companies thought that lpwus-PDCCHMonitoringTimer should not be started because the periodicity of LP-WUS occasion can be shorter than lpwus-PDCCHMonitoringTimer, so if lpwus-PDCCHMonitoringTimer is started, MR would be always waked up.

Based on this discussion, rapporteur suggested the compromised solution.

- Proposal 6: For Option 1-2, NW configures UE whether to start the lpwus-PDCCHMonitoringTimer in collision cases, i.e. when the UE is not able to monitor the LP-WUS occasion(s).

We think that the compromised solution is reasonable, which provide the flexibility to the network to control lpwus-PDCCHMonitoringTimer and the periodicity of LP-WUS occasion.

Proposal 5. For Option 1-2, NW configures UE whether to start the lpwus-PDCCHMonitoringTimer in collision cases.

If RAN2 goes with the compromised solution, the UE behaviour on single DRX group can be as follows.

- If the UE can monitor LP-WUS (i.e. Not in collision case)

  - If LP-WUS is detected, the UE starts lpwus-PDCCHMonitoringTimer.

  - If LP-WUS is not detected, the UE does not start lpwus-PDCCHMonitoringTimer.

- If the UE cannot monitor LP-WUS (i.e. in collision case)

  - If the UE is configured to start lpwus-PDCCHMonitoringTimer for the collision case, the UE starts the next lpwus-PDCCHMonitoringTimer.

  - If the UE is configured not to start lpwus-PDCCHMonitoringTimer for the collision case, the UE does not start the next lpwus-PDCCHMonitoringTimer.

In addition, the UE behaviour on dual DRX group can be as follows.

- If the UE can monitor LP-WUS (i.e. both DRX group is in non-Active time)

  - If LP-WUS is detected, the UE starts lpwus-PDCCHMonitoringTimer in both DRX group.

  - If LP-WUS is not detected, the UE does not start lpwus-PDCCHMonitoringTimer in both DRX group.

- If the UE cannot monitor LP-WUS (i.e. at least one DRX group is in Active time)

  - If the UE is configured to start lpwus-PDCCHMonitoringTimer for the collision case, the UE starts the next lpwus-PDCCHMonitoringTimer in both DRX groups.

  - If the UE is configured not to start lpwus-PDCCHMonitoringTimer for the collision case, the UE does not start the next lpwus-PDCCHMonitoringTimer in both DRX groups.

Proposal 6. For single DRX group in Option 1-2,

- If the UE can monitor LP-WUS (i.e. Not in collision case)

  - If LP-WUS is detected, the UE starts lpwus-PDCCHMonitoringTimer.

  - If LP-WUS is not detected, the UE does not start lpwus-PDCCHMonitoringTimer.

- If the UE cannot monitor LP-WUS (i.e. in collision case)

  - If the UE is configured to start lpwus-PDCCHMonitoringTimer for the collision case, the UE starts the next lpwus-PDCCHMonitoringTimer.

  - If the UE is configured not to start lpwus-PDCCHMonitoringTimer for the collision case, the UE does not start the next lpwus-PDCCHMonitoringTimer.

Proposal 7. For dual DRX groups in Option 1-2,

- If the UE can monitor LP-WUS (i.e. both DRX group is in non-Active time)

  - If LP-WUS is detected, the UE starts lpwus-PDCCHMonitoringTimer in both DRX group.

  - If LP-WUS is not detected, the UE does not start lpwus-PDCCHMonitoringTimer in both DRX group.

- If the UE cannot monitor LP-WUS (i.e. at least one DRX group is in Active time)

  - If the UE is configured to start lpwus-PDCCHMonitoringTimer for the collision case, the UE starts the next lpwus-PDCCHMonitoringTimer in both DRX groups.

  - If the UE is configured not to start lpwus-PDCCHMonitoringTimer for the collision case, the UE does not start the next lpwus-PDCCHMonitoringTimer in both DRX groups.

Meanwhile, in DCP, the collision cases are defined in 38.300 [2], and RAN2 sent LS to ask RAN1 to check the potential collision case for LP-WUS.

A UE can only be configured to monitor DCP when connected mode DRX is configured, and at occasion(s) at a configured offset before the on-duration. If short DRX cycle is configured, DCP is not applicable when short DRX cycle is used. More than one monitoring occasion can be configured before the on-duration. The UE does not monitor DCP on occasions occurring during active-time, measurement gaps, BWP switching, or when it monitors response for a CFRA preamble transmission for beam failure recovery (see clause 9.2.6), in which case it monitors the PDCCH during the next on-duration. If no DCP is configured in the active BWP, UE follows normal DRX operation.

In Reply LS [3], RAN1 confirms Active time, measurement gap and RAR window for BFR as a collision case, but BWP switching is not included.

LP-WUS monitoring Agreement: As the initial reply to RAN2 LS in R1-2503616, RAN1 confirms that at least the collision with Active Time, measurement gap, and RAR window monitoring for BFR can be considered for the cases/scenarios on when the UE is not able to monitor LP-WUS.

In our understanding, this means that the UE needs to monitor LP-WUS during BWP switching.

However, this brings a problematic situation. For example, if bwp-InactivityTimer expires near LP-WUS occasion, BWP switching may be overlapped with LP-WUS occasion. In this case, according to RAN1 agreement, the UE is required to monitor LP-WUS and to perform BWP switching simultaneously, but at BWP switching, the UE wakes up MR. In other words, the UE is required to operate MR and LR simultaneously, but this is contradic to the othe RAN1 agreement that the UE cannot operate MR and LR simultaneously.

Note that BWP switching can be happened based on DCI, RRC signaling and RA procedure, but the UE is in Active time in these cases, which is already included in the collision case.

In order to resolve this issue, we think there are three options.

- Option 1. RAN2 considers BWP switching as a collision case and sends LS to RAN1 to confirm this.

- Option 2. Upon bwp-Inactivitytimer expiry, the UE keeps monitor LP-WUS, then the UE performs BWP switching when the UE wakes up MR.

- Option 3. When the UE monitors LP-WUS, the UE stops/suspends bwp-Inactivitytimer, and when the UE detects LP-WUS, the UE restarts/resumes bwp-Inactivitytimer.

We think that Option 3 is against to the motivation of bwp-Inactivitytimer. In Rel-15, bwp-InactivityTimer was introduced to measure the time that BWP is not used. If the BWP is not used during bwp-InactivityTimer, it is considered as that the BWP is no longer suitable and the UE switches to default BWP.

Thus, while the UE monitors LP-WUS, the corresponding BWP is not used, so bwp-InactivityTimer should be kept as running.

Therefore, we proposed to discuss this issue based on Option 1 and Option 2.

Proposal 8. For the issue that BWP switching is overlapped with LP-WUS occasion, RAN2 discusses following options.

- Option 1. RAN2 considers BWP switching as a collision case and sends LS to RAN1 to confirm this.

- Option 2. Upon bwp-Inactivitytimer expiry, the UE keeps monitor LP-WUS, then the UE performs BWP switching when the UE wakes up MR.

3.	Conclusion

In this document, we discuss open issues on LP-WUS in RRC_CONNECTED, and make proposals as follows.

Proposal 1. RAN2 sends LS to RAN1 to ask whether LR is separately implemented per cell group in DC case.

Proposal 2. If RAN1 confirms that LR is separately implemented per cell group in DC case, the restriction that UE cannot operate MR and LR simultaneously is applied only for the same cell group.

Proposal 3. RAN2 does not consider the simultaneous operation of MR and LR for dual DRX group with FR1-FR2 until there is RAN1 input.

Proposal 4. In Option 1-1, when at least one DRX group is in Active time and LP-WUS occasion is overlapped with the Active time, the UE starts the next drx-onDurationTimer in both DRX groups.

Proposal 5. For Option 1-2, NW configures UE whether to start the lpwus-PDCCHMonitoringTimer in collision cases.

Proposal 6. For single DRX group in Option 1-2,

- If the UE can monitor LP-WUS (i.e. Not in collision case)

  - If LP-WUS is detected, the UE starts lpwus-PDCCHMonitoringTimer.

  - If LP-WUS is not detected, the UE does not start lpwus-PDCCHMonitoringTimer.

- If the UE cannot monitor LP-WUS (i.e. in collision case)

  - If the UE is configured to start lpwus-PDCCHMonitoringTimer for the collision case, the UE starts the next lpwus-PDCCHMonitoringTimer.

  - If the UE is configured not to start lpwus-PDCCHMonitoringTimer for the collision case, the UE does not start the next lpwus-PDCCHMonitoringTimer.

Proposal 7. For dual DRX groups in Option 1-2,

- If the UE can monitor LP-WUS (i.e. both DRX group is in non-Active time)

  - If LP-WUS is detected, the UE starts lpwus-PDCCHMonitoringTimer in both DRX group.

  - If LP-WUS is not detected, the UE does not start lpwus-PDCCHMonitoringTimer in both DRX group.

- If the UE cannot monitor LP-WUS (i.e. at least one DRX group is in Active time)

  - If the UE is configured to start lpwus-PDCCHMonitoringTimer for the collision case, the UE starts the next lpwus-PDCCHMonitoringTimer in both DRX groups.

  - If the UE is configured not to start lpwus-PDCCHMonitoringTimer for the collision case, the UE does not start the next lpwus-PDCCHMonitoringTimer in both DRX groups.

Proposal 8. For the issue that BWP switching is overlapped with LP-WUS occasion, RAN2 discusses following options.

- Option 1. RAN2 considers BWP switching as a collision case and sends LS to RAN1 to confirm this.

- Option 2. Upon bwp-Inactivitytimer expiry, the UE keeps monitor LP-WUS, then the UE performs BWP switching when the UE wakes up MR.

4.	Reference

- [Post130][213][LPWUS] Running CR for 38.321 (Apple)

- TS 38.300 NR and NG-RAN Overall Description; Stage 2;

- R2-2505020	Reply LS on LP-WUS in RRC_CONNECTED (R1-2504888; contact: NTT DOCOMO)