# 3GPP TSG RAN WG2 Meeting #131      	                           R2-2505499
Bengaluru, Indian, August 25 – 29, 2025

Agenda item:	8.5.3

Source:	Apple

Title:	Remaining issues on on-demand SIB1

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

The Rel-19 WID of Network Energy Savings (WID RP- RP-250343) was updated in RAN#107 [1]. The WI objective on on-demand SIB1 (OD-SIB1) for UEs in IDLE/INACTIVE state is copied as below:

In this contribution, we share our view on remaining issues on OD-SIB1.

# 2 Discussion

## 2.1 RRC open issues

According to RRC running CR discussion [2], 3 open issues were identified:

- Issue 1: Q12

- Issue 2: Vivo002

- Issue 3: HW006

We discuss these 3 issues in this section.

### 2.1.1 RRC open issue 1: Q12

This issue is:

Q12: Can “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1” be added to the field description of si-BroadcastStatus to address the FFS?

Conclusion: FFS remains as open issue

According to agreement in RAN2#129b [3]:

NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

we understand the intention is to clarify that it is unnecessary to introduce OD-SIB1 procedure for UE in RRC_CONNECTED state because RRC_CONNECTED UE has no mechanism to check whether stored SIB1 is the latest one in OD-SIB1 cell. In other word, the intention is not to avoid the UE triggering OD-SIB procedure due to unable to acquire si-BroadcastStatus of the SIB(s).

Observation 1: According to agreement in RAN2#129b, the intention is to clarify that it is unnecessary to introduce OD-SIB1 procedure for UE in RRC_CONNECTED state because RRC_CONNECTED UE has no mechanism to check whether stored SIB1 is the latest one in OD-SIB1 cell. In other word, the intention is not to avoid the UE triggering OD-SIB procedure due to unable to acquire si-BroadcastStatus of the SIB(s).

Thus, we prefer the original wording suggested by CR Rapporteur. If any clarification is needed, we suggest to add the highlighted text.

“The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 (including its si-BroadcastStatus) as the latest SIB1.”

So, we propose:

Proposal 1 (RRC open issue 1): Capture the following text in field description of si-BroadcastStatus:

“The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 (including its si-BroadcastStatus) as the latest SIB1.”

### 2.1.2 RRC open issue 2: vivo002

This issue is:

| vivo002 | 5.2.2.3.1 Acquisition of MIB and SIB1 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> if the UE has a stored valid version of od-SIB1-Config for this cell: 4> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 4> if the UE is in RRC_CONNECTED while T311 is running: 5> perform the actions as specified in clause 5.2.2.3.3x; 3> else: 4> perform the actions as specified in clause 5.2.2.5. NOTE 1: The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e., the broadcast and unicast/MBS multicast beams are quasi co-located. The UE in RRC_INACTIVE state while T319a is running, is only required to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located. NOTE 2: UE in RRC_INACTIVE that does not support inactiveStateNTN-r17 enters RRC_IDLE upon cell reselection between TN cell and NTN cell, and initiates the NAS signalling connection recovery (see TS 24.501 [23]). NOTE x: It is up to UE’s implementation to check if SIB1 is currently being broadcasted for that cell before triggering OD-SIB1 request procedure of that cell. [comment] During last meeting, at offline session [AT130][104][NES] (Ericsson), P6 in R2-2504037 about the UE behavior after receiving SI change notifcation was discussed. Rapporteur responded that a condition was added to address the issue: 5> if SIB1 is not broadcasted 6> perform the actions as specified in clause 5.2.2.3.3x; And therefore, at the Comeback session, it was first agreed that: Keep RRC CR as it is for the short message and UE behaviour.. Then, it is commented that the high-lighted part mandates the UEs to check whether SIB1 is broadcasting before requesting OD-SIB1 (not the case upon receiving SI change notification), which is against the RAN1 agreement that it is up to the UE’s implementation to check if SIB1 is broadcasting. Thus, the high-lighted part was removed, and instead the NOTE x is added. However, RAN2#126 agrees that: Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.. Therefore, we encourage RAN2 to review the agreement again ‘Keep RRC CR as it is for the short message and UE behaviour.’, whether the NOTE x can cover the above RAN2#126 agreement. The discussion is comprised of the following questions: Q1. For RAN2#126 agreement ‘ Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’, does it mean that the UE always assumes the NES cell will turn from transmitting NCD-SSB to transmitting CD-SSB for SIB1 update? [HONOR] For Q1, we understand that this agreement simply shows the UE can acquire the updated SIB1 from the NES cell without returning to Cell A. It does not imply that the NES cell will immediately transmit CD-SSB for SIB1 update upon broadcasting a SIB change notification (e.g., at the start of the next MP). Q2. If the answer is ‘NO’ for Q1, i.e. the NES cell can remain transmitting NCD-SSB and at the same time transmit the updated SIB1, how does the UE camping on the NES cell to receive the updated SIB1? Shall we rely on the NOTE x to say it’s up to UE’s implementation to handle this? [HONOR] For Q2, we think the UE camping on the NES cell can initiate OD-SIB1 request based on the WUS configuration to acquire the updated SIB1. We think for Q2, if the CORESET#0 in the stored OD-SIB1 request configuration has changed, the UE camping on the NES cell will have no way by implementation to receive the updated SIB1. As a result, all the UEs camping on the NES cell will trigger OD-SIB1 request since SIB1 cannot be received, which is not correct. Therefore, we propose Q1/Q2 to be listed as open issues for further discussion. [HONOR] Same view as Vivo, i.e., add Q1, Q2 and the issue of WUS configuration changes caused by CORESET#0 modifications to open issues for further study. |
| --- | --- |

We provide our view on the two questions raised by vivo002:

Q1. For RAN2#126 agreement ‘ Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’, does it mean that the UE always assumes the NES cell will turn from transmitting NCD-SSB to transmitting CD-SSB for SIB1 update?

[Apple] We think RAN2#126 agreement just means the UE still follows legacy SIB1 update procedure after reception of SIB change notification, i.e. it doesn’t imply any NW behavior. Therefore, the UE can’t assume the NES cell will always turn from transmitting NCD-SSB to transmitting CD-SSB for SIB1 update.

Q2. If the answer is ‘NO’ for Q1, i.e. the NES cell can remain transmitting NCD-SSB and at the same time transmit the updated SIB1, how does the UE camping on the NES cell to receive the updated SIB1? Shall we rely on the NOTE x to say it’s up to UE’s implementation to handle this?

[Apple] Our understanding is that it is left to NW implementation when to send short message with SIB1 change notification. From the UE side:

- It is madated UE to trigger SIB1 update procedure only if it receives short message with SIB change notification.

- It is still up to UE implementation how to check SIB1 broadcast status and trigger OD-SIB1 procedure, if it doesn’t receive receives short message with SIB change notification.

We think for Q2, if the CORESET#0 in the stored OD-SIB1 request configuration has changed, the UE camping on the NES cell will have no way by implementation to receive the updated SIB1. As a result, all the UEs camping on the NES cell will trigger OD-SIB1 request since SIB1 cannot be received, which is not correct. Therefore, we propose Q1/Q2 to be listed as open issues for further discussion.

[Apple] We are not sure how the highlighted behavior is derived. Our understanding is that if NW doesn’t want UE to trigger OD-SIB1 request, it should change to CD-SSB for SIB1 update, which however is not mandatory NW behavior.

Thus, we think the system can work without any spec change.

Proposal 2 (RRC open issue 2): RAN2 confirm that no spec change is needed for vivo002 (i.e. it is left to NW implementation when/whether to change to CD-SSB for SIB1 update and send short message with SIB1 change notification).

### 2.1.3 RRC open issue 3: HW006

This issue is:

| HW006 | #### ra-SearchSpace-r19 SearchSpaceId > based on R1-2503243, shouldn’t this be of type SearchSpace? #### In PDCCH-Config of cell A the search space (with ID) is for cell A. If SIBxx includes the search space ID for NES cell, the search space configuration if the NES cell also needs to be provided in SIBxx, it cannot be a SearchSpaceId from Cell A. | Open issue |
| --- | --- | --- |

We agree with Huawei. Legacy ra-SearchSpace in PDCCH-ConfigCommon is type of SearchSpaceId because search space is already defined in PDCCH-ConfigCommon. However, in OD-SIB1-Config-r19, PDCCH-ConfigCommon (and its included search space) is not included. This will lead to failing to decode RAR in NES cell.

Observation 2: Legacy ra-SearchSpace in PDCCH-ConfigCommon is type of SearchSpaceId because search space is already defined in PDCCH-ConfigCommon. However, in OD-SIB1-Config-r19, PDCCH-ConfigCommon (and its included search space) is not included. This will lead to failing to decode RAR in NES cell.

Thus, we propose:

Proposal 4 (RRC open issue 3): Change the IE type of ra-SearchSpace-r19 to SearchSpace.

## 2.2 Other issues

### 2.2.1 FFS: Co-existence of SDT and OD-SIB1

According to company input, it seems that the intention is to allow the UE to trigger OD-SIB1 procedure even if there is SDT ongoing.

In current on-demand SI, if there is no SDT ongoing, the on-demand SI is not allowed. The on-demand SI is performed toward current serving cell and on-demand SIB1 is toward neighbour cell usually. It is not good idea to forbid the UE to perform on-demand SIB1 to neighbour NES cell, e.g., no other cell can be reselected except NES cell, and it will impact the UE mobility.

According to current spec, after cell reselection occurs, the SDT will stop. However, performing on-demand SIB1 does not mean cell reselection will happen to this cell.

The legacy RRC specification doesn’t allow the UE to trigger OD-SIB procedure if there is SDT ongoing. We understand the reason is because only one RACH procedure can be performed in one time instance. As OD-SIB procedure is not that so essential, it is not necessary to specify the UE to terminate the ongoing SDT procedure and trigger OD-SIB procedure. Back to the OD-SIB1 issue. Because there is no hard latency requirement is put on OD-SIB1 procedure, we also don’t see the need to change legacy OD-SIB behaviour.

Proposal 4: RAN2 don’t pursue the optimization to allow the UE to trigger OD-SIB1 procedure when there is SDT ongoing (i.e. no need to change the procedure of current running RRC CR).

# 3 Conclusion

In this contribution, we discuss remaining issues of on-demand SIB1. Our observations are:

Observation 1: According to agreement in RAN2#129b, the intention is to clarify that it is unnecessary to introduce OD-SIB1 procedure for UE in RRC_CONNECTED state because RRC_CONNECTED UE has no mechanism to check whether stored SIB1 is the latest one in OD-SIB1 cell. In other word, the intention is not to avoid the UE triggering OD-SIB procedure due to unable to acquire si-BroadcastStatus of the SIB(s).

Observation 2: Legacy ra-SearchSpace in PDCCH-ConfigCommon is type of SearchSpaceId because search space is already defined in PDCCH-ConfigCommon. However, in OD-SIB1-Config-r19, PDCCH-ConfigCommon (and its included search space) is not included. This will lead to failing to decode RAR in NES cell.

Based on observations, our proposals are:

RRC open issues

Proposal 1 (RRC open issue 1): Capture the following text in field description of si-BroadcastStatus:

“The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 (including its si-BroadcastStatus) as the latest SIB1.”

Proposal 2 (RRC open issue 2): RAN2 confirm that no spec change is needed for vivo002 (i.e. it is left to NW implementation when/whether to change to CD-SSB for SIB1 update and send short message with SIB1 change notification).

Proposal 3 (RRC open issue 3): Change the IE type of ra-SearchSpace-r19 to SearchSpace.

Other issue

Proposal 4: RAN2 don’t pursue the optimization to allow the UE to trigger OD-SIB1 procedure when there is SDT ongoing (i.e. no need to change the procedure of current running RRC CR).

# 4 References

[1] RP-250343, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] R2-250xxxx, Running RRC CR for enhancements for network energy efficiency, Ericsson.

[3] RAN2#129b, Chair Notes.