# 3GPP TSG RAN WG2 Meeting #131      	                           R2-2505502
Bengaluru, Indian, August 25 – 29, 2025

Agenda item:	8.1.2.2

Source:	Apple

Title:	Remaining issues on LCM procedure of UE-sided model for AI/ML based beam management

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The revised Rel-19 WID of AI/ML for air interface (WID RP-243244) was agreed in RAN#106 [1]. The WI objective on general LCM framework, including applicable functionalities, is led by RAN2 which is copied below:

By RAN2#130 [2], most of essential issues on LCM framework for UE side model have been closed. However, there are some leftover issues to be addressed. In post-meeting email discussion on RRC running CR [3], multiple open issues are identified.

In this contribution, we discuss the following remaining issues related to LCM procedure of AI/ML based beam management:

- Open issue RRC-1: Cause of inapplicability

- Open issue RRC-17: Processing timing requirement of applicability/inapplicability report via RRCReconfigurationComplete

- Open Issue RRC-15: The time duration for an AI functionality to become available for inference when the UE reports applicability

- Open issue RRC-16: Open issue RRC-16: UE behaviour when the associated ID is not provided by the network

- Open issue RRC-44: Whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations

- Open issue RRC-45: How/where to capture activation of periodic inference CSI-ReportConfig in specifications

- Open issue RRC-48: Applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided

# 2 Discussion

## 2.1 Open issue RRC-1: Cause of inapplicability

The issue is described as follows:

###### Open issue RRC-1: Cause of inapplicability Issue description: It is FFS how to define the simple cause value of inapplicability related to model availability and how to capture it in the spec. This issue refers to the RAN2#129bis agreement: Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec. The issue is captured as an editor’s note in the endorsed RRC running CR [R2-2504349], clause 5.3.5.3. Proposed resolution: This open issue was resolved by the RAN2#130 agreement below. The remaining FFS can be addressed directly in the RRC running CR, where companies can provide comments to the text suggestions made by the rapporteur. If it is not possible to conclude from the running CR discussion, this FFS can be discussed in the next RAN2 meeting based on a proposal from the rapporteur. 1 Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified).

During online discussion of RAN2#130 [2], some specific cause value of inapplicability (e.g. model unavailability) was discussed. But no consensus can be achieved because there are too many reasons leading to “inapplicability”, e.g. model unavailability or model training incomplete. That is why the indication was finally agreed as “request for release configuration” from NW reaction perspective rather than UE’s cause value perspective.

Observation 1: No consensus can be achieved for cause value of inapplicability because there are too many reasons leading to inapplicability, e.g. model unavailability or model training incomplete. That is why the indication was finally agreed as “release configuration” from NW perspective rather than UE perspective.

Given the large divergence during online discussion, we don’t think RAN2 can make further progress on top of above agreement. Thus, we propose to not specify reason from UE side.

Proposal 1 (Open issue RRC-1): Don’t specify reason of inapplicability for “release configuration” indication.

## 2.2 Open issue RRC-17: Processing timing requirement of applicability/inapplicability report via RRCReconfigurationComplete

The issue is described as follows:

Issue description: According to Section 12 of TS 38.331, there are two processing latency requirements (10ms vs 16ms) between reception of RRCReconfiguration and reporting RRCReconfigurationComplete, depending on whether it is related to CA/DC operation. For Option A, it was agreed to report initial applicability report in RRCReconfigurationComplete. However, it is not clear what is its processing latency requirement because the applicability reporting is a new reporting different from CA/DC configuration. As Rel-19 is the first release of AI/ML, we expect new / unpredicted challenges for the UE to handle AI/ML operation. Thus, one possibility is using relaxed RRC processing latency requirements (i.e. 16ms). Proposed resolution: Adopt the proposal below. (RRC-17) RRCReconfigurationComplete containing applicability reports has a processing latency requirement of 16 ms with respect to the reception of RRCReconfiguration. FFS whether RAN4 input is needed. FFS whether this solves open issue RRC-15.

We think extra 6ms is needed for the UE to perform the following additional operations on top of existing processing of RRCReconfiguration message:

- Check whether model is available in local device for inference

- Check whether associated ID is same as the one in data collection configuration

- Check whether UE’s internal battery and memory are sufficient to complete inference

Please note that 1) doesn’t imply whether the UE needs to load AI/ML model in AS buffer because the UE may load the AI/ML model before reception of RRCReconfiguration message. Whether the UE loads AI/ML model in AS buffer or another accessible memory is UE implementation which should not be discussed in RAN2.

Observation 2: Whether the UE loads AI/ML model in AS buffer or another accessible memory is UE implementation which should not be discussed in RAN2. It is a separate issue of processing latency requirement for RRCReconfigurationComplete with applicability reporting.

In [3], some companies think RAN4 involvement is needed. Although the RRC processing delay requirement was determined by RAN2 in Rel-15 without RAN4 involvement, we are fine to send RAN2 agreement to RAN4 for issue checking. Thus, we propose:

Proposal 2 (Open issue RRC-17): RAN2 can send the agreement on 16ms of processing timing requirement of applicability/inapplicability report via RRCReconfigurationComplete to RAN4 for issue checking.

## 2.3 Open issue RRC-15: The time duration for an AI functionality to become available for inference when the UE reports applicability

The issue is described as follows:

###### Open Issue RRC-15: The time duration for an AI functionality to become available for inference when the UE reports applicability Issue description: The UE needs to synchronize the time duration for an AI functionality to become available for inference with the network when the UE reports applicability for either a full inference configuration or a set of inference parameters. Unlike conventional non-AI features, where the algorithm size is fixed, AI algorithms can vary significantly in size depending on their generalization performance and specific use cases. These sizes can range from several kilobytes to tens of megabytes. Additionally, UE implementations may use various types of memory to balance cost and performance, and these different memory types and constraints can affect the access speed of stored models. The UE should indicate the time duration for the AI functionality to become available when reporting applicability to the network. This time duration refers to how long it takes for the UE to load the AI models into RAM or another accessible memory for inference. By providing this information, the network can better understand the timeframe within which the configured radio resources and AI model can be utilized, enabling more efficient and effective resource management, especially for periodic CSI. Another option is to let RAN4 address this time duration in terms of requirements. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

We think the time duration depends on different UE implementation. In detail, the time duration will be different if the UE loads AI/ML model in AS buffer or another accessible memory.

Observation 3: The time duration for an AI functionality to become available for inference depends on different UE implementation, e.g. if the UE loads AI/ML model in AS buffer or another accessible memory.

However, as we discussed in open issue RRC-17, whether the UE loads AI/ML model in AS buffer or another accessible memory is UE implementation which should not be discussed in RAN2. Thus, we think RAN2 can conclude RRC issue-15 is up to UE implementation. Regarding to whether specifying requirement is up to RAN4, and proponents can raise it in RAN4 directly.

Proposal 3 (Open issue RRC-15): On the time duration for an AI functionality to become available for inference, RAN2 conclude that it is up to UE implementation and no need of UE reporting. Whether to specify its requirement is left to RAN4.

## 2.4 Open issue RRC-16: UE behaviour when the associated ID is not provided by the network

The issue is described as follows:

###### Open issue RRC-16: UE behaviour when the associated ID is not provided by the network Issue description: The UE behaviour when the associated ID is not provided by the network is not yet clarified. In the rapporteur’s view, if the associated ID is not provided by the network, the UE should still be able to provide the applicability status (applicable or inapplicable) of the configurations, which is anyway subject to UE implementation. Proposed resolution: Adopt the proposal below. - (RRC-16) RAN2 to discuss the following options: - (a) If the network does not provide the associated ID, the UE reports the applicability (applicable/inapplicable) based on UE-side additional conditions (internally known by UE), model availability in device and the other provided information in the configuration. FFS if the UE reports associated IDs, if available. - (b) If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

We prefer the option (b). The option (a) means that the NW-side additional conditions are always ignored by the UE (or mandating UE to regard NW-side additional conditions are met), if the network does not provide the associated ID. We don’t think it is reasonable. In our understanding, checking consistency between training and inference is the most important step for the UE to determine applicability. So, if NW doesn’t provide associated ID, one UE implementation is that UE reports “inapplicable” if the UE can’t determine the consistency between training and inference. While another UE implementation may determine based on the historical NW configuration and NW deployment information. Thus, option (a) will put an unnecessary restriction on UE implementation on how to handle NW-side additional conditions.

Observation 4: Option (a) means that the NW-side additional conditions are always ignored by the UE (or mandating UE to regard NW-side additional conditions are met), if associated ID is absent. It may be misaligned with some UE implementation (e.g. based on the historical NW configuration and NW deployment information).

We believe that the flexibility should be provided to the UE. Thus, we propose:

Proposal 4 (Open issue RRC-16): If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

On the FFS of option (a), the UE can report associated ID only if NW provides associated ID in training configuration but not in inference configuration. We don’t think it is a valid NW configuration, and 3GPP should not discuss solution based on an unreasonable NW configuration. Thus, we propose:

Proposal 5 (Open issue RRC-16): RAN2 don’t pursue UE reporting associated ID, which is an optimization for an unreasonable NW configuration (i.e. NW provides associated ID in training configuration but not in inference configuration).

## 2.5 Open issue RRC-44: Whether RRCReconfigurationComplete contains applicability/inapplicability of all inference configurations

The issue is:

###### Open issue RRC-44: Whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations Issue description: Based on the RAN2 agreements below, it is not clear whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations stored at the UE, or whether it can contain only updates for the inference configurations for which the UE already reported the applicability status in the past. This open issue refers to the RAN2#129 agreements: 3. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause. In the rapporteur’s understanding, there are two interpretations of the agreements above: - The RRCReconfigurationComplete contains the applicable/inapplicable status for all inference configurations. Only UAI contains updates. - The RRCReconfigurationComplete contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message and only applicability changes for those inference configurations received in earlier RRCReconfiguration messages, for which the UE already reported applicable/inapplicable at least once. UAI contains updates. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

Our understanding is more aligned with approach (1). For inference configuration(s) and inference related parameter set(s), they still follow RRC delta configuration. Thus, if NW sends multiple RRCReconfiguration messages, the UE will maintain only one latest set of inference configurations. It seems to be straight forward to feedback their applicability in one RRCReconfigurationComplete message. In addition, although approach (2) can provide some benefit of signaling overhead reduction, it requires the UE to remember the reporting of last time which may further increase UE complexity.

Observation 5: If NW sends multiple RRCReconfiguration messages, the UE will maintain only one latest set of inference configurations via RRC delta signaling. Thus, applicability feedback for the latest set in one RRCReconfigurationComplete message is more reasonable.

Proposal 6 (Open issue RRC-44): The RRCReconfigurationComplete contains the applicable/inapplicable status for all the latest inference configurations maintained by the UE via delta signaling. Only UAI contains updates.

## 2.6 Open issue RRC-45: How/where to capture activation of periodic inference CSI-ReportConfig in specifications

The issue is described as follows:

Open issue RRC-45: How/where to capture activation of periodic inference CSI-ReportConfig in specifications Issue description: RAN2 agreed that the UE should activate a periodic inference configuration in CSI-ReportConfig only if it reports that it is applicable. There are several possibilities to capture this in the specifications, as commented by several companies, and it should be discussed how to best do this. Furthermore, it should be clarified if the solution can be captured only in RRC specifications or whether RAN1 should also capture (part of) the solution. Some solutions suggested by companies for several aspects are: - Where exactly in RRC to capture the solution: - In Section 5.3.5.3 when setting the content of RRCReconfigurationComplete, or - In Section 5.3.5.3 when sending RRCReconfigurationComplete to the lower layers - What to write in RRC specifications: - Upon including the applicable status in RRCReconfigurationComplete, indicate to lower layers to activate the configuration, or - Submit/do not submit the configuration to lower layers, which would be the equivalent of activation/no activation. Furthermore, there is an incoming LS [R4-2508085], which is discussing this issue, so it is suggested that according to this LS the companies bring contributions about whether: 1) the applicable functionalities are already activated before reporting applicable functionalities via RRCReconfigurationComplete; or 2) UE starts to activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete. In particular, related to the periodic reporting, according to Rapporteur point of view, it should be taken into account that the gNB cannot know whether the UE is sending L1 prediction reports or not, until the NW receives successfully the applicability report from the UE. Until then, if the UE is allowed to multiplex UCI (with CSI inference predictions) in PUCCH or PUSCH, the NW will try to decode the multiplexed information, which will turn into decoding issues if the inference predictions are not sent by the UE and this will impact also other decoded information from the UE. So a standardized behaviour is needed in order to avoid the gNB incurring in such ambiguity, e.g: a) the UE always sends a L1 report in UCI for periodic CSI-ReportConfig, even if the configuration is not active/applicable; b) the UE does not multiplex UCI containing inference predictions and specify that the configuration is activated only after finishing sending RRCReconfigurationComplete. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

We think there are two aspects raised by CR Rapporteur:

Issue 1: down-select between two alternative modelling on how to indicate lower layer in RRC spec

- Alt-1: Upon including the applicable status in RRCReconfigurationComplete, indicate to lower layers to activate the configuration, or

- Alt-2: Submit/do not submit the configuration to lower layers, which would be the equivalent of activation/no activation.

Our understanding is that there is NO concept of “activation/deactivation of periodic CSI reporting” in RAN1. Thus, we think Alt-1 will lead to confusion in RAN1. So, Alt-2 is preferred.

Proposal 7 (Open issue RRC-45): As no concept of “activation/deactivation of periodic CSI reporting” in RAN1, adopt the way of “submit/do not submit the configuration to lower layers” in RRC running CR.

Issue 2: Whether to allow UCI multiplexing

First, we are not sure whether the UCI multiplexing issue is valid because it is out of RAN2 scope. If any essential issue, we believe RAN1 will discuss it.

Even if it is valid, we think neither of two solutions suggested by Rapporteur is necessary. Instead, it is sufficient to rely on NW implementation to resolve the issue (e.g. NW doesn’t configure UCI multiplexing when periodic CSI prediction is configured).

Proposal 8 (Open issue RRC-45): When periodic CSI prediction is configured, it is up to NW implementation how to resolve the UCI multiplexing issue (if any).

## 2.7 Open issue RRC-48: Applicability reporting for inference related parameter sets after full inference configuration is provided

The issue is described as follows:

###### Open issue RRC-48: Applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided Issue description: Based on current agreements, the UE reports the applicability of both full inference configurations and corresponding inference related parameter sets, even if some of them may correspond to each other. The issue was raised that once the network provides full inference configuration for applicable inference related parameter set, it would be highly inefficient if UE keeps reporting both full inference configuration ID and inference related parameter set config ID upon change in applicability. One solution is to allow the UE to either provide the applicability of inference related parameter set or inference configuration once the inference configuration is provided based on the applicability of inference related parameter set. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

We don’t think it is an essential issue, and thereby no spec change is needed.

Proposal 9 (Open issue RRC-48): Don’t pursue optimization on applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided, i.e. no spec change of current running CR is needed.

# 3 Conclusion

In this contribution, we discuss key remaining issues on LCM procedure of UE-sided model for AI/ML based beam management. Our observations are:

Observation 1: No consensus can be achieved for cause value of inapplicability because there are too many reasons leading to inapplicability, e.g. model unavailability or model training incomplete. That is why the indication was finally agreed as “release configuration” from NW perspective rather than UE perspective.

Observation 2: Whether the UE loads AI/ML model in AS buffer or another accessible memory is UE implementation which should not be discussed in RAN2. It is a separate issue of processing latency requirement for RRCReconfigurationComplete with applicability reporting.

Observation 3: The time duration for an AI functionality to become available for inference depends on different UE implementation, e.g. if the UE loads AI/ML model in AS buffer or another accessible memory.

Observation 4: Option (a) means that the NW-side additional conditions are always ignored by the UE (or mandating UE to regard NW-side additional conditions are met), if associated ID is absent. It may be misaligned with some UE implementation (e.g. based on the historical NW configuration and NW deployment information).

Observation 5: If NW sends multiple RRCReconfiguration messages, the UE will maintain only one latest set of inference configurations via RRC delta signaling. Thus, applicability feedback for the latest set in one RRCReconfigurationComplete message is more reasonable.

Based on observations, our proposals can be found below:

Proposal 1 (Open issue RRC-1): Don’t specify reason of inapplicability for “release configuration” indication.

Proposal 2 (Open issue RRC-17): RAN2 can send the agreement on 16ms of processing timing requirement of applicability/inapplicability report via RRCReconfigurationComplete to RAN4 for issue checking.

Proposal 3 (Open issue RRC-15): On the time duration for an AI functionality to become available for inference, RAN2 conclude that it is up to UE implementation and no need of UE reporting. Whether to specify its requirement is left to RAN4.

Proposal 4 (Open issue RRC-16): If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

Proposal 5 (Open issue RRC-16): RAN2 don’t pursue UE reporting associated ID, which is an optimization for an unreasonable NW configuration (i.e. NW provides associated ID in training configuration but not in inference configuration).

Proposal 6 (Open issue RRC-44): The RRCReconfigurationComplete contains the applicable/inapplicable status for all the latest inference configurations maintained by the UE via delta signaling. Only UAI contains updates.

Proposal 7 (Open issue RRC-45): As no concept of “activation/deactivation of periodic CSI reporting” in RAN1, adopt the way of “submit/do not submit the configuration to lower layers” in RRC running CR.

Proposal 8 (Open issue RRC-45): When periodic CSI prediction is configured, it is up to NW implementation how to resolve the UCI multiplexing issue (if any).

Proposal 9 (Open issue RRC-48): Don’t pursue optimization on applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided, i.e. no spec change of current running CR is needed.

# 4 References

[1] RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#130, Chair Notes.

[3] R2-2505778, Remaining RRC open issues in feature AIML PHY, Ericsson.