3GPP TSG-RAN WG2 Meeting #131	R2-2505529

Bangalore, India, 25th – 29th Aug. 2025

Agenda item:	8.4.2

Source:	Samsung

Title:	Procedure and Configuration of LP-WUS in RRC Idle /
Inactive Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1].

In this contribution, we would like to discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

Based on the LP-WUS/WUR study conducted during the Study Item (SI) phase in Release 18, it has been proposed that LP-WUS/WUR should be implemented as a power saving mechanism specifically designed for RRC_IDLE and RRC_INACTIVE mode devices. Furthermore, the concept of an "ultra-deep sleep state" has emerged as a potential power saving strategy for both RRC_IDLE and RRC_INACTIVE modes. By leveraging LP-WUS/WUR, a user equipment's (UE) Main Receiver (MR) can effectively transit into an ultra-deep sleep state, thereby achieving significant power savings. While in this ultra-deep sleep state, the UE can use its LP-WUS/WUR functionality to continuously monitor network signaling activities, waking up the MR when necessary.

- 2.1 Aspects in TS 38.304 that are already defined in TS 38.213

In the TS 38.304 running CR, there are some details regarding the locations, offsets, and UE behaviors related to LO (LP-WUS Occasion) monitoring, which are already defined in TS 38.213[4], as follows:

| [TS 38.304 CR] 7.x.0 General The UE monitors one LP-WUS occasion per DRX cycle. A LO is a set of LP-WUS monitoring occasions (LP-WUS MOs). In multi-beam operations, the UE assumes that the same LP-WUS is repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation. The time location of an LO for UE’s PO is determined by a reference PF and the configured frame-level offset: - The reference PF is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO. The reference PF for the LO of a PO is provided by (SFN for PF) – floor(iPO/NS) * T/N, where SFN for PF is determined in clause 7.1, iPO is defined in clause 10.4C in TS 38.213[4], T, Ns, and N are determined in clause 7.1. - The frame-level offset between the LO and the reference PF is provided by lo-Offset in SIB1. If single value is configured for lo-Offset, and if the gap between the LO and the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the offset, otherwise the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2. If more than one values are configured for lo-Offset, and if the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the smallest offset value that has a gap between the LO and the PO associated with the offset no less than the wake-up delay, otherwise the UE follows the paging monitoring procedure as described in clause 7.1or 7.2. | [TS 38.213 CR] A UE assumes that WUS occasions occur with a periodicity equal to the I-DRX cycle in the RRC_IDLE/RRC_INACTIVE state [17, TS 38.304]. The UE determines WUS occasions associated with a paging occasion based on PO-to-LO association. A reference frame of a WUS occasion starts a number of frames prior to the first of a number of paging frames associated with the WUS occasion. Each number of frames is provided by LO-FrameOffsets. The first WUS monitoring occasion of a WUS occasion starts at an offset provided by offset_firstMO_withinLO relative to the start of the reference frame. If multiple values for the number of frames provided by LO-FrameOffsets are larger than or equal to the value of XYZ, the UE monitors WUS starting at a WUS occasion corresponding to the smallest of the multiple values. If all values for the number of frames provided by LO-FrameOffsets are smaller than the value of XYZ, the UE monitors PDCCH according to Type2-PDCCH CSS sets associated with the paging occasion and does not monitor WUS. - A paging occasion associated with a WUS occasion has index iPO=UE_ID mod N⋅NS+i_s mod NPOWO where NPOWO is a number of paging occasions associated with a WUS occasion, N, NS, iSG, and i_s are defined in [17, TS 38.304], and UE_ID is defined in clause 7.1 of [17, TS 38.304]. If a number of NSGPO subgroups per paging occasion, provided by subgroupNumber-PO-WUS, is NSGPO>1, the codepoint for the subgroup index iSG in a PO iPO is iPO*NSGPO+1+iSG, and the codepoint for all subgroups in the PO is (iPO+1)*NSGPO+1-1; otherwise, the codepoint for the PO iPO is iPO. |
| --- | --- |

Considering the details are suitable to be captured in TS 38.213, we suggest to simplify the TS 38.304 CR to avoid redundant re-defining the same details. One possible suggested change could be :

The UE monitors one LP-WUS occasion per DRX cycle. A LO is a set of LP-WUS monitoring occasions (LP-WUS MOs) and is defined in clause 10.4C in TS 38.213[4]. In multi-beam operations, the UE assumes that the same LP-WUS is repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation.

Proposal 1: (P4 from [Post130][211])  Aspects regarding the locations, offsets, and UE behavior related to LO monitoring in TS 38.304 could be simplified since they are already defined in TS 38.213.

- 2.2 Enable/disable of LP-WUS

During the previous meeting and subsequent extensive email discussions [222], RAN2 discussed on the necessity of implementing UE-specific enabling and disabling support for LP-WUS. This topic has garnered significant attention due to its potential impact on network efficiency, user experience, and resource management.

The primary argument introducing a UE-specific enable/disable mechanism for LP-WUS has several key advantages. Companies presented diverse technical justifications for why the ability to disable and re-enable LP-WUS for individual UEs is essential.

Below is a detailed summary of the arguments:

    - Reducing false paging probability or increasing the reachability of the UE: NW (CN or RAN) decides whether to disable LP-WUS based on the conditions in the NW (e.g., radio resources) and/or characteristics of the UE (e.g., paging probability, shorter DRX cycle, etc)

    - Disabling for UEs with delay-sensitive services: the UEs with delay-sensitive services may not be suitable to monitor LP-WUS considering the possible large wake up latency caused by LP-WUS which may have unexpected impacts on the delay-sensitive service

    - Power Consumption: if the paging probability is high for some specific UEs, there will be frequent transition

And now we have to discuss the possible solutions for such enable/disable mechanism. During the discussion, there were three possible directions as follows:

    - NAS signalling

    - RRC signalling

    - No NAS/RRC spec changes

Given that this is simply an optimization complementing the existing cell-specific enable/disable method, it aligns with the third approach of making no changes to NAS or RRC specifications.

For the NAS-based solution, the current CN-assigned subgroup ID method is sufficient to achieve this functionality without the need for introducing additional signaling or mechanisms.

Regarding the RRC-based solution for UE-specific control, it is not suitable for managing idle UEs, as it would require idle UEs to establish AS/NAS connections solely for the purpose of LP-WUS disable/enable control. This would constitute an inefficient use of resources.

Proposal 2: (RRC-12) For enable/disable of LP-WUS per UE, ‘No NAS/RRC spec changes’ is enough.

- 3. Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1: (P4 from [Post130][211])  Aspects regarding the locations, offsets, and UE behavior related to LO monitoring in TS 38.304 could be simplified since they are already defined in TS 38.213.

Proposal 2: (RRC-12) For enable/disable of LP-WUS per UE, ‘No NAS/RRC spec changes’ is enough.

- References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC