3GPP TSG-RAN WG2 Meeting #131                                    	R2-2505537

Bengaluru, India, 25th - 29th August 2025

Agenda item:	8.9.3

Source:	Qualcomm Incorporated

Title:	CB-Msg3-EDT and Msg4 multicast

Document for:	Discussion and Decision

- Introduction

In RAN2#130, following agreements were made.

Agreements:

1.	HARQ process 0 is used to transmit all the CB-Msg3 replicas in the transmission window (RV0 is used to transmit the first repetition of each CB-Msg3 replica in the transmission window)

2.	We do not specify another way of starting Msg4 monitoring window, i.e. it is confirmed that the Msg4 monitoring window always starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not)

3.	A CB-Msg4 without RRC message (but with contention resolution identity) is allowed as the complete response to the CB-Msg3 in CP solution.

Working Assumption:

The formula for RNTI for mMsg4 monitoring is:

RNTI=X + Msg3_W_index modulo (Y) + Y*CE_level + 3*Y*carrier_id.

•	X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=2401 for eMTC or 4097 for NB-IoT,

•	Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19,

•	Y is ceil (Msg4_WS/Msg3_WP),

•	CE_level is the CE level, 0 <= CE_level < 3

•	carrier_id is the index of the UL carrier of the CB-Msg3 resources, anchor carrier has index 0,

0 <= carrier_id < 16

Can come back to check if the NW can also simply configure RNTI = X

Agreements – part 2:

1.	The value of X is 4097 for NB-IoT and 2401 for eMTC

2.	The value of Msg4_WS is the maximum Msg4 window size

3.	The value of Msg3_WP is the minimum Msg3 window periodicity

In this document, we provide details for DSA solution that are not converged/not addressed in the open issues summarized over email discussion.

- Discussion

## Transmission window configuration

As explained in [1], for DSA, the network should configure a number of replicas (i.e., K) allowed for transmission. There could be N EDT occasions configured for UE to choose randomly, and UE selects randomly K occasions (i.e., total number of Msg3 and its replica transmissions) out of N occasions. If K = 1, then UE will have only one PUSCH resource to select. If K = 2, then UE will choose randomly 2 PUSCH resources in different time occasions and transmit one original msg3 transmission and one replica of the Msg3 transmission.

Following is agreed.

- Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT

- We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure

In our view, this would mean when the network configures, K = 1 and N = 1, the size of window is same as on time domain transmission occasion duration, i.e., it can be assumed there is no transmission window. We also have following agreements:

- The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource.

- The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same.

PUSCH configuration

Defining larger periodicity of PUSCH occasions and then grouping consecutive PUSCH occasions into a CB-Msg3-EDT transmission window will lead to long latency for the UE to complete the CB-Msg3-EDT procedure.

For example in figure 1, if the network can allocate only 4 PUSCH occasions per second, having periodic PUSCH occasions will result in 4 PUSCH occasions starting at 0ms, 250ms, 500ms and 750ms. Assuming N=2 occasions within a window, the first two occasions belong to window 1 while the next two belong to window 2. Let us also assume K=2 is configured.

Figure 1 An example of CB Msg3 (DSA) window with larger PUSCH periodicity

A UE transmitting in Window 1 will transmit in TO#1, wait until start of TO#2, transmit in TO#2 and start the response window after the end of TO#2 plus RTT. So the earliest time the UE can receive success/failure response in this case is 250ms + PUSCH duration + RTT.

To reduce this latency the PUSCH peridiocity can be reduced. However this leads to increase amount of resources allocated for PUSCH occasions.

Instead, if the PUSCH occasions within a window are packed closer together (optimally back-to-back) the latency can be decreased without increased resource utilization.

Figure 2 An example of CB Msg3 (DSA) window with shorter PUSCH periodicity

The earliest time the UE can receive a success/failure response in this example in figure 2 would be 80ms + PUSCH duration + RTT.

We think network should have option to configure the CB-Msg3-EDT transmission window efficiently optimizing the resource, latency and probability of success. For example, there can be gap between two transmission windows and there should be no PUSCH resources configured in the gap. Network should also have option to configure the PUSCH occasions back-to-back to reduce latency. For example, when UE is in CE level 0, the minimum gap can be zero. In our view, the cleaner configuration is to introduce following parameters:

- Transmission window start time

- PUSCH time occasion periodicity within transmission window or PUSCH occasion offsets from the start of the window or PUSCH occasion offsets from end of the previous PUSCH occasion within the window

- Number of replicas within transmission window

- Number of PUSCH occasions within transmission window

- Transmission window periodicity (e.g., values X SFN)

The transmission window configuration includes start time, periodicity and within transmission window includes number of replicas, number of PUSCH occasions and periodicity of PUSCH occasions.

Network is allowed to configure the minimum gap between PUSCH occasions to be zero.

RNTI calculation

Following is working assumption from the last meeting.

Working Assumption:

The formula for RNTI for mMsg4 monitoring is:

RNTI=X + Msg3_W_index modulo (Y) + Y*CE_level + 3*Y*carrier_id.

•	X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=2401 for eMTC or 4097 for NB-IoT,

•	Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19,

•	Y is ceil (Msg4_WS/Msg3_WP),

•	CE_level is the CE level, 0 <= CE_level < 3

•	carrier_id is the index of the UL carrier of the CB-Msg3 resources, anchor carrier has index 0,

0 <= carrier_id < 16

Can come back to check if the NW can also simply configure RNTI = X

We think the working assumption can be confirmed. We think the definition of “Msg3_W_index” can be clarified better. It should be simply the integer of the value of current SFN divided by the window periodicity in SFN, i.e., Msg3_W_index = floor(SFN_id/Msg3 window periodicity in SFN).

Since the Msg4 window monitoring window is long, i.e., it can be as large as 10.24s, we think cell specific RNTI, i.e., configuring just one RNTI would be certain to result in collision across multiple overlapping Msg4 monitoring windows and further deteriorating the UE’s false decoding of PDSCH. We think this should be avoided.

Confirm the working assumption for RNTI calculation and define the Msg3_W_index = floor (SFN_id/configured transmission window periodicity in SFN).

Cell specific RNTI is not used for DSA.

TBS size

In our view, CB-Msg3 EDT concept and use case are same as that of Rel-15 EDT. Rel-15 EDT supports multiple TBSs configuration allowing UE to choose the correct TBS size based on the UL data size. This is important to maximize the benefit of EDT as network would not have idea on UL traffic TBS size. The same concept of multiple TBS size can be supported for CB-Msg3 EDT.

In addition, the smallest TBS size supported for Rel-15 EDT is 328 bits. However, we think for CB-Msg3 EDT, smaller TBS size of 144 bits should also be considered.

Support multiple-TBSs option same as that of Rel-15 EDT.

Support smaller TBS size of 144 bits for CB-Msg3 EDT.

Establishment cause

For Rel-15 EDT, only the establishment causes of mo-Data, delayTolerantAccess and mo-ExceptionData for NB-IoT allowed. This means the UE cannot use EDT resource to send any signaling data such as tracking area update request message. In our view, when UE has to perform tracking area update, it is beneficial that the UE does not have to go through Msg1 and Msg2. Therefore, UL NAS PDU for tracking area update can be allowed to use CB-Msg3 EDT resource.

Allow establishment cause of MO-Signalling for CB-Msg3 EDT, i.e., transmitting tracking area update request via CB-Msg3 EDT.

Fallback indication

There may be scenario where network was not able to decode any message from the CB-Msg3 transmission window, this may be the result of congestion or channel condition. In this case, all the UEs will waste power just monitoring the Msg4 response window just to find out the msg3 transmission was not successful at the expiry of Msg4 monitoring window. In such case, it is beneficial that network indicates a DL signalling for fall back earlier. This can be contention-free fallback indication to Rel-15 EDT.

We believe the retransmission of Msg3 EDT can be supported but it is not necessary to support the retransmission of all replicas. Since the retransmission grant can be specific to the Msg3 transmission location (time and frequency), just single replica retransmission can be supported.

In this case, same as in fallbackRAR in 2 step RACH, the response can include the UL grant field, the Msg3 transmission location (time and frequency indicator) as in RAPID, timing advance and a temporary C-RNTI.

As per current design, the field MAC subheader type “T” has four code points. Three codepoint is used for backoff indicator, success response and LCID data. We think the last remaining codepoint (value 11) can be used for network fallback indication.

Figure 3: E/T/R/R/H/TA/C MAC subheader

-	T: The Type field is a flag indicating the type of the MAC subheader. The T field is set to "00" to indicate the presence of a Backoff Indicator field in the subheader (BI). The T field is set to "01" to indicate the presence of the H field in the subheader. The T field is set to "10" to indicate the presence of a Logical Channel ID (LCID) field in the subheader. The value "11" is reserved for further use. The size of the Type field is 2 bits;

The fallback indication can be provided with or without UL grant. If T = 11 and there is indication that no UL grant is present, then no MAC control element is needed. The UE can exit the CB-Msg3 EDT and fallback to either 4 step RACH or Rel-15 EDT.

If T = 11 and there is indication of UL grant present, then MAC SDU can be present to include include the UL grant field, the location (time and frequency indicator) of transmitted Msg3 as in RAPID, timing advance and a temporary C-RNTI.

If MAC subheader field T = 11 and there is indication that no UL grant is present, then no control element is present and the UE exits the CB-Msg3 EDT to fallback to either 4 step RACH or Rel-15 EDT.

If MAC subheader field T = 11 and there is indication that UL grant is present, then control element includes the UL grant field, the location (time and frequency indicator) of transmitted Msg3 as in RAPID, timing advance and a temporary C-RNTI. Based on UL grant TBS, the UE decides whether the Msg3 includes UL data or not (i.e., update Msg3 or not).

- Conclusion

Following proposals are made:

Proposal 1	The transmission window configuration includes start time, periodicity and within transmission window includes number of replicas, number of PUSCH occasions and periodicity of PUSCH occasions.

Proposal 2	Network is allowed to configure the minimum gap between PUSCH occasions to be zero.

Proposal 3	Confirm the working assumption for RNTI calculation and define the Msg3_W_index = floor (SFN_id/configured transmission window periodicity in SFN).

Proposal 4	Cell specific RNTI is not used for DSA.

Proposal 5	Support multiple-TBSs option same as that of Rel-15 EDT.

Proposal 6	Support smaller TBS size of 144 bits for CB-Msg3 EDT.

Proposal 7	Allow establishment cause of MO-Signalling for CB-Msg3 EDT, i.e., transmitting tracking area update request via CB-Msg3 EDT.

Proposal 8	If MAC subheader field T = 11 and there is indication that no UL grant is present, then no control element is present and the UE exits the CB-Msg3 EDT to fallback to either 4 step RACH or Rel-15 EDT.

Proposal 9	If MAC subheader field T = 11 and there is indication that UL grant is present, then control element includes the UL grant field, the location (time and frequency indicator) of transmitted Msg3 as in RAPID, timing advance and a temporary C-RNTI. Based on UL grant TBS, the UE decides whether the Msg3 includes UL data or not (i.e., update Msg3 or not).

- References

[1] R2-2500534, “CB-Msg3 and Msg4 enhancements”, Athens, Greece, February 17th – 21st, 2025.