3GPP TSG-RAN WG2 Meeting #131                                    	R2-2505538

Bengaluru, India, 25th - 29th August 2025

Agenda item:	8.9.4

Source:	Qualcomm Incorporated

Title:	Discussion on PWS in NB-IoT NTN

Document for:	Discussion and Decision

- Introduction

Following agreements were made in RAN2#130 meeting.

Agreements:

1.	A UE should be able to camp on an NB-IoT cell (at least for NTN) for the purpose of receiving PWS broadcast, without the cell being a part of UEs PLMNs. Come back in the next meeting to discuss the exact solution to make this possible (i.e. not necessarily introducing an acceptable cell category for NB-IoT)

2.	Regarding support continued reception of PWS segmentation of a message from different cells, can come back in the next meeting with an actual TP. In any case we will not target other scenarios than the intra-gNB case

In this document, we provide details on remaining aspects of PWS in NB-IoT.

- Discussion

Currently NB-IoT UEs do not support “acceptable cell”, “Camped on Any cell state” and “emergency call” functionalities. NB-IoT UEs cannot also read broadcast notification message in RRC_CONNECTED. When PWS is supported in a cell (i.e., in NB-IoT RAT) as new feature, there will be mix of cells supporting and not supporting the PWS feature.

## 4.4 NB-IoT functionality in Idle Mode This specification is applicable to NB-IoT, except for the following functionality which is not applicable to NB-IoT: - Acceptable cell - Accessibility measurements - Access Control based on ACDC categories - Camped on Any cell state - CSG, including support for manual CSG selection and CSG or Hybrid cell related functionality in PLMN selection, or HNB name (SIB9), Cell selection and Cell reselection. - Emergency call - E-UTRAN Inter-frequency Redistribution procedure - Inter-RAT Cell Selection and Reselection including measurements in other RATs - Logged measurements - Mobility History Information - Mobility states of a UE - Priority based reselection - Public warning system including CMAS, ETWS, PWS.

We think NB-IoT UEs can be allowed to camp on any cell that is acceptable cell with minimum specification change and without any NAS impact. We think it can be left to UE that while being in any cell selection state, the UE supporting PWS may camp on an acceptable cell of any PLMN to monitor PWS. The following change can be considered in TS 36.304. In addition, support of the acceptable cell should be optional feature for NB-IoT.

### 5.2.8a Any Cell Selection state for NB-IoT In this state, the UE shall attempt to find a suitable cell of any PLMN to camp on and searching first for a high quality cell, as defined in clause 5.1.2.2. The UE supporting PWS may attempt to find an acceptable cell of any PLMN to monitor PWS and regularly attempt to find a suitable cell of any PLMN. The UE, which is not camped on any cell, shall stay in this state until a suitable cell is found.

Clarify in clause 5.2.8a in TS 36.304 that the UE supporting PWS may attempt to find an acceptable cell of any PLMN to monitor PWS and regularly attempt to find a suitable cell of any PLMN.

It is optional for UEs to camp in an acceptable cell.

In NB-IoT, cell reselection is based on ranking regardless of frequency priorities. This includes the existing NB-IoT RATs which are not updated to support PWS notification/broadcasting. We have already agreed to introduce UE capability signaling to indicate support of PWS for network awareness and obviously this would be only for new updated network.

If a UE is PWS capable and camps in a cell that does not support PWS while there are neighbor cells supporting PWS, then UE implementing such important feature will not be able to take benefit and is going to miss the warning message. Therefore, we think UE needs to be aware of the PWS capability of the cell to prioritize cell search and PLMN search. In addition, while searching for the suitable cell, the NB-IoT UE may be able to monitor the paging for emergency message in an acceptable cell. Such indication can be added in SIB1. But to avoid reading SIB1 and speed up the search, such indication can be added in MIB as there are enough spare bits available.

Introduce a new indication in MIB using a spare bit that a NB-IoT cell supports PWS feature.

Skipping SIB1 acquisition

When a UE receives paging notification for PWS message, it would need to acquire SIB1 to know the scheduling information of SIB10 or SIB11 or SIB12. The SystemInformationBlockType1-NB (SIB1-NB) uses a fixed schedule with a periodicity of 2560 ms ( and repetitions within). Due to coverage enhancement operation, the SIB1 acquisition itself can add delay to acquire the SIB10 or SIB11 or SIB12. An optimization that can be considered is that the scheduling information of SIB10, SIB11 and SIB12 are pre-configured in SIB1, however, they are not transmitted until indicated as valid/active or usable. The scheduling information is stored along with the valid SIB1.

When there is warning message, then only the network sends PWS notification in DCI/Paging message which will then activate/validate the stored scheduling information of SIB10 or SIB11 or SIB12.

When scheduling information of the SIB10/11/12 is being broadcast in SIB1, it is possible a UE joining the cell or UEs missing paging reacquires the SIB1, there can be confusion whether the scheduling information is valid. To avoid this confusion, a new indication can be introduced to let the UEs know that the SIB10/11/12 scheduling information being broadcast is valid or not.

Consider pre-configuring the scheduling information of SIB10/11/12 in SIB1 and validating it by the PWS notification.

Add an indication in SIB1 whether or not the scheduled SIB10/11/12 is valid (i.e., SIB10 or SIB11 or SIB12 is being broadcast) for UEs missing the paging notification or new UEs joining the cell. Change of this indication does trigger SI update notification.

The other benefit of this mechanism is to implicitly adopt the beam-based geofencing for PWS message (i.e., without geographical coordinates overhead). This means the PWS is valid only in the beams where the paging message is transmitted, especially in the scenario where several beams constitute a single large cell for larger coverage (or for longer service duration).

Observation 1: For a large cell consisting of several beams, beam-based geofencing can be possible with preconfiguring the scheduling information SIB10/11/12 in SIB1 and validating it by the PWS notification at the intended beams.

Support beam area-specific PWS message broadcast for PWS geo-fencing.

PWS assembly

When cell switch happens during IDLE mode, the UE needs to discard the old segments of PWS and acquire the PWS from new cell. The reason is the PWS size can be different for different cell. Depending on resource availability and segmentation policy of different cells or eNBs, the PWS segment size can be different and cannot be assembled across different cell. We think this cannot be achieved without close coordination between eNBs on the segmentation policy and its awareness at UE.

In addition, this may not really bring benefit to UE as satellite switching procedure will cause interruption. During this interruption, UE may miss a segment and create a hole. To fill the segment hole, the UE might have to anyway receive all the segments. Therefore, the UE should discard old segments upon new cell reselection.

Old PWS segments are discarded upon cell change.

- Conclusion

Following proposals are made:

Proposal 1	Clarify in clause 5.2.8a in TS 36.304 that the UE supporting PWS may attempt to find an acceptable cell of any PLMN to monitor PWS and regularly attempt to find a suitable cell of any PLMN.

Proposal 2	It is optional for UEs to camp in an acceptable cell.

Proposal 3	Introduce a new indication in MIB using a spare bit that a NB-IoT cell supports PWS feature.

Proposal 4	Consider pre-configuring the scheduling information of SIB10/11/12 in SIB1 and validating it by the PWS notification.

Proposal 5	Add an indication in SIB1 whether or not the scheduled SIB10/11/12 is valid (i.e., SIB10 or SIB11 or SIB12 is being broadcast) for UEs missing the paging notification or new UEs joining the cell. Change of this indication does trigger SI update notification.

Observation 1: For a large cell consisting of several beams, beam-based geofencing can be possible with preconfiguring the scheduling information SIB10/11/12 in SIB1 and validating it by the PWS notification at the intended beams.

Proposal 6	Support beam area-specific PWS message broadcast for PWS geo-fencing.

Proposal 7	Old PWS segments are discarded upon cell change.