3GPP TSG-RAN WG2 Meeting #131	R2-2505557

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:		8.18.2

Source:		Samsung

Title:	Way forward on remaining issues for RRC and MAC

Document for:	Discussion & Decision

# 1 Introduction

Following are the remaining issues for Rel-19 LTE-based 5G Broadcast:

[MAC-1]: Whether and how HARQ handling is described in the MAC specification for the Rel-19 time-interleaved PMCH transmission [MAC-2]: Whether Extended MSI is applicable for the Rel-19 time-interleaved PMCH transmission [RRC-1]: Rel-19 IEs for MBMS Interest Indication according to latest RAN1 agreements [RRC-2]: Time-interleaved PMCH transmission (as configured) exceeding UE’s soft buffer size or supported TBS according to latest RAN1 agreements [RRC-3]: FFS and Editor’s Noted in RRC running CR [RRC-4]: UE capability signalling

# 2 Discussion

[MAC-1]: Whether and how HARQ handling is described in the MAC specification for the Rel-19 time-interleaved PMCH transmission

RAN1 made the below agreement in RAN1#121 about the HARQ for time interleaved MCH and sent a LS to RAN2

Agreement RAN1 asks RAN2 whether/how to address the HARQ handling in the MAC specification for the Rel-19 time-interleaved PMCH transmission with M TBs and N RVs.

Figure 1: Time Interleaver for R19 5G Broadcast – M TBs and N RVs

Time interleaving enhancements for LTE-based 5G Broadcast are based on the reuse of the 3GPP building block behind the HARQ retransmission and combining, which allows spreading the coded bits of a transport block across multiple non-consecutive subframes [RP-242534]. RAN1 discussed the matter and reached the above agreement, and a LS is sent to RAN2 given the HARQ handling is described in MAC specs.

Similar mechanisms for the HARQ handling have been addressed in MAC specs for SIB reception (TS 36.321, TS 38.321) and NR MBS Broadcast reception (TS 38.321) and it is straight-forward to inherit the approach for time interleaved MCH reception.

Excerpt from TS 38.321: SIB reception and MR MBS Broadcast reception

#### 5.3.2.2 HARQ process … For each received TB and associated HARQ information, the HARQ process shall: 1> if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or 1> if the HARQ process is equal to the broadcast process, and this is the first received transmission for the TB according to the system information schedule indicated by RRC; or 1> if the HARQ process is associated with a transmission indicated with a MCCH-RNTI for MBS broadcast, and this is the first received transmission for the TB according to the broadcast MCCH schedule indicated by RRC; or 1> if the HARQ process is associated with a transmission indicated with a Multicast MCCH-RNTI for MBS multicast in RRC_INACTIVE, and this is the first received transmission for the TB according to the multicast MCCH schedule indicated by RRC; or 1> if the HARQ process is associated with a transmission indicated with a G-RNTI for MBS broadcast, and this is the first received transmission for the TB according to the MTCH schedule indicated by RRC or according to the scheduling indicated by DCI as specified in TS 38.214 [7]; or 1> if this is the very first received transmission for this TB (i.e. there is no previous NDI for this TB): 2> consider this transmission to be a new transmission. 1> else: 2> consider this transmission to be a retransmission. The MAC entity then shall: 1> if this is a new transmission: 2> attempt to decode the received data. 1> else if this is a retransmission: 2> if the data for this TB has not yet been successfully decoded: 3> instruct the physical layer to combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data. 1> if the data which the MAC entity attempted to decode was successfully decoded for this TB; or 1> if the data for this TB was successfully decoded before: 2> if the HARQ process is equal to the broadcast process: 3> deliver the decoded MAC PDU to upper layers. 2> else if this is the first successful decoding of the data for this TB: 3> deliver the decoded MAC PDU to the disassembly and demultiplexing entity. 1> else: 2> instruct the physical layer to replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode. 1> if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see clause 5.1.5); or 1> if the HARQ process is associated with a transmission indicated with a MSGB-RNTI and the Random Access procedure is not yet successfully completed (see clause 5.1.4a); or 1> if the HARQ process is equal to the broadcast process; or 1> if the HARQ process is associated with a transmission indicated with a MCCH-RNTI or a G-RNTI for MBS broadcast; or 1> if the HARQ process is associated with a transmission indicated with a Multicast MCCH-RNTI for MBS multicast; or 1> if the HARQ process is associated with a transmission indicated with a G-RNTI or a G-CS-RNTI or a configured downlink assignment for MBS multicast and HARQ feedback is disabled for this G-RNTI or G-CS-RNTI or the corresponding G-CS-RNTI, as specified in clause 18 of TS 38.213 [6]; or 1> if the HARQ process is associated with a transmission indicated with a G-RNTI or a G-CS-RNTI or a configured downlink assignment for MBS multicast and NACK only HARQ feedback is used for this G-RNTI or G-CS-RNTI or the corresponding G-CS-RNTI and the data for this TB is successfully decoded and the transmission is not the first transmission of PDSCH where the configured downlink assignment was (re-)initialised; or 1> if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired, and the Serving Cell is not configured with two TAGs, and if the cg-SDT-TimeAlignmentTimer, if configured, is not running; or 1> if the Serving Cell on which the HARQ feedback is to be transmitted is configured with two TAGs and if the timeAlignmentTimer of the TAG, associated with the TCI state(s) used for transmitting the HARQ feedback, is stopped or expired: 2> not instruct the physical layer to generate acknowledgement(s) of the data in this TB.

It can be noted that R19 time interleaving introduced in the WID is just not row by column interleaving but also includes redundancy versions across M x N transmission pattern of subframes and require being soft-combined. It was not the case for pre-R19 MCH.

Accordingly, the Text Proposal 1 captures the two spec changes (Change#1 and Change#2) as required in TS 36.321 to address HARQ handling for M transport blocks and N redundancy versions corresponding to the time interleaved MCH reception and essentially, requires to be specified. Notably, the proposed spec text is nothing new but the reuse of the legacy way of handling TBs with different RVs and soft combining, as has been described traditionally by MAC spec.

[MAC-1] Proposal 1: RAN2 to specify the HARQ handling for Rel-19 time-interleaved MCH transmission in MAC specification. (Adopt TP 1)

[MAC-2]: Whether Extended MSI is applicable for the Rel-19 time-interleaved PMCH transmission

In our understanding, extended MCH Scheduling Information (E-MSI) MAC CE is intended for group communication services and allows network to intimate UEs in advance of the suspension of specific broadcast service and let UE prepare to switch to unicast mode. With this regard, E-MSI seems not applicable for Rel-19 time-interleaved PMCH transmission. However, to support implementation flexibility, it seems no problem to allow usage of extended MSI for scheduling LTE-based 5G Broadcast. MSI description changes can be safely reflected to extended MSI description as well.

[MAC-2] Proposal 2: To support implementation flexibility, reflect the MSI description changes also to extended MSI description.

[RRC-1]: Rel-19 IEs for MBMS Interest Indication according to latest RAN1 agreements

RAN1 Agreement Rel-19 IE in MBMS Interest Indication message carries following parameters utilized for receiving Rel-19 LTE-based 5G Broadcast service(s): - Frequency - New values of sub-carrier spacing = {2.5 kHz, 1.25 kHz} - New values for bandwidth = {n30, n35, n40} - A new UE baseband resource Scaling factor (e.g., utilized soft buffer at the UE) used for receiving the time interleaved broadcast

Given the RAN1 agreement on Rel-19 version of MBMS Interest Indication (MII) message, below are the impacts expected on RAN2 specs:

- RRC specification should capture the procedural description changes to accommodate initiation and composing of Rel-19 version of MII

- RRC specification should add a new Rel-19 IE in MBMS Interest Indication. Placeholder can be put for actual value of the parameter in case it is yet to be received from RAN1

[RRC-1] Proposal 3: RAN2 to capture the procedural and signalling impact for the Rel-19 IE of MBMS Interest Indication. (Adopt TP2)

RRC-2: Time-interleaved PMCH transmission (as configured) exceeding UE’s soft buffer size or supported TBS according to latest RAN1 agreements:

Agreement In case the time interleaved PMCH transmission (as configured) exceeds a UE’s capability(/es) in terms of soft buffer size or maximum TBS supported in a TTI, the UE is not expected to receive the corresponding time interleaved PMCH transmission.

We think the above RAN1 agreement is informative to RAN2 and this does not really required to be captured in RAN2 specification.

[RRC-2] Proposal 4: RAN2 specification need not capture anything for time-interleaved PMCH transmission (as configured) exceeding UE’s soft buffer size or supported TBS and consider RAN1 agreement as informative.

RRC-3: FFS and Editor’s Noted in RRC running CR

-- Editor’s Note: for the following parameter, exact candidate values are ‘ffs’ as they are still under discussion in RAN1 and not included in the RAN1 parameter list yet. Following is added as placeholder.

pmch-CyclicShiftAlpha-r19 				ENUMERATED {alphaOne, alphaOther, ffs}		OPTIONAL -- Need OR

pmch-CyclicShiftAlpha

Indicates parameter α for cyclic shift for PMCH, see TS 36.211 [21] clause 6.5.1.

Editor’s Note: the description may need further update once RAN1 finalizes the parameter list.

In our understanding, RAN2 cannot make any progress about this issue and just need to wait for RAN1 update.

[RRC-3] Proposal 5: RAN2 need to wait for RAN1 progress about cyclic shift parameter for PMCH. Maintain the current placeholder in RRC specification.

# 3 Conclusion

Based on the discussion above, request RAN2 to consider the following proposals:

[MAC-1] Proposal 1: RAN2 to specify the HARQ handling for Rel-19 time-interleaved MCH transmission in MAC specification. (Adopt TP 1)

[MAC-2] Proposal 2: To support implementation flexibility, reflect the MSI description changes also to extended MSI description.

[RRC-1] Proposal 3: RAN2 to capture the procedural and signalling impact for the Rel-19 IE of MBMS Interest Indication. (Adopt TP2)

[RRC-2] Proposal 4: RAN2 specification need not capture anything for time-interleaved PMCH transmission (as configured) exceeding UE’s soft buffer size or supported TBS and consider RAN1 agreement as informative.

[RRC-3] Proposal 5: RAN2 need to wait for RAN1 progress about cyclic shift parameter for PMCH. Maintain the current placeholder in RRC specification.

# 4 Text Proposal

TP 1

Change#1: HARQ applicability for MTCH corresponding to time-interleaved MCH

### 4.2.1 MAC Entities … Figure 4.2.1-1: MAC structure overview, UE side

Change#2: TB decoding and soft combining

## 5.12 MCH reception ….. Time-interleaved MCH transmission may occur in subframes configured by upper layer for MTCH transmission. For each such subframe, dataMCS applies. The transmission of an MCH occurs in a set of subframes defined by PMCH-Config. An MCH Scheduling Information MAC control element is included in the first subframe allocated to the MCH within the MCH scheduling period to indicate the position of each MTCH and unused subframes on the MCH. The MAC entity shall assume that the first scheduled MTCH starts at the earliest in the next subframe after the subframe containing the MCCH and/or the MCH Scheduling Information MAC control element, and the other scheduled MTCH(s) start immediately after the previous MTCH, at the earliest in the next subframe (which is not containing MCCH) after the subframe where the previous MTCH stops. Unused part of the subframe, if any, is filled with padding. When the MAC entity needs to receive MCH, the MAC entity shall: - if the HARQ process is associated with a transmission indicated with a M-RNTI for time interleaved MCH: - if this is the first received transmission for the TB according to the MTCH schedule indicated by MSI: - attempt to decode the received data. - else: - if the data for this TB has not yet been successfully decoded: - combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data. - if the data which the MAC entity attempted to decode was successfully decoded for this TB; or - if the data for this TB was successfully decoded before: - deliver the decoded MAC PDU to upper layers. - else: - replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode. - do not indicate the positive or negative acknowledgement to the physical layer. ….

TP 2

#### TS 36.331 #### 5.8.5.2 Initiation An MBMS or SC-PTM capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment, upon entering or leaving the service area, upon session start or stop, upon change of interest, upon change of priority between MBMS reception and unicast reception, upon change to a PCell broadcasting SystemInformationBlockType15, upon starting and stopping of MBMS service(s) in receive only mode, upon change of receive only mode frequency, bandwidth or subcarrier spacing or scaling factor of MBMS service(s) in receive only mode. Upon initiating the procedure, the UE shall: 1> if SystemInformationBlockType15 is broadcast by the PCell; or 1> if mbms-ROM-ServiceIndication is received in SystemInformationBlockType2 from PCell: 2> ensure having a valid version of SystemInformationBlockType15 for the PCell, if present; 2> if the UE did not transmit an MBMSInterestIndication message since last entering RRC_CONNECTED state; or 2> if since the last time the UE transmitted an MBMSInterestIndication message, the UE connected to a PCell neither broadcasting SystemInformationBlockType15 nor including mbms-ROM-ServiceIndication in SystemInformationBlockType2: 3> if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, is not empty: 4> initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4; 2> else: 3> if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, has changed since the last transmission of the MBMSInterestIndication message; or 3> if at least one of the subcarrier spacing or bandwidth or scaling factor parameter of receive only mode MBMS frequency of interest, determined in accordance with 5.8.5.3, has changed since the last transmission of the MBMSInterestIndication message; or 3> if the prioritisation of reception of all indicated MBMS frequencies compared to reception of any of the established unicast bearers has changed since the last transmission of the MBMSInterestIndication message: 4> initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4; NOTE: The UE may send an MBMSInterestIndication even when it is able to receive the MBMS services it is interested in i.e. to avoid that the network allocates a configuration inhibiting MBMS reception. 3> else if SystemInformationBlockType20 is broadcast by the PCell: 4> if since the last time the UE transmitted an MBMSInterestIndication message, the UE connected to a PCell not broadcasting SystemInformationBlockType20; or 4> if the set of MBMS services of interest determined in accordance with 5.8.5.3a is different from mbms-Services included in the last transmission of the MBMSInterestIndication message; 5> initiate the transmission of the MBMSInterestIndication message in accordance with 5.8.5.4. #### …. #### 5.8.5.4 Actions related to transmission of MBMSInterestIndication message The UE shall set the contents of the MBMSInterestIndication message as follows: 1> if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, is not empty: 2> include mbms-FreqList and set it to include the MBMS frequencies of interest sorted by decreasing order of interest, using the EARFCN corresponding with freqBandIndicator included in SystemInformationBlockType1 (for serving frequency), if applicable, and the EARFCN(s) as included in SystemInformationBlockType15 (for neighbouring frequencies); NOTE 1: The EARFCN included in mbms-FreqList is merely used to indicate a physical frequency the UE is interested to receive i.e. the UE may not support the band corresponding to the included EARFCN (but it does support at least one of the bands indicated in system information for the concerned physical frequency). 2> include mbms-Priority if the UE prioritises reception of all indicated MBMS frequencies above reception of any of the unicast bearers; 2> if SystemInformationBlockType20 is broadcast by the PCell: 3> include mbms-Services and set it to indicate the set of MBMS services of interest determined in accordance with 5.8.5.3a; NOTE 2: If the UE prioritises MBMS reception and unicast data cannot be supported because of congestion on the MBMS carrier(s), E-UTRAN may initiate release of unicast bearers. It is up to E-UTRAN implementation whether all bearers or only GBR bearers are released. E-UTRAN does not initiate re-establishment of the released unicast bearers upon alleviation of the congestion. 1> if the UE is receiving MBMS service(s) in receive only mode: 2> if the supportedBandCombination the UE included in UE-EUTRA-Capability contains at least one band combination including the mbms-ROM-Freq: 3> include mbms-ROM-Freq, mbms-ROM-SubcarrierSpacing, and mbms-Bandwidth and mbms-ScalingFactor; NOTE 3: The EARFCN included in mbms-ROM-Freq is used to indicate a physical frequency the UE is interested to receive MBMS service(s) in receive only mode and is determined based on UE implementation. The UE shall submit the MBMSInterestIndication message to lower layers for transmission. .... MBMSInterestIndication message -- ASN1START MBMSInterestIndication-r11 ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE { interestIndication-r11 MBMSInterestIndication-r11-IEs, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } MBMSInterestIndication-r11-IEs ::= SEQUENCE { mbms-FreqList-r11 CarrierFreqListMBMS-r11 OPTIONAL, mbms-Priority-r11 ENUMERATED {true} OPTIONAL, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension MBMSInterestIndication-v1310-IEs OPTIONAL } MBMSInterestIndication-v1310-IEs ::= SEQUENCE { mbms-Services-r13 MBMS-ServiceList-r13 OPTIONAL, nonCriticalExtension MBMSInterestIndication-v1540-IEs OPTIONAL } MBMSInterestIndication-v1540-IEs ::= SEQUENCE { mbms-ROM-InfoList-r15 SEQUENCE (SIZE(1..maxMBMS-ServiceListPerUE-r13)) OF MBMS-ROM-Info-r15 OPTIONAL, nonCriticalExtension MBMSInterestIndication-v1610-IEs OPTIONAL } MBMSInterestIndication-v1610-IEs ::= SEQUENCE { mbms-ROM-InfoList-r16 SEQUENCE (SIZE(1..maxMBMS-ServiceListPerUE-r13)) OF MBMS-ROM-Info-r16 OPTIONAL, nonCriticalExtension MBMSInterestIndication-v19xy-IEs SEQUENCE {} OPTIONAL } MBMSInterestIndication-v19xy-IEs ::= SEQUENCE { mbms-ROM-InfoList-r19 SEQUENCE (SIZE(1..maxMBMS-ServiceListPerUE-r13)) OF MBMS-ROM-Info-r19 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } MBMS-ROM-Info-r15 ::= SEQUENCE { mbms-ROM-Freq-r15 ARFCN-ValueEUTRA-r9, mbms-ROM-SubcarrierSpacing-r15 ENUMERATED {kHz15, kHz7dot5, kHz1dot25}, mbms-Bandwidth-r15 ENUMERATED {n6, n15, n25, n50, n75, n100} } MBMS-ROM-Info-r16 ::= SEQUENCE { mbms-ROM-Freq-r16 ARFCN-ValueEUTRA-r9, mbms-ROM-SubcarrierSpacing-r16 ENUMERATED {kHz2dot5, kHz0dot37}, mbms-Bandwidth-r16 ENUMERATED {n6, n15, n25, n50, n75, n100} } MBMS-ROM-Info-r19 ::= SEQUENCE { mbms-ROM-Freq-r19 ARFCN-ValueEUTRA-r9, mbms-ROM-SubcarrierSpacing-r19 ENUMERATED { kHz15, kHz7dot5, kHz2dot5, kHz1dot25}, mbms-Bandwidth-r19 ENUMERATED {n6, n15, n25, n30, n35, n40,n50, n75, n100}, mbms-ScalingFactor-r19 FFS } -- ASN1STOP