3GPP TSG-RAN WG2 Meeting #131	R2-2505558

Bengaluru, India, 25 - 29 August, 2025

Agenda item:	8.7.6

Source:	Samsung

Title:	Discussion on UL rate control for Rel-19 XR

Document for:	Discussion & Decision

Introduction

The remaining open issues on UL rate control can be summarized as follows:

[MAC-3] Design of identifier for QoS flows indicated in the UL Rate Control MAC CE. [MAC-4] Format of the UL Rate Control MAC CE. [MAC-5] Discuss UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, e.g. whether the UE can transmit some of the queries or it must wait until it can send all pending queries together. [RRC-1] FFS whether UAI for gap cancellation ratio preference can be reported to SN [UE capability-05] Whether maximum number of QoS flows subject to rate control should be defined as UE AS capability and the value range of the capability (if to be defined).

In this paper, we provide our view for the remaining issues.

Discussion

[MAC-3] Design of identifier for QoS flows indicated in the UL Rate Control MAC CE

In the latest RRC running CR, the target QoS flows subject to UL rate control/query are configured per MAC entity (in MAC-CellGroupConfig), with additionally indicating which PDU session each QoS flow is associated with. Moreover, the releasing list of QFI (e.g., ul-RateControlConfigReleaseList-r19  SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI) currently configured in MAC-CellGroupConfig also involves the ambiguity on which QoS flow to release, given that there could be more than one QoS flows with same QFI within a cell group, necessitating further clarifying which PDU session each QFI in the releasing list is associated with.

[MAC-3] Observation 1: Current configuration structure for the target QoS flows (in MAC-CellGroupConfig) necessitates further configuration on which PDU session each QoS flow is associated with, involving signalling overhead, which could be avoided by other configuration structure.

From the signalling overhead point of view, instead of additionally configuring the PDU session ID per QoS flow, it is more efficient to configure the QoS flows per DRB configuration (DRB-ToAddMod) without the additional PDU session ID signalling, given that QoS flow ID is unique within a certain DRB. Specifically, a QFI can uniquely identify a QoS flow within the set of QoS flows served by a certain DRB, which is by default, associated with a certain PDU session configured by SDAP-Config.

[MAC-3] Proposal 1: Network configures target QoS flows subject to rate control/query per DRB configuration (DRB-ToAddMod) to avoid the signalling overhead for configuring PDU session per QoS flow.

For how to indicate QoS flow in the UL rate control MAC CE, instead of indicating the QoS flow ID explicitly, it is more efficient to use a QoS flow bitmap per DRB (given the QoS flow configuration is per DRB) to indicate which QoS flow the bit rate is associated with. That is, a QoS flow can be identified by indicating the DRB (it associated with) and the bit in the QoS flow bitmap (of the DRB), i.e., ith bit corresponding to ith QoS flow configured for the DRB.

[MAC-3] Proposal 2: A QoS flow is identified by [DRB ID + QoS Flow Bitmap] in the UL rate control MAC CE.

[MAC-4] Format of the UL Rate Control MAC CE

Considering that the UL rate control MAC CE should be able to indicate bit rates for multiple QoS flows from multiple DRBs, the UL rate control MAC CE should include extension bit per DRB to indicate whether further DRBs are involved in the bit rate recommendation/query. That is, UL rate control MAC CE should include the following fields:

- DRB ID: indicates the ID of the DRB, to which the QoS flow(s) indicated by QoS flow bitmap are associated. The length of DRB ID is 5-bit.

- QoS Flow Bitmap: each bit indicates the presence of the bit rate field of the corresponding QoS flow. QFi corresponds to the ith QoS flow configured for the DRB.

- E (Extension): indicates the whether any further DRB follows.

- Bit Rate: indicates an index of Table 6.1.3.x-1, corresponding to the recommended/queried bit rate. The number of consecutive Bit Rate fields is equal to the number of bit-1 in the corresponding QoS Flow Bitmap.

- R: Reserved bit.

The proposed UL rate control MAC CE is illustrated in Fig. 1 below.

Fig.1 Proposed UL rate control MAC CE format.

[MAC-4] Proposal 3: UL rate control MAC CE includes one or more sets of [DRB ID, QoS Flow Bitmap, Extension bit, Bit Rate field(s), and possibly reserved bit(s)].

[MAC-5] Discuss UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, e.g. whether the UE can transmit some of the queries or it must wait until it can send all pending queries together.

The rate control query must be delivered to the network promptly, as this information is closely tied to the real-time congestion status. A delayed query may no longer be necessary if delivered after congestion has been alleviated. To ensure timely delivery, it is advisable to implement a truncated version of the UL rate control MAC CE. This version would include only a portion of the pending queries, allowing them to fit within the available UL resources if the resources are insufficient to handle all queries.

For the inclusion order of QoS flows for the composition of the truncated UL rate control MAC CE, it is reasonable to reflect how long the QoS flow has been pending for the query. Accordingly, QoS flow with longer pending time, should be considered for inclusion prior to that with shorter pending time, to prevent the query information from being outdated.

[MAC-5] Proposal 4: RAN2 to introduce truncated version of UL rate control MAC CE to serve only a portion of the pending queries, when the UL resource is insufficient to handle all the pending queries.

[MAC-5] Proposal 5: When determining the inclusion order of QoS flows for the truncated UL rate control MAC CE, the QoS flow with a longer pending time is prioritized over that with shorter pending time.

[UE capability-05] Whether maximum number of QoS flows subject to rate control should be defined as UE AS capability and the value range of the capability (if to be defined).

Legacy QoS flow configuration (by 5GC) can already ensure proper number of QoS flows configured for a certain UE for a certain service requirement, and the QoS flows subject to rate control are within the QoS flows configured for the UE, such that it seems hardly possible that the number of QoS flows the network is throttling exceeds the limit that UE can support. Therefore, it is unclear what kind of benefit we can expect from defining the UE capability on the maximum number of QoS flows subject to rate control.

[UE capability-05] Proposal 6: RAN2 to not consider UE AS capability on the maximum number of QoS flows subject to rate control.

Conclusion

Based on the discussion above, we have the following observations and proposals:

[MAC-3] Observation 1: Current configuration structure for the target QoS flows (in MAC-CellGroupConfig) necessitates further configuration on which PDU session each QoS flow is associated with, involving signalling overhead, which could be avoided by other configuration structure.

[MAC-3] Proposal 1: Network configures target QoS flows subject to rate control/query per DRB configuration (DRB-ToAddMod) to avoid the signalling overhead for configuring PDU session per QoS flow.

[MAC-3] Proposal 2: A QoS flow is identified by [DRB ID + QoS Flow Bitmap] in the UL rate control MAC CE.

[MAC-4] Proposal 3: UL rate control MAC CE includes one or more sets of [DRB ID, QoS Flow Bitmap, Extension bit, Bit Rate field(s), and possibly reserved bit(s)].

[MAC-5] Proposal 4: RAN2 to introduce truncated version of UL rate control MAC CE to serve only a portion of the pending queries, when the UL resource is insufficient to handle all the pending queries.

[MAC-5] Proposal 5: When determining the inclusion order of QoS flows for the truncated UL rate control MAC CE, the QoS flow with a longer pending time is prioritized over that with shorter pending time.

[UE capability-05] Proposal 6: RAN2 to not consider UE AS capability on the maximum number of QoS flows subject to rate control.