3GPP TSG-RAN WG2 Meeting #131	R2-250xxxx

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.2.4

Source:	NEC

Title:	Ambient-IoT Data transmission

Document for:	Discussion, Decision

Introduction

In RAN2#130 meeting, a good progress was achieved on D2R scheduling, D2R data transmission and the related MAC PDU formats. This contribution further discusses remaining issues of D2R scheduling, D2R data transmission and the related MAC PDU formats.

Discussion

D2R data transmission

RAN2#130 [1] meeting reached the following agreements on “no NAS message available”.

Agreements 1. The device is expected to send a MAC response to the reader in the D2R occasion. The MAC response contains the NAS message if available at the D2R occasion. If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed.

There is also an FFS issue relate to “no NAS message available” at RAN2#130 post email discussion [2]:

| (New)Issue 3-7: more data indication handling | How to set “more data indication” value in case of no NAS response available (i.e., zero SDU) - During CR implementation, the rapp identified there is no clear conclusion how to set the “more data indication” in case of no data available, i.e., zero SDU. - Status in running CR: captured as value FFS. | Companies are invited to input views for Q#9 |
| --- | --- | --- |

Based on the companies’ views, one issue that needs clarification is whether to set “More Data Indication (MDI)” value to “1” in case of no NAS response available (i.e., zero SDU).

We argue that MDI is unnecessary in this case according to the following reasons:

- First, “0 SDU” clearly indicates that an A-IoT device has received the command but needs more time to prepare the NAS response.

- Second, MDI is designed to indicate whether there is a remaining segment or not. Thus, MDI is used only when upper layer data is available, not when upper layer data is unavailable.

(Issue 3-7) MDI should not be used when no NAS response is available.

Therefore, regarding MDI value assignment, the following options are proposed:

- Opt1: MDI is an optional field, and present only when the length of MAC PDU is larger than “0”;

- Opt2: MDI is a mandatory field, but the reader may not interpret it when the length of MAC SDU exceeds “0”. Thus, it’s value “1” or “0” is irrelevant.

(Issue 3-7) RAN2 to consider the following options for More Data Indication (MDI) value assignment:
- Opt1: MDI is an optional field, and present only when the length of MAC SDU is larger than “0”;
- Opt2: MDI is a mandatory field, but the reader may not interpret it when the length of MAC PDU exceeds “0”.Thus, it’s value “1” or “0” is irrelevant.

Indicating of D2R Scheduling Info field

There is an FFS issue relate to “scheduling info” at RAN2#130 post email discussion [2]

| Issue 4-1: RAN1 parameters | How to handle RAN1 parameters if any, e.g. scheduling info in paging, Msg2, R2D command messages. - Based on RAN1 LS in R1-2504915, the Rapp created a subclause 6.2.1.7 in the MAC running CR to capture all the RAN1 agreement parameter, companies are encouraged to check the details and make comment if any. - Status in running CR: a field named as D2R Scheduling Info is included in Paging message, Msg2 and R2D command message as a placeholder, and the details are captured in subclause 6.2.1.6 based on RAN1 inputs. | To be checked/discussed directly in CR review [POST130][027][AIoT] MAC Running CR |
| --- | --- | --- |

The total size of bits indicating Bit Duration and Frequency Resource Indication is 11 bits in the draft running CR. However, the total size of bits needed to indicate Bit Duration and Frequency Resource Indication is 9 bits. It is because there are totally i=182i-1 = 502 different cases where 9 bits is enough to indicate all of them. It’s benefit to reduce signalling overhead.

Following is an example of how to indicate Bit Duration and Frequency Resource Indication by 9 bits. Device can distinguish the bits for “bit duration” and the bits “Frequency Resource Indication” by identifying where is the first “1” in the “bit duration”&” Frequency Resource Indication” joint bitmap; after the first “1”, the remaining bits are for “ Frequency Resource Indication”.

| Tb (us) | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 266.67 | 1 | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |
| 133.33 | 0 | 1 | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |
| 66.67 | 0 | 0 | 1 | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |
| 33.33 | 0 | 0 | 0 | 1 | R=1 | R=2 | R=4 | R=8 | R=16 |
| 16.67 | 0 | 0 | 0 | 0 | 1 | R=1 | R=2 | R=4 | R=8 |
| 8.33 | 0 | 0 | 0 | 0 | 0 | 1 | R=1 | R=2 | R=4 |
| 4.17 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | R=1 | R=2 |
| 1.39 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | R=1 |

Corresponding text proposal is also provided:

- =================================Text Proposal============================= 6.2.1.6 D2R Scheduling Info field description This clause defines the child fields contained in D2R Scheduling Info field. See the Table 6.2.1.6-1. <Irrelevant Texts Omitted> Table 6.2.1.6-1: Child fields of D2R Scheduling Info field Field name; Length; Value range; Description. Bit Duration; 1 to 8 bits; {266.67, 133.33, 66.67, 33.33, 16.67, 8.33, 4.17, 1.39}; The duration in microseconds of each D2R bit: - 1-bit of ‘1’ indicates bit duration 266.67μs; - 2-bits of ‘01’ indicates bit duration 133.33μs; - 3-bits of ‘001’ indicates bit duration 66.67μs; - 4-bits of ‘0001’ indicates bit duration 33.33μs; - 5-bits of ‘00001’ indicates bit duration 16.67μs; - 6-bits of ‘000001’ indicates bit duration 8.33μs; - 7-bits of ‘0000001’ indicates bit duration 4.17μs; - 8-bits of ‘00000001’ indicates bit duration 1.39μs;. Frequency Resource Indication; 8 to 1 bits; An 8-bit to 1-bit bitmap where the size of bitmap depends on bit duration. The values of small frequency shift factor are {1, 2, 4, 8, 16, 32, 64, 128}. In the bitmap, the first/leftmost bit of the bitmap corresponds to the first value of small frequency shift factor, the second bit corresponds to the second value of small frequency shift factor, and so on. For each bit, value 0 indicates that the corresponding value is not allowed, while value 1 indicates that the corresponding value can be used.; This field indicates: - the set of NSFS potential small frequency shift factors when present in A-IoT Paging message for CBRA. Each small frequency shift factor corresponding to X access occasion(s). NSFS is the number of frequency domain resource of access occasions triggered by A-IoT Paging message or one Access Trigger message. Or - one value of small frequency shift factor when present in A-IoT Paging message for CFRA, R2D Upper Layer Data Transfer message. Or - one or multiple values of small frequency shift factor when present in Random ID Response message. A device determines its small frequency shift factor value for the following D2R transmission based on its order of Echoed Random ID field in the Random ID Response message. Regarding different Bit Duration, only the following the size of bitmap and corresponding values are can be indicated to 1 in the bitmap: - 8-bits bitmap indicating values {1, 2, 4, 8, 16, 32, 64, 128}, when Bit Duration is configured to 266.67μs; - 7-bits bitmap indicating values {1, 2, 4, 8, 16, 32, 64}, when Bit Duration is configured to 133.33μs; - 6-bits bitmap indicating values {1, 2, 4, 8, 16, 32}, when Bit Duration is configured to 66.67μs; - 5-bits bitmap indicating values {1, 2, 4, 8, 16}, when Bit Duration is configured to 33.33μs; - 4-bits bitmap indicating values {1, 2, 4, 8}, when Bit Duration is configured to 16.67μs; - 3-bits bitmap indicating values {1, 2, 4}, when Bit Duration is configured to 8.33μs; - 2-bits bitmap indicating values {1, 2}, when Bit Duration is configured to 4.17μs; - 1-bit bitmap indicating values {1}, when Bit Duration is configured to 1.39μs.. - =================================Text Proposal=============================

(Issue 4-1) The total size of bits needed to indicate Bit Duration and Frequency Resource Indication is 9 bits.

(Issue 4-1) Consider the above TP to indicate Bit Duration and Frequency Resource Indication with 9 bits instead of 11bits.

Conclusion and Proposal

We have the following observations and proposals:

- (Issue 3-7) MDI should not be used when no NAS response is available.

(Issue 4-1) The total size of bits needed to indicate Bit Duration and Frequency Resource Indication is 9 bits.

- (Issue 3-7) RAN2 to consider the following options for More Data Indication (MDI) value assignment:
- Opt1: MDI is an optional field, and present only when the length of MAC SDU is larger than “0”;
- Opt2: MDI is a mandatory field, but the reader may not interpret it when the length of MAC PDU exceeds “0”.Thus, it’s value “1” or “0” is irrelevant.

(Issue 4-1) Consider the above TP to indicate Bit Duration and Frequency Resource Indication with 9 bits instead of 11bits.

Reference

- Report of 3GPP TSG RAN WG2 meeting #130.

- [POST130][027][AIoT] MAC Running CR (Huawei)