3GPP TSG-RAN WG2 Meeting #131	R2-2505583

Bangalore, India  Aug 25th – 29th , 2025

| Agenda Item: | 8.6.2 (NR_Mob_Ph4-Core) |
| --- | --- |
| Source: | LG Electronics Inc. |
| Title: | Remaining issues on inter-CU LTM |
| Document for: | Discussion and Decision |

1.	Introduction

This paper discusses the remaining issues on inter-CU LTM: 1) FFS how NW distinguish between CLTM and fast recovery after CLTM failure; and 2) secondary key update for inter-CU MCG LTM with DC.

For the first issue, we propose a method for network to distinguish between CLTM and fast recovery after CLTM failure. About the second issue, we first review whether the RRC running CR is implemented in line with the agreement in terms of secondary key update in inter-CU MCG LTM with DC. Based on this, we propose a method to address the redundant PDCP reestablishment due to redundant secondary key update.

2.	Discussion

2.1.	FFS how NW distinguish between CLTM and fast recovery after CLTM failure

In [Post130][110], the following open issue is raised:

| Fujitsu, NTT DOCOMO | NW should be able to distinguish between CLTM and fast recovery after CLTM failure in order to deal with SN gap caused by SRB COUNT continuation when UE is performing fast recovery. If reordering timer (t-Reordering) on NW-side is set for a long duration, because NW must wait for the duration before receiving RRCReconfigurationComplete in RRC layer and sending contention resolution to the UE, T304 in the UE may expire, which results in fast recovery failure. In Rel-18, RAN2 agreed to deal with SN gap by NW implementation, and that can be done because LTM and fast recovery can be distinguished. However, in Rel-19, CLTM and fast recovery after CLTM failure cannot be distinguished, which is an open issue. | This issue is critical for operators which configures long reordering timer. If this is not resolved, this leads to failure of fast recovery after CLTM and interruption time longer than the duration of reordering timer. |
| --- | --- | --- |

A network implementation can be considered for this issue. For example, the target cell sets the reordering timer on network side to a small value regardless of whether the UE arrival is due to either CLTM or LTM based recovery. This means that the target cell does not distinguish between CLTM and LTM based recovery. Setting the reordering timer short may result in a lot of packet loss, which may cause data loss problem. Otherwise, the target cell sets the reordering timer on network side to a long value regardless of whether CLTM or LTM based recovery. In this case, as described in [Post130][110], it finally can lead to failure of LTM based recovery due to T304 expiry. Therfore, in order to avoid data loss problem or recovery failure, it is necessary to introduce a method how the target cell distinguishes between CLTM and LTM based recovery.

In Rel-18 LTM, when the UE arrives at the target cell, the target cell can distinguish between LTM cell switch and LTM based fast recovery based on whether the corresponding Cell Switch Notification F1 message has been received or not. Particularly, the UE arrival is due to the LTM cell switch (i.e., LTM CSC MAC CE) if the Cell Switch Notification message has been received. Otherwise (i.e., if the Cell Switch Notification message has not been received), the UE arrival is due to LTM based fast recovery. Based on this distinction, the target cell can set the reordering timer on network side to an appropriate value.

However, there is no way to timely initiate Cell Switch Notification procedure in Rel-19 CLTM execution procedure. This is because the source cell has no idea when the UE triggers the CLTM execution. Assume that the source cell can somehow detect the CLTM execution in advance. In this case, the source cell can transmit the Cell Switch Notification message to the target cell and consequently the target cell can distinguish between CLTM and fast recovery based on whether the corresponding Cell Switch Notification has been received or not. To enable the source cell to send Cell Switch Notification to the target cell in a timely manner, it may be helpful for the UE to notify the source cell just prior to CLTM execution. For example, the UE sends a goodbye message to the source cell upon a CLTM condition is being satisfied and then the UE performs the CLTM execution.

Proposal 1.  In order for network to distinguish between CLTM and fast recovery after CLTM failure, UE sends a goodbye message to the source cell before CLTM execution. Then, the source cell notifies the target cell of CLTM execution via e.g. Cell Switch Notification.

2.2.	Secondary key update for inter-CU MCG LTM with DC

In RAN2#129bis meeting, RAN2 agreed the following:

NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

Based on the agreement, secondary key update should be performed only when security key update is performed in MCG. However, the latest RRC running CR [1] does not reflect the agreement, i.e., the secondary key update relys on the exsiting procedure given below.

5.3.5.3 Reception of an RRCReconfiguration by the UE […unrelated texts are omitted…] 1> if the RRCReconfiguration includes the sk-Counter: 2> perform security key update procedure as specified in 5.3.5.7; […unrelated texts are omitted…]

Based on above procedure, whether to perform the secondary key update during MN RRC reconfiguration (e.g., MN handover) does not depend on whether the MN security update is performed but rather on whether sk-Counter is provided. In other words, the secondary key change at inter-CU MCG LTM cell switch with DC is not controlled by ltm-NoSecurityChangeID-r19, but rather by whether or not the LTM candidate configuration includes sk-Counter.

Observation 1. The secondary key change at inter-CU MCG LTM cell switch with DC is not controlled by ltm-NoSecurityChangeID-r19, but rather by whether or not the LTM candidate configuration includes sk-Counter.

In RAN2#125bis meeting, RAN2 agreed the following:

Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

As subsequent LTM cell switch is supported, intra-CU MCG LTM cell switch can happen without RRC reconfiguration after inter-CU MCG LTM cell switch. In this scenario, the secondary key update will be performed during the subsequent intra-CU MCG LTM cell switch, if the candidate configuration of the intra-CU MCG LTM cell switch includes sk-Counter value.

For instance, current PCell is Cell#0, and Cell#1 and Cell#2 are configured as candidate PCells for inter-CU LTM. As Cell#1 and Cell#2 are candidates for inter-CU LTM, the configurations for Cell#1 and Cell#2 include sk-Counter value for secondary key update. If the UE performs LTM cell switch to Cell#1 and then subsequently to Cell#2, the UE will perform the secondary key update in both cell switches, even though the subsequent cell switch, i.e. Cell#1 Cell#2, is intra-CU mobility (i.e., no MCG security update). Such redundant secondary key update can happen during subsequent LTM cell switches in the current RRC running CR.

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to termination point change for the bearer, reconfiguration with sync, resuming an RRC connection, or the first reconfiguration after reestablishment. It is also applicable for LTE procedures when NR PDCP is configured. Network doesn't include this field for DRB if the bearer is configured as DAPS bearer, if the RadioBearerConfig IE is part of an RRCReconfiguration message within the LTM-Config IE, or if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE

Since the security key update leads to the PDCP reestablishment as shown in the field description of reestablishPDCP, redundant secondary key update during subsequent intra-CU MCG LTM cell switches results in redundant PDCP reestablishment at the PDCP entities associated with SN. Consequently, the UE undergoes data interruption due to redundant PDCP reestablishment.

Observation 2. Redundant secondary key update during subsequent intra-CU MCG LTM cell switches can result in redundant PDCP reestablishment at the PDCP entities associated with SN and consequently data interruption can occur.

To avoid the data interruption due to redundant secondary key update during subsequent LTM cell switches, the procedure text needs to be updated. Particularly, for LTM case, the procedure 5.3.5.3 needs to reflect that UE uses the configured value for generating the secondary key when security key update is performed in MCG. We also propose the corresponding text proposal in TP1 in Annex.

Proposal 2. For LTM, the procedure 5.3.5.3 reflects that UE uses the configured value for generating the secondary key when security key update is performed in MCG (i.e., TP1 in Annex).

3.	Conclusion

In this paper, we have made the following observations and proposals:

FFS how NW distinguish between CLTM and fast recovery after CLTM failure

Proposal 1.  In order for network to distinguish between CLTM and fast recovery after CLTM failure, UE sends a goodbye message to the source cell before CLTM execution. Then, the source cell notifies the target cell of CLTM execution via e.g. Cell Switch Notification.

Secondary key update for inter-CU MCG LTM with DC

Observation 1. The secondary key change at inter-CU MCG LTM cell switch with DC is not controlled by ltm-NoSecurityChangeID-r19, but rather by whether or not the LTM candidate configuration includes sk-Counter.

Observation 2. Redundant secondary key update during subsequent intra-CU MCG LTM cell switches can result in redundant PDCP reestablishment at the PDCP entities associated with SN and consequently data interruption can occur.

Proposal 2. For LTM, the procedure 5.3.5.3 reflects that UE uses the configured value for generating the secondary key when security key update is performed in MCG (i.e., TP1 in Annex).

4.	References

[1] [POST130][110][MOB] RRC CR (Ericsson) Running RRC CR for mobility enhancements phase 4

5. Annex

5.1.	TP1

5.3.5.3	Reception of an RRCReconfiguration by the UE

[…unrelated texts are omitted…]

- if the RRCReconfiguration includes the sk-Counter:

2>	if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6:

3>	if the field ltm-NoSecurityChangeID is configured for the LTM-Candidate IE in ltm-Config indicated by lower layers and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or

3>	if the value of field ltm-NoSecurityChangeID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID:

4>	if the LTM cell switch is triggered on the MCG:

5>	if the LTM-Candidate IE indicated by lower layers does not include an mrdc-SecondaryCellGroupConfig set to release:

6>	perform security key update procedure as specified in 5.3.5.7;

2>	else:

3>	perform security key update procedure as specified in 5.3.5.7;

[…unrelated texts are omitted…]