3GPP TSG-RAN WG2 Meeting #131		R2-2505585

Bengaluru, India, 25th – 29th Aug. 2025

Source:	vivo

Title:	Discussion on open issues for asymmetric DL sTRP/UL mTRP

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- Introduction

In the email discussion on [Post130][218][MIMO_Ph5] Running CR for 38.331 (Ericsson), one open issue is identified as below [1]:

| Asymmetric DL sTRP/UL mTRP | In the last meeting, we have agreed to introduce a new parameter to enable Rel-19 2TA configuraiton, currently named singleDCI-MultiTRP-2TA. According to RAN1 UE feature, Rel-19 2TA is introduced for both intra-cell 2TA with no mDCI mTRP and inter-cell 2TA with no mDCI mTRP. The new parameter should be applied to both and LS to RAN1 may be needed to inform this. | This could be discussed based on company contributions. |
| --- | --- | --- |

In this contribution, we will discuss the above open issues related to asymmetric DL sTRP/UL mTRP.

- Discussion

  - singleDCI-MultiTRP-2TA for inter-cell and intra-cell 2TA (RRC-x)

- In RAN2#130 meeting, it was agreed to introduce a new RRC parameter (i.e. singleDCI-MultiTRP-2T) per BWP that explicitly enables the Rel-19 sDCI-mTRP 2TA, as below:

Introduce a new RRC parameter per BWP that explicitly enables the Rel-19 sDCI-mTRP 2TA, and clarify in FD of tag2 to include all cases where tag2 is configured that “it is optionally configured in a serving cell for mDCI mTRP 2TA if coresetPoolIndex for a BWP is configured with more than one value, and for sDCI mTRP 2TA if [the new parameter] is configured.”;

In the RAN1 feature list provided in the LS [2], RAN1 agreed to introduce 2TA enhancements for both intra-cell and inter-cell cases, with two separate sub-features, i.e. 59-4-4a/4b, as below.

| 59. NR_MIMO_Ph5 | 59-4-4a | Support two TAs enhancement for intra-cell beam management | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management | 23-1-1 | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management is not supported |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-4b | Support two TAs enhancement for inter-cell beam management operation | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management | 23-1-1a | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management is not supported |

The remaining issue is whether the new RRC parameter for Rel-19 sDCI-mTRP 2TA, i.e. singleDCI-MultiTRP-2TA, can be applicable for both inter-cell and intra-cell 2TA.

In our understanding, dual-TAG operations should be independently enabled for intra-cell and inter-cell beam management, due to the below two reasons:

- Downlink reference timing assumptions differ between scenarios: For inter-cell mTRP, UEs measure SSBs from both neighbor and serving cells, whereas intra-cell beam management requires measurement of SSBs from only a single TRP.

- sDCI-based multi-TRP is unsupported for inter-cell operation. Consequently, Feature Group 59-4-4b (FG 59-4-4b) provides dual-TAG support specifically for single-TRP (sTRP) configurations, which is distinct from FG 59-4-4a.

With this, we think introducing two separate RRC parameters to enable inter-cell and intra-cell 2TA, respectively.

(RRC-x) Two RRC parameter for Rel-19 sDCI-mTRP 2TA, e.g. singleDCI-MultiTRP-IntraCell2TA, and singleDCI-MultiTRP-InterCell2TA, are introduced in RAN2 to enable intra-cell and inter-cell 2TA, respectively.

- Conclusion

In this contribution, we discuss the RAN2 RRC open issues for asymmetric DL sTRP/UL mTRP. Based on the discussion above, we have the following proposals:

RRC open issues:

- (RRC-x) Two RRC parameter for Rel-19 sDCI-mTRP 2TA, e.g. singleDCI-MultiTRP-IntraCell2TA, and singleDCI-MultiTRP-InterCell2TA, are introduced in RAN2 to enable intra-cell and inter-cell 2TA, respectively.

- References

- R2-2505807, Open issues for MIMO on 38.331, Ericsson

- R2-2505010, LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#121, RAN1/Docomo