3GPP TSG-RAN WG2 #131	R2-2505588

Bengaluru, India, 25 – 29 August 2025

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on LP-WUS in IDLE and INACTIVE

Document for:	Discussion and decision

Agenda Item:	8.4.2

- Introduction

In the email discussion on [Post130][222][LPWUS] Potential solution to support enabling/disabling LP-WUS monitoring in IDLE/INACTVE per UE (Huawei) [1], the following open issue was discussed with no conclusion:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RRC-12 | whether/how to enable/disable LP-WUS, e.g. by RRC/NAS | Issue Type: not essential How to address it: can be discussed based on companies’ contribution |

In this contribution, we will further discuss the above issue for RRC_IDLE/ INACTIVE procedures with LP-WUR.

- Discussion

In the email discussion, the following observation and proposal are made based on the companies’ inputs.

Observation (9/15): Many prefer to specify either NAS or RRC signalling to support enabling/disabling LP-WUS per UE. Proposal: (RRC-12) RAN2 to down-select from NAS signalling or RRC signalling to support enabling/disabling LP-WUS per UE.

Regarding the down-selection between NAS signalling or RRC signalling to support enabling/disabling LP-WUS per UE, we prefer RRC signalling. We believe that LP-WUS enabling/disabling handling is tightly related to gNB implementation to control the IDLE/CONNECTED. The decision of whether or not to transition the UE to IDLE mode is determined by the implementation of the gNB. In the implementation of the gNB, for example, the decision to transition to IDLE mode may be based on considerations such as the resource utilization status of the gNB or the availability of other frequency bands. When disabling LP-WUS under NAS control, these considerations cannot be taken into account, which may result in the unnecessary disabling of LP-WUS.

Details of RRC signalling method can be found in section 2.2.2 in [1]. In summary, enabling/disabling indication of LP-WUS per UE can be included in RRCRelease, and it indicates UE to enable/disable LP-WUS for the UE.

Proposal 1: RRC signalling is supported for enabling/disabling LP-WUS per UE. Enabling/disabling indication of LP-WUS per UE can be included in RRCRelease.

When NAS signalling or RRC signalling is introduced to support enabling/disabling LP-WUS per UE, discussion in RAN3 is needed. For example, the following are the discussion points in our view:

- NAS signalling method: indication of LP-WUS enabling/disabling by paging message

- RRC signalling method: indication of LP-WUS enabling/disabling by paging message and indication of LP-WUS enabling/disabling from gNB to AMF

Considering the remaining TU of RAN3, it is recommended that RAN2 sends an urgent LS to RAN3, which requests RAN3 to discuss the related procedures. A draft LS is presented in Annex in this document.

Proposal 2: RAN2 sends an urgent LS to RAN3 requesting RAN3 to discuss the related procedures when RRC signalling or NAS signalling is supported for enabling/disabling LP-WUS per UE.

It should also be discussed whether to define UE behavior when the enabling/disabling indication is not included in RRCRelease if RRC signalling method is supported. It is straightforward that UE monitors LP-WUS based on CN or UE-ID based subgrouping as defined in the running CR of TS 38.304 [2]. However, it is better to disable LP-WUS monitoring by default when the enabling/disabling indication is not included in RRCRelease in some cases. For example, if UE moves to a cell of gNB supporting LP-WUS feature after initial access on another cell of gNB NOT supporting LP-WUS feature, paging loss will occur due to misunderstanding on LP-WUS configuration between UE and gNB. The UE will assume LP-WUS monitoring, while the gNB supporting LP-WUS feature will not assume LP-WUS monitoring for the UE because it does not know that the UE is LP-WUS capable. In order to avoid such a paging false issue, we think LP-WUS monitoring should be disabled by default when the enabling/disabling indication is not included in RRCRelease.

Proposal 3: If RRC signalling is supported, LP-WUS monitoring is disabled by default when enabling/disabling indication of LP-WUS per UE is not included in RRCRelease.

- Conclusion

In this contribution, we discuss the remaining issues for RRC_IDLE/ INACTIVE procedures with LP-WUR. Based on the discussion, we have the following proposals.

Proposal 1: RRC signalling is supported for enabling/disabling LP-WUS per UE. Enabling/disabling indication of LP-WUS per UE can be included in RRCRelease.

Proposal 2: RAN2 sends an urgent LS to RAN3 requesting RAN3 to discuss the related procedures when RRC signalling or NAS signalling is supported for enabling/disabling LP-WUS per UE.

Proposal 3: If RRC signalling is supported, LP-WUS monitoring is disabled by default when enabling/disabling indication of LP-WUS per UE is not included in RRCRelease.

- References

[1]	R2-2505381, Summary of [Post130][222][LPWUS] Potential solution to support enabling/disabling LP-WUS monitoring in IDLEI/NACTVE per UE (Huawei), Huawei

[2]	R2-2505234, 38.304 Running CR for LP-WUS, CATT

# Annex: LS to RAN3

3GPP TSG-RAN WG2 #131	R2-25xxxx

Bengaluru, India, 25 – 29 August 2025

Title:	[Draft] LS on enabling/disabling LP-WUS monitoring in IDLE/INACTIVE per UE

Response to:	-

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	NTT DOCOMO (to be RAN2)

To:	RAN3

Cc:	-

Contact Person: 	Yuki Takahashi

yuki.takahashi.sd@nttdoomo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall Description:

RAN2 has discussed solution to support enabling/disabling LP-WUS monitoring in IDLE/INACTIVE per UE and agreed that [NAS signalling/RRC signalling] method is supported at ongoing RAN2#131 meeting. Detailed method can be found in R2-25xxxx[RAN2 discussion summary].

RAN2 think there are some spec impacts on RAN3 and would like to request RAN3 to take them into account on their specification work.

- Actions:

To RAN WG3

RAN2 respectfully requests RAN3 to take the above information into account for their specification work, and provide feedback, if any.

- Date of Next RAN2 Meetings:

RAN2#131-bis	2025-10-13 – 2025-10-17		Prague, CZ

RAN2#132	2025-11-17 – 2025-11-21		Dallas, US