3GPP TSG-RAN WG2 Meeting #131                                      R2-2505592

Bangalore, India, Aug 25th – 29th, 2025

Source:        ZTE Corporation

Title:           Discussion on multiple carrier and measurements in SBFD

Agenda item:   8.11.3

Document for:  Discussion and Decision

- Introduction

This contribution provides our view on multiple carrier and L3 measurement with SBFD feature introduced.

- Discussion

SBFD+NR-DC

RAN1#120 has made the following agreement to support the co-existence of multiple carrier and Rel-19 SBFD:

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

RAN4 also has the following agreements achieved:

Agreement Rel-19 requirement shall consider both (1) single carrier operation for SBFD (2) SBFD operates in only one BS carrier, and legacy TDD operates in other intra-band BS carrier(s) contiguous or non-contiguous to the SBFD carrier.

So the SBFD is confirmed supportive in CA (e.g., intra-band CA), i.e., some of the UE’s serving cells are SBFD cell, while other serving cells can be legacy cell; or all of the UE’s serving cells are SBFD cell including Pcell and Scells.

RAN3#126 meeting also has agreement to coordinate SBFD time/frequency configuration between gNBs in XnAP message:

RAN3#126 agreement The SBFD time and frequency location configuration with cell granularity in Served Cell Information NR IE shall be introduced in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

The XnAP setup message is also the initial step for two RAN nodes to establish DC. So if gNBs can coordinate SBFD time/frequency configuration using this message already, and it means the SBFD together with NR-DC are supported at least from signaling perspective, implicitly.

Observation 1: RAN1, RAN3 and RAN4 has already agreed the SBFD cell exists in multi-carrier scenario, i.e., CA.

RAN1 has replied the LS R1-2504858[1] indicating that SBFD+NR-DC are feasible:

Agreement - From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier. - Inform RAN2 that specification impact to support this feature is not restricted to RAN1 but might result in specification impact on RAN4 as well. If supported, at least the following specification change to TS 38.213 should be adopted and the RAN4 specification for the configured transmitted power level for NR-DC in TS38.101-1 Clause 6.2B.4.1 may also need to be updated accordingly.

Observation 2: From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC.

Based on above, from RAN2 perspective, RAN2 does not have any reason to exclude this co-existance. We think supporting SBFD cell to be configured in NR-DC framework is feasible, since the two gNB can both be Rel-19 gNB and both are aware of the SBFD configuration. In addition, it makes the feature to be more useful in different deployment. So we have the following proposal:

Proposal 1: Support SBFD cell to be configured in NR-DC. Either MCG or SCG can configure SBFD cell. SBFD operation is applicable on only one NR TDD carrier in NR-DC.

Regarding the spec impact of SBFD+NR-DC：

- Since RAN1 asks that SBFD operation is applicable on only one NR TDD carrier, not both CGs, there is no need to interact configuration restriction (such as maximum number of L1 CLI resource, etc.) between MN and SN.

- Also, OAM can decide it should be MCG or SCG to configure the SBFD cell, so there is no need to coordinate between MN and SN that whether the MN/SN has configured SBFD cell or not.

- Since the inter-node SBFD time/frequency resource interaction has already be captured in RAN3 signaling (XnAP XN SETUP REQUEST, XnAP XN SETUP RESPONSE, XnAP NG-RAN NODE CONFIGURATION UPDATE and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE), so there is no additional RAN2 spec impact on coordinating SBFD time/frequency resource between MN and SN.

Thus, there is no stage-3(RRC) spec impact on supporting SBFD+NR-DC.

Proposal 2: There is no stage-3 spec impact of supporting SBFD+NR-DC.

For the stage-2 TS37.340 [2] spec impact:

- At least the RAN1’s conclusion ‘SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier’ should be captured;

- Since only one SBFD carrier, the L1 CLI measurement should only be configured in NR-DC by either MCG or SCG, not both, and this should be captured in the session 7.2, Measurement.

The corresponding TS37.340 TP is provided in Annex.

Proposal 3: RAN2 to adopt the TS37.340 (stage-2) CR for SBFD+NR-DC. Take the CR in Annex as baseline.

L3 measurement filtering

According to 38.300, the UE measures multiple beams (at least one) of a cell and the measurements results (power values) are averaged to derive the cell quality. In doing so, the UE is configured to consider a subset of the detected beams. Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams.

Usually the interference of DL reception on SBFD symbols is larger than the DL reception on non-SBFD symbols. So even if gNB transmits same RS power on the two symbol types, the received RSRP/RSRQ/SINR on SBFD symbols will be different than the received RSRP/RSRQ/SINR on non-SBFD symbols.

Observation 3: Due to the interference, the received RS-RSRP/RSRQ/SINR on SBFD symbols will be different than the received RS-RSRP/RSRQ/SINR on non-SBFD symbols.

For CSI-RS measurement, RAN1 has made the agreement that UE only uses one symbol type (SBFD symbol or non-SBFD symbol) to generate L1 CSI report:

Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig.

For CSI-RS this is ok, because CSI-RS is configured very flexible. However SSB configuration/transmission is more fixed. The SSB indexes/beams on the SBFD symbol can be fixed considering the SSB pattern is fixed and SBFD pattern is also fixed for the cell. This means some specific SSB beam directions may always be on the SBFD symbol, so the SSB beams on SBFD symbol will have different level of RSRP/RSRQ/SINR compared to other SSB beams. When generating L3 measurements, such SSB beams on the SBFD symbols should also participate in L3 filtering with equal possibility compared to the SSB beams on the non-SBFD symbols, in order to evaluate the cell quality fairly/accurately. Otherwise, those SSB beams on the SBFD symbols will always be absent in deriving cell measurement.

Observation 4: Since the SBFD configuration and SSB beam/index/periodicity are cell-specific and pretty fixed, it is very likely that some specific SSB beam directions are always on the SBFD symbols.

Observation 5: If UE only takes the several specific SSB beam directions on SBFD symbol and non-SBFD symbol for L3 measurement, the UE cannot well evaluate the cell quality equally/accurately since a cell has multiple beams.

Observation 6: When generating L3 measurements, the SSB beams on the SBFD symbols should also be taken into L3 filtering with equal possibility compared to the SSB beams on the non-SBFD symbols, in order to evaluate the cell quality fairly/accurately.

Legacy beam measurement selection is based on one configured power threshold. Only the beam measurement that the received power is beyond the power threshold will be selected to derive the cell measurement. However if the beam measurement on SBFD symbols also adopt the same power threshold, it is likely that beam measurement on SBFD symbols will easily fail to derive the cell measurement since the receive power on SBFD symbols will be lower.

Based on the above observations, RAN2 should consider how to apply proper filtering for UE to generate a cell measurement using the beam measurements on SBFD symbols. The procedure from L1 measurement to L3 measurement is as following:

TS38.300: Figure 9.2.4-1: Measurement Model

There can be at least the following methods:

- Method 1: From A to A1, UE itself adds a power offset to the RS beam measurement (RSRP/RSRQ/SINR) on SBFD symbols, to compensate the interference occurred during SBFD symbols. This method may require RAN4’s further feasibility analysis.

- Method 2: From A1 to B, gNB can configure different power threshold on SBFD symbols and non-SBFD symbols, i.e., gNB configures additional power threshold to UE, which is dedicated for the beam measurement made on SBFD symbols. Only when the RS beam measurement on the SBFD symbols is beyond the additional power threshold, the RS beam measurement is allowed to consolidate a cell measurement by the UE. The method 2 is more flexible since gNB can determine the value of the additional power threshold based on current interference level and the RS transmit power on SBFD symbols, which makes the cell measurement more accurate and credible.

Based on above, we propose that RAN2 supports method 2.

Proposal 4: gNB configures additional power threshold to UE, the additional power threshold is used for the consolidation of cell measurement from the beam measurements made on SBFD symbols.

- Conclusion

In this contribution, we propose the following observation and proposals:

SBFD+NR-DC

Observation 1: RAN1, RAN3 and RAN4 has already agreed the SBFD cell exists in multi-carrier scenario, i.e., CA.

Observation 2: From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC.

Proposal 1: Support SBFD cell to be configured in NR-DC. Either MCG or SCG can configure SBFD cell. SBFD operation is applicable on only one NR TDD carrier in NR-DC.

Proposal 2: There is no stage-3 spec impact of supporting SBFD+NR-DC.

Proposal 3: RAN2 to adopt the TS37.340 (stage-2) CR for SBFD+NR-DC. Take the CR in Annex as baseline.

L3 measurement filtering

Observation 3: Due to the interference, the received RS-RSRP/RSRQ/SINR on SBFD symbols will be different than the received RS-RSRP/RSRQ/SINR on non-SBFD symbols.

Observation 4: Since the SBFD configuration and SSB beam/index/periodicity are cell-specific and pretty fixed, it is very likely that some specific SSB beam directions are always on the SBFD symbols.

Observation 5: If UE only takes the several specific SSB beam directions on SBFD symbol and non-SBFD symbol for L3 measurement, the UE cannot well evaluate the cell quality equally/accurately since a cell has multiple beams.

Observation 6: When generating L3 measurements, the SSB beams on the SBFD symbols should also be taken into L3 filtering with equal possibility compared to the SSB beams on the non-SBFD symbols, in order to evaluate the cell quality fairly/accurately.

Proposal 4: gNB configures additional power threshold to UE, the additional power threshold is used for the consolidation of cell measurement from the beam measurements made on SBFD symbols.

- Reference

- R1-2504858, Reply LS on simultaneous configuration of SBFD and DC, RAN1, LS in, 2025-05.

- TS37.340-i60, ‘Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Overall Description; Stage 2’, Release 18, 2025-06.

- Annex: TP to TS37.340-i60

- 3	Definitions, symbols and abbreviations

- 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1], TS 36.300 [2] and TS 38.300 [3].

BFD	Beam Failure Detection

CHO	Conditional Handover

CLI	Cross Link Interference

CPA	Conditional PSCell Addition

CPAC	Conditional PSCell Addition or Change

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DC	Intra-E-UTRA Dual Connectivity

DCP	DCI with CRC scrambled by PS-RNTI

EN-DC	E-UTRA-NR Dual Connectivity

IAB	Integrated Access and Backhaul

IDC	In-Device Coexistence

LTM	L1/L2 Triggered Mobility

MCG	Master Cell Group

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

MUSIM	Multi-Universal Subscriber Identity Module

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR-DC	NR-NR Dual Connectivity

QMC	QoE Measurement Collection

QoE	Quality of Experience

RLM	Radio Link Monitoring

SBFD	Subband Full Duplex

SCG	Secondary Cell Group

SMTC	SS/PBCH block Measurement Timing Configuration

SN	Secondary Node

SPR	Successful PSCell Addition/Change Report

V2X	Vehicle-to-Everything

<Unchanged part omitted>

- 7	RRC related aspects

- 7.2	Measurements

If the measurement is configured to the UE in preparation for the Secondary Node Addition procedure described in clause 10.2, the Master node should configure the measurement to the UE.

In case of the intra-secondary node mobility described in clause 10.3, the SN should configure the measurement to the UE in coordination with the MN, if required.

The Secondary Node Change procedure described in clause 10.5 can be triggered by both the MN (only for inter-frequency secondary node change) and the SN. For secondary node changes triggered by the SN, the RRM measurement configuration is maintained by the SN which also processes the measurement reporting, without providing the measurement results to the MN.

Measurements can be configured independently by the MN and by the SN (intra-RAT measurements on serving and non-serving frequencies). The MN indicates the maximum number of frequency layers and measurement identities of intra-frequency and inter-frequency measurement that can be used in the SN to ensure that UE capabilities are not exceeded. In MR-DC, to assist MN to identify the measurement type, the SN indicates to the MN the list of SCG serving frequencies. In NR-DC, to assist SN to identify the measurement type, the MN indicates also to SN the list of MCG serving frequencies. The SN can also request the MN for new maximum values of the number of measurement identities that it can configure, and it is up to the MN whether to accommodate the SN request, based on the capability coordination principles as described in 7.3. If the SN receives from the MN a new value for the maximum number of measurement identities, is SN responsibility to ensure that its configured measurement identities to comply with the new limit.

If MN and SN both configure measurements on the same carrier frequency then the configurations need to be consistent (if the network wants to ensure these are considered as a single measurement layer). Each node (MN and SN) can configure independently a threshold for the SpCell quality. In (NG)EN-DC scenario, when the PCell quality is above the threshold configured by the MN, the UE is still required to perform inter-RAT measurements configured by the MN on the SN RAT (while it's not required to perform intra-RAT measurements); when the PSCell quality is above the threshold configured by the SN, the UE is not required to perform measurements configured by the SN. In NR-DC or NE-DC scenario, when the PCell quality is above the threshold configured by the MN, the UE is not required to perform measurements configured by the MN; when the PSCell quality is above the threshold configured by the SN, the UE is not required to perform measurements configured by the SN.

NOTE:	The SN cannot renegotiate the number of frequency layers allocated by the MN in this version of the protocol.

In MR-DC, both the MN and the SN can configure CGI reporting. The MN can configure CGI reporting for intra-RAT and inter-RAT cells but the SN can only configure CGI reporting of intra-RAT cells. At any point in time, the UE can be configured with at most one CGI reporting configuration. For CGI reporting coordination, the SN sends the CGI measurement request and the embedded CGI reporting configuration to the MN. Optionally, the SN sends the unknown cell information to the MN. If there is no ongoing CGI reporting measurement on UE side, the MN forwards the SN CGI measurement configuration to UE. Otherwise the MN rejects the request by sending X2/Xn reject message. In case the SN indicates the unknown cell information, and the CGI information of the requested cell is already available in the MN, the MN can also reject the request, and sends the CGI information of the requested cell to the SN. The SN cannot configure the CGI measurement using the SRB3.

Both MN-configured and SN-configured RRM measurements are supported while the SCG is deactivated. The PSCell measurement cycle when in deactivated SCG state is configured by RRC.

When SRB3 is not configured or the SCG is deactivated, reports for measurements configured by the SN are sent on SRB1. When SRB3 is configured and SCG transmission of radio bearers is not suspended and the SCG is not deactivated, reports for measurements configured by the SN are sent on SRB3.

Measurement results related to the target SN can be provided by MN to target SN at MN initiated SN change procedure. Measurement results of target SN can be forwarded from source SN to target SN via MN at SN initiated SN change procedure. Measurement results related to the target SN can be provided by source MN to target MN at Inter-MN handover with/without SN change procedure.

Measurement results according to measurement configuration from the MN are encoded according to SN RRC when they are provided by MN to SN in SgNB Addition Request message / SN Addition Request message. During SN initiated SN change procedure, measurement results according to measurement configuration from SN are encoded according to SN RRC when they are provided by MN to SN in SgNB Addition Request message / SN Addition Request message.

Per-UE or per-FR measurement gaps can be configured, depending on UE capability to support independent FR measurement and network preference. Per-UE gap applies to both FR1 (E-UTRA, UTRA-FDD and NR) and FR2 (NR) frequencies. For per-FR gap, two independent gap patterns (i.e. FR1 gap and FR2 gap) are configured for FR1 and FR2 respectively. The UE may also be configured with a per-UE gap sharing configuration (applying to per-UE gap) or with two separate gap sharing configurations (applying to FR1 and FR2 measurement gaps respectively) [8].

A measurement gap configuration is always provided:

-	In EN-DC, NGEN-DC and NE-DC, for UEs configured with E-UTRA inter-frequency measurements as described in table 9.1.2-2 in TS 38.133 [8];

-	In EN-DC and NGEN-DC, for UEs configured with UTRAN and GERAN measurements as described in table 9.1.2-2 in TS 38.133 [8];

-	In NR-DC, for UEs configured with E-UTRAN measurements as described in table 9.1.2-3 in TS 38.133 [8];

-	In NR-DC, NE-DC, for UEs configured with UTRAN measurements as described in table 9.4.6.3-1 and 9.4.6.3-2 in TS 38.133 [8];

-	In MR-DC, for UEs that support either per-UE or per-FR gaps, when the conditions to measure SSB based inter-frequency measurement or SSB based intra-frequency measurement as described in clause 9.2.4 in TS 38.300 [3] are met;

If per-UE gap is used, the MN decides the gap pattern and the related gap sharing configuration. If per-FR gap is used, in EN-DC and NGEN-DC, the MN decides the FR1 gap pattern and the related gap sharing configuration for FR1, while the SN decides the FR2 gap pattern and the related gap sharing configuration for FR2; in NE-DC and NR-DC, the MN decides both the FR1 and FR2 gap patterns and the related gap sharing configurations.

In EN-DC and NGEN-DC, the measurement gap configuration from the MN to the UE indicates if the configuration from the MN is a per-UE gap or an FR1 gap configuration. The MN also indicates the configured per-UE or FR1 measurement gap pattern and the gap purpose (per-UE or per-FR1) to the SN. Measurement gap configuration assistance information can be exchanged between the MN and the SN. For the case of per-UE gap, the SN indicates to the MN the list of SN configured frequencies in FR1 and FR2 measured by the UE. For the per-FR gap case, the SN indicates to the MN the list of SN configured frequencies in FR1 measured by the UE and the MN indicates to the SN the list of MN configured frequencies in FR2 measured by the UE.

In NE-DC, the MN indicates the configured per-UE or FR1 measurement gap pattern to the SN. The SN can provide a gap request to the MN, without indicating any list of frequencies.

In NR-DC, the MN indicates the configured per-UE, FR1 or FR2 measurement gap pattern and the gap purpose to the SN. The SN can indicate to the MN the list of SN configured frequencies in FR1 and FR2 measured by the UE.

In (NG)EN-DC and NR-DC, SMTC can be used for PSCell addition/PSCell change to assist the UE in finding the SSB in the target PSCell. In case the SMTC of the target PSCell is provided by both MN and SN it is up to UE implementation which one to use.

L3 CLI measurements can be configured for NR cells in all MR-DC options. In EN-DC and NGEN-DC, only the SN can configure L3 CLI measurements. In NE-DC, only the MN can configure L3 CLI measurements. In NR-DC, both the MN and the SN can configure L3 CLI measurements, and the MN informs the SN about the maximum number of L3 CLI measurement resources that can be configured by the SN to ensure that the total number of L3 CLI measurement resources does not exceed the UE capabilities. L1 CLI measurement can only be configured in NR-DC, and either MCG or SCG can configure the L1 CLI measurement.

For MUSIM operation, when the UE is configured to operate in NR-DC in Network A (as described in TS 38.300 [3]), the MN indicates the per-UE MUSIM gap configuration to the SN.

<Unchanged part omitted>

- 13	Other aspects

- 13.1	Interference avoidance for in-device coexistence

IDC solution as described in TS 36.300 [2] and TS 38.300 [3] is extended to address EN-DC/NR-DC operation. For the FDM solution, the list of NR carriers or NR frequency ranges suffering from IDC problems is signalled in IDC report. For the TDM solution, a periodic pattern can be signalled per-CG in IDC report. In EN-DC, the MN can configure the UE to report FDM assistance information with affected carriers. In NR-DC, the MN can configure the UE to report FDM assistance information with affected carriers/frequency ranges and/or TDM assistance information. For both EN-DC and NR-DC, the SN can configure the UE to report FDM assistance information with affected carriers/frequency ranges and/or TDM assistance information to the SN via SRB1 or SRB3, if SRB3 is configured and the SCG is activated. The network can also configure autonomous denial per-CG for the UE to solve IDC problems. The requirement on RRM/RLM/CSI measurements in different phases of IDC interference defined in TS 36.300 [2] is applicable except that for NR serving cell, the requirements in TS 38.133 [8] and TS 38.101-1 [12], TS 38.101-2 [13], TS 38.101-3 [14] apply.

- 13.2	Sidelink

NR Sidelink Communication, V2X Sidelink Communication, NR Sidelink Discovery and Ranging/Sidelink Positioning cannot be configured in MR-DC in this release.

- 13.3	SCG UE history information

The MN stores and correlates the UE History Information from MN and SN(s) as long as the UE stays in MR-DC, forwards UE History Information and optional UE History Information from the UE to its connected SNs. The resulting information is then used by SN for dual-connectivity operation. The SN is in charge of collecting SCG UE history information and providing the collected information to the MN.

If the UE stays in a PSCell for a duration exceeding the maximum value of the Time Stay parameter, the SN may store the PSCell information with consecutive entries using the same PSCell identity. The total stay time in this PSCell is the sum of stay time for all consecutive PSCell with the same identity.

The SN shall provide the collected SCG UE history information, if available, to the MN in the following procedures:

-	the SN Release, and SN initiated SN Change procedures

-	the MN initiated SN Modification procedure if requested by the MN in this procedure

-	the SN initiated SN modification procedure upon PSCell change if subscribed in the SN Addition procedure

When the target NG-RAN node receives the SCG UHI from the source NG-RAN node via Handover Request message for CHO, the target NG-RAN node updates the time UE stayed in cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target NG-RAN node. The updated value of the time UE stayed in the source PSCell is equal to the value received from the source NG-RAN node during the Handover Preparation plus the time from receiving Handover Request message from the source NG-RAN node to receiving RRC Reconfiguration Complete message from the UE.

When the target SN receives the SCG UHI from the MN via SN Addition Request message for CPC, the target SN updates the time UE stayed in the cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target SN. The updated value of the time UE stayed in the latest PSCell is equal to the value received from the MN via the SN Addition Request message plus the time from receiving SN Addition Request message from the MN to receiving SN Reconfiguration Complete from the MN.

<Unchanged part omitted>

- 13.5	SBFD operation

SBFD operation and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier.