3GPP TSG-RAN WG2 Meeting #131	R2-2505619

Bangalore, India  Aug 25th – 29th , 2025

Source:				ETRI

Title:					Remaining Issues on DSR enhancements

Agenda Item:		8.7.4 Scheduling enhancements

Document for:		Discussion and Decision

1. Introduction

During the discussion of DSR enhancements at the RAN2#130 meeting, agreements on DSR enhancements have been made as the followings:

Agreements on DSR enhancements

- (MAC-03) For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR.

- (MAC-03) In DC, no enhancements are needed when a pending DSR is cancelled because all its associated PDCP SDUs have been discarded or included in a MAC PDU.

- (MAC-03) An understanding is that if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.

During the e-mail discussion on MAC running CR, the open issues related to the DSR enhancements was identified as the followings:

Open issues on the DSR enhancements [MAC-1] Discuss whether a pending DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported. [MAC-2] Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE. [MAC-6] Whether any spec change is needed to capture the agreement on DSR cancellation in the DC case.

In this contribution, we provide discussions on the open issues for the DSR enhancements.

2. Discussion

Cancelling the pending DSR

[MAC-1]		Discuss whether a pending DSR should be cancelled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported.

The Multiple Entry DSR MAC CE can report delay information with multiple reporting thresholds for an LCG and a data volume with a reporting threshold are calculated and included in the DSR MAC CE. When an LCG is configured with remaingTimeThreshold for triggering a DSR, the MAC entity triggers the DSR for a logical channel within the LCG if the smallest remaining value of the delay critical PDCP PDU becomes below the remaingTimeThreshold of the LCG. The pending DSR can be cancelled when the delay status for all the PDCP PDU associated with the DSR has been reported or there are no delay critical data associated with the DSR in the buffer, which has been discarded or transmitted.

The DSR reports delay status of data in the buffer, enabling the scheduler to prioritize resource allocation for delay critical data. When UE is configured with a dsr-ReportingThresList for a LCG, a reporting threshold can be applied for either non-delay critical data or delay critical data. If the reporting threshold is smaller than the threshold used for declaration of delay-critical data, the volume of PDCP SDUs within the range is delay-critical data. Otherwise, the volume within the rage is calculated for non-delay critical data.

The current running MAC CR propose that the pending DSR shall be cancelled if no associated delay-critical data remains in the buffer. Upon cancellation, the delay status of any non-delay critical data related to that DSR is no longer reported to the gNB with that DSR. This non-delay critical data remains in the buffer and is managed through typical report procedures. As time elaspes, this data may eventually become delay critical data and trigger a new DSR, or its delay status may be included in a subsequent DSR. This is considered a basic operational procedure for handling non-delay critical data. Even after the pending DSR is cancelled, the associated non-delay critical data continues to be managed by the typical BSR and DSR mechanism.

Observation 1) Even after the pending DSR is cancelled, the associated non-delay critical data continues to be managed by the typical BSR and DSR mechanism.

Reporting the delay status of non-delay critical data, which is included in a pending DSR, provides valuable information for efficient resource management. While this is distinct from the primary goal of prioritizing delay critical data, there are benefits to reporting the delay status for non-delay critical that meet the conditions defined in the dsr-ReportingThresList. As an optional feature, the pending DSR is not required to be cancelled. As a result, a DSR MAC CE can be sent containing only the delay status of non-delay critical data, even when there is no delay-critical data in the buffer and its delay status is not part of the DSR. This provides the gNB with valuable information for effective resource management. It allows the network to stay informed about the overall buffer conditions and to make more proactive scheduling decisions.

Observation 2) Reporting the delay status of only non-delay critical data in a DSR MAC CE is beneficial, even when no delay-critical data is present.

The current MAC CR under discussion focuses on the cancellation of a pending DSR when there is no longer any delay critical data present. To enhance network awareness and resource management, we propose adding an optional feature that allows for more flexible DSR behavior. As an optional functionality, a DSR can be transmitted containing only the delay status of non-delay critical data. This allows the UE to report buffer conditions to the gNB even when no delay critical data is available, providing valuable information for proactive scheduling and more efficient resource utilization.

Proposal 1) A MAC entity may transmit a DSR MAC CE containing only the delay status of non-delay critical data if configured. Otherwise, the pending DSR is cancelled.

Cancelling the pending BSR

[MAC-2]		Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

A DSR includes data volume and delay status when a LCG is configured with dsr-ReportingThresList. A BSR, triggered for either regular or periodic events, reports the total data volume ofr all data in the buffer, encompassing both delay critical data and non-delay critical data. Conversely, a DSR reports the volume of data that satisfies the reporting threshold criteria. It is noted that under certain conditions, the data volume reported by a BSR may be identical to the data volume reported by a DSR. In Rel-18, DSR and BSR operated independently, and optimizations—such as canceling a pending BSR upon DSR transmission—were not specified for this particular scenario.

The need to transmit a BSR is currently under discussion when a DSR carrying the data volume of all the PDCP SDUs has already been sent. Optimizing signaling procedures by canceling a pending BSR upon transmission of a DSR containing the same data volume and delay status information would be beneficial, as it prevents redundant signaling. However, such an optimization introduces an interdependency between BSR and DSR operations, which requires careful consideration. It is a preferred design principle that BSR and DSR operations should operate independently. This ensures that the functionality of each report remains clear and self-contained, avoiding the complexities and potential side effects that could arise from a shared dependency.

Proposal 2) DSR and BSR operations remain independent without optimization to ensure clear and simple behavior.

3. Conclusion

In the discussion we made the following observations:

Observation 1) Even after the pending DSR is cancelled, the associated non-delay critical data continues to be managed by the typical BSR and DSR mechanism.

Observation 2) Reporting the delay status of only non-delay critical data in a DSR MAC CE is beneficial, even when no delay-critical data is present.

We propose the followings:

Proposal 1) A MAC entity may transmit a DSR MAC CE containing only the delay status of non-delay critical data if the option is configured. Otherwise, the pending DSR is cancelled.

Proposal 2) DSR and BSR operations remain independent without optimization to ensure clear and simple behavior.

4. References