3GPP TSG-RAN WG2 Meeting #131	R2-2505632

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on UL capacity enhancement for IoT NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

As one target of WID[1], support of capacity enhancement for UL needs to be studied and specified. One of the motivations is to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction. More specifically, it includes enhancement to transmit Msg3 without Msg1/Msg2 and enhancement to efficiently deliver Msg4.

- Support of Capacity enhancements for uplink - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In the post-130 meeting  [306] and [307] email discussions for RRC and MAC, some open issues were identified to be further discussed. In this paper, we provide further view on these remaining issues. Specifically, we also provide our view on two additional issues (i.e, the network-indicated fallback for CB-Msg3 and MPDCCH narrowband set for CB-Msg4 monitoring).

# 2	Discussion

## 2.1 MAC-2: CB-RNTI calculation

In last meeting, RAN2 made the working assumption on CB-RNTI calcuation as below:

Working Assumption: The formula for RNTI for mMsg4 monitoring is: RNTI=X + Msg3_W_index modulo (Y) + Y*CE_level + 3*Y*carrier_id. - X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=2401 for eMTC or 4097 for NB-IoT, - Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19, - Y is ceil (Msg4_WS/Msg3_WP), - CE_level is the CE level, 0 <= CE_level < 3 - carrier_id is the index of the UL carrier of the CB-Msg3 resources, anchor carrier has index 0, 0 <= carrier_id < 16 - The value of Msg4_WS is the maximum Msg4 window size - The value of Msg3_WP is the minimum Msg3 window periodicity

In the post-meeting email discussion [2], rapporteur proposed RAN2 to further discuss the two issues related to CB-RNTI calculation:

Proposal 2: For the agreed CB-RNTI formula from P1, RAN2 to discuss - Whether to define Msg3_W_index to floor(start SFN_id of Tx window / minimum Tx window length or periodicity] - Whether to define Y = 16.

In the rapporteur proposal, Msg3_W_index is calculated by using the minimum Tx window length or periodicity to calculate. This will make the Msg3_W_index not consecutive numbers corresponding to the adjacent Msg3 Tx windows. For instance, if the configured Tx window periodicity is 500ms (or 50 SF) and the SFN_id for the first Tx window is 10, the SFN_id for the next Tx window will be 60. If the minimum Tx window periodicity is 20ms (or 2 SF), then the Msg3_W_index of the first Tx window is 5 and the Msg3_W_index of the next Tx window will be 30 using the floor operation as in Proposal 2 above. With non-consecutive numbers of Msg3_W_index for adjacent Tx windows, the modular operation of Msg3_W_index with Y may not end up with different RNTI values corresponding to adjacent Tx windows. So it cannot guarantee that the calculated RNTIs corresponding to adjacent Tx windows do not conflict with each other if the minimum Tx window periodicity is used to calculate Msg3_W_index.

Observation 1: Using minimum Tx window length or periodicity to calculate Msg3_W_index will lead to non-consecutive Msg3_W_indexes corresponding to the adjacent CB-Msg3 transmission windows, which may cause the conflict of the derived RNTIs corresponding to adjacent Tx windows.

For Msg3_W_index, the RAN2 agreed working assumption uses the descriptive definition as “Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19”. In the MAC running CR [6], Msg3_W_index is defined as below:

“Msg3_W_id is floor((SFN_id - startSFN) / windowPeriodicity), where the SFN_id is the first SFN of the selected CB-Msg3 transmission window, startSFN is the CB-Msg3 transmission window start SFN defined by IE [FFS], the windowPeriodicity is the CB-Msg3 transmission window periodicity defined by IE [FFS].”

If the calculation of Msg3_W_index defined in the running CR is followed, it is actually the same as the descriptive definition of Msg3_W_index in the RAN working assumption. It will not also cause the issue of non-consecutive Msg3_W_index as raised in Observation 1 because the SFN_id interval of the adjacent CB-Msg3 transmission windows is the same as the configured CB-Msg3 transmission window periodicity. For example, if the SFN_id of the first CB-Msg3 Tx window is 10 and CB-Msg3 transmission window periodicity is configured to 500ms (or 50 SFs), the SFN_id of the next CB-Msg3 Tx window will be 60. Using the floor operation, the Msg3_W_index corresponding to the first CB-Msg3 Tx window is 0 and corresponding to the next CB-Msg3 Tx window is 1 and so on. The only aspect to be clarified for the calculation defined in the running CR is SFN_id may be used instead of “SFN_id – startSFN” as the latter may be negative value. If SFN_id is used for calculating Msg3_W_index, the windpwPeriodcity should be the CB-Msg3 transmission window periodicity defined in the unit of system frames (i.e. in the unit of 10 ms) or CB-Msg3 transmission window periodicity defined in the unit of ms divided by 10ms. Thus we propose:

Proposal 1: Msg3_W_index is floor (SFN_id/windowPeriodicity), where the SFN_id is the first SFN of the selected CB-Msg3 transmission window and the windowPeridocity is the CB-Msg3 transmission window periodicity defined by IE [FFS] if the IE [FFS] is defined in the unit of system frames or the CB-Msg3 transmission window periodicity defined by IE [FFS] divided by 10ms if the IE [FFS] is defined in the unit of milli-second.

For the definition of ”Y”, in our view, there are 3 options on the table:

- Option 1: Y = ceil (Msg4_WS/Msg3_WP), where the value of Msg4_WS is the maximum Msg4 window size and the value of Msg3_WP is the minimum Msg3 window periodicity.

- Option 2: define fixed value of Y, e.g. Y=16

  - Option 2a: value Y is configurable by the NW

- Option 3: Y = ceil (Msg4_WS/Msg3_WP), where the value of Msg4_WS is the configured Msg4 window size and the value of Msg3_WP is the configured Msg3 window periodicity.

As company commented in RAN2 email discussion [2], Y defined in option 1 may lead to the calculated RNTI out of the RNTI range. So option 1 is not preferred option.

Instead, the fixed value of Y e.g. Y=16 was proposed in the email discussion and captured in rapporteur proposal for further discussion. However, using fixed value of Y may set the limitation on NW implementation for configuring the parameters of Msg4 window size and Msg3 window periodicity. For example, if the configured Msg4 window size is larger than 16 times of Msg3 window periodicity, the fixed value of Y as 16 cannot avoid possible conflict of RNTIs within the Msg4 window size and corresponding to the different Msg3 Tx windows. So it might be better not to define the fixed Y value in the specification, but rather to make it configurable by the NW as option 2a indicated above. So NW can configure the value of Y based on the other configured parameters such as Msg4 window size and/or Msg3 window perodicity. With configurable Y value, it also provides the most efficient RNTI value range depending on the configured CB-Msg3 and Msg4 parameters, e.g. to have only one RNTI derived by the UE for CB-Msg4 monitoring if there is no overlapping for CB-Msg4 windows corresponding to different CB-Msg3 transmission windows.

Option 3 can achieve the same benefits as option 2a, but without additional configuration parameter of Y value. On the other hand, company commented in RAN2 email discussion [2] that option 3 may have different Y value derived for different carrier or CE level, which may cause RNTI collision. However, different CE level or carrier may have different PDCCH (e.g. different PDCCH searching space or carrier) configured for monitoring CB-Msg4. Collision of RNTI may not cause the real issue. Or if different Y value is calculated for different CE level or carrier, the maximum Y value among all the calculated Y values can be used for CB-RNTI calculation. Then the collision of RNTI for different CE level or carrier will not happen. But we understand this may complicate the UE implementation. Thus option 3 might not be preferred option from UE perspective.

Proposal 2: RAN2 to discuss and agree to make Y configurable by the NW as in option 2a.

## 2.2 MAC-6: CB-Msg3 power ramping

In the latest R19 TS36.213, the NPUSCH power control is defined in section 16.2.1.1.1, in which three cases were defined for different type of NPUSCH power calculation. That is:

- j = 1, used for NPUSCH (re)transmissions allocated by dynamic scheduled grant or semi-persistent grant.

- j = 2, used for NPUSCH (re)transmissions allocated in random access response grant;

- j= 3, used for NPUSCH (re)transmissions using preconfigured uplink resources (PUR).

However, for CB-Msg3, it seems the NPUSCH transmission cannot be categorized into any of above type. Therefore, it is not clear which j should be followed by UE for CB-Msg3 UL transmission power calculation.

TS36.213 Section 16.2.1.1.1: For NPUSCH (re)transmissions corresponding to the random access response grant if enhanced random access power control is not applied, and for all other NPUSCH transmissions except for NPUSCH (re)transmission corresponding to preconfigured uplink resource, when the number of repetitions of the allocated NPUSCH RUs is greater than 2: [dBm] otherwise PPUSCH,c(i)=min&PCMAX,c(i),&10log10(MNPUSCH,c(i))+PO_NPUSCH,c(j)+αc(j)⋅PLc+ΔTF,c(i)) [dBm] where, - is the configured UE transmit power defined in [6] in NB-IoT UL slot i for serving cell . - MNPUSCH,c(i) is the NPUSCH transmission resource bandwidth normalized by 15 kHz, where {1/4} is used for 3.75 kHz subcarrier spacing and {1, 3, 6, 12} are used for 15kHz subcarrier spacing - PO_NPUSCH,cj is a parameter composed of the sum of a component provided from higher layers and a component provided by higher layers for j=1, 3 and for serving cell where . For NPUSCH (re)transmissions corresponding to a dynamic scheduled grant or a semi-persistent grant then j=1, for NPUSCH (re)transmissions corresponding to the random access response grant then j=2 and for NPUSCH transmission using preconfigured uplink resource then j=3. PO_UE_NPUSCH,c2=0. If enhanced random access power control is not applied, PO_NOMINAL_NPUSCH,c(2)=PO_PRE+∆PREAMBLE_Msg3, where the parameter preambleInitialReceivedTargetPower [8] () and ∆PREAMBLE_Msg3 are signalled from higher layers for serving cell . If enhanced random access power control is applied, PO_NOMINAL_NPUSCH,c2=MSG3_RECEIVED_TARGET_POWER +ΔPREAMBLE_Msg3 - For j=1, for NPUSCH format 2, =1; for NPUSCH format 1, is provided by higher layers for serving cell . For j=2, For j=3, is the parameter alpha in PUR-Config-NB provided by higher layers for serving cell .

Observation 2: The CB-Msg3 UL transmission power calculation is not clearly defined in TS36.213 (i.e., which j should be followed for CB-Msg3 UL transmission power calculation).

For Msg3 scheduled by RAR (j=2), since the UE may have multiple RACH attempts, the enhanced random access power control was introduced to improve the Msg3 transmission successful rate, in which the NPUSCH transmission power can be adjusted dynamically based on the number of RACH attempts. How to dynamically change the Msg3 received target power is defined in MAC specification as below.

TS36.321 Section 5.1.3: - the MSG3_RECEIVED_TARGET_POWER is set to preambleInitialReceivedTargetPower + (PREAMBLE_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep;

Furthermore, in legacy Msg3 transmission, on top of power ramping in one CE level, the UE can also increase its CE level to have more number of NPUSCH repeititions for robust transmission in order to improve its RACH successful rate.

Observation 3: In legacy Msg3 transmission, UE can not only perform Msg3 power ramping but also increase its CE level to improve the Msg3 transmission successful rate.

For CB-Msg3 transmission mechanism, RAN2 agreed that the UE can try multiple number of attempts of CB-Msg3 EDT within the selected CE level (with RRC parameter cb-Msg3-MaxAttemptNum). However, it is not allowed to increase its CE level even the max number of re-attempts has been reached in the selected CE level.

RAN2-129bis agreement:

Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached.

RAN2-130 agreement:

When max re-attempt number for current CE level has been reached, the UE does not move to the next CE level (FFS on the details of the failure behaviour)

Therefore, in case of CB-Msg3 transmission failure, if the NPUSCH power ramping is not supported and the UE’s CE level cannot be increased, the UE has no means to improve its transmission reliability at all. Indeed, the UE may try multiple CB-Msg3 transmissions with the same transmission power, but it is not helpful if the UE’s transmission power is low (for example, in a UL and DL PL imbalance case). In the end, the UE may fail to transmit Msg3 after multiple CB-Msg3 re-attempts which is a waste of UE’s power and NW resource. One may argue NW may configure aggresive open loop power control parameter (e.g., alpha or p0-UE-NPUSCH for CB-Msg3) to increase the UE’s transmission power hence improve the UL transmission reliability. However, it is a cost of system capacity to increase the interference among UEs. We don’t think it is a reasonable way-forward.

Observation 4: The UE has no means to improve its CB-Msg3 transmission reliablity if both the CB-Msg3 power ramping and the CE level increase are not supported.

In RAN2-129bis meeting, RAN2 assumes power ramping should be supported for CB-Msg3-EDT. But in the LS response [4], RAN1 did not give a clear answer on whether the CB-Msg3 power-ramping should be supported or not because they have no time for performance evaluation. Based on the discussions captured in RAN1 moderator summary [5], it seems companies decide to let RAN2 to make the final decision. Please note the RAN2 agreement to forbid the CE level increase happens after RAN2 sent the LS to RAN1. Based on the comments in the moderator summary [5], it seems RAN1 did not consider this latest agreement in their discussions.

R1-2504905 Reply LS on CB Msg3 EDT for IoT NTN Ph3: RAN1 has not evaluated the potential performance of power ramping for CB-msg3-EDT, and it is likely that there will not be sufficient time to evaluate this topic within the R19 timeframe - For open loop power control the following UL power control parameters can be reused for CB-msg3-EDT - p0-UE-NPUSCH-r16 and alpha-r16 for NB-IoT NTN

Observation 5: There is no clear conclusion in RAN1 whether the CB-Msg3 power ramping should be supported because they have no time for performance evaluation. RAN2 can make the final decision considering the CB-Msg3 CE level increase is forbidden.

In our understanding, RAN2 should not exclude both the CE level increase and Msg3 power ramping functions for the CB-Msg3 procedure. Otherwise, the CB-Msg3 performance will be impacted. In other words, we fail to see the motivation to disable the CB-Msg3 power ramping feature which was supported in legacy Msg3 transmisison. Instead, we believe the power ramping between CB-Msg3 windows is beneficial for the UE that remains in a certain CE level.

On the specification impact, as legacy, the parameter of powerRampingStep should be introduced for CB-Msg3. On top of that, we assume the impact in both RAN1 and RAN2 specification are limited as the same principle for legacy Msg3 (j=2) can be followed in both TS36.213 and TS36.321.

Proposal 3: Power ramping between CB-Msg3 windows is supported.

Proposal 4: The same principle as for legacy Msg3 power ramping (j=2) should be followed to limit the specification impact.

## 2.3 MAC-9: Whether NW/UE processing time is needed when determine the Msg4 monitoring starts

In RAN2#129 meeting, it has been agreed that the Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT. FFS whether NW/UE processing time needs to be considered or not.

RAN2-129bis meeting agreements: - The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

In TS36.321, the start time of the Msg3 response window were defined in both the legacy RACH and PUR, in which 4 ms were considered in PUR while it is not added in RACH contention resolution.

TS 36.321 5.1.5 for Contention Resolution: Once Msg3 is transmitted, the MAC entity shall: - if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage: - if Msg3 is transmitted on a non-terrestrial network: … - start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission of the bundle in the subframe containing the last repetition of the corresponding PUSCH transmission plus UE-eNB RTT. TS 36.321 5.4.7 for Msg3 response window in PUR: After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer: - if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT.

For CB-Msg3, we tend to keep the specification simple and align the UE behaviour with the contention resolution timer as defined in legacy RACH procedure. Furthermore, the processing time may not be needed in order to start the transmission of CB-Msg4 earlier since the network can process the Msg3 transmission and prepare for Msg4 transmission well in advance for those UEs who transmit the Msg3 at the early part of CB-Msg3 transmission window.

Additionally, even the UE sends the last replica close to the end of the CB-Msg3 window, as company commented in the email discussion [2], it is not necessary to add the processing time (3ms) since the Msg4 monitor window length is very large.

Proposal 5: When the Msg4 monitoring window starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT, there is no need to consider additional delays for NW/UE processing time.

## 2.4 MAC-15:  FFS on the details of the failure behaviour

In the post-meeting email discussion [2], it seems common understanding that MAC layer should notify the RRC layer when the maximum number of CB-Msg3 re-attempts has been reached while the CB-Msg3 Response Timer expires. However, it is not clear on RRC layer behaviour when the RRC layer receives the CB-Msg3 EDT failure indication from MAC.

Firstly, the RRC layer should decide the type of fallback procedure. We think fallback to PUR is not necessary as we don’t see the motivation to allocate PUR resource dedicated for a UE if it can directly use CB-Msg3 without Msg1 and Msg2 transmission. For the fallback to legacy 4-step RACH or EDT, we believe the legacy mechanism should be followed.

Proposal 6: When the RRC layer receives a CB-Msg3 EDT failure indication from the MAC layer, it may let the UE fall back to legacy 4-step RACH or EDT. The UE shall follow the legacy mechanism to determine whether to trigger EDT.

Secondly, such CB-Msg3 EDT failure indication from MAC to RRC layer means the ongoing RACH procedure is not completed yet.  No matter which procedure (e.g., legacy 4-step RACH, EDT) will be selected upon RRC layer receives the failure indication, there is no need to start random access again from scratch. Specifically, the Msg3 buffer built for CB-Msg3 transmission (as described in MAC running CR section 5.1x.2) should not be flushed when the UE fallback to legacy 4-step RACH or EDT procedure.

However, in the current MAC specification, the UE shall always flush the Msg3 buffer upon legacy RACH procedure initialization (section 5.1.1 as below). It is true for a new reinitiating or restarting RACH procedure while it is not needed for the 4-step RACH or EDT procedure which is the continuation of the CB-Msg3 EDT. To enable the MAC layer to distinguish whether a RACH procedure is a continuation of a CB-Msg3 EDT procedure or a completely new RACH procedure from scratch, the RRC layer should provide the CB-Msg3 fallback indication to the MAC layer.

### 5.1.1 Random Access Procedure initialization … The Random Access procedure shall be performed as follows: - flush the Msg3 buffer; - set the PREAMBLE_TRANSMISSION_COUNTER to 1;

Proposal 7: The Msg3 buffer shall not be flushed when the UE falls back from the CB-Msg3 EDT procedure to the legacy 4-step RACH or EDT.

Proposal 8: The RRC layer shall provide a CB-Msg3 fallback indication to the MAC layer.

## 2.5 MAC-17:  Whether to allow multiple TBSs as in EDT

In legacy EDT, UE can select smaller TBS than the defined edt-TBS for Msg3, to reduce the redundant padding on Msg3, thereby saving some power and time.

If network set the edt-SmallTBS-Enabled to TRUE, the UEs can adapt their TBS selection within a small TBS subset based on its pending data size (as 36.213 table 16.3.3-2 for NB-IoT) to avoid unnecessary padding. Furthermore, for MT-EDT, the TBS required for Msg3 is much smaller than the TBS required for MO because the Msg3 is only allowed to include the RRC message (indicating the MT cause and approximately 40 bit UE ID information) but not any user data. Assuming the network configures the edt-TBS to 1000 bits, while the UE transmits only 100 bits of data (e.g., for MT-CB-Msg3), 900 padding bits (90% of the TB) are added and this is really a waste of resources.

Observation 6: In legacy EDT, for both MO-EDT and MT-EDT, UE can use smaller TBS than edt-TBS for Msg3 to avoid excessive padding.

edt-SmallTBS-Enabled Value TRUE indicates UE performing EDT is allowed to select TBS smaller than edt-TBS for Msg3 in the corresponding CE level, as specified in TS 36.213 [23]. 36.213: Table 16.3.3-2: EDT TBS for Msg3 NPUSCH with edt-SmallTBS-Enabled set to 'true' edt-TBS; edt-SmallTBS-Subset; Allowable TBS values. 408; not configured; 328, 408. 504; not configured; 328, 408, 504. 504; enabled; 408, 504. 584; not configured; 328, 408, 504, 584. 584; enabled; 408, 584. 680; not configured; 328, 456, 584, 680. 680; enabled; 456, 680. 808; not configured; 328, 504, 680, 808. 808; enabled; 504, 808. 936; not configured; 328, 504, 712, 936. 936; enabled; 504, 936. 1000; not configured; 328, 536, 776, 1000. 1000; enabled; 536, 1000.

For CB-Msg3, we believe the legacy design principle should be followed, especially RAN2 has agreed to support MT-EDT. One simple way is to introduce the similar mechanism as edt-SmallTBS-Enabled. Or alternatively, to avoid any RAN1 impact, NW may configure multiple TBS for a CE level hence UE can select the TBS based on its pending data size to avoid unnecessary bits padding.

Proposal 9: Multiple TBSs and TBS selection are supported for CB-Msg3 EDT.

Proposal 10: RAN2 to downselect below two options:

- Option1: NW can configure multiple CB-Msg3 TBSs for a CE level (to avoid RAN1 impact)

- Option2: UE is allowed to select TBS smaller than the configured CB-Msg3 TBS for the corresponding CE level.

## 2.6 MAC-20:  Should the CQI report in Msg3 in NB-IoT be supported in CB-Msg3-EDT ?

Based on the desription in email discussion [2], we understand the NB-IoT CQI report included in Msg3 is to assist network for NPDCCH repetition level selection.

#### – CQI-NPDCCH-NB The IE CQI-NPDCCH-NB represents the downlink channel quality measurement of the NB-IoT carrier where the random access response is received. The codepoints for the CQI-NPDCCH measurements are according to the mapping table in TS 36.133 [16]. The value noMeasurements indicates no measurement reporting. CQI-NPDCCH-NB information element -- ASN1START CQI-NPDCCH-NB-r14 ::= ENUMERATED { noMeasurements, candidateRep-A, candidateRep-B, candidateRep-C, candidateRep-D, candidateRep-E, candidateRep-F, candidateRep-G, candidateRep-H, candidateRep-I, candidateRep-J, candidateRep-K, candidateRep-L} -- ASN1STOP #### 36.133 9.1.22.15 MSG3-based Measurement Report Mapping for UE Category NB1 Table 9.1.22.15-1: Downlink channel quality measurement report mapping of CQI-NPDCCH-NB when the DL channel quality reporting is supported [7] Reported value; NPDCCH repetition level. noMeasurement; No measurement reporting. candidateRep-A; 1. candidateRep-B; 2. candidateRep-C; 4. candidateRep-D; 8. candidateRep-E; 16. candidateRep-F; 32. candidateRep-G; 64. candidateRep-H; 128. candidateRep-I; 256. candidateRep-J; 512. candidateRep-K; 1024. candidateRep-L; 2048.

In our understanding, the NB-IoT CQI report is not that useful for CB-Msg3 because most of the CB-Msg3 procedure terminate with Msg4 and then UE goes to RRC idle. Furthermore, one CB-Msg4 is designed to address multiple UEs therefore network would have to use conservative PDCCH link adaptation to cover the worst UE. Even in legacy, it is an optional feature in UE which means not all UEs will report that to network. Therefore, we think the CQI report in CB-Msg3 is not needed.

Proposal 11: CB-Msg3 CQI report for NB-IoT is not needed.

## 2.7 Additional issue#1: Whether to support NW-indicated fallback

Although RAN2 agreed that the CB-Msg3 re-attempt may apply backoff as indicated in CB-Msg4, repeated re-attempts using the CB-Msg3 resource within the same CE level may continue to fail if the CB-Msg3 resource is overloaded.

From the UE’s perspective, it is unaware of whether the CB-Msg3 resource is overloaded. When the feature is enabled, the UE transmits Msg3 directly using the CB-Msg3 PUSCH resource indicated in SIB. RAN2 has not discussed mechanisms to mitigate congestion on the CB-Msg3 resource when a large number of UEs attempt to transmit Msg3 simultaneously using this resource. We believe it would be beneficial for the network to have the capability to trigger a fallback from CB-Msg3 to the legacy RACH or EDT procedure, which could help balance the load between CB-Msg3 and legacy RACH resources.

Therefore, we propose RAN2 to discuss whether to support the network-indicated fallback to legacy RACH, EDT.

Observation 7: When the CB-Msg3 resource is overloaded, repeated CB-Msg3 transmission attempts may result in continuous failures. It would be beneficial for the network to fall back UEs to the 4-step RACH procedure to balance the load.

Proposal 12: RAN2 to discuss whether to support the network-indicated fallback to legacy RACH or EDT.

If the above propsal can be agreed, it seems stright-forward to use the reserved value ”11” of type field T to indicate the NW-initiated fallback to 4-step RACH/EDT. The impact to specificaiton is limited.

### 6.2.x MAC header for CB-Msg4 Figure 6.1.x-2: E/T/R/R/H/TA/C MAC subheader - T: The Type field is a flag indicating the type of the MAC subheader. The T field is set to "00" to indicate the presence of a Backoff Indicator field in the subheader (BI). The T field is set to "01" to indicate the presence of the H field in the subheader. The T field is set to "10" to indicate the presence of a Logical Channel ID (LCID) field in the subheader. The value "11" is reserved for further use. The size of the Type field is 2 bits;

Proposal 13: The reserved value for MAC subheader type indication can be used for NW-indicated fallback.

## 2.7 Additional issue#2:  Whether the MPDCCH narrowband configuration should be defined as a set

In the RAN2 LS[3], RAN2 indicates the MPDCCH narrowband maybe configured as a set instead of one single narrow band. However, according to the RAN1 LS response [4], there is no consensus in RAN1 yet. We believe it is up to RAN2 to decide the way-forward.

Q2: For eMTC, RAN2 agrees to have MPDCCH configuration for shared resource configuration and decides to reuse the parameters from PUR MPDCCH configuration (in IE PUR-MPDCCH-Config-r16, as below) as baseline. RAN2 think TDD parameter is not needed and maybe narrowband configuration should be defined as a set, not one single narrow band. RAN2 would like to check with RAN1 on how to define the detail MPDCCH configuration.

In legacy, the RRC specification defines a single MPDCCH narrowband for the PUR procedure (as specified in PUR-Config), whereas multiple MPDCCH narrowbands are defined for the legacy RACH procedure (as in PRACH-Config). For CB-Msg3, the question arises as to whether the configuration should follow the principle in PUR-Config or PRACH-Config.

PUR-Config information element -- ASN1START <omitted part> PUR-MPDCCH-Config-r16 ::= SEQUENCE { mpdcch-FreqHopping-r16 BOOLEAN, mpdcch-Narrowband-r16 INTEGER (1..maxAvailNarrowBands-r13), <omitted part> } PRACH-Config information elements -- ASN1START <omitted part> temptCE-r13 ENUMERATED {n3, n4, n5, n6, n7, n8, n10} OPTIONAL, -- Need OP numRepetitionPerPreambleAttempt-r13 ENUMERATED {n1,n2,n4,n8,n16,n32,n64,n128}, mpdcch-NarrowbandsToMonitor-r13 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13), <omitted part> }

In our understanding, the MPDCCH monitoring for CB-Msg4 is more like the Msg2/Msg4 monitoring in the legacy RACH procedure than to that in PUR. In the PUR case, network allocates dedicated resources to individual UEs, making it sufficient for a UE to be assigned with only one MPDCCH narrowband at one configuration. In contrast, for CB-Msg3, the SIB-indicated resources are shared among multiple UEs, enabling simultaneous system access by many UEs—similar to the legacy RACH procedure. Therefore, we believe the MPDCCH narrowbands should be configured as a set. This approach provides greater flexibility and capacity for the network control channel, which aligns with the main objective of this WI.

Proposal 14: To support capacity enhancement in line with the objective of the WI, the MPDCCH narrowband configuration for CB-Msg4 monitoring shall be defined as a set, following the legacy RACH procedure.

# 3	Conclusion

This document has made the following observations and proposals for the remaining open issues:

MAC-2: CB-RNTI calculation:

Observation 1: Using minimum Tx window length or periodicity to calculate Msg3_W_index will lead to non-consecutive Msg3_W_indexes corresponding to the adjacent CB-Msg3 transmission windows, which may cause the conflict of the derived RNTIs corresponding to adjacent Tx windows.

Proposal 1: Msg3_W_index is floor (SFN_id/windowPeriodicity), where the SFN_id is the first SFN of the selected CB-Msg3 transmission window and the windowPeridocity is the CB-Msg3 transmission window periodicity defined by IE [FFS] if the IE [FFS] is defined in the unit of system frames or the CB-Msg3 transmission window periodicity defined by IE [FFS] divided by 10ms if the IE [FFS] is defined in the unit of milli-second.

Proposal 2: RAN2 to discuss and agree to make Y configurable by the NW as in option 2a.

MAC-6: CB-Msg3 power ramping

Observation 2: The CB-Msg3 UL transmission power calculation is not clearly defined in TS36.213 (i.e., which j should be followed for CB-Msg3 UL transmission power calculation).

Observation 3: In legacy Msg3 transmission, UE can not only perform Msg3 power ramping but also increase its CE level to improve the Msg3 transmission successful rate.

Observation 4: The UE has no means to improve its CB-Msg3 transmission reliablity if both the CB-Msg3 power ramping and the CE level increase are not supported.

Observation 5: There is no clear conclusion in RAN1 whether the CB-Msg3 power ramping should be supported because they have no time for performance evaluation. RAN2 can make the final decision considering the CB-Msg3 CE level increase is forbidden.

Proposal 3: Power ramping between CB-Msg3 windows is supported.

Proposal 4: The same principle as for legacy Msg3 power ramping (j=2) should be followed to limit the specification impact.

MAC-9: Whether NW/UE processing time is needed when determine the Msg4 monitoring starts

Proposal 5: When the Msg4 monitoring window starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT, there is no need to consider additional delays for NW/UE processing time.

MAC-15:  FFS on the details of the failure behaviour

Proposal 6: When the RRC layer receives a CB-Msg3 EDT failure indication from the MAC layer, it may let the UE fall back to legacy 4-step RACH or EDT. The UE shall follow the legacy mechanism to determine whether to trigger EDT.

Proposal 7: The Msg3 buffer shall not be flushed when the UE falls back from the CB-Msg3 EDT procedure to the legacy 4-step RACH or EDT.

Proposal 8: The RRC layer shall provide a CB-Msg3 fallback indication to the MAC layer.

MAC-17:  Whether to allow multiple TBSs as in EDT

Observation 6: In legacy EDT, for both MO-EDT and MT-EDT, UE can use smaller TBS than edt-TBS for Msg3 to avoid excessive padding.

Proposal 9: Multiple TBSs and TBS selection are supported for CB-Msg3 EDT.

Proposal 10: RAN2 to downselect below two options:

- Option1: NW can configure multiple CB-Msg3 TBSs for a CE level (to avoid RAN1 impact)

- Option2: UE is allowed to select TBS smaller than the configured CB-Msg3 TBS for the corresponding CE level.

MAC-20:  Should the CQI report in Msg3 in NB-IoT be supported in CB-Msg3-EDT

Proposal 11: CB-Msg3 CQI report for NB-IoT is not needed.

Additional issue#1: Whether to support NW-indicated fallback

Observation 7: When the CB-Msg3 resource is overloaded, repeated CB-Msg3 transmission attempts may result in continuous failures. It would be beneficial for the network to fall back UEs to the 4-step RACH procedure to balance the load.

Proposal 12: RAN2 to discuss whether to support the network-indicated fallback to legacy RACH or EDT.

Proposal 13: The reserved value for MAC subheader type indication can be used for NW-indicated fallback.

Additional issue#2:  Whether the MPDCCH narrowband configuration should be defined as a set

Proposal 14: To support capacity enhancement in line with the objective of the WI, the MPDCCH narrowband configuration for CB-Msg4 monitoring shall be defined as a set, following the legacy RACH procedure.

# Reference

[1] RP-250472	Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2] R2-2505555	Remaining MAC open issues in IoT NTN, MediaTek

[3]	R2-2503175	LS on CB-msg3-EDT

[4]	R1-2504905	Reply LS on CB Msg3 EDT for IoT NTN Ph3

[5] R1-2504803	Moderator summary on reply LS on CB Msg3 EDT, Mediatek

[6] R2-2505201	MAC Running CR for Rel-19 IoT NTN, MediaTek Inc.