3GPP TSG RAN WG2 #131	R2-2505642

Bangalore, India, Aug. 25th – 29th, 2025

Agenda item:	8.8.2

Source: 	ITRI

Title: 	Discussion on supporting multiple SMTC periodicities for inter-frequency neighbour cells

Document for:	Discussion

1	Introduction

One of the remaining RRC open issues for NTN DL coverage enhancements after RAN2#130 meeting is listed as in the following:

- Editor’s Note: In RAN2#129, RAN2 assumed that “it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer”. Thus, it is FFS whether multiple SMTC periodicities are needed in frequency layers different from the serving cell frequency (e.g., inter-frequency neighbours).[1]

In this contribution we discuss supporting inter-frequency multiple SMTC periodicities with beneficial scenario.

2	Discussion

After RAN2#130 meeting, one of the remaining RRC open issues is whether to support multiple SMTC periodicities for inter-frequency measurements. [1]

2.1 Multiple SMTC offsets for inter-freqency measurements

The surrounding neighbour cells could be deployed on the SSB frequency/ies different from the serving cell. RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer. [2]. That is, the SSBs of neighbour cells may be transmitted at different times. According to the assumption, network could configure different SMTC offsets for inter-frequency neighbour cells to indicate the difference of the SSB transmission time corresponding to the serving cell.

UE is not required to perform SSB-based RRM measurements outside the SMTC occasion [3]. However, current configuration could not fit two cells with more than 5 ms SSB offset difference into a SMTC duration. Supporting multiple SMTC offsets for inter-frequency SSB based Layer 3 measurements is needed.

Observation 1: Multiple neighbour cells operated on the same frequency may have different SSB offsets. Current configuration could not fit two cells with more than 5 ms SSB offset difference into a SMTC duration.

Proposal 1: Support multiple SMTC offsets for inter-frequency neighbour cell measurements.

2.2 Multiple SMTC periodicities for inter-frequency measurements

Neighbour cells operated on the same SSB frequency could be configured with different SSB periodicities (e.g., regarding the population disparity between different NTN served regions).

The legacy SMTC mechanism allows the network to configure one SMTC periodicity for each SSB frequency different form the serving cell,  SMTC4 further supports the configuration of cell specific SMTC offsets.

For SSB based RRM, the measurement period is based on SMTC period as shown in the following table. [4]

Table 9.3.5-1: Measurement period for inter-frequency measurements with gaps (Frequency FR1) [4]

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, Ceil(8 * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(8 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent measurement GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

For inter-frequency measurements, using the larger SSB periodicity of the frequency as the SMTC periodicity would prolong the measurement period of cells with shorter SSB periodicity. Therefore, it is proposed to support multiple SMTC periodicities for inter-frequency L3 measurements.

Observation 2: Neighbour cells operated on the same SSB frequency could be configured with different SSB periodicities. Multiple SMTC periodicities could prevent prolonged L3 measurement periodicity for SSB-based inter-frequency neighbour cell measurements.

Proposal 2: Support multiple SMTC periodicities in frequency layers different from the neighbour cell frequency.

A new SMTC5 is introduced in Rel-19 to configure periodicity and offsets from scratch. Therefore, the configuration could be included in SMTC5 to indicate the additional SMTC periodicity of a frequency and the PCIs of associated neighbour cells. The configuration of SMTC5 could override the SMTC4 configuration and Rel-19 UE supporting DL coverage enhancements would apply the SMTC periodicities and offsets configured in SMTC5.  In addition, Pre-19 UE could not understand and should omit the configuration of multiple inter-frequency SMTC periodicities.

Proposal 3: It is proposed that Multiple SMTC periodicities for inter-frequency measurements is configured by SMTC5.

3	Conclusion

For the FFS of whether to support multiple SMTC periodicity for inter-frequency neighbour cell measurements, the following observations and proposals are provided:

Observation 1: Multiple neighbour cells operated on the same frequency may have different SSB offsets. Current configuration could not fit two cells with more than 5 ms SSB offset difference into a SMTC duration.

Proposal 1: Support multiple SMTC offsets for inter-frequency neighbour cell measurements.

Observation 2: Neighbour cells operated on the same SSB frequency could be configured with different SSB periodicities. Multiple SMTC periodicities could prevent prolonged L3 measurement periodicity for SSB-based inter-frequency neighbour cell measurements.

Proposal 2: Support multiple SMTC periodicities in frequency layers different from the neighbour cell frequency.

Proposal 3: It is proposed that Multiple SMTC periodicities for inter-frequency measurements is configured by SMTC5.

4	References

[1] R2-25xxxxx	Remaining RRC open issues for NR NTN Rel-19, Bangalore, India.

[2] R2-2501702	Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece.

[3] 3GPP TS 38.331 V18.4.0: "NR; Radio Resource Control (RRC); Protocol specification".

[4] 3GPP TS 38.133: "NR; Requirements for support of radio resource management".