3GPP TSG-RAN WG2 Meeting #131  	                            R2-2505656

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.5.3

Source: 	Sony

Title:	On-demand SIB1 request on SUL

Document for:	Discussion

# Introduction

In RAN2#130 meeting, the agreements on OD-SIB1 were agreed as follows:

Agreements on OD-SIB1

- Not to include access related information of NES into WUS configuration.

- Normative text for 2nd check, e.g. “if SIB1 is not broadcast”, is removed. RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before triggering OD-SIB1 request procedure of that cell.

- TP1 in R2-2503636 is agreed with the removal of “until receiving SIBxx from this cell”.

- Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

- If there is no common searchspace configuration on current active BWP, the network will ensure the RRC_CONNECTED UE can acquire the updated UL WUS configuration, i.e., the dedicatedSystemInformationDelivery IE in RRCReconfiguration is also applied to new SIB containing UL WUS configuration.

- Include the following parameters from the RAN1 parameter list in SIBxx: frequencyBandList, Prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, zeroCorrelationZoneConfig, preambleReceivedTargetPower, powerRampingStep, ra-ResponseWindow, searchSpaceZero, controlResourceSetZero, ra-SearchSpace, n-TimingAdvanceOffset, ssb-PeriodicityServingCell, od-sib1-WindowDuration, od-sib1-windowStartOffset.

In this contribution, we discuss the remaining open issue on whether to support OD-SIB1 request on SUL.

# Discussion

It was discussed in RAN2#130 meeting on whether to support OD-SIB1 request on SUL or not. Companies have different views on whether this has been excluded in RAN1 or not and it is our view that RAN1 didn’t discuss this issue before, therefore this issue can be discussed and decided in RAN2.

From MAC specification, UE should select the UL carrier at the very beginning of the RACH procedure as follows.

1> else if the carrier to use for the Random Access procedure is not explicitly signalled; and 1> if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5]; and 1> if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL: 2> select the SUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the SUL carrier. 1> else: 2> select the NUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the NUL carrier.

And then perform the OD-SIB1 request procedure, if necessary, as included in the running CR.

1> if the Random Access procedure was initiated for SIB1 request (as specified in TS 38.331 [5]) and the Random Access Resources for SIB1 request have been provided by RRC: 2> set the RA_TYPE to 4-stepRA.

There is no issue to support the OD-SIB1 request transmission on SUL as it is in the running CR and we don’t see any reason this should be prohibited. In the case that network doesn’t specify which carrier (NUL or SUL) to use, and if UE selects the SUL and is not allowed to send OD-SIB1 request on SUL, it means that UE has to re-select to NUL to perform the RACH and initiate OD-SIB1 request. This unnecessarily prolongs the RACH procedure and imposes extra restrictions on specification as well. Therefore, we propose:

Proposal 1: OD-SIB1 request is supported on SUL. There is no MAC specification impact.

# Conclusion

We propose RAN2 to discuss following proposal:

Proposal 1: OD-SIB1 request is supported on SUL. There is no MAC specification impact.