3GPP TSG-RAN2 Meeting #131	R2-2505666

# Bengaluru, India, 25 - 29 August 2025

Agenda Item:	8.11.2

Source:	Ericsson

Title:	SBFD RA remaining aspects

Document for:	Discussion

# 1	Introduction

In the WID [1], the following objective regarding random access operation has been defined for RAN2

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

We discuss the RAN2 aspects for the above objective in this paper. In the below, we use the term SBFD RA operation to stand for random access operation in SBFD symbols unless otherwise stated. We use the term legacy RA operation to stand for random access operation during non SBFD symbols.

In this paper, a SBFD aware UE means a UE which is capable of operating in a cell configured with SBFD feature, i.e., the cell/the gNB transmits DL and receive UL simultaneously in SBFD slots and symbols within a carrier. The UE can be aware of SBFD configurations so that the UE knows which slots/symbols are SBFD capable, which are also referred to as SBFD slots/symbols. This doesn’t mean that the UE needs to support full duplex operation.

In this document, we discuss some remaining RA issues.

# 2	Discussion

## 2.1	Open issues in MAC running CR

The rapporteur has summarized the below MAC open issues:

- [Open Issue MAC-2-1] Do companies think any additional MAC specification changes (compared to the latest version of the MAC running CR) are needed for capturing the condition of the agreement: SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA?

- [Open Issue MAC-2-2] If so, please clarify what the changes should be.

- [Open Issue MAC-3-1] Do companies think any additional specification changes (in MAC and/or RRC) are essential to address the issue on how to derive the SBFD RO locations for CFRA (ZTE’s concern)? Please provide your input with the rationale.

- [Open Issue MAC-3-2] If so, please provide your preferred solution with the rationale, including but not limited to:

- Network can indicate RO type as SBFD RO for CFRA, only if an SBFD RACH configuration is configured for CBRA (proposed by LGE).

- [Open Issue MAC-4-1] Do companies think the parameter initialization is needed after RO type switching in the case of RACH configuration Option 2? Please provide your input with the rationale.

- [Open Issue MAC-4-2] If so, please further clarify whether all parameters in RACH-ConfigCommon should be considered for the initialization, or if certain parameters should be excluded from the initialization, along with the rationale.

- [Open Issue MAC-5] Do companies support introducing a power offset to compensate the power ramping difference between the two RO types (in SBFD RACH configuration Option 2) for RO type switching? Please provide your input with the rationale.

### 2.1.1 Open Issue MAC-2-1

Since RAN2 has already made clear agreement on this issue, we see no reason to not capture the agreement clearly. In addition, fallback between CFRA and CBRA is a legacy feature, without clear capturing of the agreement, the UE behaviours may be remaining unclear.

Capture explicitly in MAC spec that SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

### 2.1.2 Open Issue MAC-3-1

Based on the comments provided by other companies in the email discussion, the use case where the network only provides CFRA SBFD resources without CBRA SBFD resources is unclear. Based on RAN1 discussions, the use cases/intentions for SBFD RA include latency reduction, capacity increase, range extension or coverage enhancement. For any use case, a RACH event can be triggered for either CBRA procedure or CFRA procedure. Therefore, it is reasonable that the network decides to enable the SBFD RA operation or disable SBFD RA operation in a cell depending on the target use case. When SBFD RA operation is enabled, the network will provide both CBRA SBFD resources and CFRA SBFD resources. However, this is pure network implementation, nothing needs to be specified.

It is reasonable that the network decides to enable the SBFD RA operation or disable SBFD RA operation in a cell depending on the target use case. When SBFD RA operation is enabled, the network will provide both CBRA SBFD resources and CFRA SBFD resources.

Therefore, we don’t agree with the way forward suggested by companies

- Network can indicate RO type as SBFD RO for CFRA, only if an SBFD RACH configuration is configured for CBRA (proposed by LGE).

It is up to the network implementation to ensure that SBFD RO type is signalled to a UE for CFRA only when an SBFD RACH configuration is configured for CBRA, i.e., no spec change is needed.

### 2.1.3 Open Issue MAC-4-1

Regarding the issue MAC-4-1, we agree it is beneficial to perform the parameter initialization after the RO type switching in the case of RACH configuration option 2, since there is separate SBFD RACH configuration, which may give different RACH settings compared to the legacy RACH configuration. If initialization is needed, we can assume all parameters in RACH-ConfigCommon are considered for the initiation, there is no need to exclude any parameter artificially, which may limit the network implementation unnecessarily.

After RO type switching in the case of RACH configuration Option 2, the MAC entity conducts the initialization of (all) RACH parameters according to the SBFD RACH configuration.

### 2.1.4 Open Issue MAC-5

Regarding the issue MAC-4-1, since RAN1 has already discussed this issue and made the below agreement

For RACH configuration Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, there is no RAN1 consensus to support a power offset to compensate the power ramping difference

No consensus in RAN1 to support a power offset to compensate the power ramping difference in case of RO type switch.

In addition, it was also observed that the issue can be avoided if the gNB configures the same power ramping step for both RO types.

The issue can be avoided if the gNB configures the same power ramping step for both RO types.

Therefore, we see the issue is an optimization. RAN2 shall not pursue it in the last meeting of the work item.

Not support introducing a power offset to compensate the power ramping difference between the two RO types (in SBFD RACH configuration Option 2) for RO type switching.

## 2.2	Open issue RRC-1 in RRC running CR

[Proposal for RRC-1] For the NT indicating RO type, use 1 bit signalling (as in the current RRC running CR) [8/13].

For this issue, the solutions proposed by companies were already discussed during previous online session. RAN2 already agreed the single bit indication based option to address the issue per current RAN2 agreement. In our view, the other options are not categorized as signalling details, rather than different options for the same issue. Given there is only one meeting left for RAN2 to conclude the work item. We see no point to reopen the discussion on this issue.

RAN2 already agreed the single bit indication based option to address the issue per current RAN2 agreement.

The other options are not categorized as signalling details, rather than different options for the same issue.

For the NT indicating RO type, stick to the current RAN2 agreement, i.e., use 1 bit indicator-based signalling (as captured in the RRC running CR).

## 2.3	LTM

RAN2 has made the below agreement at RAN2#129bis

- Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

Regarding whether SBFD operation is supported for LTM, RRC Rapporteur thinks companies can discuss here and try to down-select from the below options, please clarify the impact on RRC spec (as well as on MAC spec):

Option 1: Not to support both "early UL synchronization with an LTM candidate cell in SBFD symbols” and ”RACH-based LTM in SBFD symbols".

Option 2:  Only support ”RACH-based LTM in SBFD symbols".

Option 3:  Support both "early UL synchronization with an LTM candidate cell in SBFD symbols” and ”RACH-based LTM in SBFD symbols".

For early UL sync, the spec effort would be more as RACH for UL sync is a separate config.

For RACH-based LTM in SBFD symbols, we could further discuss whether the NW indication on RO type is carried by RRC (no further change needed as RACH-ConfigDedicated already has RO type indicator) or LTM Command MAC CE (MAC should be changed). From our perspective, using LTM Command MAC CE to indicate RO type can be supported, at least for intra-CU scenario.

Regarding if SBFD can be supported for LTM, adopt Option 2, i.e., only support ”RACH-based LTM in SBFD symbols".

## 2.4	Msg1 repetition

RAN2 has made the below agreement regarding Msg1 repetition

- The UE is allowed to switch to an RO type that is configured with the same Msg1 repetition number. FFS on higher Msg1 repetition number, if the same is not available.

The above agreement was mainly intended to limit the UE to not fallback unnecessary Msg1 repetition number in order to avoid overlapping/redundant Msg1 transmissions when fallback between RO types occur. Give this in mind, if the same Msg1 repetition number if not available in the new RO type, it is reasonable to allow the UE to apply higher Msg1 repetition number.

The UE is allowed to switch to an RO type that is configured with higher Msg1 repetition number, if the same Msg1 repetition number is not available for the RO type.

## 2.5	SBFD awareness early indication

Regarding SBFD awareness early indication, RAN2 has made the below agreement:

- From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

The remaining issue is whether a SBFD aware UE can indicate SBFD awareness to the gNB via Msg3 or Msg5. We think this is not needed because the gNB will learn this from UE capabilities (received from the UE or CN). We do acknowledge that if the gNB gets this info in Msg3/Msg5 it allows the gNB to schedule any transmission on SBFD resources for the SBFD aware UEs, but this would be an optimization with marginal gain at best. In contrast, if one compares with the Msg3-indication for RedCap, that indication was critical since scheduling a RedCap UE on a larger-than 20 MHz bandwidth would simply not be possible and this was the motivation for adding the RedCap Msg3-indication. An SBFD-aware UE on the other hand can be scheduled on the legacy resources so in this case RAN2 should not waste bits/add complexity, i.e. it is not motivated and not worth adding new indications in MAC or RRC. Further, it should be noted that an SBFD-aware UE that uses SBFD RA resources will by this make the gNB aware that it is SBFD aware, i.e. it is really questionable to add even more complexity to the SBFD feature since this would increase implementation effort and delay time to market.

Support SBFD awareness early indication via Msg3 or Msg5 would require RAN2 to introduce an indicator in either MAC layer or RRC signalling, which would increase unnecessary design efforts for RAN2.

With early indication via Msg3 or Msg5, the gNB may be only feasible to schedule SBFD resources to subsequent signalling transmissions. The achieved latency reduction or other performance enhancements would be marginal given that the gNB will anyway receive the UE capability from UE or CN after the RACH procedure.

If the UE sees no need to apply SBFD RA (to exploit latency reduction or coverage extension), the UE would not choose SBFD RO for Msg1 transmission.

Therefore, in our view, it is unnecessary to introduce early indication for SBFD awareness. Anyway, an SBFD aware UE can be identified by the gNB at an early stage when the UE uses SBFD RO, which is sufficient.

SBFD awareness early indication via Msg3 or Msg5 is not pursued for SBFD aware UE.

## 2.6	RA-RNTI collision

At RAN2#127bis, some companies have proposed to study the issue on RA-RNTI collision. They declare that:

For PRACH configuration Option 2 (i.e., an SBFD-aware UE is configured with a legacy and additional PRACH configurations), there is a possibility that the SBFD ROs configured by the additional configurations may overlap with the legacy ROs. This could happen in flexible symbols (as both legacy and SBFD ROs can be configured in flexible symbols). When this overlapping happens, there is a non-negligible possibility that at least s_id and t_id of the overlapping legacy RO and SBFD RO are the same (and of course, the ul-carrier_id is the same since we are talking about the same carrier). Now the tricky part is the f_id. If the legacy specs is kept as is, then at the point when there is such overlapping, there is a mismatch between the frequency indices being used by legacy UEs and the ones being used by SBFD UEs. This leads to the same RA-RNTI and confuses these UE when receiving the same RA-RNTI.

More specifically, the UE calculates the RA-RANTI when sending the Msg 1 of the RA procedure. According to TS38.321, the RA-RNTI is computed as

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where,

- s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

- t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

- f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

- ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

In our view, we think the issue is invalid due to the below reasons

- According to RAN1 agreements,

a RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default. A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE:

- The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH.

- UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols

- There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH.

- Other assumptions are not precluded.

NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB.

For a SBFD RO across SBFD symbols and non-SBFD symbols, if the SBFD RO is overlapping with the legacy RO on a 2nd symbol or beyond, there is no issue, in such case, at least the s_id would be different, which would be sufficient to distinguish RA-RNTIs between the SBFD RO and the legacy RO which are overlapping.

Regarding whether a SBFD RO across SBFD symbols and non-SBFD symbols, may be overlapping with a legacy RO in the first symbol, we think it can be either avoided by NW configuration, or RA-RNTI collision can addressed by allocating different preamble list to the legacy RACH configuration and additional RACH configuration.

- In addition, a configured RO starting from non SBFD symbols and ending in SBFD symbols, is invalid.

For a RO across SBFD symbols and non-SBFD symbols, the RO may be overlapping with a legacy RO on 2nd symbol or beyond, in such case, at least the s_id would be different, which would be sufficient to distinguish RA-RNTIs between the SBFD RO and the legacy RO which are overlapping.

A configured RO starting from non SBFD symbols and ending in SBFD symbols, is invalid.

RAN2 has discussed this issue, however no consensus has been reached. We have further queried our RAN1 colleagues and learned additional information the following:

RACH configuration option 2 is typically to be configured with different PRACH preamble format (e.g., longer PRACH preamble format) compared to the legacy PRACH configuration. Given this, the network can easily detect PRACH preamble transmissions from UEs on the overlapping ROs, wherein the SBFD aware UE applies a longer PRACH preamble format while the legacy UE applies a shorter PRACH preamble format.

All in all, we see no issue for RA-RNTI confusion. Suggest RAN2 to conclude no further discuss is needed for this issue.

Update the RA-RNTI formula for SBFD RA configuration is not pursued.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	It is reasonable that the network decides to enable the SBFD RA operation or disable SBFD RA operation in a cell depending on the target use case. When SBFD RA operation is enabled, the network will provide both CBRA SBFD resources and CFRA SBFD resources.

Observation 2	No consensus in RAN1 to support a power offset to compensate the power ramping difference in case of RO type switch.

Observation 3	The issue can be avoided if the gNB configures the same power ramping step for both RO types.

Observation 4	RAN2 already agreed the single bit indication based option to address the issue per current RAN2 agreement.

Observation 5	The other options are not categorized as signalling details, rather than different options for the same issue.

Observation 6	Support SBFD awareness early indication via Msg3 or Msg5 would require RAN2 to introduce an indicator in either MAC layer or RRC signalling, which would increase unnecessary design efforts for RAN2.

Observation 7	With early indication via Msg3 or Msg5, the gNB may be only feasible to schedule SBFD resources to subsequent signalling transmissions. The achieved latency reduction or other performance enhancements would be marginal given that the gNB will anyway receive the UE capability from UE or CN after the RACH procedure.

Observation 8	If the UE sees no need to apply SBFD RA (to exploit latency reduction or coverage extension), the UE would not choose SBFD RO for Msg1 transmission.

Observation 9	For a RO across SBFD symbols and non-SBFD symbols, the RO may be overlapping with a legacy RO on 2nd symbol or beyond, in such case, at least the s_id would be different, which would be sufficient to distinguish RA-RNTIs between the SBFD RO and the legacy RO which are overlapping.

Observation 10	A configured RO starting from non SBFD symbols and ending in SBFD symbols, is invalid.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Capture explicitly in MAC spec that SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

Proposal 2	It is up to the network implementation to ensure that SBFD RO type is signalled to a UE for CFRA only when an SBFD RACH configuration is configured for CBRA, i.e., no spec change is needed.

Proposal 3	After RO type switching in the case of RACH configuration Option 2, the MAC entity conducts the initialization of (all) RACH parameters according to the SBFD RACH configuration.

Proposal 4	Not support introducing a power offset to compensate the power ramping difference between the two RO types (in SBFD RACH configuration Option 2) for RO type switching.

Proposal 5	For the NT indicating RO type, stick to the current RAN2 agreement, i.e., use 1 bit indicator-based signalling (as captured in the RRC running CR).

Proposal 6	Regarding if SBFD can be supported for LTM, adopt Option 2, i.e., only support ”RACH-based LTM in SBFD symbols".

Proposal 7	The UE is allowed to switch to an RO type that is configured with higher Msg1 repetition number, if the same Msg1 repetition number is not available for the RO type.

Proposal 8	SBFD awareness early indication via Msg3 or Msg5 is not pursued for SBFD aware UE.

Proposal 9	Update the RA-RNTI formula for SBFD RA configuration is not pursued.

# 4	References

- [1] 	RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei,

- Shanghai, China, June 17-20, 2024, RAN#104.