3GPP TSG-RAN WG2 Meeting #131	R2-2505685

Bengaluru, India, August 25-29, 2025

Agenda item:	 8.10.2

Source:		Lenovo

Title:		Discussion on MRO enhancements for mobility

Document for:		Discussion and Decision

1	Introduction

RAN2#130 meeting agreed for LTM that:

We will not introduce UE reporting regarding TAs that were “unnecessarily” acquisitioned.

We will not add TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment.

Introduce new values of raPurpose for RACH-based LTM-related RA reports.

RAN2#130 meeting agreed for CHO with candidate SCG(s) that:

FFS if we add some correlation indication.

We will not add any optimizations to avoid duplicated info in correlated reports

In this paper, we further discuss left issues of MRO enhancements for LTM and CHO with candidate SCG(s).

2	Discussion

RA report enhancements for LTM

In a LTM procedure, depending on the availability of a valid TA value, the UE performs either a RACH-less LTM or RACH-based LTM cell switch:

- the UE performs RACH-less LTM cell switch upon receiving the LTM cell switch command:

  - if the TA value is provided in the cell switch command, the UE applies the TA value as instructed by the network. For this case, before LTM cell switch command, the UE performs RACH to an LTM candidate cell triggered by PDCCH order for early TA acquisition/UL synchronization; when RACH-less LTM cell switch fails, the UE performs RACH based LTM failure recovery or RRC-re-establishment.

  - if UE-based TA measurement is configured, but no TA value is provided in the cell switch command, the UE applies the TA value by itself if available; when RACH-less LTM cell switch fails, the UE performs RACH based LTM failure recovery or RRC-re-establishment.

- if no valid TA value is available, the UE performs RACH-based LTM cell switch upon receiving the LTM cell switch command:

  - the UE performs CFRA to the target cell, if contention-free Random Access Resources are provided in LTM Cell Switch Command MAC CE;

  - or, the UE performs CFRA to the target cell, if no contention-free Random Access Resource is provided in LTM Cell Switch Command MAC CE, but contention-free Random Access Resources are provided in the RRC Reconfiguration message for the LTM procedure;

  - or, the UE performs CBRA to the target cell, if no contention-free Random Access Resource is provided.

Currently, RA report is used to log RACH information related with RA procedure, and at most eight RA reports can be included in a RACH report list. In an LTM procedure, the UE may perform both RACH for early TA acquisition and RACH based access (e.g. LTM failure recovery/RRC-re-establishment, or RACH-based LTM for the case that network does not send the early TA value to the UE due to the early TA is outdated). Besides RACH information for RACH based access (e.g. LTM failure recovery or RRC-re-establishment, or RACH-based LTM for the case that network does not send the early TA value to the UE due to the early TA is outdated), it is beneficial to include RACH information for early TA acquisition in RA report, in order to optimize RACH related parameters for early TA acquisition/UL synchronization.

Proposal 1: RACH information for early TA acquisition can be included in RA report.

When both RACH information for RACH based access (e.g. LTM failure recovery or RRC re-establishment, or RACH-based LTM for the case that network does not send the early TA value to the UE due to the early TA is outdated) and RACH information for early TA acquisition need to be logged in the RA report, RAN2 should discuss how to log them:

- Option 1: One entry can be used to log both RACH information related with early TA acquisition and RACH information related with RACH based access (e.g. LTM failure recovery or RRC re-establishment, or RACH-based LTM for the case that network does not send the early TA value to the UE due to the early TA is outdated) in the RA report.

- Option 2: Separate entries can be used to log RACH information related with early TA acquisition and RACH information related with RACH based access (e.g. LTM failure recovery or RRC re-establishment, or RACH-based LTM for the case that network does not send the early TA value to the UE due to the early TA is outdated) in the RA report.

One entry for logging is the better solution to reduce spec impacts on legacy RA report structure and RACH information logging behaviour at UE side. Therefore, Option 1 is preferred.

Proposal 2: One entry can be used to log both RACH information related with early TA acquisition and RACH information related with RACH based access (e.g. LTM failure recovery or RRC re-establishment, or RACH-based LTM for the case that network does not send the early TA value to the UE due to the early TA is outdated) in RA report.

MRO enhancements for CHO with candidate SCG(s)

For the FFS about if we add a correlation indication for SHR and SPR generated for the same CHO with candidate SCG execution (i.e. Open issue RRC-1), the potential options are summarized as below:

- Opt 1: Correlation indication in RRC_Complete message (a flag beside availability indication)

- Opt 2: Correlation indication inside the SHR and SPR

- Opt 3: Logging C-RNTI of PCell and timeSinceSPR in SPR

- Opt 4: The duplicated information in correlated SHR and SPR is sufficient. no further information is needed for correlation.

We can find that RAN3#127bis agreed no support of near failure case in R19, which means that from NW point of view, any enhancement for SPR is not supported for the CHO with candidate SCG procedure, the better way is that, to align with RAN3 to reduce WG workload, neither RAN2 needs to add the correlation indication for both SHR and SPR. On the other hand, if correlation is really a critical issue to be solved, we think duplicated information in correlated SHR and SPR is sufficient, no additional information for correlation is needed.

Proposal 3 (RRC-1): RAN2 will not add additional information for correlation SHR and SPR generated for the same CHO with candidate SCG execution.

3	Conclusion

In this paper, MRO related enhancements for LTM and CHO with candidate SCG(s) are discussed. We have the following proposals:

RA report enhancements for LTM

Proposal 1: RACH information for early TA acquisition can be included in RA report.

Proposal 2: One entry can be used to log both RACH information related with early TA acquisition and RACH information related with RACH based access (e.g. LTM failure recovery or RRC re-establishment, or RACH-based LTM for the case that network does not send the early TA value to the UE due to the early TA is outdated) in RA report.

MRO enhancements for CHO with candidate SCG(s)

Proposal 3 (RRC-1): RAN2 will not add additional information for correlation SHR and SPR generated for the same CHO with candidate SCG execution.