3GPP TSG-RAN WG2 #131	R2-2505686

Bangaluru, India, August 25th – 29th, 2025

Agenda item:	   8.1.2.2

Source:		Lenovo

Title:	Left issues for AIML based BM

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss left issues related to applicability information reporting.

# Discussion

## Open Issue RRC-15: The time duration for an AI functionality to become available for inference when the UE reports applicability

Issue description: The UE needs to synchronize the time duration for an AI functionality to become available for inference with the network when the UE reports applicability for either a full inference configuration or a set of inference parameters. Unlike conventional non-AI features, where the algorithm size is fixed, AI algorithms can vary significantly in size depending on their generalization performance and specific use cases. These sizes can range from several kilobytes to tens of megabytes. Additionally, UE implementations may use various types of memory to balance cost and performance, and these different memory types and constraints can affect the access speed of stored models. The UE should indicate the time duration for the AI functionality to become available when reporting applicability to the network. This time duration refers to how long it takes for the UE to load the AI models into RAM or another accessible memory for inference. By providing this information, the network can better understand the timeframe within which the configured radio resources and AI model can be utilized, enabling more efficient and effective resource management, especially for periodic CSI. Another option is to let RAN4 address this time duration in terms of requirements. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

To address the raised issue, it’s suggested to align companies understanding on the scenario when UE is expected to load the model to memory (i.e., the last step before performing inference), for which the “model loading delay” will occur.

- Scenario 1: When receiving the set of inference parameters and determined to be appliable

- Scenario 2: When receiving the Inference Configuration and determined to be applicable (may or may not activate immediately)

- Scenario 3: When receiving the configuration to activate the inference (e.g., receiving periodic CSI report configuration, or activation MAC CE command for semi-persistent CSI report, or activation DCI command for aperiodic CSI report)

In our understanding, Scenario 1 is unnecessary and may lead to a waste of memory usage, since UE does not know when the gNB will configure UE with inference configuration eventually.

Scenario 2 and Scenario 3 could be feasible and can be considered by RAN2.

RAN2 is suggested to clarify the “model loading” will occur in which scenario of the following:

  - Scenario 1: When receiving the set of inference parameters and determined to be appliable.

  - Scenario 2: When receiving the Inference Configuration and determined to be applicable (may or may not activate immediately)

  - Scenario 3: When receiving the configuration to activate the inference (e.g., receiving periodic CSI report configuration, or activation MAC CE command for semi-persistent CSI report, or activation DCI command for aperiodic CSI report)

When it comes to a potential RAN2 impact, we believe that the “model loading delay” may be additionally indicated to NW only in case that UE receives RRC Reconfiguration containing the inference configuration in Scenario 2, and model loading has not finished before RRC Reconfiguration Complete message transmission.

Comparatively, UE will not indicate the “model loading delay” in the other two cases:

- For Scenario 3, for semi-persistent CSI report and aperiodic CSI report, which are activated by MAC CE and DCI, there is no responding signalling for UE to indicate the “model loading delay”

- Upon the change of applicability, from non-appliable to appliable, UE informs the NW using UAI. Since there is no restriction of when UAI must be sent, UE can simply send the UAI after the model has been loaded to the memory.

The legacy RRC processing delay can be up to 16 ms, whether it is enough or not for UE to load the model loading would rely on RAN4’s requirement analysis. Thus, it is suggested to wait for RAN4’s input before the final decision.

Upon RAN4’s input, RAN2 discusses the need of additional “model loading delay” indication from UE to NW only in the following case:

  - UE receives RRC Reconfiguration containing the inference configuration in Scenario 2, and model loading has not finished before RRC Reconfiguration Complete message transmission.  .

## Open issue RRC -44: Whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations

Issue description: Based on the RAN2 agreements below, it is not clear whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations stored at the UE, or whether it can contain only updates for the inference configurations for which the UE already reported the applicability status in the past. This open issue refers to the RAN2#129 agreements: 3. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause. In the rapporteur’s understanding, there are two interpretations of the agreements above: - The RRCReconfigurationComplete contains the applicable/inapplicable status for all inference configurations. Only UAI contains updates. - The RRCReconfigurationComplete contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message and only applicability changes for those inference configurations received in earlier RRCReconfiguration messages, for which the UE already reported applicable/inapplicable at least once. UAI contains updates. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

As observed also by some other companies, it remains unclear in the current CR what applicability information will UE report in the RRCReconfigurationComplete message after receiving a RRCReconfiguration containing inference configuration or inference related parameter sets. There are three possibilities on the table:

- Option 1: Applicability information of the inference configuration or inference related parameter sets configured (i.e., add or modify) for the first time by the received RRCReconfiguration. (Agreed previously in RAN2)

- Option 2: Option 1 + any changed applicability information of existing inference configuration or inference related parameters sets

- Option 3: Applicability Information of ALL inference configuration and inference related parameter sets (i.e., existing ones and newly configured ones). (Implemented in the current CR)

In our understanding, Option 3 that enforces UE to always report Applicability Information of ALL inference configuration and inference related parameter sets, will certainly cause heavy signaling overhead.

Besides, Option 2 and Option 3 may cause confusion to UE whether a changed applicability should be reported via RRCReconfigurationComplete message or a generated UAI message, in an extreme example, when the applicability of an AI functionality changed at the moment when a RRCReconfiguration message is received.

Therefore, we think it clearer from UE behavior point of view that the RRCReconfigurationComplete message only contains Applicability information of the inference configuration or inference related parameter sets configured (i.e., add or modify) for the first time by the received RRCReconfiguration. (Agreed previously in RAN2)

The RRCReconfigurationComplete message only contains applicability information of the inference configuration or inference related parameter sets configured (i.e., add or modify) in the respective received RRCReconfiguration.

## Open Issue RRC-x: Applicability information transfer over HandoverPreparationInformation

In the previous RAN2 discussion, it was believed the existing UAI framework can be reused for the source gNB to transfer the applicability information to the target gNB. - Source cell UAI (as is) can be sent from source cell to target cell using existing signaling. No further optimizations will be considered in RAN2 related to UAI. However, source gNB will only send the last received UAI to the target gNB following the current spec, which only contains an incomplete list of lastly changed applicability information. Proposed resolution: Source gNB transmits to target gNB a complete applicability information list currently maintained at the source gNB considering the UAI and RRCReconfigurationComplete received in the past. Via UAI (needs to update the description), or via a dedicated IE (maybe cleaner?).

As discussed in RAN2 before, companies believed that, during handover procedure, the source gNB could transfer the applicability information received from UE to the target gNB via UAI information. However, according to the legacy, HandoverPreparationInformation will only include the UAI that is last reported by the UE. Following the design of applicability reporting, that means the target gNB will not receive the applicability information of the complete list of inference configuration or inference related parameters. Instead only those has changed their applicability in the last reported UAI will be transferred to target gNB.

AS-Context field descriptions

configRestrictInfoDAPS Includes fields for which source cell explicitly indicates the restriction to be observed by target cell during DAPS handover.

mbsInterestIndication Includes the information last reported by the UE in the NR MBSInterestIndication message, where the plmn-Index (if included by the UE in tmgi) is replaced by the PLMN ID, if needed. A TMGI for which the plmn-Index points to a non-serving SNPN is removed from the NR MBSInterestIndication message.

needForGapsInfoNR Includes measurement gap requirement information of the UE for NR target bands. The field includes needForGapsInfoNR in RRCReconfigurationComplete message, needForGapsInfoNR in RRCResumeComplete message or musim-needForGapsInfoNR in UEAssistanceInformation message that is last reported by the UE, if any.

selectedBandCombinationSN Indicates the band combination selected by SN in (NG)EN-DC, NE-DC, and NR-DC.

sidelinkUEInformationEUTRA This field includes SidelinkUEInformation IE as specified in TS 36.331 [10].

sidelinkUEInformationNR This field includes SidelinkUEInformationNR IE.

ueAssistanceInformation Includes for each UE assistance feature the information last reported by the UE, if any.

ueAssistanceInformationSCG Includes for each UE assistance feature associated with the SCG, the information last reported by the UE in the NR UEAssistanceInformation message for the SCG, if any.

To resolve this, RAN2 is suggested to first confirm the support of source gNB sending to target gNB a complete applicability information list currently maintained at the source gNB considering the UAI and RRCReconfigurationComplete received in the past. Then, RAN2 can further discuss if the complete applicability information is conveyed in either

- UAI in HandoverPreparationInformation message, then the description needs to be updated, or

- A dedicated IE in HandoverPreparationInformation message.

RAN2 is suggested to first confirm the support of source gNB sending to target gNB a complete applicability information list currently maintained at the source gNB considering the UAI and RRCReconfigurationComplete received in the past.

RAN2 further clarify if the complete applicability information list is conveyed by the UAI (field description needs update) or a dedicated IE in HandoverPreparationInformation message.

# 3	Conclusion

Based on the discussion above, we observe:

No table of figures entries found.

Based on the discussion above, we propose:

Proposal 1	RAN2 is suggested to clarify the “model loading” will occur in which scenario of the following:

a.	Scenario 1: When receiving the set of inference parameters and determined to be appliable.

b.	Scenario 2: When receiving the Inference Configuration and determined to be applicable (may or may not activate immediately)

c.	Scenario 3: When receiving the configuration to activate the inference (e.g., receiving periodic CSI report configuration, or activation MAC CE command for semi-persistent CSI report, or activation DCI command for aperiodic CSI report)

Proposal 2	Upon RAN4’s input, RAN2 discusses the need of additional “model loading delay” indication from UE to NW only in the following case:

a.	UE receives RRC Reconfiguration containing the inference configuration in Scenario 2, and model loading has not finished before RRC Reconfiguration Complete message transmission.  .

Proposal 3	The RRCReconfigurationComplete message only contains applicability information of the inference configuration or inference related parameter sets configured (i.e., add or modify) in the respective received RRCReconfiguration.

Proposal 4	RAN2 is suggested to first confirm the support of source gNB sending to target gNB a complete applicability information list currently maintained at the source gNB considering the UAI and RRCReconfigurationComplete received in the past.

Proposal 5	RAN2 further clarify if the complete applicability information list is conveyed by the UAI (field description needs update) or a dedicated IE in HandoverPreparationInformation message.

# Reference