3GPP TSG-RAN WG2 Meeting #131	R2-2505724

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.10.2

Source:	Nokia

Title:	MRO enhancements for Rel-18 mobility features (RRC-1, RRC-2, RRC-9)

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core – Rel-19

Document for:	Discussion and Decision

# 1	Introduction

This paper presents proposals for open issue RRC-1 (Correlation of SHR and SHR generated for a CHO with Candidate SCG), open issue RRC-2 (CHO with candidate SCG with complementary CHO only configuration) and open issue RRC-9 (CPAC configuration logging).

Moreover, it discusses an issue related to near failure handling for CHO with candidate SCGs.

# 2	Discussion

## 2.1	RRC-1: Correlation of SHR and SPR for CHO with candidate SCGs

The following agreements were reached on the topic of MRO for CHO with candidate SCG with regard to information to be logged in case of near failure in SHR and SPR:

RAN2#125bis: RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

RAN2#128: Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

RAN2#129bis

UE includes the target PSCell ID in SHR for successful CHO with candidate SCGs.

UE includes the target PCell ID in SPR for successful CHO with candidate SCG.

In general, and where applicable, agreements valid for SHR, RLF reports and SCG failure info applies also to SPR.

RAN2#130

FFS if we add some correlation indication.

We will not add any optimizations to avoid duplicated info in correlated reports.

In the spirit of aligning the report content with RLF report and legacy SHR, some agreements were made in RAN2 on enhancing SHR and SPR with content relating CHO with candidate SCGs. There are cases where both SHR and SPR are generated for the same CHO with candidate SCGs execution and therefore the root cause analysis should include both near failure reports as they are both pointing to the same CHO with candidate SCGs execution. In this way, the maximum information is gathered and can be used by the network for optimization actions. An individual treatment of SHR and SPR would not embrace the strong inter-dependency of the two links and each report represents only the link-specific view of the issue. Not knowing that there was another near failure on the second link can result in wrong root cause decision. If wrong counters are concluded by separate analysis, a mutual statistical post-processing will not help.

For that reason, the two near failure cases need to be analyzed in a unified manner and need to be correlated. This issue was also identified as an open issue during the 38.331 CR review after RAN2#129 [[Post129bis][603][SONMDT] Running NR RRC CR for SONMDT (Ericsson)]. Although the topic was discussed during the last RAN2 meeting, the companies could not agree if this is needed at all nor on a solution.

We still believe that, in the absence of such correlation, the network may have a hard time understanding what happened and may even take wrong corrective actions.

Some of the companies opposing a dedicated mechanism to correlate SHR and SPR are of the opinion that the network can already do this based on the cell pairing included in the reports. We think however that this method is not full proof. For one, in the absence of a UE identifier and with multiple UEs that may perform CHO with candidate SCGs handovers between the same cell pairs, it will be hard to understand which reports belong together. There may also be the case that for some CHO with candidate SCGs executions, only one report will be generated, SHR or SPR which may leave the network waiting for the retrieval of a report that does not exist. The other issue is that matching the SHRs and SPRs would require significant effort from the network side as all reports should be checked to find the correlated ones.

Proposal 1.1: RAN2 to decide that a correlation mechanism for SHR and SPR generated for the same CHO with candidate SCG execution is needed.

Such a correlation may be easily achieved by including for instance a coexistence flag, in each report pointing to the existence of the other report. The UE is aware of both near failures occurring for same CHO with candidate SCGs execution and can add such an indication to both reports. From implementation point of view, the reports may be generated independently upon successful execution of CHO and CPAC to the respective target cells and the co-existence flags can be added to each report when indicating their availability to the network via the RRCReconfigurationComplete message.

Proposal 1.2: RAN2 to agree that the correlation of SHR and SPR generated for the same CHO with candidate SCG execution can be achieved by adding a co-existence flag in both SHR and SPR indicating the presence of the other report when their availability is indicated. (Text proposal is in Annex 1.)

## 2.2	RRC-2: CHO with candidate SCG with complementary CHO only configuration

In the previous RAN2 meeting, it was discussed and agreed that the network may configure the UE operating in dual connectivity (DC) mode both with CHO with candidate SCG(s) and CHO-only in complementary manner [1], i.e., there is in addition to the preferred CHO with candidate SCG(s) a fallback option for the conditional PCell change being carried without preparation of the secondary link.

In this context the following was agreed during RAN2#130:

We will look in to if/what to specify for the scenario with CHO with candidate SCG alongside a CHO-only configuration. Proponents should have clear and well-defined proposals next meeting preferably with text proposals.

This contribution starts to look deeper and provides our view how an UE operating in DC mode being configured both with CHO with candidate SCG(s) and with CHO-only behaves, discusses the relevant issues and comes up with clear proposals to tackle them. For sake of simplicity, CHO with candidate SCG(s) is abbreviated with CwcS in this section

For UEs operating in DC, it is ensured by the standard that the CHO with candidate SCG(s) (CwcS) is the preferred conditional PCell change option among other CHO with SN options given by section 5.3.5.13.5 on “Conditional reconfiguration execution” [2]. That means the complementary CHO-only configuration per se will not harm the preference of CwcS even though CHO part (condExecutionCond) is identical for two options.

Observation 2.1: The standard ensures that CHO with candidate SCG(s) (CwcS) is carried out if both CHO with SN options (CHO with candidate SCG(s) and CHO-only) are triggered simultaneously.

There are basically two scenarios when CHO-only will overrule CwcS

- The criterion condExecutionCond of the condReconfigId representing CHO-only typically triggered by A3-like relative event criterion is more aggressive, or

- The CPAC criterion (condExecutionCondPSCell) which is an absolute A4-like criterion where at least one candidate PSCell including the current serving PSCell is not fulfilled when CHO part of CwcS is triggered (assuming that condExecutionCond for CwcS and CHO-only is configured identically).

We think that identical PCell related conditional criterion condExecutionCond for CHO-only and CwcS can be assumed in practice because the PCell change refers to the same cell border.

Observation 2.2: There are two scenarios where preferred CwcS option could be pre-empted by the complementary CHO-only mode.

This pre-emption is per se not a failure, but the advantages of CwcS are lost. As discussed in [1], it would be beneficial for the network to understand the reason why CwcS was not carried out as expected, and then it could retune the corresponding parameters of execution conditions for candidate PCells and/or associated candidate PSCells.

The missed CwcS execution caused by unexpected pre-emption by CHO-only is not a failure but performance-disturbing and, therefore, it is an issue that network should be aware about.

Proposal 2.1: RAN2 discusses and agrees that the unwanted pre-emption by CHO-only is a scenario that network should be aware of.

As discussed above, to help the network to understand the reason why CwcS was pre-empted by the complementary CHO-only configuration, UE should be able to detect and report this issue, including further information to analyse the root cause for the missed CwcS.

Proposal 2.2: RAN2 discusses and agrees that the UE should be able to detect and report the pre-emption by CHO-only.

As this pre-emption can be seen as near-failure case, introduction of a new trigger criterion for a successful handover report (SHR) would an obvious solution.

The new trigger criterion for that SHR, which is indicating a missed CwcS is triggered when all the following three criteria are met:

- UE is operating in DC mode, and

- UE is configured for both with CwcS and CHO-only, and

- UE did execute CHO-only instead of CwcS.

With fulfilment of all three conditions an SHR is created and should be delivered to the network after successful completion of the CHO-only based PCell handover.

Proposal 2.3: RAN2 agrees on a new trigger criterion for SHR that reports the issue of performing a CHO-only handover when CwcS was also configured including the necessary information to detect the root cause of the issue.

To make the network aware of potential near failure case by means of SHR the UE needs to be configured, i.e., the successHO-Config is to be amended accordingly. It might be useful to add a flag (e.g. “CwcS_Usage_Check-r19”, which indicates the UE to check usage of CHO with candidate SCG(s), when it is configured for both CHO-only and CwcS.

Proposal 2.4: RAN2 agrees on the amendment of successHO-Config to configure the UE for creating the SHR when a CHO-only handover is performed in spite of having CwcS configuration as well.

With successful completion of the CHO-only handover the network will be informed about the issue of a “missed CwcS”. The node controlling the target PCell (or the newly serving PCell after handover) will be informed about SHR availability and can start retrieving the SHR by means of UEInformationRequest/Response message exchange.

For a detailed issue analysis in MRO context, the SHR will consist of a new SHR-Cause, for instance, “CwcS not triggered”) as well as further information and measurements being logged at the time when CwcS was preempted by CHO-only, i.e., when CHO-only handover was executed.

The needed information elements (IE) like sourcePCellId, sourcePSCellId, targetPCellId are almost identical to what has been specified so far in the failure and near failure reports in context of CwcS. The new ChoWithCandidateSCGInfo can be reused for this purpose as well, even though the timing criteria therein are not needed. As they are optional elements, they can be left out from the reports without any issue.

Apart from a new SHR cause, the currently specified SHR (successHO-Report-r18) already provides many of the IEs to carry the needed information. Only the candidate PSCells measurements might be missing. But those are important to see whether any of the PSCells did pass the condEventA4 condition for condExecutionCondPSCell needed for CwcS.

Relevant is also the time when needed information is to be dumped, namely at that moment when CHO-only was executed, when the CwcS is preempted.

From MRO perspective, the following information might be needed for proper root cause analysis, where all the listed elements can be provided with the SHR already or are at least currently proposed for release-19:

- List of candidate PCells configured for CwcS: covered in ChoWithCandidateSCGInfo proposed in the running CR

- List of associated candidate PSCells configured for CwcS: covered in ChoWithCandidateSCGInfo proposed in the running CR

- Id of sourcePSCell: covered by sourcePSCellInfo-r19 proposed in the running CR

- Id of sourcePCell: covered by sourceCellInfo_r17

- Id of targetPCell used for CHO-only: covered by targetCellInfo_r17

- Measurement of serving PCell: covered by sourceCellInfo_17

- Measurement of serving PSCell: covered by sourcePSCellInfo-r19 proposed in the running CR

- Measurements of all candidate PCells: covered by measResultNeighCells-r17

- Measurements of all associated candidate PSCells covered by measResultNeighCells-r17

Proposal 2.5: RAN2 agrees that the new SHR will consist of a new SHR-Cause, for instance, “CwcS not triggered” which requires new trigger condition.

The advantage of this approach is that the MRO/SON mechanism is immediately stipulated when the issue occurs. Based on the included information the corresponding MRO instance will be able to analyse the root cause for the missed CwcS per PCell pair.

Proposal 2.6: Adopt the text proposals of Annex 2 to implement Proposal 2.3, 2.4 and 2.5 in the RRC specification.

## 2.3	RRC-9: CPAC configuration logging

In the current specifications, details regarding to CHO dual event execution (first triggered event and time between events) are logged as per RAN2 agreements:

RAN2#129: Enhance RLF report for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any).

This information is logged in the RLF report and the SCGFailureInformation. Additionally, the current CR also logs the CHO configurations under choConfig IE in case of radio link failure but not in the case of SCG failure. However, the CPAC configuration and dual event information is not logged in either report.

As the RLF and SCG failure may be due to the either of the execution conditions (CHO or CPAC) not being fulfilled, we think the current agreements and specifications are not complete and deprive the network from obtaining all relevant information that may aid it in determining corrective actions.

Proposal 3.1: CHO configuration should also be logged in case of SCG failure, i.e. choConfig should be also included in the SCGFailureInformation message.

We also propose that CPAC configuration and dual event information is also logged in RLF report and SCGFailureInformation message via additional new IEs similar to ones used for CHO.

Proposal 3.2: RAN2 agrees to log CPAC configuration and dual event information in RLF report and SCGFailureInformation via additional new IEs similar to ones used for CHO.

## 2.4		Near failure handling for CHO with candidate SCGs

The following agreements were reached on the topic of MRO for CHO with candidate SCG with regard to information to be logged in case of near failure in SHR and SPR:

RAN2#125bis: RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

RAN2#128: Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

RAN2#129bis

UE includes the target PSCell ID in SHR for successful CHO with candidate SCGs.

UE includes the target PCell ID in SPR for successful CHO with candidate SCG.

In general, and where applicable, agreements valid for SHR, RLF reports and SCG failure info applies also to SPR.

RAN2#130

FFS if we add some correlation indication.

We will not add any optimizations to avoid duplicated info in correlated reports.

In the spirit of aligning the report content with RLF report and legacy SHR, some agreements were made in RAN2 on enhancing SHR and SPR with content relating CHO with candidate SCGs.

We believe that the case of a T310-percentage triggered SHR or SPR, respectively, is to be treated differently depending on which cell change condition was fulfilled already when near failure is experienced. Therefore, another interesting aspect in case of SHR/SPR triggered by T310-percentage criterion would be knowing at which point T310 was started in relation to the two execution conditions’ fulfillments. For example, if T310 on either link was started after the fulfillment of one execution condition but before fulfilment of the second execution condition, this information could more clearly point to the root cause of the near failure, e.g. the second fulfilled condition needs to be updated. From an implementation point of view, the UE may log the point in time when the first condition was fulfilled and compared to the elapsed T310 at the moment of CHO with Candidate SCG(s) execution.

Proposal 4: RAN2 to agree that there is a need to distinguish if the T310 started before or after fulfilment of the first execution conditions for CHO with Candidate SCG(s) and an indication about this in SHR and SPR.

# 3	Conclusion

This document has made the following proposals on correlation of SHR and SPR for CHO with candidate SCGs (open issue RRC-1):

Proposal 1.1: RAN2 to decide that a correlation mechanism for SHR and SPR generated for the same CHO with candidate SCG execution is needed.

Proposal 1.2: RAN2 to agree that the correlation of SHR and SPR generated for the same CHO with candidate SCG execution can be achieved by adding a co-existence flag in both SHR and SPR indicating the presence of the other report when their availability is indicated. (Text proposal is in Annex 1.)

This document has made the following observations and proposals on CHO with candidate SCG with complementary CHO only configuration (open issue RRC-2):

Observation 2.1: The standard ensures that CHO with candidate SCG(s) (CwcS) is carried out if both CHO with SN options (CHO with candidate SCG(s) and CHO-only) are triggered simultaneously.

Observation 2.2: There are two scenarios where preferred CwcS option could be pre-empted by the complementary CHO-only mode.

Proposal 2.1: RAN2 discusses and agrees that the unwanted pre-emption by CHO-only is a scenario that network should be aware of.

Proposal 2.2: RAN2 discusses and agrees that the UE should be able to detect and report the pre-emption by CHO-only.

Proposal 2.3: RAN2 agrees on a new trigger criterion for SHR that reports the issue of performing a CHO-only handover when CwcS was also configured including the necessary information to detect the root cause of the issue.

Proposal 2.4: RAN2 agrees on the amendment of successHO-Config to configure the UE for creating the SHR when a CHO-only handover is performed in spite of having CwcS configuration as well.

Proposal 2.5: RAN2 agrees that the new SHR will consist of a new SHR-Cause, for instance, “CwcS not triggered” which requires new trigger condition.

Proposal 2.6: Adopt the text proposals of Annex 2 to implement Proposal 2.3, 2.4 and 2.5 in the RRC specification.

This document has made the following proposals on CPAC configuration logging (open issue RRC-9):

Proposal 3.1: CHO configuration should also be logged in case of SCG failure, i.e. choConfig should be also included in the SCGFailureInformation message.

Proposal 3.2: RAN2 agrees to log CPAC configuration and dual event information in RLF report and SCGFailureInformation via additional new IEs similar to ones used for CHO.

This document has made the following proposal on near failure handling for CHO with candidate SCGs:

Proposal 4: RAN2 to agree that there is a need to distinguish if the T310 started before or after fulfilment of the first execution conditions for CHO with Candidate SCG(s) and an indication about this in SHR and SPR.

# Annex 1: Text proposal for open issue RRC-1

Changes are indicated over the latest version of 38.331.

Start of changes

#### 5.7.10.6	Actions for the successful handover report determination

The UE shall for the PCell:

<Text Omitted>

3>	if available, set the locationInfo as in 5.3.3.7;

3>	if any of the conditions of generating a successful PSCell change or addition report as in 5.7.10.7 are also met;

4>	include the successPSCell-ReportGenerated;

1>	release successHO-Config configured by the source PCell and thresholdPercentageT304 if configured by the target PCell.

The UE may discard the successful handover information, i.e., release the UE variable VarSuccessHO-Report, 48 hours after the last successful handover information is added to the VarSuccessHO-Report.

Next changes

#### 5.7.10.7	Actions for the successful PSCell change or addition report determination

The UE shall for the PSCell:

<Text Omitted>

3>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured:

4>	if available, set the locationInfo as in 5.3.3.7 according to the otherConfig associated with the source PSCell;

4>	include sn-InitiatedPSCellChange;

3>	else:

4>	if available, set the locationInfo as in 5.3.3.7 according to the otherConfig associated with the PCell;

3>	if any of the conditions of generating a successful handover report as in 5.7.10.6 are also met;

4>	include the successHO-ReportGenerated;

1>	release successPSCell-Config configured by the source PSCell if available and thresholdPercentageT304 if configured by the target PSCell.

The UE may discard the successful PSCell change or addition information, i.e., release the UE variable VarSuccessPSCell-Report, 48 hours after the last successful PSCell change or addition information is added to the VarSuccessPSCell-Report or upon deregistration from the network as specified in TS 23.502 [43].

Next changes

#### –	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

Signalling radio bearer: SRB1 or SRB2 (when logged measurement information is included)

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

UEInformationResponse message

-- ASN1START

-- TAG-UEINFORMATIONRESPONSE-START

UEInformationResponse-r16 ::=        SEQUENCE {

rrc-TransactionIdentifier            RRC-TransactionIdentifier,

criticalExtensions                   CHOICE {

ueInformationResponse-r16            UEInformationResponse-r16-IEs,

criticalExtensionsFuture             SEQUENCE {}

}

}

<Text Omitted>

SuccessHO-Report-r17 ::=                 SEQUENCE {

sourceCellInfo-r17                       SEQUENCE {

sourcePCellId-r17                        CGI-Info-Logging-r16,

sourceCellMeas-r17                       MeasResultSuccessHONR-r17                       OPTIONAL,

rlf-InSourceDAPS-r17                     ENUMERATED {true}                               OPTIONAL

},

targetCellInfo-r17                       SEQUENCE {

targetPCellId-r17                        CGI-Info-Logging-r16,

targetCellMeas-r17                       MeasResultSuccessHONR-r17                       OPTIONAL

},

measResultNeighCells-r17                 SEQUENCE {

measResultListNR-r17                     MeasResultList2NR-r16                           OPTIONAL,

measResultListEUTRA-r17                  MeasResultList2EUTRA-r16                        OPTIONAL

}                                                                                            OPTIONAL,

locationInfo-r17                         LocationInfo-r16                                    OPTIONAL,

timeSinceCHO-Reconfig-r17                TimeSinceCHO-Reconfig-r17                           OPTIONAL,

shr-Cause-r17                            SHR-Cause-r17                                       OPTIONAL,

ra-InformationCommon-r17                 RA-InformationCommon-r16                            OPTIONAL,

upInterruptionTimeAtHO-r17               UPInterruptionTimeAtHO-r17                          OPTIONAL,

c-RNTI-r17                               RNTI-Value                                          OPTIONAL,

...,

[[

targetCell-PCI-ARFCN-r17                 PCI-ARFCN-NR-r16                                    OPTIONAL

]],

[[

eutra-TargetCellInfo-r18                 SEQUENCE {

targetPCellId-r18                        CHOICE {

cellGlobalId-r18                         CGI-Info-Logging-r16,

pci-arfcn-r18                            PCI-ARFCN-EUTRA-r16

},

targetCellMeas-r18                       MeasQuantityResultsEUTRA                       OPTIONAL

}                                                                                           OPTIONAL,

measResultServCellRSSI-r18                   RSSI-Range-r16                                 OPTIONAL,

measResultNeighFreqListRSSI-r18              MeasResultNeighFreqListRSSI-r18                OPTIONAL,

eutra-C-RNTI-r18                             EUTRA-C-RNTI                                   OPTIONAL,

timeSinceSHR-r18                             TimeSinceSHR-r18                               OPTIONAL

]],

[[

successPSCell-ReportGenerated-r19            ENUMERATED {true}                                    OPTIONAL

]]

}

SuccessPSCell-Report-r18 ::=             SEQUENCE {

pCellId-r18                              CGI-Info-Logging-r16,

sourcePSCellInfo-r18                     SEQUENCE {

sourcePSCellId-r18                       CHOICE {

cellGlobalId-r18                         CGI-Info-Logging-r16,

pci-arfcn-r18                            PCI-ARFCN-EUTRA-r16

},

sourcePSCellMeas-r18                     MeasResultSuccessHONR-r17                       OPTIONAL

}                                                                                            OPTIONAL,

targetPSCellInfo-r18                     SEQUENCE {

targetPSCellId-r18                       CHOICE {

cellGlobalId-r18                     CGI-Info-Logging-r16,

pci-arfcn-r18                        PCI-ARFCN-NR-r16

},

targetPSCellMeas-r18                     MeasResultSuccessHONR-r17                       OPTIONAL

},

measResultNeighCells-r18                 SEQUENCE {

measResultListNR-r18                     MeasResultList2NR-r16                           OPTIONAL,

measResultListEUTRA-r18                  MeasResultList2EUTRA-r16                        OPTIONAL

}                                                                                            OPTIONAL,

spr-Cause-r18                            SPR-Cause-r18                                       OPTIONAL,

timeSinceCPAC-Reconfig-r18               TimeSinceCPAC-Reconfig-r18                          OPTIONAL,

locationInfo-r18                         LocationInfo-r16                                    OPTIONAL,

ra-InformationCommon-r18                 RA-InformationCommon-r16                            OPTIONAL,

sn-InitiatedPSCellChange-r18             ENUMERATED {true}                                   OPTIONAL,

...,

[[

successHO-ReportGenerated-r19            ENUMERATED {true}                                    OPTIONAL

]]

}

MeasResultNeighFreqListRSSI-r18 ::=      SEQUENCE(SIZE (1..maxFreq)) OF MeasResultNeighFreqRSSI-r18

<Text Omitted>

SuccessHO-Report field descriptions

c-RNTI This field indicates the C-RNTI assigned by the target PCell of the handover for which the successful HO report was generated.

eutra-TargetCellInfo This field is used to indicate the target EUTRA PCell and the last measurement results of the target PCell of a handover in which the successful handover triggers the SuccessHO-Report.

eutra-C-RNTI This field indicates the C-RNTI assigned by the E-UTRA target PCell of the mobility from NR command for which the successful HO report was generated.

measResultListNR This field refers to the last measurement results taken in the neighboring NR Cells when a successful handover is executed.

measResultNeighFreqListRSSI This field is used to log the RSSI measurement results in dBm (see TS 38.215 [9]) taken for the neighbouring frequencies upon successful handover execution.

measResultServCellRSSI This field refers to the log RSSI measurement results in dBm (see TS 38.215 [9]) taken for the frequency of the source PCell upon successful handover execution.

rlf-InSourceDAPS This field indicates whether a radio link failure occurred at the source cell while T304 was running.

shr-Cause This field is used to indicate the cause of the successful HO report.

sourceCellMeas This field refers to the last measurement results taken in the source PCell of a handover in which the successful handover triggers the SuccessHO-Report.

sourcePCellId This field is used to indicate the source PCell of a handover in which the successful handover triggers the SuccessHO-Report.

successPSCell-ReportGenerated This field is used to indicate that a successPSCell-Report is also generated.

targetPCellId This field is used to indicate the target PCell of a handover in which the successful handover triggers the SuccessHO-Report.

targetCellMeas This field refers to the last measurement results taken in the target PCell of a handover in which the successful handover triggers the SuccessHO-Report.

timeSinceCHO-Reconfig This field is used to indicate the time elapsed between the initiation of the last conditional reconfiguration execution towards the target cell and the reception of the latest conditional reconfiguration for this target cell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

timeSinceSHR This field is used to indicate the time elapsed since the execution of the last MobilityFromNRCommand towards the target EUTRA cell. Value in seconds. The maximum value 172800 means 172800s or longer.

upInterruptionTimeAtHO This field is used to indicate the time elapsed between the time of arrival of the last PDCP PDU received from the source cell for any data radio bearer and the time of arrival of the first non-duplicate PDCP PDU received from the target cell for any data radio bearer, and it is measured at the time of arrival of the first non-duplicate PDCP PDU received from the target cell for any data radio bearer. The field is set only in case of DAPS handover. Value in milliseconds. The maximum value 1023 means 1023ms or longer.

<text omitted>

SuccessPSCell-Report field descriptions

measResultListNR This field refers to the last measurement results according to the initiating node configuration taken in the neighboring NR Cells when a successful PSCell change/addition is executed.

pCellId This field is used to indicate the PCell to which the UE was connected when the successful PSCell change or addition triggers the SuccessPSCell-Report.

sn-InitiatedPSCellChange This field indicates whether the PSCell change procedure for which the successful PSCell change report is logged is SN initiated or not.

spr-Cause This field is used to indicate the cause of the successful PSCell change or addition report.

sourcePSCellId This field is used to indicate the source PSCell of a PSCell change in which the successful PSCell change triggers the SuccessPSCell-Report.

sourcePSCellMeas This field refers to the last measurement results taken in the source PSCell of a PSCell change in which the successful PSCell change triggers the SuccessPSCell-Report.

successHO-ReportGenerated This field is used to indicate that a successHO-Report is also generated.

targetPSCellId This field is used to indicate the target PSCell of a PSCell change/addition in which the successful PSCell change or addition triggers the SuccessPSCell-Report.

targetPSCellMeas This field refers to the last measurement results taken in the target PSCell of a PSCell change/addition in which the successful PSCell change or addition triggers the SuccessPSCell-Report.

timeSinceCPAC-Reconfig This field is used to indicate the time elapsed between the initiation of the last conditional reconfiguration execution towards the target PSCell and the reception of the latest conditional reconfiguration for this target PSCell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

End of changes

# Annex 2: Text proposal for open issue RRC-2

Changes are indicated over the latest version of 38.331.

Start of changes

#### 5.7.10.6	Actions for the successful handover report determination

The UE shall for the PCell:

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the ratio between the value of the elapsed time of the timer T304 and the configured value of the timer T304, included in the last applied RRCReconfiguration message including the reconfigurationWithSync, is greater than thresholdPercentageT304 if included in the successHO-Config received before executing the last reconfiguration with sync; or

<text omitted>

1>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, and if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the Mobility from NR to E-UTRA and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PCell before executing the last Mobility from NR to E-UTRA, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last Mobility from NR to E-UTRA; or

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the mobility procedure was triggered by a condReconfigId in the MCG VarConditionalReconfig including only condExecutionCond while operating in dual connectivity mode and being also configured with another condReconfigId in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell and the CHOwithCandidateSCG-Check included in the successHO-Config received before executing the last reconfiguration with sync:

2>	store the successful handover information in VarSuccessHO-Report and determine the content in VarSuccessHO-Report as follows:

3>	clear the information included in VarSuccessHO-Report, if any;

3>	if the UE is not in SNPN access mode, set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e., includes the RPLMN);

3>	else if the UE is in SNPN access mode, set the snpn-IdentityList to include the list of equivalent SNPNs stored by the UE (i.e., including the registered SNPN identity), if available;

3>	for intra-NR handover, set the c-RNTI to the C-RNTI assigned by the target PCell of the handover;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, for the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied; or

3>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for the source PCell in which the last MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA was applied; or

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the mobility procedure was triggered by a condReconfigId in the MCG VarConditionalReconfig including only condExecutionCond while operating in dual connectivity mode and being also configured with another condReconfigId in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell:

4>	set the sourcePCellID in sourceCellInfo to the global cell identity and tracking area code, if available, of the source PCell;

<text omitted>

3>	if the procedure is triggered due to successful completion of reconfiguration with sync and if sourceDAPS-FailureReporting included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync is set to true, and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

4>	set sourceDAPS-Failure in shr-Cause to true;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the mobility procedure was triggered by a condReconfigId in the MCG VarConditionalReconfig including only condExecutionCond while operating in dual connectivity mode and being also configured with another condReconfigId in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell:

4>	set CHO-WithSCG-NotTrigerred in shr-Cause to true;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, for each of the measObjectNR, configured by the source PCell, in which the last RRCReconfiguration message including reconfigurationWithSync was applied;or:

<text omitted>

3>	if available, set the locationInfo as in 5.3.3.7;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the mobility procedure was triggered by a condReconfigId in the MCG VarConditionalReconfig including only condExecutionCond while operating in dual connectivity mode and being also configured with another condReconfigId in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in choWithCandidateSCGInfoList and set the values as follows:

4>	set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

4>	set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

1>	release successHO-Config configured by the source PCell and thresholdPercentageT304 if configured by the target PCell.

The UE may discard the successful handover information, i.e., release the UE variable VarSuccessHO-Report, 48 hours after the last successful handover information is added to the VarSuccessHO-Report.

Next changes

#### –	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

Signalling radio bearer: SRB1 or SRB2 (when logged measurement information is included)

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

UEInformationResponse message

-- ASN1START

-- TAG-UEINFORMATIONRESPONSE-START

<text omitted>

SHR-Cause-r17 ::=                    SEQUENCE {

t304-cause-r17                       ENUMERATED {true}                                       OPTIONAL,

t310-cause-r17                       ENUMERATED {true}                                       OPTIONAL,

t312-cause-r17                       ENUMERATED {true}                                       OPTIONAL,

sourceDAPS-Failure-r17               ENUMERATED {true}                                       OPTIONAL,

...,

[[

CHO-WithSCG-NotTrigerred-r19         ENUMERATED {true}                                       OPTIONAL

]]

}

<text omitted>

-- TAG-UEINFORMATIONRESPONSE-STOP

-- ASN1STOP

Next changes

#### –	OtherConfig

The IE OtherConfig contains configuration related to miscellaneous other configurations.

OtherConfig information element

-- ASN1START

-- TAG-OTHERCONFIG-START

<text omitted>

SuccessHO-Config-r17 ::=                SEQUENCE {

thresholdPercentageT304-r17             ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT310-r17             ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT312-r17             ENUMERATED {p20, p40, p60, p80, spare4, spare3, spare2, spare1}         OPTIONAL, --Need R

sourceDAPS-FailureReporting-r17         ENUMERATED {true}                                                       OPTIONAL, --Need R

...,

[[

CHOwithCandidateSCG-Check-r19           ENUMERATED {true}                                                       OPTIONAL, --Need R

]]

}

<text omitted>

-- TAG-OTHERCONFIG-STOP

-- ASN1STOP

End of changes