3GPP TSG RAN WG2 Meeting #131	 R2-2505727

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.2.2

Source: 	ITL

Title: 	Remaining issues on A-IoT paging

Document for:	Discussion and decision

Introduction

In the previous RAN2 meeting [1], agreements related to A-IoT paging are as follows:

Agreements on parallel service request

1	Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation.   Capture this in stage 2 specification.

2	The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device.  Capture this in stage 3 specification.

3	Send LS to RAN3 to notify them of agreements 1 and 2

4	Parallel service request for overlapping reader scenario can be addressed in Rel-20

Agreements on paging

1.	For CFRA, as a baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent.  FFS on the need for the transaction ID for command case.

2.	For CFRA, the device always responds to paging regardless of transaction ID (if we put a transaction ID) (i.e. as long as it is addressed to the corresponding device).

3.	To ensure forward compatibility for paging with multiple identifiers, introduce at least one R field.   FFS if more than one R bit is required.

4.	Rel-19 devices would ignore the content of future release instead of ignoring the whole paging message.

5.	Issue (1-4) For number of access occasions introduce exponential way, 4 bits, value range FFS

This paper presents our views on the A-IoT paging remaining issues that were not addressed in the previous meeting.

Discussion

Transaction ID length

In previous discussions, the primary reason for considering a longer Transaction ID or specifying its generation method was to handle potential multi-reader scenarios. During the last meeting, it was agreed that avoiding parallel service requests to a device should be handled by network implementation. This also implies that any potential Transaction ID collisions could be managed at the network side if needed.

Based on this understanding, there is no need for RAN2 to specify how the Transaction ID should be generated, as this can be left to implementation without impacting the specification.

Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation.   Capture this in stage 2 specification

In the last meeting, the following candidate Transaction ID lengths were proposed: 2 bits, 3 bits, 4 bits, and 6 bits. Preferences vary based on the trade-off between signaling overhead (shorter IDs) and flexibility/extensibility (longer IDs).

- 2 bits: Minimal overhead, short ID preferred.

- Minimal overhead: Maximizes efficiency in paging messages. Sufficient under current assumptions: With the network preventing parallel service requests, the device only needs to distinguish “new request” vs. “retransmission”. Simplified state handling: Short reuse cycles can be managed by network coordination

- Low differentiation capability: Insufficient for simultaneously representing different readers, services, and delta paging status, Limited future scalability: Risk of collisions in more complex Rel-20 environments.

- 3 bits: Minimal length with basic differentiation capability.

- Small overhead increase: Doubles the value space compared to 2 bits while still being compact. Basic combinability: Allows simple schemes such as 2 bits for reader/service + 1 bit for delta indicator.

- Tight margin: Could be insufficient in denser or more complex multi-reader/service situations.

- 4 bits: Balance between signaling overhead and flexibility, seen as sufficient for Rel-19 and adaptable for future needs.

- Balanced trade-off: Offers reasonable flexibility while keeping overhead low. Operational simplicity: Works well under the assumption of a 1:1 mapping between TID and Correlation ID.

- Less headroom for complex cases: Might be constrained if multiple dimensions (reader, service, delta) must all be encoded independently.

- 6 bits: Greater differentiation capability, beneficial for multi-reader/multi-service scenarios and potential Rel-20 extensions.

- Higher overhead: Slightly larger paging message size and potential power impact. Policy complexity: More bits require consistent reuse/release rules (AS/Transaction ID release handling).

We prefer a 4-bit Transaction ID as it provides a balanced trade-off between signaling overhead and flexibility.

Proposal 1: Introduce 4-bit Transaction ID in paging message

Transaction ID for CFRA

In last meeting discussion, it was concluded that, for CFRA, the paging message does not need to include the transaction ID, the indication of whether a paging ID is present, and the number of access occasions. However, some companies expressed the view that the transaction ID might be needed in the case of a command.

In the current RAN2 baseline and the running CR, the CFRA paging message does not include the Transaction ID, the indication of paging ID present/absent, and the number of access occasions.
In the last meeting, it was noted that some companies raised a potential need for the Transaction ID in the case of a command, but there was no time to elaborate the use case, and the topic was left for further study.

For CFRA, as a baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent.  FFS on the need for the transaction ID for command case.

We support maintaining the current baseline for command case as following reasons:

- Nature of CFRA Procedure: In CFRA, a device should respond to every paging message it receives, regardless of whether it is for a command or inventory. The Transaction ID does not affect the device’s decision to respond in CFRA.

- Parallel CFRA/CBRA Scenario Not Typical: We think running CFRA and CBRA in parallel for the same service request as atypical and avoidable by reader implementation.

Therefore, unless additional issues are identified, we do not prefer presenting a Transaction ID in the paging message for the command case.

Proposal 2: For CFRA, It is not necessary to present a Transaction ID in the paging message for the command case

Paging ID length

In previous RAN2 discussions, some companies considered reducing the size of the paging ID length field by indicating a “type” instead of the number of bits. However, after reviewing the CT4 and SA2 reply LS in C4-252466 and S2-2505793, it became clear that:

- The format of the device ID and filtering information defined by CT4 is highly flexible, supporting bit-level granularity and potentially uncountable variations.

- As per CT4 agreed CR C4-252464, the filtering information can be up to 256 bits in length, and is not necessarily byte-aligned.

Necessity of a Length Indicator: Deriving the paging ID length from the total message size is unreliable, since devices may encounter extended message formats in later releases and filtering information can be non-byte-aligned. Therefore, a dedicated field indicating length in bits is still necessary.

Based on the current SA2/CT4 agreements, adopting an 8-bit field (in unit of bit) to indicate paging ID length is the most practical and compatible choice for Rel-19. This solution balances flexibility with interoperability, and ensures readiness for future extension.

Proposal 3: RAN2 adopts an 8-bit field, in units of bit, to indicate the Paging ID length, in alignment with the current SA2/CT4 conclusion

Conclusion

In this contribution, we discussed the A-IoT paging. According to discussion in section 2, we have the following proposals:

Proposal 1: Introduce 4-bit Transaction ID in paging message

Proposal 2: For CFRA, It is not necessary to present a Transaction ID in the paging message for the command case

Proposal 3: RAN2 adopts an 8-bit field, in units of bit, to indicate the Paging ID length, in alignment with the current SA2/CT4 conclusion

Reference

RAN2#130 meeting report