3GPP TSG-RAN WG2 Meeting #131	R2-2505751

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.11.2

Source:	Nokia

Title:	Random Access Operation of SBFD

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# SBFD Random Access Operation

For initial access, the UE selects the index of the preferred SSB index and decodes the associated PBCH for MIB, and obtains relevant information for decoding the SIB1, from where it obtains the RACH configuration for sending Msg1. The RACH configuration contains, among others, the PRACH configuration index, the frequency domain multiplexing of the RACH occasions (ROs), the frequency offset of the ROs, and information about the power ramp up during the RACH procedure. For UEs in RRC_CONNECTED, dedicated RACH configuration, including CFRA, can be indicated by the network. If the CFRA configuration is missing, UEs in RRC_CONNECTED use the common RACH configuration for CBRA.

To enable support of initial access in SBFD, i.e., in the UL subband, RAN1 has been discussing the above aspects of random access procedures in SBFD since RAN1#116. In fact, RAN1 observed that support of initial access in SBFD provides at least one or more following benefits: reduced random access latency, reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots, improved coverage of PRACH with sparse UL resources, and increased cell range of PRACH with sparse UL resources. As mentioned in section 1, WI was updated in RAN#104 [RP-241614] to specify SBFD operation to support random access in SBFD symbols by UEs in RRC_IDLE/INACTIVE mode, along with RRC_CONNECTED mode in RAN1 and RAN2.

# RRC and MAC open issues

In this contribution, we share our views on the RRC and MAC open issues discussed during [Post130][216][SBFD] Running CR for 38.331 (Huawei)  and  [Post130][217][SBFD] Running CR for 38.321 (Samsung)

## RRC Open Issue

### RO Type Indication for CBRA

During the during [Post130][216][SBFD] Running CR for 38.331 (Huawei), the following has been agreed by majority of the companies.

[Proposal for RRC-1] For the NT indicating RO type, use 1 bit signalling (as in the current RRC running CR) [8/13].

The main purpose of the network indication is to dynamically manage the distribution of UEs between additional and legacy ROs. Therefore, the network indication should support dynamic changes, based on the network load. If the NW  indicate a 1-bit value, then it would force all the SBFD UEs to use the indicated RO type (SBFD RO or legacy RO), which is not aligned with the load balancing goal. Meaning, what NW want to achieve is that the network indicate via broadcast signalling that a portion of the SBFD UEs to use a type of RO based on the network load condition.

Observation 1:  A 1-bit indication in RACH-ConfigCommon is not suitable for dynamically distributing  the RA attempts from the SBFD UEs in legacy and additional RO type.

Hence we propose:

Proposal 1.1: For CBRA RO type indication, that NW includes a value between 0 and 1 based on the NW load in RACH-ConfigCommon. FFS the granularity of the value generated (how many bits).

Proposal 1.2: Upon receiving the indicated value, the UE generate a uniformly distributed random number between 0 and 1(using pseudo random number generator algorithm), and if the generated value is less than the value indicated by the NW, the UEs use the SBFD RO for random access.

## MAC Open Issue

### RA-RNTI collision in SBFD

For the additional RACH configuration in option 2, designed for SBFD-aware UEs, a challenge is avoiding RA-RNTI confusion caused by having two sets of RACH resources. The RA-RNTI is used by a UE to receive the Random Access Response (RAR) once the PRACH is transmitted. Different RACH occasions within a system frame are assigned unique RA-RNTI values to ensure that the RAR for a given PRACH transmission is directed only to the UE that used that specific RACH occasion. If two RACH occasions share the same RA-RNTI value, there is a risk that two UEs using different RACH occasions could incorrectly act on the same RAR message.

More specifically, the UE calculates the RA-RANTI when sending the Msg 1 of the RA procedure. According to TS38.321, the RA-RNTI is computed as

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where,

- s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

- t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

- f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

- ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

For PRACH configuration Option 2 (i.e., an SBFD-aware UE is configured with a legacy and additional PRACH configurations), there is a possibility that the SBFD ROs configured by the additional configurations may overlapped with the legacy ROs. This could happen in flexible symbols (as both legacy and SBFD ROs can be configured in flexible symbols). When this overlapping happens, there is a non-negligible possibility that at least s_id and t_id of the overlapping legacy RO and SBFD RO are the same (and of course the ul-carrier_id is the same since we are talking about a same carrier).

Now the tricky part is the f_id. If the legacy specs is kept as is, then at the point when there is such overlapping, there is a mismatch between the frequency indices being used by legacy UEs and the one being used by SBFD UEs. Indeed, as shown in figure below, although there is no issue at the non-overlapping time instance, there exist the possibility that two independent legacy and SBFD ROs may end up having the same f_id. This leads to the same RA-RNTI and making confusion for these UE when receiving the same RA-RNTI. Please note that  contention resolution in the current spec (msg 4) handles the case when two or UEs use the same RO and preamble. Here RA-RNTI collision happens for the case when two different RO share the same RA-RNTI due to misinterpretation of the f_id.

More illustration is shown below

- SBFD f_id : is the frequency index of any RO seen by SBFD-aware UE (not the f_id of the SBFD ROs only).

- Legacy f_id: is the frequency index seen by legacy UE.

Observation 2: In PRACH configuration Option 2, RA-RNTI collision happens when the SBFD ROs configured by the additional configurations overlap with the legacy ROs in flexible symbols.

Proposal 2: RAN2 to confirm that RA-RNTI collision may happen when the SBFD ROs configured by the additional configurations overlap with the legacy ROs in flexible symbols.

Proposal 3:  When there is an overlap between the SBFD ROs and additional ROs in flexible symbols (RACH configuration Option 2), the SBFD UEs uses  s_id + 1 for the calculation of the RA-RNTI.

Please note that s_id range is 0-13, the s_id can be used max value is 12 in spec 38.211 table 6.3.3.2, then using s_id+1 will not exceed the spec range. So, the change will not impact the current formula.  Also, the minimum interval for multiple ROs is 2 symbols, and since the minimum PRACH duration requires 2 symbols. The minimum interval for S_ID is 2 symbols in one PCI, therefore, using s_id+1 will not conflict with the current spec.

### RO type switching from non-SBFD symbols to SBFD symbols.

In RAN2#129bis RAN2 made following agreement on initial attempt and re-attempt of the RACH procedure for an SBFD-aware UE.

RAN2#129bis Agreements RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

A successful RA may not only depend on the interference condition in the RO. A RA failure for SBFD UEs using the legacy symbols could be due to two main reasons:

- Case 1: The SBFD-aware UE did not receive an RAR response from the NW (Msg1 failed to be received by the NW). A failure in the RAR response is an indication of bad coverage of PRACH.

- Case 2: The SBFD-aware UE did not receive the contention resolution message (Msg4). A failure in the Msg4 response is an indication of a collision happening between different UEs.

  - Note that the probability of collision in the RACH procedure may depend at least on the number of users trying to perform random access on RACH resources. For SBFD, collision probability may be very different between SBFD ROs and legacy ROs considering only SBFD-aware (Rel-19 or later) UEs may be capable of transmitting in the SBFD ROs.

If the RA failure is due to Case 1, there is limited advantage in having SBFD-aware UEs switch to additional RACH Occasions (ROs), as the Cross-Link Interference (CLI) could be higher in those ROs. However, if the failure is due to Case 2, switching to additional ROs can be beneficial since these ROs are likely to be less congested than the legacy RO. The UEs are capable of distinguishing between these two failure scenarios. Therefore, we propose:

Proposal 4: The fallback from legacy RO to an additional RO in SBFD symbols is supported only when the contention resolution is not succeeded for the SBFD-aware UEs for its RA attempts in legacy RO.

The following agreements are made in RAN2#127bis.

RAN2#127Bis Agreements - For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

However, the agreed way of RO type switching has the issue mentioned below.

If a UE initially selects an SBFD RO, but after multiple attempts, the NW observes that the UL signal strength does not improve and is unlikely to change in the next several attempts (e.g., 5 attempts), or NW observes that there is preamble congestion in the selected RO type,  the NW realize that a legacy RO would provide a better chance of success. However, the current procedure requires the UE to exhaust all RA attempts in the SBFD RO (RRC configured value) before switching to the more suitable legacy RO.  This results in unnecessary latency, as the UE keeps retrying in an inefficient RO type before switching, even when the network has a better alternative available.

Proposal 5: Discuss whether to allow the NW to indicate to the SBFD UEs via RAR message to switch the RO type.

At the last meeting following agreement has been made for CFRA to CBRA fallback scenario.

SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

It was agreed that when CBRA resources are configured for the same RO type as CFRA, the SBFD-aware UE uses that RO type when performing CFRA to CBRA fallback. However, the UE behaviour in cases where such CBRA resources are absent has not been discussed. This leads to the question of whether an SBFD RO can be indicated for CFRA if it is not configured for CBRA, and, if so, whether the UE may switch from an SBFD RO in CFRA to a legacy RO in CBRA during fallback. Based on offline discussions, our view, aligned with Huawei and LGE, is that indicating an SBFD RO only in CFRA without a matching CBRA configuration is illogical and adds unnecessary complexity. A simpler approach is to indicate an SBFD RO in CFRA only when the same SBFD RO is also configured in CBRA.

Proposal 6: Agree to indicate SBFD RO in CFRA only if the same SBFD RO is configured in CBRA resources.

## Access to Legacy and Additional PreambleReceivedTargetPower for Msg3

In the RAN1#120bis meeting, we reached the following agreement:

Agreement in RAN1#120bis For determination of the Msg3 PUSCH transmission power for RACH configuration Option 2: - preambleReceivedTargetPower configured for legacy-RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols; - preambleReceivedTargetPower configured for additional-RO is used if Msg3 PUSCH is transmitted in SBFD symbols; FFS whether/how to support separate msg3-DeltaPreamble for non-SBFD symbols and SBFD-symbols."

Based on this agreement, the selection of preambleReceivedTargetPower is correlated with the symbol type scheduled for Msg3. However, for RACH configuration Option 2, an SBFD-aware UE can access only one of the RACH configuration instances—either legacy or additional—depending on the type of ROs selected for the PRACH. Each RACH configuration instance contains its own preambleReceivedTargetPower (legacy or additional).

As a result, an SBFD-aware UE that selects the SBFD ROs for PRACH transmission can access only the preambleReceivedTargetPower from the additional RACH Configuration. Consequently, if Msg3 is scheduled on non-SBFD symbols, the UE would require the legacy preambleReceivedTargetPower for Msg3, which is not accessible because it only has access to the additional configuration. To address this issue, we propose supporting both legacy and additional preambleReceivedTargetPower within each RACH configuration instance. This approach would allow an SBFD-aware UE to access both preambleReceivedTargetPower values, regardless of the symbol type selected for the PRACH.

Proposal 7: For RACH Configuration Option 2, support separate preambleReceivedTargetPower parameters for both legacy and additional RACH configuration.

# Conclusion

Observation 1:  A 1-bit indication in RACH-ConfigCommon is not suitable for dynamically distributing  the RA attempts from the SBFD UEs in legacy and additional RO type.

Observation 2: In PRACH configuration Option 2, RA-RNTI collision happens when the SBFD ROs configured by the additional configurations overlap with the legacy ROs in flexible symbols.

Proposal 1.1: For CBRA RO type indication, that NW includes a value between 0 and 1 based on the NW load in RACH-ConfigCommon. FFS the granularity of the value generated (how many bits).

Proposal 1.2: Upon receiving the indicated value, the UE generate a uniformly distributed random number between 0 and 1(using pseudo random number generator algorithm), and if the generated value is less than the value indicated by the NW, the UEs use the SBFD RO for random access.

Proposal 2: RAN2 to confirm that RA-RNTI collision may happen when the SBFD ROs configured by the additional configurations overlap with the legacy ROs in flexible symbols.

Proposal 3:  When there is an overlap between the SBFD ROs and additional ROs in flexible symbols (RACH configuration Option 2), the SBFD UEs uses  s_id + 1 for the calculation of the RA-RNTI.

Proposal 4: The fallback from legacy RO to an additional RO in SBFD symbols is supported only when the contention resolution is not succeeded for the SBFD-aware UEs for its RA attempts in legacy RO.

Proposal 5: Discuss whether to allow the NW to indicate to the SBFD UEs via RAR message to switch the RO type.

Proposal 6: Agree to indicate SBFD RO in CFRA only if the same SBFD RO is configured in CBRA resources.

Proposal 7: For RACH Configuration Option 2, support separate preambleReceivedTargetPower parameters for both legacy and additional RACH configuration.