3GPP TSG-RAN WG2 #131	R2-2505761

Bengaluru, India, August 25th– 29th, 2025

Agenda Item:	8.20.1 NR others (RAN4)

Source:	Ericsson

Title:	Discussion on ATG LS

Document for:	Discussion, Decision

# 1	Introduction

RAN2 has received LS in [1] with the below content:

1. Overall Description:

In ATG network, when a UE supporting antennaArrayType-r18 on the SCC measures a neighbour cell on SCC, there may be scheduling restrictions on the serving cell due to different Rx beam directions between serving cell and neighbour cell. RAN4 agreed to introduce a network signaling to indicate whether an ATG UE supporting antennaArrayType-r18 on the SCC shall perform the neighbour cell measurements or not on the frequency configured by servingcellMO for SCC, i.e. to indicate whether the UE is only required to measure the serving cell or both serving and neighbour cell on the frequency configured by servingCellMO for SCC.

RAN4 also agreed that the network signaling is based on RRC.

2. Actions:

To RAN2

ACTION: 	RAN4 kindly requests RAN2 to specify RRC based network signaling to indicate whether an ATG UE supporting antennaArrayType-r18  on the SCC shall perform the neighbour cell measurements or not on the frequency configured by servingcellMO for SCC.

# 2	Discussion

The signalling that is asked to be included in the RRC specification has two values, UE measures neighbour cells on a serving cell frequency or UE measures only the serving cell on that frequency. This parameter may be placed in the IE ServingCellConfig or in the IE MeasObjectNR as shown below:

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                                OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                                    OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                               OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink                         OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                                                                   OPTIONAL,   -- Cond SyncAndCellAdd

bwp-InactivityTimer                 ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30,

ms40,ms50, ms60, ms80,ms100, ms200,ms300, ms500,

ms750, ms1280, ms1920, ms2560, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1 }    OPTIONAL,   --Need R

defaultDownlinkBWP-Id               BWP-Id                                                                  OPTIONAL,   -- Need S

uplinkConfig                        UplinkConfig                                                            OPTIONAL,   -- Need M

supplementaryUplink                 UplinkConfig                                                            OPTIONAL,   -- Need M

pdcch-ServingCellConfig             SetupRelease { PDCCH-ServingCellConfig }                                OPTIONAL,   -- Need M

pdsch-ServingCellConfig             SetupRelease { PDSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

csi-MeasConfig                      SetupRelease { CSI-MeasConfig }                                         OPTIONAL,   -- Need M

sCellDeactivationTimer              ENUMERATED {ms20, ms40, ms80, ms160, ms200, ms240,

ms320, ms400, ms480, ms520, ms640, ms720,

ms840, ms1280, spare2,spare1}       OPTIONAL,   -- Cond ServingCellWithoutPUCCH

crossCarrierSchedulingConfig        CrossCarrierSchedulingConfig                                            OPTIONAL,   -- Need M

tag-Id                              TAG-Id,

dummy1                              ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

pathlossReferenceLinking            ENUMERATED {spCell, sCell}                                              OPTIONAL,   -- Cond SCellOnly

servingCellMO                       MeasObjectId                                                            OPTIONAL,   -- Cond MeasObject

...,

*OMITTED*

[[

srs-PosTx-Hopping-r18               SetupRelease { SRS-PosTx-Hopping-r18 }                                  OPTIONAL,   -- Need M

enablePL-RS-UpdateForType1CG-PUSCH-r18  ENUMERATED {enabled}                                                OPTIONAL,   -- Need R

powerBoostPi2BPSK-r18               BOOLEAN                                                                 OPTIONAL,   -- Need R

powerBoostQPSK-r18                  BOOLEAN                                                                 OPTIONAL    -- Need R

]],

[[

neigborCellMeas-r19          ENUMERATED{true}                               OPTIONAL    -- Need R

]]

}

*OMITTED*

-- TAG-SERVINGCELLCONFIG-STOP

-- ASN1STOP

ServingCellConfig field descriptions

neigborCellMeas Indicates that UE shall omit neigborcell measurements on this serving cell frequency, see TS XXXXX. Network configures this field only if UE supports antennaArrayType-r18.

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                          OPTIONAL,   -- Need R

nrofCSI-RS-ResourcesToAverage       INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)                   OPTIONAL,   -- Need R

quantityConfigIndex                 INTEGER (1..maxNrofQuantityConfig),

offsetMO                            Q-OffsetRangeList,

cellsToRemoveList                   PCI-List                                                        OPTIONAL,   -- Need N

cellsToAddModList                   CellsToAddModList                                               OPTIONAL,   -- Need N

excludedCellsToRemoveList           PCI-RangeIndexList                                              OPTIONAL,   -- Need N

excludedCellsToAddModList           SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

allowedCellsToRemoveList            PCI-RangeIndexList                                              OPTIONAL,   -- Need N

allowedCellsToAddModList            SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

...,

*omitted*

[[

measSequence-r18                    MeasSequence-r18                                                OPTIONAL,   -- Need R

cellsToAddModListExt-v1800          CellsToAddModListExt-v1800                                      OPTIONAL    -- Need N

]] ,

[[

neigborCellMeas-r19           ENUMERATED{true}                              OPTIONAL    -- Cond SCellMO

]]

}

*omitted*

-- TAG-MEASOBJECTNR-STOP

-- ASN1STOP

MeasObjectNR field descriptions

neigborCellMeas Indicates that UE shall omit neigborcell measurements on this serving cell frequency, see TS XXXXX. Network configures this field only if UE supports antennaArrayType-r18.

| Conditional Presence | Explanation |
| --- | --- |
| SCellMO | This field is optionallly present, Need R, in the measObject associated with a serving cell. Otherwise the field is absent. |

RAN2 to adopt the new parameter in measObjectNR.

# 5	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss whether to place the new parameter in servingcellConfig or measObjectNR.

# References

R4-2508329, LS on UE signaling design for NR ATG enh