3GPP RAN WG2 Meeting #131	R2-2505762

Bengaluru, India, August 25th – 29th 2025

Agenda Item:	8.1.2.2

Source:	InterDigital

Title:	Remaining open issues: LCM for UE-sided model for BM use case

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback.

A list of remaining open issues related to the LCM procedure has been provided as part of a post meeting TS 38.331 running CR review [2]. This contribution discusses these remaining issues, specifically those related to UE sided model for BM use case.

# Remaining Open Issues

The following issues and descriptions have been extracted from the Open issues list as part of the TS 38.331 running CR review.

## [RRC-44] Whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations

A current open issue is whether RRCReconfigurationComplete contains the (in)applicability of all inference configurations stored at the UE, or it contains only updates for the inference configuration(s) for which the UE already reported the applicability status in the past. Two interpretations have been described:

- The RRCReconfigurationComplete contains the applicable/inapplicable status for all inference configurations. Only UAI contains updates.

- The RRCReconfigurationComplete contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message and only applicability changes for those inference configurations received in earlier RRCReconfiguration messages, for which the UE already reported applicable/inapplicable at least once. UAI contains updates.

Interpretation 1 better aligns with previous agreements, where the UE reports the (in)applicability status of all configurations and in the initial report (via RRCReconfigurationComplete) and only those that have been changes during subsequent reporting (via UAI).

Proposal 1:	[RRC-44] Initial applicability reporting via RRCReconfigurationComplete contains the (in)applicable status for all inference configurations stored at the UE. Only UAI contains updates (i.e., support interpretation 1).

## [RRC-45] Clarification on activating periodic inference CSI-ReportConfig

RAN2 agreed that the UE should activate a periodic inference configuration in CSI-ReportConfig only if it reports that it is applicable. An incoming LS from RAN4 [R4-2508085] provides two interpretations of this agreement and seeks to clarify the intended meaning:

- the applicable functionalities are already activated before reporting applicable functionalities via RRCReconfigurationComplete; or

- UE starts to activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete.

The above behaviour is clearly agreed in RAN2#129 via the following:

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5

The common understanding should therefore be interpretation 2.

Proposal 2a:	[RRC-45] UE activates the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete (i.e., interpretation 2).

The Rapporteur further provides an example related to the periodic reporting, noting that it should be taken into account that the gNB cannot know whether the UE is sending L1 prediction reports or not until the NW receives successfully the applicability report from the UE. Until then, if the UE is allowed to multiplex UCI (with CSI inference predictions) in PUCCH or PUSCH, the NW will try to decode the multiplexed information. This will turn into decoding issues if the inference predictions are not sent by the UE and this will impact also other decoded information from the UE.

The Rapporteur suggests that a standardized behaviour is needed in order to avoid the gNB incurring in such ambiguity, e.g:

- the UE always sends a L1 report in UCI for periodic CSI-ReportConfig, even if the configuration is not active/applicable;

- the UE does not multiplex UCI containing inference predictions and specify that the configuration is activated only after finishing sending RRCReconfigurationComplete.

If the UE follows the behaviour of (a), the gNB cannot identify whether the UE reported dummy information unless there is a predefined information which is indicating that this is for not active/applicable CSI report configuration. Therefore, the preferred behaviour is (b), that the UE does not multiplex UCI containing inference predictions until the configuration is activated (i.e., UE has reported the functionality as applicable via RRCReconfigurationComplete).

Proposal 2b:	[RRC-45] UE does not multiplex UCI containing inference predictions until applicabile functionality is activated.

## [RRC-47] Implementing RAN1 parameters in CSI-ReportConfig

There are currently two options on how to implement RAN1 parameters in CSI-ReportConfig in ASN.1:

- List all parameters (for inference, performance monitoring, UE data collection) one after another without any hierarchy; or

- Group some of the parameters with a CHOICE structure, to clarify which parameters are allowed to be configured together. For instance, the latest version of the RRC running CR proposes a structure with a CHOICE branch for inference (and UE data collection) and a second one for performance monitoring.

Although both of the above would work, use of a CHOICE structure allows simplification by limiting the number of potential configuration combinations supported by the specification. As mentioned by the RRC rapporteur, one way to limit configuration without introducing too much restriction is to link one branch to inference and another for performance monitoring, which contain largely independent configurations. The current RRC implementation seems a reasonable baseline, and can be revisited if required by subsequent agreements.

Proposal 3:	[RRC-47] Group RAN1 parameters with a CHOICE structure (i.e., Option 2)

## [RRC-48] Applicability reporting for parameter sets after the full inference configuration is provided

Based on current agreements, the UE reports the applicability of both full inference configurations and corresponding inference related parameter sets, even if some of them may correspond to each other.  The issue was raised that once the network provides full inference configuration for applicable inference related parameter set, it would be highly inefficient if UE keeps reporting both full inference configuration ID and inference related parameter set config ID upon change in applicability.

If the network has provided a full inference configuration corresponding to a parameter set, and the UE maintains configuration IDs for both, a change in applicability status would impact both configurations. There is clearly no need to include both IDs within the subsequent applicability report. Although including either one (but not both) would work, the UE may simply indicate the full inference configuration ID when both are available.

Proposal 4:	[RRC-48] In a subsequent applicability report, UE only includes one configuration ID (e.g., the full inference configuration ID) if both a full inference configuration ID and corresponding parameter set configuration ID are available.

# Conclusion

In this contribution the following proposals are made regarding remaining open issues for LCM for beam management use case:

Proposal 1:	[RRC-44] Initial applicability reporting via RRCReconfigurationComplete contains the (in)applicable status for all inference configurations stored at the UE. Only UAI contains updates (i.e., support interpretation 1).

Proposal 2a:	[RRC-45] UE activates the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete (i.e., interpretation 2).

Proposal 2b:	[RRC-45] UE does not multiplex UCI containing inference predictions until applicable functionality is activated.

Proposal 3:	[RRC-47] Group RAN1 parameters with a CHOICE structure (i.e., Option 2)

Proposal 4:	[RRC-48] In a subsequent applicability report, UE only includes one configuration ID (e.g., the full inference configuration ID) if both a full inference configuration ID and corresponding parameter set configuration ID are available.

# References

RP-243244 – WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface – Qualcomm

Report from [POST129bis][016][AI PHY] 38.331 Running CR (Ericsson)