3GPP TSG-RAN2 Meeting #131	R2-2505819

Bangalore, India, Aug 25th – 29th, 2025

Agenda item:	8.2.4

Source:	Qualcomm Incorporated

Title:	Data Transmission and Other General Aspects of Ambient IoT

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

The Rel-19 Ambient IoT WI was approved in [1] and the RAN2 objectives were described as follows:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included. - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

RAN2 #129 made the following agreements regarding the A-IoT data transmission and general aspects in terms of AS ID, segmentation, MAC PDU format and etc.

Agreements - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - From device perspective, it is only required to use one AS ID. - CFRA is not supported for group ID - RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2

- Agreements on segmentation

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

- R2D segmentation is not supported for R19 A-IoT.

Agreements

From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”.

Agreements on MAC PDU format design - Aim to design simple MAC PDU format design - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages - At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - FFS whether for D2R we need message type field, any length and need for padding - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

RAN2 #129bis made the following agreements regarding the A-IoT data transmission.

Agreements

1	AS ID is applied for Inventory + command case;

2	AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed.

3	For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits.

4	Do not specify the reader behaviour on how exactly the ASID is generated.

5	The device releases the AS ID upon power off (no stage 3 specification impact);

6	The device only keeps one AS ID at a time.

7	For CFRA, command message is used for AS ID assignment

8	For CBRA, Msg 2 is used for AS ID assignment

9	The device releases the AS ID at least:

- upon receiving Paging with new transaction id for that device, i.e. different session/service

- when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA)

- FFS other cases for release ASID to avoid keeping it indefinitely.

Agreements on segmentation

- For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again.  FFS whether offset zero is always included.

- FFS whether the reader always includes the command for retransmission of segments.

- 1-bit indication is sufficient to indicate whether more D2R data will be sent

- For inventory response, RAN2 assumes that segmentation is not applied.  RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size.  Notify SA2 about this assumption.

Agreements on MAC PDU format

- The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte.  The actual TBS value depends on RAN1.   FFS for R2D trigger message

- RAN2 assumes that the upper layer data SDU is byte-aligned, and an LS can be sent to CT1.

- The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message

- The MAC padding is supported at least for D2R from RAN2 perspective.   The device includes padding bits if there is no more data and there is still space available in the TBS.

- In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required.  FFS how this is provided (i.e. SDU length field or padding length field).  The size of length field is FFS.

RAN2 #130 made the following agreements regarding the A-IoT data transmission.

Agreements 1 Use as baseline the following message names, field names and definitions are to be used in A-IoT MAC: − Message name: A-IoT Paging message, Access Trigger message, Random ID message, Random ID Response message, R2D Upper Layer Data Transfer message, D2R Upper Layer Data Transfer message. − Field name: R2D Message Type, RA Type, Indication of Paging ID Presence, Length of Paging ID, Paging ID, Transaction ID, Number of Access Occasions, D2R Scheduling Info, Random ID, Echoed Random ID, AS ID, Assigned AS ID, More Data Indication, SDU Length, MAC Padding, Received Data Size. − Definitions: o Access occasion: A time-frequency resource for device(s) to transmit Msg1 (i.e., the Random ID message) during a CBRA procedure. o AS ID: The AS layer identifier to address the specific device for R2D reception and D2R scheduling 2 One bit indication is needed for each echoed random ID in Msg2 to indicate whether AS ID is present (i.e., assigned by reader) for this random ID. 3 NACK feedback is defined as an explicit message (i.e. new message type). AS ID(s) is/are included to indicate the failure for given device(s). Multiplexing of NACK feedback is supported in one message 4 Assume two transport channels are introduced between A-IoT MAC and PHY. One is for R2D, and the other is for D2R. Neither logical channel concept nor SAP is defined for the interface between A-IoT MAC and upper layers.

Agreements

- R2D message scheduling non-first segment (re)transmission does not include upper layer command.

- For the first segment and unsegmented packet (re)transmission, the “offset” indicator in R2D is not present.

- This implies that the R2D message will either have command or offset (but not both).  FFS whether we define two message types or one message type with optional fields.

- Agreements

- The device is expected to send a MAC response to the reader in the D2R occasion.   The MAC response contains the NAS message if available at the D2R occasion.   If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed.

- Send LS to CT1 to inform the agreement 1 to CT1 and explain that we have an issue with delayed NAS write success response.   RAN2 would prefer that this is handled by CT1 (and give the example of sending NAS response upon successful reception of write command).    Ask if this can be handled by CT1

Agreement on MAC PDU format

- A mandatory length field directly indicates the length of D2R data MAC SDU to support varying lengths of D2R data.    The size of length field is 7-bit in bytes.

- The offset indication for transmission/retransmission of the segments after the first segment of a D2R message is 7-bit length in bytes.  Segmented SDUs are also byte aligned.

- FFS D2R message type.  Current running CR will capture no message type,  but we can revisit this next meeting and also consider if any other bits are needed for the MAC header

- The length field inside MAC for SDU is not needed for R2D messages, assuming R2D MAC padding is not needed.  FFS can come back if padding is needed depending on granularity of TBS  (only if needed)

Agreements

-	For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not.   FFS for CFRA

-	FFS for need for release message

In this paper, we will discuss the data communication and other aspects of Ambient IoT (A-IoT).

# R2D TBS length field

RAN1 sent LS [2] to inform RAN2 the following RAN1 agreements.

Agreement R2D postamble is specified with 4 ON chips corresponding to M value of the PRDCH - R2D postamble is added immediately after the PRDCH - R2D postamble has always 4 ON chips - Note: For M=24, 2 ON chips at the end of OFDM symbol for CP handling are in addition to R2D postamble, and are not part of the R2D postamble - R2D padding duration is determined after R2D postamble insertion TBS information for R2D is supported via higher layer R2D control signalling. - Send LS to RAN2 asking to include R2D TBS information (excluding CRC length) in higher layer signaling, at least for messages with variable size. Note: Exact method for determining the end of PRDCH at the device is not specified.

As requested by RAN1 in the LS, an R2D TBS field should be introduced after the R2D message type field for all R2D message with variable size (e.g., A-IoT paging message, Random ID Response message, R2D Upper Layer Data Transfer message, and NACK Feedback message). The length of R2D TBS field should be 7-bit which would be enough to indicate up to ~1000 bits size.

For R2D messages with variable length (e.g., A-IoT paging message, Random ID Response message, R2D Upper Layer Data Transfer message, and NACK Feedback message), an R2D TBS field is placed after the R2D message type field.

The length of R2D TBS field is 7-bit.

# D2R message type

One open issue is whether the D2R message type is introduced or not in Rel-19 A-IoT. In our view, without D2R message type will require the Reader to fully track the R2D message and D2R response without any exception case. The Reader has to rely on the triggered R2D message to detect the potential D2R response. This is obviously not a robust system design.

Considering the Rel-20 forward compatibility, not studying the coexistence of device 1 and active devise does not necessarily mean there is no possibility to have different types of devices in the same area.

Consider a future case where Rel-20 device legitimately responds to a Rel-20 reader (perhaps with a message Type header in Rel-20 or some other information without message Type header). However, for a nearby Rel-19 reader, that message is still a sequence of 0s and 1s, and in lack of a proper way to know whether it is a Rel-19 message or Rel-20 message, the Rel-19 reader may wrongly parse the reception (misinterpreting the fields) and whole operation can easily fail due to interference/misunderstanding.

On the other hand, if Rel-19 A-IoT devices unexpectedly responds to the command (targeted to Rel-20 devices), the Rel-20 reader may parse the first few bits according to Rel-20 definition (whatever that may be in the future) instead of discarding the wrongly received packet, it may cause interference and error to the whole system.

Thus, we prefer to introduce the D2R message type not only for the future proofing but also for the robust system design of Rel-19 A-IoT communication.

D2R message type should be added from Rel-19 for robustness and future compatibility.

For robustness and future compatibility, D2R message type is introduced in Rel-19.

# Delayed NAS feedback

The delayed NAS feedback issue was discussed in the RAN2#130. The following were agreed.

- The device is expected to send a MAC response to the reader in the D2R occasion.   The MAC response contains the NAS message if available at the D2R occasion.   If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed.

If the NAS response is not ready at the D2R occasion, besides the 0 SDU indication is sent from A-IoT device to Reader, it is beneficial to indicate an estimated time on when the NAS command response will be ready. The A-IoT device may estimate the execution time e.g., based on the writing speed to estimate when the NAS command response could be generated.

It can facilitate Reader to provide subsequent D2R resource on time. Otherwise, Reader may have no knowledge on when to provide the D2R occasion. Eventually it is up to Reader implementation to provide more subsequent resources based on the estimated time message from A-IoT device.

When there is no NAS response available to transmit at the D2R occasion, besides 0 SDU indication to Reader, A-IoT device sends to Reader an estimated time about when the NAS command response will be ready.

When no NAS response is available, besides 0 SDU indication, there is an open issue on how to set ‘more data indication’ in D2R Upper Layer Data Transfer message. In our view, the ‘more data indication’ field is introduced for segmentation case. The ‘0 SDU’ is already clear to indicate that the NAS response is not ready. The information sent from A-IoT Reader is already clear enough for Reader. If ‘more data indication’ is set to 1, Reader may assume more data are in A-IoT device and will immediately send the D2R resource. However, NAS response message may still not be ready.

When there is no NAS response available, A-IoT device sends 0 SDU indication and set ‘more data indication’ field to ‘0’.

# Overall Protocol stack

During the SI phase, it was agreed that based on the targeted lower complexity and cost of A-IoT devices, traditional Control Plane functions like RRC connection management, RRM L3 measurement reporting, ASN.1 encoding/decoding, legacy periodical SIB/MIB, legacy paging message, legacy RRC layer, legacy DRX, tracking area update from the device etc. will not be supported by A-IoT. Subsequently, RAN2#128 agreed:

=> RAN2 assumes that there is no other L2 AS layer (i.e. A-IoT MAC layer only).   There is no CP/UP protocol stack differentiation on A-IoT interfaces.

Subsequently, the current draft stage-2 running CR [3] captures the following figure which is applicable to both A-IoT CP and UP (i.e. no differentiation between CP and UP protocol stacks).

Figure 1 A-IoT protocol stack (in current stage-2 running CR [3])

As some companies commented during the CR review process, there has been no explicit RAN2 agreement about the protocol layer above A-IoT MAC. We tend to agree with other companies which suggested that a more comprehensive figure should be captured in the stage-2 specifications. So, we propose the below figure replace the current figure in the stage-2 CR.

Figure 2 A-IoT protocol stack (to update the Figure 16.x.3-1 in the stage-2 running CR [3])

Update Figure 16.x.3-1 in the current stage-2 running CR to include the AOTF as well as A-IoT NAS layer (take Figure 2 from R2-2505819 as baseline).

# Conclusion

Based on the discussion above, we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1:	For R2D messages with variable length (e.g., A-IoT paging message, Random ID Response message, R2D Upper Layer Data Transfer message, and NACK Feedback message), an R2D TBS field is placed after the R2D message type field.

Proposal 2:	The length of R2D TBS field is 7-bit.

Proposal 3:	For robustness and future compatibility, D2R message type is introduced in Rel-19.

Proposal 4:	When there is no NAS response available to transmit at the D2R occasion, besides 0 SDU indication to Reader, A-IoT device sends to Reader an estimated time about when the NAS command response will be ready.

Proposal 5:	When there is no NAS response available, A-IoT device sends 0 SDU indication and set ‘more data indication’ field to ‘0’.

Proposal 6:	Update Figure 16.x.3-1 in the current stage-2 running CR to include the AOTF as well as A-IoT NAS layer (take Figure 2 from R2-2505819 as baseline).

# References

- RP-250796, Solutions for Ambient IoT (Internet of Things) in NR.

- R1-2504915, LS on Ambient IoT R2D control information, CMCC

- R2-2504398, Stage-2 38.300 Running CR for Ambient IoT, CMCC