3GPP TSG-RAN WG2 Meeting #131 	R2-2505851

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.2.4

Source:	Panasonic

Title:	AS ID release upon receiving paging message

Document for:	Discussion

# Introduction

This paper addresses the issues related to the following agreement that we made in previous meeting, RAN2#130.

Agreements (RAN2#130)

-	For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not.   FFS for CFRA

-	FFS for need for release message

We will explain why this agreement, especially the yellow highlighted part, can create problems and the possible ways to solve the problems.

# Discussion

The agreement as shown in Section 1 is an extension of the following agreement in RAN2#129bis.

Agreements (RAN2#129bis) - The device releases the AS ID at least: - upon receiving Paging with new transaction id for that device, i.e. different session/service - when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA)

As seen, the earlier agreement says that the device releases AS ID upon receiving paging with new transaction ID for that device. However, in later meeting RAN2#130, some companies pointed out an issue that reader may not know which device(s) are actually selected by the paging identification information provided by CN. Therefore, reader cannot know whether the overlapped device(s) are selected by the old paging (i.e. with old transaction ID) and new paging (i.e. with new transaction ID). Consequently, reader cannot know which previously assigned AS ID has actually been released by device(s), which makes the earlier agreement useless from reader AS ID management point of view. With this understanding, the new agreement is made in a way that receiving a paging with a new transaction ID is used to indicate “all devices” an end of procedure and to release AS ID (not only the targeted devices in paging message).

However, the new agreement only works under the assumption that no parallel services are requested from reader, meaning that it will not happen that device receives a new paging request while the procedure associated with old paging is still ongoing. While this assumption seems feasible in Rel-19, it cannot be guaranteed in Rel-20 D2T2 where UE is a reader. When neighboring UE readers perform A-IoT operation independently, device may receive (e.g. overhear) paging messages for other devices triggered by neighboring UE readers which it is still in the middle of service round. If the new agreement is followed, device would mistakenly flush the AS ID, resulting in communication/service failure with its serving reader.

There could be one misunderstanding that Rel-20 will support only active devices (Device 2b/C) but not Rel-19 devices (i.e. Device 1), due to the note in WID “Coexistence between Device 1 and Device 2b/C is not considered in the same deployment in the same band”. However, this is not true. The note just means no coexistence. It is clearly captured in WID that Device 1 needs to be supported for D2T2. See the following Objective 2 from WID:

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - (rest part is omitted here)

Therefore, while we are finalizing the specification for Device 1 behavior in Rel-19, we need to consider the fact that this type of devices should be able to work properly in D2T2 in Rel-20 deployment as well.

Therefore, our first proposal is to revert the new agreement back to the previous agreement in RAN2#129bis.

Observation 1. If device releases AS ID upon receiving paging message with different transaction ID no matter the paging message is for it or not, device could mistakenly flush its AS ID in Rel-20 D2T2 due to pagings from neighbouring readers.

Proposal 1. The device releases the AS ID upon receiving paging message with different transaction ID no matter the paging message is for it or not for that device. (revert agreement in RAN2#130)

On the other hand, we do agree on the issue of AS ID release misalignment between reader and device as pointed out by companies in the previous meeting. But we should solve the problem by another way. Logically speaking, since AS ID is assigned by reader, reader should be able to release the AS ID(s) of target device(s) which are defined or understood by the reader. In the sense, an explicit release indication (1-bit) with associated AS ID(s) would be a cleaner solution. With this method, reader can unambiguously indicate which AS ID to release. This explicit release indication with target AS ID can be a new message type, or carried by existing R2D messages.

Proposal 2. If proposal 1 is agreed, RAN2 consider to introduce explicit release indication (1-bit) with target AS ID(s) to be released. FFS a message type or enhance existing R2D message with optional field for explicit release indication.

Furthermore, in some case, reader may want to recycle all assigned AS IDs. Relying on Proposal 2 to indicate each AS ID would be inefficient. In this case, using broadcast message without specific AS ID(s) becomes useful. One may argue that the extended agreement in RAN#130 (see Section 1) serves exactly this purpose. However, the root cause why it cannot work properly in multi-reader scenario is that it relies on a “new transaction ID” as “proxy” for release indication. This can work in single or isolated reader scenario. But in multi-reader overlapping scenario, the “new transaction ID” can come from another reader but the device is not aware of. Therefore, the unintended release occurs. To correct this, the release operation should be triggered by an explicit indication (such as a flag) associated with the current transaction ID where the AS ID has been assigned. Since only the reader that initiated the service and assigned the AS ID can generate the same transaction ID for releasing, interference from neighboring reader can be avoided. Here it is assumed that probability of transaction ID collision from different readers can be ignored by proper size of transaction ID.

Proposal 3. If proposal 1 is agreed, RAN2 consider to introduce explicit release indication (1-bit) with the target transaction ID to release all assigned AS ID in this transaction. FFS a message type or enhance existing R2D message with optional field for explicit release indication.

# Conclusion

In this document, we have explained issues related to previous meeting agreement and the possible ways to solve the problems. The following are observations and proposals.

Observation 1. If device releases AS ID upon receiving paging message with different transaction ID no matter the paging message is for it or not, device could mistakenly flush its AS ID in Rel-20 D2T2 due to pagings from neighbouring readers.

Proposal 1. The device releases the AS ID upon receiving paging message with different transaction ID no matter the paging message is for it or not for that device. (revert agreement in RAN2#130)

Proposal 2. If proposal 1 is agreed, RAN2 consider to introduce explicit release indication (1-bit) with target AS ID(s) to be released. FFS a message type or enhance existing R2D message with optional field for explicit release indication.

Proposal 3. If proposal 1 is agreed, RAN2 consider to introduce explicit release indication (1-bit) with the target transaction ID to release all assigned AS ID in this transaction. FFS a message type or enhance existing R2D message with optional field for explicit release indication.