3GPP TSG-RAN WG2 Meeting #131 	R2-2505852

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.2.2

Source:	Panasonic

Title:	Paging consideration for forward compatibility of Device 1

Document for:	Discussion

# Introduction

The scope of Rel-19 A-IoT standardization has been limited to Device type 1 (having ~1 μW power consumption, and with D2R transmission by backscattering), and the usage is limited in D1T1 (i.e. by direct connection between a BS reader and A-IoT device). See [1] for more details.

Recently, RAN plenary has approved for Rel-20 a new work item on A-IoT phase 2, where active devices (i.e. Device 2b and Device C) will be considered. Another enhancement is to support D2T2 with intermediate UE as Reader in Rel-20 (see [2] for WID).

There could be one misunderstanding that Rel-20 will support only active devices (Device 2b/C) but not Device 1, due to the note in WID “Coexistence between Device 1 and Device 2b/C is not considered in the same deployment in the same band”. However, this is not true. The note just means no coexistence. It is clearly captured in WID that Device 1 needs to be supported for D2T2. See the following Objective 2 from WID:

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - (rest part is omitted here)

Observation 1. Device 1 needs to be supported in Rel-20 D2T2.

Therefore, while we are finalizing the specification for Device 1 behavior in Rel-19, we need to consider the fact that this type of devices should be able to work properly in D2T2 in Rel-20 deployment as well, i.e. being forward compatible.

In this paper, we discuss the impact on paging message by considering the forward compatibility of Device 1.

# Discussion

In Re-19 previous discussion, RAN2 group has made a very important assumption that Rel-19 device is not expected to receive parallel service requests, meaning that it will not happen that device receives a new paging request while the procedure is still ongoing. See below agreement from RAN#130.

Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in RAN2 stage 2 specification.

Since Rel-19 deployment considers only D1T1 where reader is fixed base station, it is feasible to place BS reader well separated in order to ensure no overlapping multi-reader scenario. However, Rel-20 deployment supports D2T2 where UE is a reader. Since UE reader can be moving around, it could happen that device is within coverage of multiple readers. When neighboring UE readers perform A-IoT operation independently, device may receive parallel service requests from different UE readers. This could lead to unintended interruption due to the following device behavior agreed in previous meeting.

The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in RAN2 stage 3 specification.

In other words, the above agreed device behavior is not forward compatible. We suggest to revisit this agreement in this meeting.

Observation 2. If Device 1 always responds to the new service indicated by the received paging message applicable for that device, it cannot work properly in Rel-20 D2T2.

Our solution is to introduce one control bit in the paging message to indicate whether device should respond to new paging (i.e. with a new transaction ID) and abandon the ongoing procedure (i.e. device is forced to terminate); or ignore the new paging and continue with ongoing procedure (i.e. device is not forced to terminate). In this sense, this control bit can be interpreted as “forced termination” indication. With such mechanism, for Rel-19 deployment, this “forced termination” bit can be set to true, then the above agreed device behavior in Rel-19 can be preserved. On the other hand, for Rel-20 D2T2 deployment, UE reader can set this “forced termination” bit to false in order not to interrupt devices unnecessarily.

To summarize, we propose the following.

Proposal 1. Paging message includes one-bit “forced termination” indication.

- When indicated as True, target device should respond to the paging with a new transaction ID, regardless of whether there is procedure related to previous transaction ID ongoing or not

- When indicated as False, target device should ignore the paging with a new transaction ID, if there is procedure related to previous transaction ID still ongoing.

One question related to the above proposal is how device determines whether a procedure is ongoing or already ended. This has been discussed intensively and progressed a lot in the previous meeting. More specifically, as agreed in previous meeting for CBRA, after sending Msg1, device has a time boundary (exact definition of the boundary needs to be downselected from 2 options, see agreement below) to receive Msg2. With this time boundary, device is monitoring Msg2 and therefore naturally assumes that the procedure is still ongoing. Then after sending Msg3, there is also a clear defined time window for device to monitor NACK, which is time until the subsequent paging or command addressed to this device. Until then (or until a NACK is received), the procedure is considered as ongoing. See the following agreements.

A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. - For option C, further discuss in terms of complexity at the device vs reader flexibility. After MSG3 transmission, upon receiving NACK with its AS ID before subsequent paging or command addressed to this device from the reader, device determines it will perform re-access. FFS how to specify.

For CFRA, the case is even simpler. RAN2 agreed in some meeting ago that for CFRA, NACK feedback and re-access is not support. Consequently, device can assume the procedure ends successfully after sending first D2R transmission (i.e. response to paging). Note that since device still needs to monitor R2D (as long as device has power), device would not miss the possible follow-up command from the reader even if device has assumed the procedure has ended.

To summarize, we have the following observations on definition of ongoing procedure.

Observation 3. For CBRA, after sending Msg1 in response to an A-IoT paging, device assumes the procedure associated with this paging is ongoing, until one of the following events occurs:

      - Msg2 is not received within a time boundary, in which case the procedure is ended as failure

      - After sending Msg3 (i.e. inventory response),

        - Device receives NACK for re-access determination until next paging or trigger message, in which case the procedure is concluded as failure

        - No R2D is received until next paging or command message addressed to this device, in which case the procedure is concluded as successful

Observation 4. For CFRA, after sending first D2R data transmission, device assumes that the procedure associated with this paging ends.

# Conclusion

In this paper, we have discussed the impact on paging message by considering the forward compatibility of Device 1. The following are observations and proposals.

Observation 1. Device 1 needs to be supported in Rel-20 D2T2.

Observation 2. If Device 1 always responds to the new service indicated by the received paging message applicable for that device, it cannot work properly in Rel-20 D2T2.

Observation 3. For CBRA, after sending Msg1 in response to an A-IoT paging, device assumes the procedure associated with this paging is ongoing, until one of the following events occurs:

      - Msg2 is not received within a time boundary, in which case the procedure is ended as failure

      - After sending Msg3 (i.e. inventory response),

        - Device receives NACK for re-access determination until next paging or trigger message, in which case the procedure is concluded as failure

        - No R2D is received until next paging or command message addressed to this device, in which case the procedure is concluded as successful

Observation 4. For CFRA, after sending first D2R data transmission, device assumes that the procedure associated with this paging ends.

Proposal 1. Paging message includes one-bit “forced termination” indication.

- When indicated as True, target device should respond to the paging with a new transaction ID, regardless of whether there is procedure related to previous transaction ID ongoing or not

- When indicated as False, target device should ignore the paging with a new transaction ID, if there is procedure related to previous transaction ID still ongoing.

# Reference

[1] RP-250796, Revised WID for A-IoT (Rel-19)

[2] RP-251885, Rel-20 A-IoT phase 2 WID