3GPP TSG-RAN2 Meeting #131	R2-2505856

Bengaluru, India, 25 - 29 August 2025	Revision of R2-2504288

Agenda Item:	8.4.2 Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Source: 	Ericsson

Title:  	LP-WUS in Idle and Inactive

Document for:	Discussion and Decision

# Introduction

In this contribution the open issues for LP-WUS in Idle and Inactive are discussed further.

For the RAN1 and RAN2 agreements in previous meetings the reader is referred to the Annex.

# Discussion

## LP-WUS in lastUsedCellOnly (38304-8)

The option to restrict the use of LP-WUS to the last used cell was discussed during RAN2#130, but no agreement was reached, i.e. it is noted as open issue 38304-8.

In SIB1 PEI can be configured to be used all the time or PEI can be restricted to be used in the last used cell only (aka the cell where the UE was last in RRC_CONNECTED):

DownlinkConfigCommonSIB ::=     SEQUENCE {

…

pei-Config-r17                  PEI-Config-r17                           OPTIONAL,  -- Need R

…

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}              OPTIONAL,  -- Need R

...

}

DownlinkConfigCommonSIB field descriptions

lastUsedCellOnly When present, the field indicates that the UE monitors PEI only if the latest received RRCRelease without noLastCellUpdate is from this cell. A PEI-capable UE stores its last used cell information.

In case the lastUsedCellOnly flag is set the UE only monitors PEI when it is camped on the cell where it was released from RRC_CONNECTED the last time. In case the flag is not set the UE monitors PEI in every cell where PEI is configured.

There can be cases where the UE and CN disagree about the last used cell, e.g. when there was SDT without UE context relocation or when security was not established on NGAP interface. In such case the gNB sets noLastCellUpdate in the RRCRelease message, i.e. the last visited cell info is not updated in the CN nor in the UE.

When lastUsedCellOnly is configured then the LP-WUS transmissions are limited to the first paging attempt of the NW, i.e. LP-WUS is not transmitted during paging escalation when the UE did not respond to the first paging attempt. During paging escalation the NW may page in all the cells of the registration / RNA area to reach the UE that apparently has been moving. In case LP-WUS is used during paging escalation then the LP-WUS overhead is significant because LP-WUS may be transmitted in hundreds of cells. But also from a UE perspective there is a negative impact because all the UEs belonging to the same subgroup ID as the UE being paged need to wake-up when the UE is paged in the complete registration area during the second paging attempt. LP-WUS supports more subgroups (31) compared to PEI (8) and the false alarm probability in LP-WUS is expected to be lower compared to PEI. But this depends on the number of subgroups that is configured, and the false alarm probability may be small but it will not be zero.

More importantly, there is a power saving benefit from a UE perspective when noLastCellUpdate is set. For discussion sake: stationary UEs are UEs camped on their last visited cell and moving UEs are not camped on their last visited cell:

- Cell A is the cell where the UE is expected to be, i.e. it is the last visited cell of the UE being paged. When lastUsedCellOnly is set then LP-WUS is only used in cell A when paging the UE. In the first paging attempt:

  - Moving UEs camped on cell A will not wake-up during this first paging attempt.

  - Stationary UEs camped on cell A will wake up if they belong to the same subgroup ID as the UE being paged (false alarm).

- In case the UE does not reply to the first paging attempt, paging is escalated to a larger area, e.g. the UE is paged in the complete registration area to limit the paging delay (it is up to NW implementation to page in cell A again). During paging escalation:

  - Stationary UEs do not wake up during paging escalation because LP-WUS is not used.

  - Moving UEs wake up when they belong to the same subgroup ID as the UE being paged.

- In practice UEs are found with a high probability (e.g. 95%) on its last used cell (or surrounding cells), i.e. UEs are not that mobile. Thus, there is not a great power saving gain when LP-WUS is also used during mobility. Instead, there is an increased power consumption because all the UEs that have the same subgroup ID as the UE being paged need during paging escalation.

In case the NW has the strategy to immediately pages the UE in the complete registration area, with the purpose to limit the paging latency (e.g. for voice calls), then:

In case the NW pages the UE in the complete registration area immediately in the first paging attempt, then there is a huge increase in the number of LP-WUS transmissions and a huge increase in the number of wake-ups for the UE, even when the false alarm probability is lower compared to PEI because more subgroups are used.

To enable flexibility in how LP-WUS is used and limit the system impact during paging escalation:

Similar as with PEI LP-WUS can be configured to be used in lastUsedCellOnly or in any cell where LP-WUS is configured.

## LP-WUS subgroup and eDRX (38304-9)

For PEI it is specified:

In RRC_INACTIVE state with CN configured PTW the SubgroupID used outside CN PTW is the same as the SubgroupID used inside CN PTW.

The reason for this is the following (R2-2305910):

- In RRC_INACTIVE the UE monitors CM-IDLE paging and RAN paging to detect potential RRC state mismatch. When the UE receives CM-IDLE paging the UE goes to RRC-IDLE to resolve the RRC state mismatch.

- The UE may be configured with eDRX in both RRC_IDLE and RRC_INACTIVE, but with a different eDRX cycle and PTW length. Thus in some cases the PTWRRC_IDLE and PTWRRC_INACTIVE overlap and in some cases do not overlap. The UE uses the same subgroup ID in RRC_IDLE and RRC_INACTIVE, i.e. the subgroup ID in RRC_INACTIVE is determined based on the DRX cycle in RRC_IDLE.

- The UE in RRC_INACTIVE monitors CM-IDLE and RAN paging during PTWRRC_INACTIVE. Thus when the UE is monitoring paging in RRC_INACTIVE the UE may be inside or outside the PTW used in RRC_IDLE. It is good to clarify that the subgroup ID monitored outside the PTWRRC_IDLE in RRC_INACTIVE is the same as the subgroup ID monitored inside PTWRRC_IDLE.

Therefore proposal 6 in R2-2504677 can be confirmed:

In RRC_INACTIVE state with CN configured PTW the LP-WUS SubgroupID used outside CN PTW is the same as the LP-WUS SubgroupID used inside CN PTW.

## LP-WUS and SDT (38304-10)

An open issue (38304-10) was defined during the post email discussion #211:

- FFS whether/how LP-WUS with SDT is supported

- Note: the open issue was discussed online in RAN2#130 (R2-2504264) without conclusion.

When SDT is ongoing (T319a is running) the UE does not monitor Paging PDSCH but the UE is monitoring Paging PDCCH:

LP-WUS monitoring during SDT can be used detect ETWS/CMAS indication or SI-change.

In case SDT is used frequently, then the power consumption in RRC_INACTIVE is mainly determined by the UL transmissions during SDT. In such case using LP-WUS during SDT is not going to lower the power consumption significantly.

In case SDT is used rarely then LP-WUS monitoring during SDT also does not reduce the power consumption significantly, because this event only happens rarely:

Monitoring LP-WUS during SDT is not critical from a power saving perspective.

However SDT can impact the latency significantly:

Increased latency for UL data

When LP-WUS is used during MO-SDT, then the latency for UL data is increased, because the MR may be deactivated when the UE wants to initiate SDT, i.e. when UL data arrives. Dependent on the minimum wake-up delay of the MR and the configured time offset, LP-WUS introduces a considerable additional delay (e.g. up to 900 ms).

There is an additional measurement latency when UL data arrives and the MR is in ultra-deep sleep. In that case, after the MR has woken up, MR has to start measurements from scratch. The MR measurements have to be filtered using 2 measurement samples spaced ½ DRX cycle apart. Thus the additional measurement latency is an additional DL data latency of 640 ms when a DRX cycle of 1.28 sec is configured.

The UL latency is increased both when SDT or DRB in RRC_CONNECTED is used.

Increased latency for DL data

When LP-WUS is used during MT-SDT, then the latency for DL data is also increased. When the DL latency arrives during the time offset between LP-WUS Occasion (LO) and PO, then the DL latency is increased with one DRX because only the LO in the next DRX cycle can be used to wakeup the MR:

When LP-WUS is used during SDT then the UL/DL latency is increased significantly, because UL data may arrive when the MR is in ultra-deep sleep and DL data may arrive during the LP-WUS time offset, i.e. the UE is paged in the next DRX cycle. Furthermore the MR has to start RSRP measurements from scratch after it woke up from ultra-deep sleep increasing the delay further.

In case SDT thresholds would be introduced on the LR, e.g. see R2-2503838, then the measurement latency can be reduced, i.e. using the LR measurements the UE can immediately check whether the LP-RSRP is above the SDT threshold and whether SDT can be initiated. However this does not compensate for the MR wake-up latency, i.e. it does not really solve the increased latency with LP-WUS:

The UE can initiate MO-SDT or MT-SDT while monitoring LP-WUS without further enhancements, i.e. for SDT operation only MR functions are used.

The use of LP-WUS during SDT can be disabled in the UE to avoid the additional latency due to LP-WUS, e.g. via NAS signalling).

## Enabling/disabling LP-WUS operation per UE (RRC-12)

Many companies see the need to enable/disable LP-WUS per UE. But there is no common view which approach to use:

- NAS signalling

- RRC signalling

- No specification changes:

  - LP-WUS can be enabled/disabled per UE when CN-assigned subgrouping is configured only.

  - Hard code the condition under which LP-WUS is enabled/disabled in the specification, e.g. LP-WUS is disabled when the time offset is longer than the DRX cycle.

In our view NAS signalling is preferred, because the CN-assigned subgrouping enabling/disabling is already specified in NAS. And NAS signalling provides a central point where the decision to enable /disable are made.

In our view enabling/disabling LP-WUS is not a local gNB decision, i.e. RRC signalling is not suitable.

Solution 3a cannot be used when only UE-ID based subgrouping is configured or when both UE-ID based subgroup and CN-assigned subgrouping are configured.

Solution 3b is not flexible, i.e. once the condition is specified, it cannot be changed:

LP-WUS can be enabled/disabled per UE via NAS signalling, also when only UE-ID based subgrouping is configured.

The NAS signalling could look like this:

## Receiver sensitivity and UE type

RAN4 agreed that for the receiver sensitivity (REFSENS requirements) for LP-WUS in FR1 there are two possible Noise Figures (NF) [1,2]. The Implementation Margin (IM) is 2.5 dB:

- Set 1: IM+NF = 18 dB

- Set 2: IM+NF = 13.5 dB

An LP-WUS implementation supporting 18 dB requires more guarband to prevent degradation in the paging performance compared to an LP-WUS implementation supporting 13.5 dB [4]:

RAN4 has not decided yet on the mapping of the NF requirements on the possible LP-WUS implementations, for example (other mappings are possible):

Mapping A:

- OFMD based WUR supports 13.5 dB

- OOK based WUR supports 18 dB

Mapping B:

- OFMD based WUR Type 1 supports 13.5 dB

- OFMD based WUR Type 2 supports 18 dB

- OOK based WUR Type 1 supports 13.5 dB

- OOK based WUR type 2 support 18 dB

Perhaps OFDM based WUR using LP-SS should be added to this discussion.

The mapping that is eventually agreed may impact the UE capabilities and configuration options. The RAN1 UE feature list and the RAN2 running CRs only support mapping A for the moment.

The initial agreement on entry/exit thresholds assumed OFDM-based and OOK-based WUR. But in later agreement this was generalized a bit more to account for RAN1/RAN4 agreements:

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

RAN2 has one meeting left to close the LP-WUS work item. In case more UE capabilities and NW configuration options need to be supported, then this has big impact on the signalling, ASN.1 and specification text:

Introduction of new WUR types hasd big impact on RAN2, i.e. RAN2 may not be able to close LP-WUS work item in RAN2#131.

One option is that the supported NF is not signalled to the network, e.g. an OFDM based WUR may support 13.5 or 18 dB. In that case RAN2 may want to discuss whether there is a need to indicate in SIB whether 13.5 and/or 18 dB WUR is supported. For example to support 18 dB WUR the network cannot use all of the channel bandwidth to configure LP-WUS. In case LP-WUS is configured close the channel edge then LP-WUS UEs that only support 18 dB should not be allowed to use LP-WUS to prevent degradation in the paging performance:

In SIB it is indicated when WUR with IM+NF of 18 dB is not allowed to be used.

## Radio paging capabilities

RAN2 agreed to introduce a new Rel-19 UE radio paging capability container [2]. The UE creates the Rel-19 container ue-RadioPagingInfo-r19 IE (including Rel-19 and later UE paging capabilities) and the gNB copies that container received in the UECapabilityInformation message into the UE-RadioPagingInfo IE to the CN:

The LP-WUS UE capability in Idle/Inactive is included in ue-RadioPagingInfo-r19 IE.

The running Mega CR 38.331 for the Rel-19 UE capabilities does not include LP-WUS capabilities yet, see [POST130][004][UE capabilities] Running Mega CR (Xiaomi).

## LP-WUS and emergency PDU session

SA2 and CT1 have discussed whether the UE is allowed to use LP-WUS during an emergency PDU session. The configured LP-WUS time offset, i.e. the minimum wake-up delay of the MR, introduces some delay in receiving an emergency callback. SA2 and CT1 decided that the UE is allowed to negotiate LP-WUS via NAS signalling during an emergency PDU session, and that the UE is allowed to use LP-WUS during an emergency PDU session [4, 5].

In the running CR 38.304 LP-WUS monitoring is not restricted when the UE has an emergency PDU session, i.e. no specification changes are required in RAN2 to align with SA2/CT1:

There is no restriction to use LP-WUS during an emergency PDU session, i.e. there is no specification impact on 38.304.

## Configured time offset and minimum wake-up delay

To support UE implementations that require different minimum wake-up delays for the MR RAN1 reached the following agreements:

- The UE signals to support a minimum wake-up delay of 70, 500 or 900 ms.

- The NW configures one or two time offset value(s) in SIB i.e. LoFrameOffset 8 .. 200 frames.

- If the configured time offset(s) is/are shorter than the supported minimum wake-up delay then the UE does not monitor LP-WUS.

With this approach there are the following drawbacks:

The network is allowed to configure a time offset shorter than the minimum latency of the UE, in which case the UE is not able to monitor LP-WUS.

The number of LP-WUS transmissions is increased when two time offsets are configured, because fewer paging attemps share the same LP-WUS transmission.

When two time offsets are configured, e.g. blue and orange LP-WUS occasions below, then the number of LP-WUS transmissions is increased, compared to the case when only the blue LP-WUS occasion is configured:

When only one LP-WUS occasion is associated with the PO then some UEs may have to wake-up too early or some UEs may not be able to use LP-WUS.

But it is possible to configure a single time offset, but support UEs with different wake-up delays:

There is no RAN1 impact to support this and there is also limited RAN2 specification impact:

The UE monitors the LP-WUS occasion that is closest to the UEs PO but is further away from the UEs PO than the minimum wake-up delay of the UE.

With this approach there are no restrictions for the UEs that can monitor LP-WUS.

## LP-WUS entry/exit condition

To avoid ping pong with cell reselection the NW should be able to configure a time to trigger:

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better than the serving cell according to the cell reselection criteria specified above during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

The IE T-Reselection concerns the cell reselection timer TreselectionRAT for NR and E-UTRA. Value in seconds. For value 0, behaviour as specified in 7.1.2 applies.

T-Reselection ::=                   INTEGER (0..7)

To avoid ping pong with entry/exit for LP-WUS monitoring:

For the entry condition for LP-WUS monitoring the NW can configure a Time To Trigger (TTT), i.e. a minimum time during which the entry condition has to be fulfilled before the UE may start LP-WUS monitoring.

## LP-WUS monitoring rules

RAN1 agreed that when the UE does not detect LP-WUS or LP-WUS is received but the LP-WUS  subgroup is not included then the UE is not required to monitor the following PO. But there can also be cases where the UE is not able to monitor LP-WUS, e.g. during cell re-selection. For PEI it has been captured in 38.304:

If the UE is unable to monitor the PEI occasion (i.e. all valid PDCCH MO for PEI) corresponding to its PO, e.g. during cell re-selection, the UE monitors the associated PO according to clause 7.1.

A similar requirement should be captured for LP-WUS:

If the UE is not able to monitor the LP-WUS then the UE is required to monitor the following PO.

## Band restrictions

The UE may not support LP-WUS in all the bands that the UE supports. RAN2 agreed that to mitigate this UE restriction the gNB may redirect an LP-WUS UE to a frequency where the UE supports LP-WUS and allocate a high dedicated frequency priority in the release. Dependent on NW implementation this may be complex to implement and redirection in SIB is less complex.

For example it is straightforward to introduce in SIB an indication that a UE supporting LP-WUS is allowed to consider to have the highest priority for cell reselection (lpwus-PrioritizationAllowed).

But there can be cases where “redirection to” may not be sufficient when:

- Congestion status on the LP-WUS frequency is missing (e.g. standalone midband)

- The cell overlaps with more than one cell on the LP-WUS frequency

In these use cases the UE may reselect to a cell on the LP-WUS frequency that is congested causing further congestion, while an LP-WUS UE is able to receive paging on a frequency where LP-WUS is not supported. Furthermore, to have an indication “redirect from” on the LP-WUS frequency (lpwus-PrioritizationNotAllowed) is also useful when the frequency becomes congested:

Introduce indication in SIB to redirect an LP-WUS UE to and from a frequency.

# Summary

RAN2 is kindly asked to discuss LP-WUS in Idle and Inactive:

Observation 1	In case the NW pages the UE in the complete registration area immediately in the first paging attempt, then there is a huge increase in the number of LP-WUS transmissions and a huge increase in the number of wake-ups for the UE, even when the false alarm probability is lower compared to PEI because more subgroups are used.

Observation 2	Monitoring LP-WUS during SDT is not critical from a power saving perspective.

Observation 3	When LP-WUS is used during SDT then the UL/DL latency is increased significantly, because UL data may arrive when the MR is in ultra-deep sleep and DL data may arrive during the LP-WUS time offset, i.e. the UE is paged in the next DRX cycle. Furthermore the MR has to start RSRP measurements from scratch after it woke up from ultra-deep sleep increasing the delay further.

Observation 4	Introduction of new WUR types hasd big impact on RAN2, i.e. RAN2 may not be able to close LP-WUS work item in RAN2#131.

Observation 5	The network is allowed to configure a time offset shorter than the minimum latency of the UE, in which case the UE is not able to monitor LP-WUS.

Observation 6	The number of LP-WUS transmissions is increased when two time offsets are configured, because fewer paging attemps share the same LP-WUS transmission.

Proposal 1	Similar as with PEI LP-WUS can be configured to be used in lastUsedCellOnly or in any cell where LP-WUS is configured.

Proposal 2	In RRC_INACTIVE state with CN configured PTW the LP-WUS SubgroupID used outside CN PTW is the same as the LP-WUS SubgroupID used inside CN PTW.

Proposal 3	The UE can initiate MO-SDT or MT-SDT while monitoring LP-WUS without further enhancements, i.e. for SDT operation only MR functions are used.

Proposal 4	The use of LP-WUS during SDT can be disabled in the UE to avoid the additional latency due to LP-WUS, e.g. via NAS signalling).

Proposal 5	LP-WUS can be enabled/disabled per UE via NAS signalling, also when only UE-ID based subgrouping is configured.

Proposal 6	In SIB it is indicated when WUR with IM+NF of 18 dB is not allowed to be used.

Proposal 7	The LP-WUS UE capability in Idle/Inactive is included in ue-RadioPagingInfo-r19 IE.

Proposal 8	There is no restriction to use LP-WUS during an emergency PDU session, i.e. there is no specification impact on 38.304.

Proposal 9	The UE monitors the LP-WUS occasion that is closest to the UEs PO but is further away from the UEs PO than the minimum wake-up delay of the UE.

Proposal 10	For the entry condition for LP-WUS monitoring the NW can configure a Time To Trigger (TTT), i.e. a minimum time during which the entry condition has to be fulfilled before the UE may start LP-WUS monitoring.

Proposal 11	If the UE is not able to monitor the LP-WUS then the UE is required to monitor the following PO.

Proposal 12	Introduce indication in SIB to redirect an LP-WUS UE to and from a frequency.

# References

- R4-2503003 LS on LP-WUS UE RF, LS out, RAN4 (vivo), To: RAN1, Cc: RAN2, RAN4#114

- R1-2504943 Reply LS on LP-WUS UE RF, LS out, RAN1 (vivo), To: RAN4, Cc: RAN2, RAN1#121

- R250xxx, On blocking performance of WUR, Ericsson, DISC, RAN4#116

- S2-2502428, Removing unnecessary restrictions for emergency PDU sessions, CR 23.501 (Rel-19), QC, SA2#167

- C1-251603, LP-WUSPS and emergency, CR 24.501 (Rel-19), Ericsson, CT1#154

# Annex: Agreements

## RAN1 agreements (selection)

R1-2504332	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns):

| SSB periodicity (ms) | Wake-up delay (ms) UE capability 1 | Wake-up delay (ms) UE capability 2 | Wake-up delay (ms) UE capability 3 |
| --- | --- | --- | --- |
| 5/10/20 | [70] | [500] | [900] |
| 40 | [x] | [x] | [x] |
| 80 | [x] | [x] | [x] |
| 160 | [x] | [x] | [x] |

Agreement

If the number of POs associated with a LO is less than Ns (the number of POs per PF), to determine the LP-WUS MOs for the multiple LOs with the same reference PF:

- Alt 1: additional frame-level offset(s) are configured.

  - Common slot-level or symbol level offsets are shared across all LOs

R1-2504333	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

For PO-to-LO association and codepoint determination, assume NPOLO is the number of POs associated with a LO, NSGPO is the number of subgroups per PO for LP-WUS, and  iSG is the subgroup ID of a UE (0≤iSG<NSGPO).

- The PO index within the LO is defined as

iPO=UE_ID mod N∙Ns+is mod NPOLO,

- The reference PF for the LO of a PO is provided by (SFN for PF) – floor(iPO/NS) * T/N, which is the first PF of the PF(s) associated with the LO.

- For the codepoints,

  - If NSGPO>1,

    - The number of information bits in LP-WUS is log2NPOLO*(NSGPO+1).

    - Alt 2: (the codepoints for each PO are consecutive)

      - The codepoint for subgroup iSG in PO iPO is iPO*(NSGPO+1)+iSG.

      - The codepoint for all the subgroups in PO iPO is (iPO+1)*NSGPO+1-1.

  - If NSGPO=1, the number of information bits in LP-WUS is log2NPOLO, and the codepoint for PO iPO is iPO.

- Here UE_ID (for LP-WUS), N, NS, is and T are defined in TS 38.304

R1-2504334	Summary #4 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

For UE to determine whether a symbol is available for LP-WUS, at least the following is supported:

- A unit-level bitmap with a periodicity 10, 20, or 40 units and a 14-bit or 28-bit symbol-level bitmap that covers 1 or 2 slots can be configured, where each unit is 1 or 2 slots for 14-bit or 28-bit symbol-level bitmap, respectively, with a maximum periodicity of 40ms.

  - ‘1’ in unit-level bitmap means the symbol level bitmap is applied to determine which symbols are unavailable in the unit for LP-WUS (‘0’ means unavailable).

  - ‘0’ in unit-level bitmap means: all the symbols in the unit are unavailable for LP-WUS

- If the slot-level bitmap is not configured, UE assumes all 1’s for the bitmap.

- If the symbol-level bitmap is not configured, UE assumes all 1’s for the bitmap.

Agreement

For the handling of SSB for IDLE mode, the SSB symbols are considered as unavailable for LP-WUS, if SSB and LP-WUS overlap in frequency domain

R1-2504335	Summary #5 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

Nominal MO duration (X1, in unit of OFDM symbols) and actual LP-WUS duration (X2, in unit of OFDM symbols) are configured. (Alt C)

- A LP-WUS MO spans the nominal MO duration (i.e., the LP-WUS MO duration is the same as the nominal MO duration.)

- If the number of available OFDM symbols within the nominal MO duration is no less than the actual LP-WUS duration, UE monitors LP-WUS on the first X2 available symbols within the LP-WUS MO.

- Otherwise, UE does not monitor LP-WUS in this MO (i.e., the MO is dropped).

- Note: Any symbols that are not defined as unavailable are available symbols for LP-WUS.

Agreement

The starting time location of the first LP-WUS MO in a LO is configured by an offset w.r.t. the reference point, where the offset is a symbol-level offset

Agreement

If LP-SS is configured, the periodicity is configured with the candidate value set of {160ms, 320ms}.

- A time offset is configured for the first LP-SS occasion with reference to SFN0

  - If the periodicity is 160ms, the candidate value set for the time offset is {0, 1, …, 159}ms.

  - If the periodicity is 320ms, the candidate value set for the time offset is {0, 1, …, 319}ms.

Agreement

The number of MOs per beam per LO is configured by gNB, with candidate value set of {1, 2, 3, 4}.

R1-2504335	Summary #5 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

R1-2504891	Summary #6 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

For the EPRE ratio between LP-WUS/LP-SS and SSB (i.e. LP-WUS/LP-SS EPRE divided by SSB EPRE), separate configurations are provided for LP-WUS and LP-SS.

- The candidate value set for the EPRE ratio is {-3dB, 0 dB, 3 dB, 6 dB}.

- If M=1 for both LP-WUS and LP-SS, or M>1 for both LP-WUS and LP-SS, the difference between the two EPRE ratios shall be no larger than 3 dB.

- If M=1 for LP-WUS and M>1 for LP-SS, the ERPE ratio for LP-SS minus the EPRE ratio for LP-WUS should be within the range of -6 dB to 0 dB.

Note: EPRE refers to EPRE in one OFDM symbol with non-zero power (from baseband perspective) LP-WUS/LP-SS transmission.

## RAN2 agreements (selection)

RRC-2, whether entry/exit condition is mandatory or optional

The entry/exit condition for LP-WUS monitoring is mandatory in LP-WUS configuration, if the LP-WUS configuration is provided by the NW.

Full coverage for LP-WUS is not precluded, e.g., if there needs to be a threshold value so that the condition is always fulfilled for all LPWUS UEs.

It is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement based conditions are used for LP-WUS monitoring entry/exit condition, if UE support both measurement types.

RRC-12, whether/how to enable/disable LP-WUS, e.g. by RRC/NAS

RAN2 aim at supporting enabling/disabling LP-WUS monitoring in IDLE/INACTVE per UE, if the solution can be concluded in the August meeting.