3GPP TSG-RAN2 Meeting #131	R2-2505857

Bengaluru, India, 25 - 29 August 2025	Revision of R2-2504289

Agenda Item:	8.4.3 RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Source: 	Ericsson

Title:  	LP-WUS and RRM measurements

Document for:	Discussion and Decisaion

# Introduction

In this contribution the open issues for LP-WUS and RRM measurements are discussed further.

For the RAN2 and RAN4 agreements in previous meetings the reader is referred to the Annex.

# Discussion

## Exit condition for RRM relaxation (38304-3/RRC-7)

There is an open issue for the exit condition for RRM relaxation (38304-3/RRC-7):

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. (Same as the open issue in RRC, i.e., FFS on exit condition for serving cell RRM relaxation)

For RRM offloading the entry condition is based on MR measurements and optionally on LR measurements as well. The exit condition is based on LR measurements only. When only RRM relaxation is configured, it is straightforward and obvious, that RRM relaxation should use the same principle as RRM offloading, i.e. the RRM relaxation entry condition is based on MR measurements and optionally LR measurements and RRM relaxation exit condition is based on LR measurements only.

Some companies see the benefit to configure RRM offloading and relaxation together. In that case, when the UE is in offloading state and moving towards the cell border, then the UE performs LR measurements to check when it should exit RRM offloading. But to enable a smooth transition from offloading to relaxation state, the UE needs to check if the entry condition for relaxation is met, while the UE is in offloading state. However checking the entry condition for relaxation requires MR measurements, i.e. the power saving gains in offloading state would be impacted. A similar “problem” exists when the UE is moving in the opposite direction, i.e. while in relaxation state the UE needs to check the entry condition for offloading which requires MR measurements. It is clear that there is a trade off between smooth state transition between RRM offloading and relaxation and the UE power saving gains. It is proposed to leave this trade off to UE implementation:

For both RRM offloading and RRM relaxation the entry condition is based on MR measurements with optionally LR measurements and the exit condition is based on LR measurements. When both RRM offloading and relaxation are configured, then it is left to UE implementation when the UE checks whether the entry condition for RRM offloading when the UE is in relaxation state.

## LP-WUS threshold dependencies (RRC-8)

There is an open issue for the LP-WUS threshold dependencies (RRC-8):

FFS on whether/how to reduce the threshold number for LP-WUS/WUR

Editor’s NOTE: FFS on the relationship between the thresholds for serving cell relaxation/offloading, neighboring cell relaxation and entry/exit condition of using LP-WUS, [and potential pre-condition between RRM relaxation/offloading criteria and entry/exit condition of using LP-WUS].

RAN2 made the following agreements for this topic already:

RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both.

Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation), if there is such configuration. Capture this in the field description. Details will be discussed in the running CR.

It is up to NW to configure the condition for LP-WUS monitoring and/or R19 serving/ neighboring cell RRM relaxation /R19 serving cell RRM offloading, as in the current RRC running CR.

The threshold of the MR based entry condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, which is the maximum of {SIntraSearchP, SnonIntraSearchP}. Capture this in the field description. Details will be discussed in the running CR.

The following issues are discussed further:

- Dependency between the entry condition for RRM offloading and legacy thresholds (see the highlighted agreement above).

- Dependencies between RRM offloading and RRM relaxation.

Dependencies for RRM offloading

First RAN2 agreed that full cell coverage is not excluded when LP-WUS is configured:

Full coverage for LP-WUS is not precluded, e.g., if there needs to be a threshold value so that the condition is always fulfilled for all LPWUS UEs.

Full cell coverage means that the UE can use LP-WUS directly after cell reselection, provided that the cell is suitable to camp and the entry condition is met. Thus it is possible that the UE reselects to a cell where the measured RSRP is below SIntraSearchP and SnonIntraSearchP:

In case LP-WUS provides full cell coverage then it should be possible that the UE monitors LP-WUS below SIntraSearchP and SnonIntraSearchP.

RAN2 made the highlighted agreement because there are a similar requirements for Rel-16 and 17 RRM relaxation and the not-at-cell-edge threshold(s), i.e. the not-at-cell-edge thresholds  s-SearchThresholdP and s-SearchThresholdP2:

relaxedMeasurement-r16              SEQUENCE {

lowMobilityEvaluation-r16           SEQUENCE {

…

cellEdgeEvaluation-r16              SEQUENCE {

s-SearchThresholdP-r16              ReselectionThreshold,

s-SearchThresholdQ-r16              ReselectionThresholdQ                       OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition-r16     ENUMERATED {true}                               OPTIONAL,       -- Need R

highPriorityMeasRelax-r16           ENUMERATED {true}                               OPTIONAL        -- Need R

}

…

relaxedMeasurement-r17                  SEQUENCE {

…

cellEdgeEvaluationWhileStationary-r17   SEQUENCE {

s-SearchThresholdP2-r17                 ReselectionThreshold,

s-SearchThresholdQ2-r17                 ReselectionThresholdQ                   OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition2-r17        ENUMERATED {true}                           OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

OPTIONAL        -- Need R

s-SearchThresholdP, s-SearchThresholdP2 Parameters "SSearchThresholdP" and "SSearchThresholdP2"in TS 38.304 [20]. The network configures s-SearchThresholdP and s-SearchThresholdP2 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP.

For neighbour cell relaxation that makes sense, i.e. when the UE moves from cell centre to cell edge:

- UE is not required to measure

- UE has to measure with a scaling factor

- UE has to measure continuously

The agreement is captured in the running CR 38.331 for LP-WUS:

relaxedMeasurementForServingAndNeighboringCell-r19                  SEQUENCE {

cellEdgeEvaluationOnMR-ForLR-OnSSB-r19   SEQUENCE {

s-SearchThresholdP3-r19                 ReselectionThreshold,

s-SearchThresholdQ3-r19                 ReselectionThresholdQ                   OPTIONAL        -- Need R

}                                                                                  OPTIONAL,    -- Cond SupportLR-OnSSB

cellEdgeEvaluationOnMR-ForLR-OnLPSS-r19   SEQUENCE {

s-SearchThresholdP4-r19                 ReselectionThreshold,

s-SearchThresholdQ4-r19                 ReselectionThresholdQ                   OPTIONAL        -- Need R

}                                                                                   OPTIONAL,    -- Cond SupportLR-OnLPSS

…

}                                                                                       OPTIONAL,        -- Need R

offloadMeasurementForServingCell-r19                  SEQUENCE {

cellEdgeEvaluationOnMR-ForLR-OnSSB-r19   SEQUENCE {

s-SearchThresholdP5-r19                 ReselectionThreshold,

s-SearchThresholdQ5-r19                 ReselectionThresholdQ                   OPTIONAL     -- Need R

}                                                                                   OPTIONAL,    -- Cond SupportLR-OnSSB

cellEdgeEvaluationOnMR-ForLR-OnLPSS-r19   SEQUENCE {

s-SearchThresholdP6-r19                 ReselectionThreshold,

s-SearchThresholdQ6-r19                 ReselectionThresholdQ                   OPTIONAL     -- Need R

}                                                                                   OPTIONAL,    -- Cond SupportLR-OnLPSS

…

}                                                                                       OPTIONAL        -- Need R

]]

}

s-SearchThresholdP, s-SearchThresholdP2, s-SearchThresholdP3, s-SearchThresholdP4, s-SearchThresholdP5, s-SearchThresholdP6 Parameters "SSearchThresholdP", "SSearchThresholdP2", "SSearchThresholdP3", "SSearchThresholdP4", "SSearchThresholdP5", and "SSearchThresholdP6" in TS 38.304 [20]. The network configures s-SearchThresholdP and s-SearchThresholdP2 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP. The network configures both s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-IntraSearchP and s-NonIntraSearchP. The network configures s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-SearchThresholdP3 and s-SearchThresholdP4.

FYI: RAN4 made the following agreement in accordance with RAN2:

Issue 1-1-10: Higher priority frequency layer measurement requirements

Agreement:

For case 1:

- MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60

  - Note: RAN4 assumes Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met for case 1.

However it is not correct to compare Rel-16/Rel-17 RRM relaxation with Rel-19 RRM offloading. Because with Rel-19 offloading the UE continues to measure serving cell using LR. Furthermore in case LP-WUS provides full cell coverage then Rel-19 RRM offloading should not be restricted and be allowed in the complete cell:

It is incorrect to compare Rel-16/Rel-17 RRM relaxation with Rel-19 RRM offloading because the LR continues serving cell measurements with offloading. And in case LP-WUS provides full cell coverage then Rel-19 RRM offloading should not be restricted, i.e. the UE is allowed to offload serving cell in the complete cell.

It should be left to NW implementation how to configure the RRM offloading threshold, i.e. whether partial or full LP-WUS cell coverage is provided:

It is left to NW implementation to configure full or partial LP-WUS cell coverage, i.e. configuration of s-SearchThresholdP5 and s-SearchThresholdP6 is left to NW implementation.

Similar as for Rel-16/Rel-17 RRM relaxation it is reasonable that the Rel-19 RRM offloading thresholds are configured below s-IntraSearchP and s-NonIntraSearchP. And it makes also sense that the threshold for the Rel-19 scaling factors is higher than the threshold for the Rel-16/Rel-17 scaling factors:

The NW configures s-SearchThresholdP3 and s-SearchThresholdP4 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP.

The NW configures s-SearchThresholdP3 and s-SearchThresholdP4 to be higher than s-SearchThresholdP-r16 and s-SearchThresholdP2-r17, if configured.

Dependencies between RRM offloading and RRM relaxation

To achieve optimal power saving it is required that:

- When the UE is in LP-WUS monitoring state, then the UE is also in the offloading state

- When the UE is in LP-WUS monitoring state, then the UE is also in the relaxation state

When only offloading is configured:

- The same entry/exit condition is used for LP-WUS monitoring and offloading. The UE goes in and out of LP-WUS monitoring state and offloading state at the same time.

When only relaxation is configured:

- The same entry/exit condition is used for LP-WUS monitoring and relaxation. The UE goes in and out of LP-WUS monitoring state and relaxation state at the same time

When both offloading and relaxation are configured:

- The UE goes out of LP-WUS monitoring state and relaxation state at the same, i.e. the LP-WUS monitoring exit condition and the relaxation exit  condition are the same.

- The UE goes into LP-WUS monitoring state and relaxation state at the same, i.e. the LP-WUS monitoring entry condition and the relaxation entry  condition are the same.

- The UE does not go out of offloading state and LP-WUS monitoring state at the same time, i.e. the offloading exit threshold is higher than the LP-WUS monitoring exit threshold.

- The UE does not go into offloading state and LP-WUS monitoring state at the same time, i.e. the offloading entry thresholds are higher than the LP-WUS monitoring entry thresholds.

Thus for the entry/exit for LP-WUS, offloading and relaxation:

If only offloading or only relaxation is configured then the same entry/exit condition is used for LP-WUS monitoring, offloading and relaxation.

When both offloading and relaxation are configured the offloading exit threshold is configured higher than the LP-WUS monitoring exit threshold. And the entry threshold for offloading is configured higher than the entry threshold for LP-WUS monitoring entry condition.

## Rel-16 RRM relaxation (RRC-10, RRC-15)

The open issues related to LP-WUS and Rel-16 RRM relaxation are listed below:

| RRC-10 | FFS on low mobility criteria Editor’s NOTE: FFS on “low mobility” criteria. |
| --- | --- |
| RRC-15 | FFS on impact on R16 RRM relaxation criteria (e.g. can LR can be used to evaluate Rel-16 “not-at-cell-edge” and “low mobility” criterion). As raised by Ericsson above. RRC-10: The description of “FFS on low mobility criteria” is not very clear. In our understanding this is for the use case when LP-WUS and Rel-16 RRM relaxation are configured, i.e. Rel-19 RRM relaxation is not configured. This is not about introducing low mobility criteria in the entry condition (which I think has been discussed already). It needs to be clarified whether the LR can be used to evaluate the Rel-16 “not-at-cell-edge” and “low mobility” criterion. Proposed wording: FFS on low mobility criteria (e.g. can LR can be used to evaluate Rel-16 “not-at-cell-edge” and “low mobility” criterion). NOTE: this open issue overlaps with open issue 38304-2. |
| 38304-2 | FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. |
| 38304-6 | Whether UE low mobility criterion or stationary criterion should be considered for RRM relaxation/offloading. (Same as the open issue in RRC, i.e., FFS on low mobility criteria) |

Related to this topic RAN2 agreed:

It is up to NW to configure the condition for LP-WUS monitoring and/or R19 serving/ neighboring cell RRM relaxation /R19 serving cell RRM offloading, as in the current RRC running CR.

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

There are two questions to be answered:

- Is the Rel-16 low mobility criterion part of the entry/exit condition for RRM relaxation/offloading?

- If the answer to question 1 is no: what are the UE requirements when Rel-19 serving and neighbor cell relaxation is not configured but Rel-16 neighbour cell relaxation is configured?

It is optional for the NW to configure Rel-19 relaxation with LP-WUS. When Rel-19 relaxation is not configured, then the NW configures Rel-19 offloading. The following configuration options can be identified w.r.t. neighbour cell relaxation:

- LP-WUS is configured without Rel-16 RRM relaxation and without Rel-19 RRM relaxation

- LP-WUS is configured with Rel-16 RRM relaxation and without Rel-19 RRM relaxation

- LP-WUS is configured without Rel-16 RRM relaxation and with Rel-19 RRM relaxation

- LP-WUS is configured with Rel-16 RRM relaxation and with Rel-19 RRM relaxation

Option 1 is the legacy case, i.e. when the UE is above SIntraSearch or above SNonIntraSearch then the UE is not required to measure intra-frequency and inter-frequency neighbours respectively.

Concerning options 2, 3 and 4, i.e. when one or both relaxation options are configured: the UE is expected to apply the NW configuration when LP-WUS, Rel-16 RRM relaxation and/or Rel-19 RRM relaxation is configured. But it may be good to clarify the scenario when NW configures both Rel-16 and Rel-19 RRM relaxation and the UE supports both features:

When LP-WUS is configured then it is up to UE implementation whether to apply Rel-16 or Rel-19 RRM relaxation when both are configured.

The UE does not apply Rel-19 RRM relaxation when the UE does not support LP-WUS.

The Rel-16 RRM relaxation requirements are the same for a UE supporting LP-WUS and a UE not supporting LP-WUS, i.e. the UE behavior w.r.t. neighbour cell measurements is the same in both cases.

When Rel-19 RRM relaxation is configured with LP-WUS we identified the following risk:

When Rel-19 RRM relaxation is configured and the UE is above the Rel-19 relaxation threshold, then a moving UE (e.g. with pedestrian/car speeds) may refrain from serving/neighbour cell measurements up to 20 sec (scaling factor 16). In an urban environment with irregular cell coverage this may imply that the UE is not always camped on the strongest cell, i.e. cell reselection and paging performance may be degraded (or the UE can only use LP-WUS in the cell partially).

In our view using the Rel-16 RRM relaxation with LP-WUS is a safer approach, i.e. when low mobility is configured only stationary UEs are allowed to relax (and monitor LP-WUS). This is safer because when the UE was moving and performed neighbour cell measurements, but did not find a stronger neighbour, then it is fine that the UE does not continue neighbour cell measurements when the UE is stationary.

When the UE is in low mobility state then the UE continuously checks whether the low mobility criterion is still fulfilled based on the serving cell measurements. When the serving cell measurements have dropped more than SSearchDeltaP then the UE is no longer in low mobility state. A legacy UE uses the MR for serving cell measurements. But in case an LP-WUS UE would use the MR for mobility detection, then there are no power saving gains. Thus to make Rel-16 RRM relaxation work together with LP-WUS monitoring:

A UE for which the Rel-16 low mobility criterion and the LP-WUS entry condition are fulfilled is allowed to monitor LP-WUS without MR neighbour cell measurements. The UE uses LR serving cell measurements to determine when the low mobility criterion is no longer fulfilled.

The MR should not be required to continuously measure the serving cell to detect when the low mobility criterion is no longer fulfilled.

However the LR measurements are expected to provide lower accuracy compared to MR measurements, especially when the LR uses LP-SS. For the LR accuracy RAN4 assumes a [2 or 2.5] dB RF impairment margin. To compensate for the reduced accuracy it is proposed:

A Time To Trigger (TTT) can be configured for the low mobility criterion, i.e. the low mobility criterion has to be fulfilled for TTT time before the UE is allowed to enter low mobility state when the UE is using LR serving cell measurements using LP-SS.

## LP-RSRP and LP-RSRQ value range and mapping (RRC-6)

In the email discussion #210 it is proposed that:

The IE ThresholdLP is used to indicate a measured RSRP threshold for LP-WUS. Actual value of threshold = field value * 2 [dBm].

ThresholdP-LP ::=               INTEGER (-80..0)

The IE ThresholdQ-LP is used to indicate a measured RSRQ threshold for LP-WUS. Actual value of threshold = field value [dB].

ThresholdQ-LP ::=               INTEGER (-34..0)

This might be ok but it is not RAN2 that can actually decide this, i.e. it requires input from RAN4. RAN2 does not know what the LP-RSRP/LP-RSRQ value range is and thus what are the possible threshold settings.

More importantly RAN2 does not know the mapping of LP-RSRP and LP-RSRQ on RSRP and RSRQ respectively. In case the NW wants to configure the entry thresholds for both MR and LR, then the NW should understand what is the corresponding LP-RSRP/LP-RSRQ value for a certain RSRP/RSRQ threshold on MR. This does not mean that the NW has to aim that the MR and LR thresholds typically would be fulfilled at the same time. But the NW has to have some understanding how the MR and LR measurements relate to each other. RAN4 is expected specify a mapping table or use an offset as was done for a 1 Rx UE: the “offset” is a fixed value in dB specified in RAN4 specification and corresponds to the magnitude of the difference between RSRP accuracies for 1 Rx branch and 2Rx branches [2]:

Ask RAN4 for the LP-RSRP/LP-RSRQ value range and mapping to RSRP/RSRQ.

## EMR (Xiaomi 002)

EMR can briefly be summarized as:

- The UE can be requested to measure up to 5 min after RRCRelease on specific frequencies in specific area. The UE starts T331 when it receives RRCRelease with measIdleDuration and the UE starts the EMR measurements. The UE may stop EMR measurements when T331 expires. Measurements specifically after cell selection can be requested (Rel-18).

- The UE can report the measurements or provide an availability indication in RRCSetupComplete or RRCResumeComplete.

- The UE can be requested not to report measurements older than 5, 10, 20, 50 or 100 sec (Rel-18).

In 38.133 it is specified that the Rel-16 low mobility and/or not-at-cell-edge criterion can only be fulfilled when T331 is not running e.g.:

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	T331 timer is not running for EMR measurements on inter-RAT E-UTRAN, and

-	UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, or

-	UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] is not configured, and

-	UE has fulfilled only the lowMobilityEvaluation [2] criterion.

Rel-19 RRM serving/neighbour cell relaxation and EMR is discussed in RAN4 and the requirements will be captured in 38.133 [1]:

RAN2 assumes that EMR measurements are prioritized over Rel-19 RRM relaxation (similar as with Rel-16 RRM relaxation) but leaves it to RAN4 to agree and capture the detailed requirements.

## Time To Trigger

To avoid ping pong with cell reselection the NW should be able to configure a time to trigger:

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better than the serving cell according to the cell reselection criteria specified above during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

The IE T-Reselection concerns the cell reselection timer TreselectionRAT for NR and E-UTRA. Value in seconds. For value 0, behaviour as specified in 7.1.2 applies.

T-Reselection ::=                   INTEGER (0..7)

To avoid ping pong with entry/exit for LP-WUS monitoring:

For LP-WUS exit/entry the NW can configure a time to trigger (seconds), i.e. minimum time during which the entry condition has to be fulfilled, before the UE may start LP-WUS monitoring (again).

## Band restrictions

The UE may not support LP-WUS in all the bands that the UE supports. RAN2 agreed that to mitigate this UE restriction the gNB may redirect an LP-WUS UE to a frequency where the UE supports LP-WUS and allocate a high dedicated frequency priority in the release. Dependent on NW implementation this may be complex to implement and redirection in SIB is less complex.

For example it is straightforward to introduce in SIB an indication that a UE supporting LP-WUS is allowed to consider to have the highest priority for cell reselection (lpwus-PrioritizationAllowed).

But there can be cases where “redirection to” may not be sufficient when:

- Congestion status on the LP-WUS frequency is missing (e.g. standalone midband)

- The cell overlaps with more than one cell on the LP-WUS frequency

In these use cases the UE may reselect to a cell on the LP-WUS frequency that is congested causing further congestion, while an LP-WUS UE is able to receive paging on a frequency where LP-WUS is not supported. Furthermore, to have an indication “redirect from” on the LP-WUS frequency (lpwus-PrioritizationNotAllowed) is also useful when the frequency becomes congested:

Introduce indication in SIB to redirect an LP-WUS UE to and from a frequency.

# Summary

RAN2 is kindly asked to discuss LP-WUS and RRM measurement requirements:

Observation 1	In case LP-WUS provides full cell coverage then it should be possible that the UE monitors LP-WUS below SIntraSearchP and SnonIntraSearchP.

Observation 2	It is incorrect to compare Rel-16/Rel-17 RRM relaxation with Rel-19 RRM offloading because the LR continues serving cell measurements with offloading. And in case LP-WUS provides full cell coverage then Rel-19 RRM offloading should not be restricted, i.e. the UE is allowed to offload serving cell in the complete cell.

Observation 3	When Rel-19 RRM relaxation is configured and the UE is above the Rel-19 relaxation threshold, then a moving UE (e.g. with pedestrian/car speeds) may refrain from serving/neighbour cell measurements up to 20 sec (scaling factor 16). In an urban environment with irregular cell coverage this may imply that the UE is not always camped on the strongest cell, i.e. cell reselection and paging performance may be degraded (or the UE can only use LP-WUS in the cell partially).

Proposal 1	For both RRM offloading and RRM relaxation the entry condition is based on MR measurements with optionally LR measurements and the exit condition is based on LR measurements. When both RRM offloading and relaxation are configured, then it is left to UE implementation when the UE checks whether the entry condition for RRM offloading when the UE is in relaxation state.

Proposal 2	It is left to NW implementation to configure full or partial LP-WUS cell coverage, i.e. configuration of s-SearchThresholdP5 and s-SearchThresholdP6 is left to NW implementation.

Proposal 3	The NW configures s-SearchThresholdP3 and s-SearchThresholdP4 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP.

Proposal 4	The NW configures s-SearchThresholdP3 and s-SearchThresholdP4 to be higher than s-SearchThresholdP-r16 and s-SearchThresholdP2-r17, if configured.

Proposal 5	If only offloading or only relaxation is configured then the same entry/exit condition is used for LP-WUS monitoring, offloading and relaxation.

Proposal 6	When both offloading and relaxation are configured the offloading exit threshold is configured higher than the LP-WUS monitoring exit threshold. And the entry threshold for offloading is configured higher than the entry threshold for LP-WUS monitoring entry condition.

Proposal 7	When LP-WUS is configured then it is up to UE implementation whether to apply Rel-16 or Rel-19 RRM relaxation when both are configured.

Proposal 8	The UE does not apply Rel-19 RRM relaxation when the UE does not support LP-WUS.

Proposal 9	The Rel-16 RRM relaxation requirements are the same for a UE supporting LP-WUS and a UE not supporting LP-WUS, i.e. the UE behavior w.r.t. neighbour cell measurements is the same in both cases.

Proposal 10	A UE for which the Rel-16 low mobility criterion and the LP-WUS entry condition are fulfilled is allowed to monitor LP-WUS without MR neighbour cell measurements. The UE uses LR serving cell measurements to determine when the low mobility criterion is no longer fulfilled.

Proposal 11	A Time To Trigger (TTT) can be configured for the low mobility criterion, i.e. the low mobility criterion has to be fulfilled for TTT time before the UE is allowed to enter low mobility state when the UE is using LR serving cell measurements using LP-SS.

Proposal 12	Ask RAN4 for the LP-RSRP/LP-RSRQ value range and mapping to RSRP/RSRQ.

Proposal 13	RAN2 assumes that EMR measurements are prioritized over Rel-19 RRM relaxation (similar as with Rel-16 RRM relaxation) but leaves it to RAN4 to agree and capture the detailed requirements.

Proposal 14	For LP-WUS exit/entry the NW can configure a time to trigger (seconds), i.e. minimum time during which the entry condition has to be fulfilled, before the UE may start LP-WUS monitoring (again).

Proposal 15	Introduce indication in SIB to redirect an LP-WUS UE to and from a frequency.

# References

- R4-251xxxx, Remaining issues on LP-WUS RRM requirement, Ericsson, DISC, RAN4#116

- R4-2206951, LS on configuring margin for 1 Rx RedCap UEs, LS out, RAN4, To: RAN2, RAN4#120-e

# Annex: Agreements

## RAN2 agreements (selection)

RRC-8, 38304-4, on whether/how to reduce the threshold number for LP-WUS/WUR

R2-2504739	Discussion report on [AT130][205][LPWUS], vivo, Report

The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation), if there is such configuration. Capture this in the field description. Details will be discussed in the running CR.

It is up to NW to configure the condition for LP-WUS monitoring and/or R19 serving/ neighboring cell RRM relaxation /R19 serving cell RRM offloading, as in the current RRC running CR.

The threshold of the MR based entry condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, which is the maximum of {SIntraSearchP, SnonIntraSearchP}. Capture this in the field description. Details will be discussed in the running CR.

[RRC-10, 38304-6, on low mobility criteria] in CB

R2-2503882	Discussion on RRM measurement relaxation and offloading, Sharp, Disc

Noted

## RAN4 agreements (selection)

Issue 1-1-10: Higher priority frequency layer measurement requirements

Agreement:

For case 1:

- MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60

  - Note: RAN4 assumes Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met for case 1.

For case 3:

- When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60

- When the condition of Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is NOT met, the same requirement for higher priority, equal priority and lower priority carriers:

  - 16 times of Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter are applied

Issue 1-1-17 Reply RAN1 LS R1-2503103

Agreement on the reply LS to RAN1:

- Confirm to RAN1 that for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

- RAN4 agreed that: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Issue 1-2-4-1: On LR measurement filtering requirement

Agreement:

No need to define filtering requirement for LP-RSRP and LP-RSRQ measurements.

Issue 1-2-6-1: On the number of SSBs (for RAN1 LS R1-2501624)

Agreement:

For the 400ms and 800ms ramping up time: {5, 5} samples

For the 10ms ramping up time: 3 samples

Agreement

For the side conditions, RAN4 assume the side conditions is the side conditions of the LP-WUS monitoring used by