3GPP TSG-RAN WG2 Meeting #131	R2-2505871

Bengaluru, India, August 25 – 29 2025

Agenda item:	8.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RAN impacts for IoT-NTN SF Operations

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Following are the agreements in RAN2 on the SF operation for IoT-NTN up to RAN2-130

RAN2-129 Agreements: 1. RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change 2. The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information) 3. Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time). 4. A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

RAN2-129bis Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

RAN2-130 Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

In this paper we analyse the list of open issues indicated in the post e-mail discussion for TS36.304 and provide our observations and proposals for wav forward.

# 2	Discussion

## 2.1 UE behavior for handling SF mode parameter for neighbor cell.

In RAN2-130 meeting, RAN2 made a working assumption to introduce SF-operating mode parameter for neighbour-cell. However, whether this parameter needs to be considered in cell reselection for prioritization of neighbour cells was not concluded. This parameter was introduced in serving cell mainly for SF mode capable UE to know that the cell is operating without ground connectivity and it can decide on whether to trigger data transmission for delay tolerant services or not.  However, when the UE is in idle mode this parameter does not play role in starting the cell search overriding the current intra-search /inter-search parameters and RAN2 concluded to leave this up to UE implementation.

Observation 1: The SF mode parameter broadcasted for serving cell is used by UE implementation to decide on its data transmission type and not for triggering cell search in current specification. It is up to the UE implementation to make use of this parameter to start early search of neighbour cells.

Extending the above analysis for UE capable of SF mode whether neighbour cell operating in SF mode or not need not be criteria for idle mode cell reselection as it can start data transmission in either of the neighbour-cell hence this information need not be criteria for cell reselection. In case there is pending MT data or NAS registration for the SF capable UE, use of SF mode in neighbour cell may be beneficial to complete the pending transaction. However this depends on application status and can be left to UE implementation.

Observation 2: Awareness of SF mode operation of neighbour cell is beneficial for completing the pending MT transaction after cell reselection.

Rel-19 UE not supporting SF mode but currently served by normal mode cell may use the neighbour cell information to prioritise the cell reselection. However, without knowledge of upcoming data transmission this prioritization is not needed. For idle mode operation in target cell its radio condition is sufficient for cell reselection. In current specification the cell reselection priority is impacted only if the UE is configured for some services such as MBMS or V2X in current cell and the support of such services in neighbour cell is needed for continuity. For SF mode parameters it is not the case.

Observation 3: For Rel-19 UE not capable of SF mode usage of neighour cell SF mode parameter for cell reselection can be left to UE implementation.

Observation 4: The cell reselection priority is considered for cell reselection in current specification for scenarios related to continuity of specific service configured in current cell (i.e V2X, MBMS).

Based on the above observations the SF mode of neighbour cell can be considered for prioritization of neighbour cell for cell reselection needed only for certain scenarios related to UE awareness of upcoming application data. As this information is related to UE internal communication, we propose to leave the usage of SF mode in neighbour cell for cell reselection up to UE implementation.

Proposal 1:  RAN2 to confirm the working assumption for introduction of SF mode for neighbour-cell without specification changes related to usage of this parameter for cell reselection.

## 2.2 Paging Enhancements for SF mode operation

In the network architecture that supports store and forward operation the satellite will receive application data from CN when it has ground connectivity. On disconnection of ground connectivity, the satellite (cell) operates in SF mode and the cell that  triggers paging message towards the UE for delivery of stored application data only. After delivery of all the stored data this cell will not generate any paging message. In such scenario it is beneficial if the UE is aware of the status that further paging for MT notification is not possible and UE can start relaxed monitoring of paging in such scenario.

Observation 4: Relaxing the paging monitoring when the satellite operation in SF mode without any stored downlink data is beneficial for UE power saving.

Whether paging monitoring can be relaxed or not in current cell operating SF mode can be indicated as part of system information as the status of pending paging transmission or as additional status for SF mode on pending MT status.

Proposal 2: Status of MME Initiated paging is included in system information for relaxed paging monitoring for cell operating in SF mode.

.

## 2.3.NAS Configured Satellite list for AS operation changes

As per TS23.401, for SF mode capable UE the MME can provide multiple assistance information for the UE to continue the pending NAS registration or resuming pending application data transmission in new SF mode capable cell. However, these parameters are provided as assistance information for the UE to determine on how to resume the pending activity in the new cell.

Observation 1:  NAS Configured Satellite list and Timer parameters provided from MME are considered as assistance information for the UE operation in next SF mode capable cell. Usage of this parameter is left to UE implementation in NAS specification.

Based on TS23.401 the UE may still camp onto cell outside the NAS configured list or outsider the time window for starting new application layer transaction or registration procedure. Hence using the  the NAS configured satellite list for idle mode operation will bring restriction for UE in such scenario.

Observation 2: New registrations or mobile originated data transmission is allowed in cells outside NAS configured satellite list.

Based on the above observations we propose that NAS configured satellite list is not considered for any idle mode operation behaviour changes. It can be left to UE implementation without RAN2 specification impact.

Proposal 3: RAN2 confirms that NAS configured satellite list is not considered for idle mode operation changes

## 2.4 Support for NB-IoT User Plane Option for SF Mode

In earlier RAN2 meetings RAN2 decided to wait for completion of SA2 work for further confirmation of NB-IoT-UP operation in SF mode. When UE is configured with NB-IoT UP option the security context is preserved at UE and eNB on release of RRC connection.  How the UE triggers the RRC-Resume procedure in new SF mode capable cell and the awareness of whether new cell have valid UE context plays role in UE determining the RRC-Resume operation or restart from RRC-Idle mode.

Proposal 4 : RAN2 to conclude on support for NB-IoT UP option for Store-Forward operation.

For the SF operation for UE operating in NB-IoT UP the security key context needed to resume in new satellite also needs be transferred along with pending data. In this case awareness of whether new satellite have valid UE context for resuming the services is beneficial. In this regard the network may need to provide information on the list of satellites where the RRC Resume is possible.

Proposal 4A: If NB-IoT UP operation is supported indication of list of satellites where the UE can resume from RRC-IDLE(suspended) state is indicated from network via dedicated signaling.

# 3	Conclusion

In this paper we discuss the remaining open issues for store-and-forward operation for IoT-NTN and make following observations and proposals

SF Mode Parameter in neighbor cell

Observation 1: The SF mode parameter broadcasted for serving cell is used by UE implementation to decide on its data transmission type and not for triggering cell search in current specification. It is up to the UE implementation to make use of this parameter to start early search of neighbour cells.

Observation 2: Awareness of SF mode operation of neighbour cell is beneficial for completing the pending MT transaction after cell reselection.

Observation 3: For Rel-19 UE not capable of SF mode usage of neighour cell SF mode parameter for cell reselection can be left to UE implementation.

Observation 4: The cell reselection priority is considered for cell reselection in current specification for scenarios related to continuity of specific service configured in current cell (i.e V2X, MBMS).

Proposal 1:  RAN2 to confirm the working assumption for introduction of SF mode for neighbour-cell without specification changes related to usage of this parameter for cell reselection.

Relaxed Paging Monitoring for SF Operation

Observation 4: Relaxing the paging monitoring when the satellite operation in SF mode without any stored downlink data is beneficial for UE power saving.

Proposal 2: Status of MME Initiated paging is included in system information for relaxed paging monitoring for cell operating in SF mode.

NAS Configured Satellite list for AS operation

Observation 5:  NAS Configured Satellite list and Timer parameters provided from MME are considered as assistance information for the UE operation in next SF mode capable cell. Usage of this parameter is left to UE implementation in NAS specification.

Observation 6: New registrations or mobile originated data transmission is allowed in cells outside NAS configured satellite list.

Proposal 3: RAN2 confirms that NAS configured satellite list is not considered for idle mode operation changes

NB-IoT UP Solution for SF Mode Operation

Proposal 4 : RAN2 to conclude on support for NB-IoT UP option for Store-Forward operation.

Proposal 4A: If NB-IoT UP operation is supported indication of list of satellites where the UE can resume from RRC-IDLE (suspended) state is indicated from network via dedicated signaling.

# Reference

[1]	RP-240776 WID for IoT-NTN Enhancements