3GPP TSG RAN WG2 Meeting #131	R2-2505878

Bangalore, India, Aug. 25th - 29th, 2025

Agenda Item:	8.9.2

Source:	ETRI, Korea University

Title:		Remaining issues on Store and Forward satellite operation

Document for:	Discussion and Decision

1	Introduction

In the previous RAN2 meeting #130, the following agreements and working assumption were made for Store and Forward satellite operation [1].

Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

3.	Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

In this paper, we would like to discuss remaining issues on Store and Forward satellite operation.

2	Discussion

2.1	More assistance information for the neighbour cells that can operate in S&F mode

In the previous meeting, RAN2 #129bis, RAN2 discussed an open issue of “(RRC-2) RAN2 to further discuss whether to introduce the following assistance information for the neighbour cells: operation mode and/or mode transition time”. There was a heated discussion about this, but no conclusions were reached, and only the working assumption shown above was made.

An indication whether a cell in the neighbour cell list operates in S&F mode or not

From our perspective, an indication whether a cell in the neighbour cell list operates in S&F mode or not is necessary in order to fully support and reap the benefits of S&F satellite operation.

Suppose that all neighbour cells are operating in S&F mode. As per the existing specifications, those cells cannot be broadcasted as neighbour cells. This is because they cannot serve the UEs not supporting S&F operation due to loss of the feeder link even if neighbor cell measurements for them are possible. However, the neighbour cells operating in S&F mode can serve the UEs supporting S&F operation. Consequently, the indication is necessary to enable the UEs supporting S&F operation to (re-)select a cell operating in S&F mode.

- An indication of whether a cell in the neighbour cell list operates in S&F mode or not is introduced.

Information of the neighbour cells operating in S&F mode needs to be delivered to only the UEs supporting S&F operation. That is, the UEs not supporting S&F operation do not need to be aware of neighbour cells operating in S&F mode. On the other hand, information of the neighbour cells operating in normal mode needs to be delivered to all UEs whether they support S&F operation or not. This is because, for the UEs supporting S&F operation, it is clear that (re-)selecting a neighbour cell operating in normal mode is highly likely to be a better choice than a neighbour cell operating in S&F mode if possible.

- Information of the neighbour cells operating in S&F mode is delivered to only the UEs supporting S&F operation, and information of the neighbour cells operating in normal mode is delivered to all UEs whether they support S&F operation or not.

If the indication whether a cell in the neighbour cell list operates in S&F mode or not is introduced, the UEs supporting S&F operation need to prioritize the neighbour cells operating in normal mode for (re-)selection. As we mentioned above, (re-)selection of a neighbour cell operating in normal mode rather than S&F mode is highly likely to be a better choice. Consequently, support of prioritizing the neighbour cells operating in normal mode for (re-)selection is required.

- UEs supporting S&F operation can prioritize the neighbour cells operating in normal mode for (re-)selection.

Mode transition time of a cell in the neighbour cell list

Regarding mode transition time of a cell in the neighbour cell list, we think the mode transition time is required, where the time can represent mode transition of either direction, i.e., S&F  normal or normal  S&F. Suppose that all neighbour cells are operating in normal mode but all or some of them are scheduled to change their operating mode from normal to S&F mode. In this case, how long a neighbour cell stays in normal mode could be very important in cell (re-)selection; the longer the better. Therefore, introduction of mode transition time in the assistance information for the neighbour cells that can operate in S&F mode is required. Therefore, introduction of mode transition time of a cell in the neighbour cell is required.

- Mode transition time of a cell in the neighbour cell list is introduced.

2.2	Measurement initiation before transition from normal to S&F mode

In the previous meeting, RAN2 agreed to indicate the time information of transition from normal to S&F mode by using t-ModeSwitching in conjunction with sf-OperationMode [1, 2]. For UEs not supporting S&F operation, the serving cell abruptly stops serving them at the time of mode transition from normal to S&F mode. Thus, we understand that t-Service is supposed to set to this time by NW to initiate UEs’ measurements for cell re-selection. However, t-Service is the time information on when the cell is going to stop. For UEs supporting S&F operation, if the serving cell supports S&F operation and performs S&F operation after stopping normal mode operation, the serving cell does not stop serving UEs supporting S&F operation. Even though the serving cell can keep serving the UEs supporting S&F operation after mode transition from normal to S&F mode, it can be beneficial for the UEs supporting S&F operation to try to find a neighbour cell operating in normal mode and re-select the neighbour cell. This is because re-selecting a neighbour cell operating in normal mode is highly likely to be a better choice than staying in the current serving cell operating in S&F mode. Therefore, even for UEs supporting S&F operation, measurement initiation before transition from normal to S&F mode is required.

- Measurement initiation before transition from normal to S&F mode applies even to UEs supporting S&F operation.

If RAN2 does not revise the description of t-Service in the current spec., another trigger other than t-Service is required for UEs supporting S&F operation to initiate measurements for re-selection. According to the current running RRC CR [2], the time of transition from normal to S&F mode is represented by t-ModeSwitching without presence of sf-OperationMode. Both t-ModeSwitching and sf-OperationMode are only for Rel-19 UEs supporting S&F operation. Thus, introduction of measurement initiation before transition from normal to S&F mode has no impact on either legacy UEs before Rel-19 and Rel-19 UEs not supporting S&F operation.

- UEs supporting S&F operation initiate intra-frequency, inter-frequency or inter-RAT measurements, before the time represented t-ModeSwitching without presence of sf-OperationMode.

2.3	Utilization of NAS information: S&F Monitoring List and S&F Wait Timer

Utilization of either S&F Monitoring List or S&F Wait Timer for cell (re-)selection enhancements

In the previous meeting, RAN2 discussed a proposal of utilizing either S&F Monitoring List or S&F Wait Timer for cell (re-)selection enhancements as follows:

- Cell (re)selection enhancements Observation 5: If a UE has received a “S&F Wait Timer” and/or a “S&F Monitoring List” from a MME operating in S&F mode, the UE is not expected to perform a NAS procedure: • Condition A: In any satellite operating in S&F mode of the same PLMN, as long as the S&F Wait Timer is running. • Condition B: In a satellite operating in S&F mode of the same PLMN which is not included in the S&F Monitoring List. Proposal 1: Enhance the definition of suitable cell in 36.304 so that a UE may consider a detected cell as unsuitable and not treating it as a candidate for reselection, if the detected cell is handled by a satellite operating in S&F mode for which conditions (A) and/or (B) are met. - Nokia thinks we already agreed not to have impacts to AS. Ericsson agrees - Apple thinks this is too much. QC/IDC agree - Google supports this for condition B (while condition A is too complicated). CATT agrees. HW also thinks condition B should be considered. Xiaomi sees some benefit to consider condition B but thinks this implies that the operation mode of neighbour cell should be provided - Samsung thinks that changing the definition would also imply that we bar the cell. IDC agrees - Can come back to this

In short, we agree with the comments of the companies against the discussed proposal. The discussed proposal tries to make UEs unable to re-reselect a satellite that is not included in the S&F Monitoring List. In RAN2 #129bis in Apr. 2025, RAN2 received reply LS [3] from SA2 with respect to the S&F Monitoring List as follows:

■ RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list. [SA2 response]: SA2 has discussed the scenario and clarified this in the TS 23.401 clause 4.13.9.

We understand that the SA2 response corresponds to the following note in TS 23.401 [4, Sec. 4.13.9]:

NOTE 7: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one

That is, (re-)selecting a cell provided by a satellite that is not included in S&F Monitoring List is NOT prevented if UEs can accept a possibility of failing to complete the ongoing NAS procedure. Therefore, utilization of either S&F Monitoring List or S&F Wait Timer for cell (re-)selection enhancements is not needed and it seems appropriate to leave it as an implementation issue.

- Do not pursue cell (re-)selection enhancement by utilizing NAS information such as S&F Monitoring List or S&F Wait Timer.

Clarification of the discontinuous coverage procedure considering S&F Monitoring List

In the previous meeting, regarding the discontinuous coverage procedure, it was left to FFS how S&F Monitoring List can be considered in the discontinuous coverage procedure. We think S&F Monitoring List needs not to be considered in the discontinuous coverage procedure.

S&F Monitoring List can be regarded as a concept that differentiates satellites based on whether a satellite has the context for ongoing NAS signalling procedures. On the other hand, in the discontinuous coverage procedure, satellites are differentiated based on the coverage of a satellite, where the coverage varies in time and shape depending on a satellite. We cannot find any reason to differentiate satellites in the discontinuous coverage procedure. In addition, independent design of NAS and AS signalling is preferred to reduce complexity.

- Do not consider S&F Monitoring List in the discontinuous coverage procedure.

2.4	Consideration of S&F operation in the discontinuous coverage procedure

Coverage of a satellite operating in S&F mode can be different for UEs whether they support S&F operation or not. UEs supporting S&F operation need to determine that it is in-coverage if it is within the footprint of the satellite providing the serving cell operates in S&F mode. Note that even though a lot of other assistance information such as ephemeris of the satellite is required for a UE to determine whether it is in-coverage or out-of-coverage, we focus on the operation mode of a satellite, i.e., normal or S&F mode.

The current discontinuous coverage does not consider satellites operating in S&F mode. If a UE supporting S&F operation is within the footprint of a satellite operating in S&F mode, it cannot determine that it is in-coverage. In this case, even though the UE can be served by the satellite with the help of S&F operation and thus it is in-coverage, it has no choice but to determine that it is out-of-coverage. This causes the UE not to perform any idle mode tasks related to NTN, which means that the UE cannot (re-)select a cell served by the satellite operating in S&F mode because S&F operation is not considered in the discontinuous coverage procedure. To overcome this problem, the operation mode of a satellite needs to be considered in the discontinuous coverage procedure.

- A UE considers the operation mode of a satellite providing the serving cell to determine that it is in the coverage of the satellite.

3	Conclusion

In this paper, we propose the followings:

Proposal 1	An indication of whether a cell in the neighbour cell list operates in S&F mode or not is introduced.

Proposal 2	Information of the neighbour cells operating in S&F mode is delivered to only the UEs supporting S&F operation, and information of the neighbour cells operating in normal mode is delivered to all UEs whether they support S&F operation or not.

Proposal 3	UEs supporting S&F operation can prioritize the neighbour cells operating in normal mode for (re-)selection.

Proposal 4	Mode transition time of a cell in the neighbour cell list is introduced.

Proposal 5	Measurement initiation before transition from normal to S&F mode applies even to UEs supporting S&F operation.

Proposal 6	UEs supporting S&F operation initiate intra-frequency, inter-frequency or inter-RAT measurements, before the time represented t-ModeSwitching without presence of sf-OperationMode.

Proposal 7	Do not pursue cell (re-)selection enhancement by utilizing NAS information such as S&F Monitoring List or S&F Wait Timer.

Proposal 8	Do not consider S&F Monitoring List in the discontinuous coverage procedure.

Proposal 9	A UE considers the operation mode of a satellite providing the serving cell to determine that it is in the coverage of the satellite.

References

R2-2504673, Report from Break-out session on NR-NTN and IoT-NTN, Session chair (ZTE), RAN2 #130, May 2025.

[Post130][306][R19 IoT NTN] RRC CR, Huawei.

R2-2501753, Reply LS on Satellite IDs for store-and-forward operation, SA2, RAN2 #129bis, Apr. 2025.

TS 23.401, General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access, v19.3.0, Jun. 2025.